| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 5th Meeting: Geneva, CH, 16-23 March, 2011 | Document: JCTVC-E083 WG11 Number: m19597 |
| --- | --- |

| Title: | Evaluations and suggestions for TU representation |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Shan Liu Zhi Zhou Shawmin Lei 2860 Junction Ave. San Jose, CA95134, USA | Emails: | shan.liu@mediatek.com zhi.zhou@mediatek.com shawmin.lei@mediatek.com |
| Source: | MediaTek USA |  |  |

_____________________________

# Abstract

This contribution first evaluates the TU representation for current HEVC. Results reportedly show that about 80% of coding gain can be obtained by setting the maximum depths of RQT to 2 compared with 3 as in the current anchor condition (HE) and single size transform. Secondly, this contribution proposes a method for reducing overall encoder complexity by reducing the number of transforms for the MERGE_2Nx2N mode during the RDO process. This method results in 6-7% overall encoding time reduction with averagely 0.3% BD-rate increase compared with anchor (HE). Furthermore, this contribution proposes two alternative schemes for TU representation. Method one reports about 10% encoding time reduction in average, with averagely 0.4% BD-rate increase. Method two reports averagely 8% encoding time reduction, with averagely 0.2% BD-rate increase, in HE conditions.

- Introduction

In the current HM, a CU can be split into multiple PUs, i.e. 2Nx2N, 2NxN, Nx2N and NxN for INTER; 2Nx2N and NxN for INTRA. For transform, a hierarchy RQT (Residual Quad Tree) is used and the TU size is constrained by CU size, max RQT depth, max TU size and min TU size. TU size is independent from PU size. The max RQT depth for Inter and Intra, max TU size and min TU size are sent in the SPS header.

The current max TU size is 32x32 and min TU size is 4x4. The default RQT depth in HM 2.0 is:

|  | HE | LC |
| --- | --- | --- |
| Inter | 3 | 2 |
| Intra | 3 | 1 |

# Evaluation about RQT depth

In order to evaluate the influence to coding efficiency by using various maximum RQT depths, simulations were conducted with maximum RQT depth set to 1, 2 and 3, for High Efficiency cases. Results are shown in Table 1 and Table 2, respectively. Anchor used HM2.0 software following common test conditions defined in JCTVC-D600 [1].

Table 1 Results with maximum RQT depth set to Inter = 2, Intra = 2 (HE)

|  | Intra |  |  |
| --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate |
| Class A | 0.1 | 0.1 | 0.1 |
| Class B | 0.2 | 0.2 | 0.2 |
| Class C | 0.1 | 0.1 | 0.1 |
| Class D | 0.1 | 0.1 | 0.1 |
| Class E | 0.2 | 0.1 | 0.1 |
| All | 0.1 | 0.1 | 0.1 |
| Enc Time[%] | 95% |  |  |
| Dec Time[%] | 100% |  |  |
|  |  | Random access |  |
|  | Y BD-rate | U BD-rate | V BD-rate |
| Class A | 0.3 | 0.2 | -0.2 |
| Class B | 0.4 | 0.4 | 0.4 |
| Class C | 0.3 | 0.2 | 0.2 |
| Class D | 0.3 | -0.1 | 0.1 |
| Class E |  |  |  |
| All | 0.3 | 0.2 | 0.1 |
| Enc Time[%] | 90% |  |  |
| Dec Time[%] | 100% |  |  |
|  | Low delay |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate |
| Class A |  |  |  |
| Class B | 0.4 | 1.0 | 0.9 |
| Class C | 0.3 | 0.8 | 0.7 |
| Class D | 0.3 | 0.9 | 0.3 |
| Class E | 0.1 | -0.9 | 0.9 |
| All | 0.3 | 0.6 | 0.7 |
| Enc Time[%] | 92% |  |  |
| Dec Time[%] | 100% |  |  |

Table 2 Results with maximum RQT depth set to Inter = 1, Intra = 1

|  | Intra |  |  | Intra LoCo |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | 0.2 | 0.2 | 0.1 | #NAME? | #NAME? | #NAME? |
| Class B | 0.5 | 0.4 | 0.4 | #NAME? | #NAME? | #NAME? |
| Class C | 0.5 | 0.4 | 0.4 | #NAME? | #NAME? | #NAME? |
| Class D | 0.4 | 0.3 | 0.3 | #NAME? | #NAME? | #NAME? |
| Class E | 0.5 | 0.2 | 0.4 | #NAME? | #NAME? | #NAME? |
| All | 0.4 | 0.3 | 0.3 | #NAME? | #NAME? | #NAME? |
| Enc Time[%] | 91% |  |  | #NUM! |  |  |
| Dec Time[%] | 100% |  |  | #NUM! |  |  |
|  | Random access |  |  | Random access LoCo |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | 1.1 | 0.5 | -0.1 | 1.2 | 0.7 | 0.7 |
| Class B | 1.5 | 0.6 | 0.4 | 1.5 | -0.7 | -0.9 |
| Class C | 1.8 | 0.4 | 0.4 | 2.0 | -0.7 | -0.8 |
| Class D | 2.7 | 0.6 | 0.7 | 3.2 | -1.1 | -0.9 |
| Class E |  |  |  |  |  |  |
| All | 1.7 | 0.5 | 0.4 | 1.9 | -0.5 | -0.5 |
| Enc Time[%] | 80% |  |  | 95% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |
|  | Low delay |  |  | Low delay LoCo |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A |  |  |  |  |  |  |
| Class B | 1.8 | 1.4 | 1.2 | 2.3 | -1.5 | -1.4 |
| Class C | 2.6 | 0.9 | 1.0 | 3.3 | -1.0 | -1.6 |
| Class D | 3.7 | 1.7 | 0.9 | 4.9 | -1.7 | -1.7 |
| Class E | 0.8 | -1.1 | 1.3 | 1.4 | -2.3 | -1.6 |
| All | 2.3 | 0.9 | 1.1 | 3.0 | -1.6 | -1.6 |
| Enc Time[%] | 83% |  |  | 96% |  |  |
| Dec Time[%] | 100% |  |  | 101% |  |  |

Table 1 and Table 2 show that by setting maximum RQT depth to 2 (HE), most of coding gain can be obtained, compared with anchor (maximum RQT depth = 3) and single size transform (i.e. setting maximum RQT depth to 1.) At the same time, the encoding time can be reduced up to 10%.

# Proposed Methods

## Encoder complexity reduction for 2Nx2N merge mode decision

In the HM2.0 software, a fast RD-cost estimation is used for selecting the best merge candidate from up to five candidates, when the partition type is Nx2N or 2NxN and the CU is considered for partial merge. That is, in the RDO loop, for each Nx2N or 2NxN PU, only one set of transforms (in multiple transform depths that are allowed) is calculated. In contrast, when evaluating the full CU merge (i.e. Merge2Nx2N) mode, the set of transform is calculated for all (up to) five merge candidates. In this contribution we propose to apply the fast RD-cost estimation method that has been adopted in HM2.0 to full CU merge as well. Results are shown in Table 6.

## Alternative TU representation

Inspired by the observations from section 2, two alternative TU representations are proposed in this contribution.

- Scheme 1: Constrain TU size to be no larger than PU, and no smaller than a quarter of CU size (i.e. half in both vertical and horizontal directions). That is, the maximum TU width/height (assuming TU is a square block) equals to minimum (PU_width, PU_height), the minimum TU width/height equals to half of CU width/height (assuming CU is a square block.) As a result, a 2Nx2N CU with partition types Nx2N, 2NxN and NxN are only allowed NxN TU; therefore no “split_transform_flag” for these partition types are needed. If the partition type is 2Nx2N, it is allowed to use either 2Nx2N or NxN TU, signaled by a “split_ transform _flag”.

- Scheme 2: Allow the maximum TU width/height equals to maximum (PU_width, PU_height), and the minimum TU width/height equals to half of CU width/height. As a result, a 2Nx2N CU with partition type NxN is only allowed NxN TU; therefore no “split_transform_flag” is needed. Otherwise, it is allowed to use either 2Nx2N or NxN TU, signaled by a “split_ transform _flag”.

Both proposed schemes can significantly simplify the existing syntax for TU representation. The modified syntaxes are shown in Table 4 and Table 5, respectively.

Table 4 Syntax for TU representation as in proposed scheme 2.

| transform_unit( x0, y0, log2TrafoSize, trafoDepth, blkIdx ) { | Descriptor |
| --- | --- |
| ... |  |
| if ( residualDataPresentFlag) { |  |
| split_transform_flag = 1 |  |
| if(trafoDepth==0 && PartMode==PART_2Nx2N ) { |  |
| split_transform_flag | u(1) ¦ ae(v) |
| ... |  |
| } |  |
| ... |  |
| } |  |
| } |  |

Table 5 Syntax for TU representation as in proposed scheme 2.

| transform_unit( x0, y0, log2TrafoSize, trafoDepth, blkIdx ) { | Descriptor |
| --- | --- |
| ... |  |
| if ( residualDataPresentFlag) { |  |
| split_transform_flag = 1 |  |
| if(trafoDepth==0 && PartMode!=PART_NxN ) { |  |
| split_transform_flag | u(1) ¦ ae(v) |
| ... |  |
| } |  |
| ... |  |
| } |  |
| } |  |

# Experimental results

Experimental results for the proposed encoder complexity reduction scheme in Section 3.1 are shown in Table 6. The test conditions are as defined in JCTVC-D600 Error: Reference source not found.

Table 6 Results for proposed encoder complexity reduction scheme (3.1), compared with HM2.0 anchor.

|  |  | Random access |  |
| --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate |
| Class A | 0.2 | 0.2 | 0.2 |
| Class B | 0.3 | 0.3 | 0.2 |
| Class C | 0.2 | 0.2 | 0.3 |
| Class D | 0.1 | 0.0 | 0.1 |
| Class E |  |  |  |
| All | 0.2 | 0.2 | 0.2 |
| Enc Time[%] | 93% |  |  |
| Dec Time[%] | 100% |  |  |
|  | Low delay |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate |
| Class A |  |  |  |
| Class B | 0.7 | 0.1 | -0.1 |
| Class C | 0.4 | 0.3 | 0.3 |
| Class D | 0.3 | 0.4 | -0.4 |
| Class E | 0.3 | -0.4 | 0.6 |
| All | 0.4 | 0.1 | 0.1 |
| Enc Time[%] | 94% |  |  |
| Dec Time[%] | 100% |  |  |

Experimental results for the two proposed TU size schemes in Section 3.2 are shown in Table 7 and 8, respectively.

Table 7 Results for proposed scheme 1(3.2-1) compared with HM2.0 anchor.

|  | Intra |  |  | Intra LoCo |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | 0.1 | 0.1 | 0.1 |  |  |  |
| Class B | 0.2 | 0.2 | 0.2 |  |  |  |
| Class C | 0.1 | 0.1 | 0.1 |  |  |  |
| Class D | 0.1 | 0.1 | 0.1 |  |  |  |
| Class E | 0.2 | 0.1 | 0.1 |  |  |  |
| All | 0.1 | 0.1 | 0.1 |  |  |  |
| Enc Time[%] | 95% |  |  |  |  |  |
| Dec Time[%] | 100% |  |  |  |  |  |

|  | Random access |  |  | Random access LoCo |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | 0.3 | 0.3 | 0.1 | 0.0 | -0.1 | 0.0 |
| Class B | 0.4 | 0.3 | 0.3 | 0.1 | 0.3 | 0.3 |
| Class C | 0.4 | 0.3 | 0.3 | 0.1 | 0.2 | 0.4 |
| Class D | 0.4 | 0.1 | 0.2 | 0.1 | 0.1 | 0.2 |
| Class E |  |  |  |  |  |  |
| All | 0.4 | 0.2 | 0.2 | 0.1 | 0.1 | 0.2 |
| Enc Time[%] | 89% |  |  | 99% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |

|  | Low delay |  |  | Low delay LoCo |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A |  |  |  |  |  |  |
| Class B | 0.5 | 1.2 | 1.4 | 0.2 | 0.5 | 0.7 |
| Class C | 0.5 | 0.9 | 0.8 | 0.2 | 0.6 | 0.5 |
| Class D | 0.4 | 1.0 | 0.7 | 0.2 | 0.8 | 0.7 |
| Class E | 0.4 | 0.6 | 1.1 | 0.3 | 1.6 | 1.0 |
| All | 0.5 | 0.9 | 1.0 | 0.2 | 0.8 | 0.7 |
| Enc Time[%] | 91% |  |  | 100% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |

Table 8 Results for proposed scheme 2 (3.2-2) compared with HM2.0 anchor.

|  | Intra |  |  | Intra LoCo |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | 0.1 | 0.1 | 0.1 |  |  |  |
| Class B | 0.2 | 0.2 | 0.2 |  |  |  |
| Class C | 0.1 | 0.1 | 0.1 |  |  |  |
| Class D | 0.1 | 0.1 | 0.1 |  |  |  |
| Class E | 0.2 | 0.1 | 0.1 |  |  |  |
| All | 0.1 | 0.1 | 0.1 |  |  |  |
| Enc Time[%] | 95% |  |  |  |  |  |
| Dec Time[%] | 100% |  |  |  |  |  |

|  | Random access |  |  | Random access LoCo |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | 0.2 | 0.2 | -0.2 | 0.1 | -0.1 | 0.0 |
| Class B | 0.3 | 0.3 | 0.2 | 0.0 | 0.1 | 0.1 |
| Class C | 0.3 | 0.2 | 0.2 | 0.0 | 0.0 | 0.0 |
| Class D | 0.3 | -0.1 | 0.2 | 0.0 | -0.2 | 0.0 |
| Class E |  |  |  |  |  |  |
| All | 0.3 | 0.2 | 0.1 | 0.0 | 0.0 | 0.0 |
| Enc Time[%] | 92% |  |  | 101% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |

|  | Low delay |  |  | Low delay LoCo |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A |  |  |  |  |  |  |
| Class B | 0.3 | 0.8 | 0.9 | 0.0 | 0.1 | 0.2 |
| Class C | 0.3 | 0.6 | 0.5 | 0.0 | 0.1 | -0.1 |
| Class D | 0.2 | 0.8 | -0.1 | 0.0 | 0.1 | 0.1 |
| Class E | -0.1 | -0.4 | -0.2 | 0.0 | 0.0 | 0.1 |
| All | 0.2 | 0.5 | 0.3 | 0.0 | 0.1 | 0.1 |
| Enc Time[%] | 93% |  |  | 101% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |

# Conclusions

This contribution first evaluated the RQT depth in the current HEVC. Results show that by setting the maximum depth to two (i.e. two level transform) most of the coding gains can be obtained, compared with multiple level transform and single size transform with reduced encoder complexity. Secondly, this contribution proposes a method for reducing the overall encoder complexity by reducing the number of transforms for the MERGE_2Nx2N mode during the RDO process. Furthermore, this contribution proposed two alternative schemes for TU representation in HEVC. Both schemes simplified the TU syntax and reduced encoding complexity, with small BD-rate increase. We recommend the committee to include the proposed encoder complexity reduction approach in the future HM. We also recommend to the committee to consider using the proposed TU representation schemes. If not, consider setting the maximum RQT depth to be two, for both Inter and Intra, High Efficiency configurations.

# Patent rights declaration(s)

MediaTek Inc. may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

- Frank Bossen, “Common test conditions and software reference configurations”, JCTVC-D600, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T VCEG and ISO/IEC MPEG, Daegu, Korea, January 2011.

- Gisle Bjontegaard, “Improvements of the BD-PSNR model”, ITU-T Q.6/SG16 VCEG, VCEG-AI11, Berlin, Germany, July 2008.