| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 5th Meeting: Geneva, CH, 16-23 March, 2011 | Document: JCTVC-E084 WG11 Number: m19598 |
| --- | --- |

| Title: | CE1: Report of self derivation of motion estimation techniques at video decoder side on HM2.0 |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yi-Jen Chiu 2200 Mission College Blvd. Santa Clara CA USA 95054 | Tel: Email: | 408-765-9494 yi-jen.chiu@intel.com |
|  | Lidong Xu 8F Raycom Infotech Park A, No. 2 KeXueYuan South Rd., Haidian District, Beijing, China, 100190 | Tel: Email: | 86-10-82171425 lidong.xu@intel.com |
|  | Wenhao Zhang 8F Raycom Infotech Park A, No. 2 KeXueYuan South Rd., Haidian District, Beijing, China, 100190 | Tel: Email: | 86-10-82171657 wenhao.zhang@intel.com |
|  | Hong Jiang 2200 Mission College Blvd. Santa Clara CA USA 95054 | Tel: Email: | 916-356-0457 hong.h.jiang@intel.com |
| Source: | Intel Corp. |  |  |

_____________________________

# Abstract

This contribution provides test result for CE1 topic of Self Derivation of Motion Estimation (SDME) techniques on HM 2.0 reference software. The skip-mode only rounded-candidate based SDME are tested for bi-prediction mode on HM2.0 reference software. To quantize the impact of the memory access bandwidth in checking multiple candidate motion vectors, the tests of SDME techniques only consider the candidate motion vectors within a predefined range to confine the required pixels used for the process of sum-of-absolute-difference (SAD) and of motion compensation filtering. Two test points of Skip-mode only rounded-candidate based SDME with 8-point MV refinement has reported that an overall 1.3% BD bitrate reduction (up to 2.4%) for the test of random access (high efficiency) category under the common test conditions with an overall 3% increase in encoding time and an overall 11% increase in decoding time increase. Two test points of Skip-mode rounded-candidate based SDME without MV refinement has reported an overall 0.7% BD bitrate reduction (up to 1.4%) for the test of random access (high efficiency) category with an overall 1% increase in encoding time and an overall 4% increase in decoding time.

# Introduction

Rounded-candidate based SDME was presented in [1] to improve the coding efficiency of bi-prediction mode of B pictures by deriving motion vectors (MV) at video decoder side. The candidate MVs for B-picture is a pair of motion vectors. The candidate MVs can be obtained from the existing (bi-predictional) MVs of temporal/spatial neighboring PUs, or can be derived from the mirroring MVs projected from the existing uni-directional MVs of temporal/spatial neighboring PUs. A study of 6 and 9 candidate MV set is reported in [1]. The candidate MVs are then rounded to integer pixel positions to reduce the amount of operation in sum of absolute difference (SAD) calculation for the process of deriving the final motion vector predictor. The final motion vector predictor is obtained with the candidate MVs to achieve minimum SAD. The location of candidate MVs can be unbounded and it will increase the amount of memory bandwidth access in the process of SAD calculation. Therefore, the core experiment to study the impact of coding efficiency versus the amount of memory access bandwidth for SDME is defined in CE1 [3].

In this report, the tests for two type of Skip-mode only candidate-based SDME techniques are reported for bi-prediction mode: 8-point-MV-refinement and no-MV-refinement. Compared to C-DMVD techniques tested in [2], the coding mode of Skip-mode is in use for SDME in this report. Same process as [1], when the SDME refinement flag in SPS is set to 0, the test case of no-MV-refinement is invoked and the best candidate will be used directly for motion compensation (MC). On the other hand (when SDME refinement flag in SPS is set to 1), a two-step 8-point MV refinement process within 3/4 pixel search range is applied after the best candidate is selected. The techniques of simplified bi-linear interpolation filter and sampled SAD are applied in the process of MV refinement as described in [1].

To quantize the impact of memory access bandwidth, a process of confining the candidate MVs is utilized with the range of pixels required by candidate MVs is limited within a pre-defined window. That is, the pixel used for the processes of SAD calculation and of MC filtering operation is confined to two windows loaded only once from the reference pictures in reference list 0 and reference list 1 respectively. The remaining SDME operation is identical as described in [1]. The following of this contribution has Section 2 to provide an introduction of the proposed candidate MV confinement, and Section 3 to report the performance data of the coding efficiency and coding complexity of tests. Section 4 concludes this contribution.

# Candidate MV confinement

In the process of MC filtering for a 2Nx2N PU, the integer pixel windows of size of (2N+8)x(2N+8) are loaded and the extra 8 pixels in horizontal/vertical direction are required for fractional pixel interpolation. In this report, the extended integer pixel windows of size of (2N+8+2M)x(2N+8+2M) are loaded once for each PU coded with SDME mode with the pixels required for both SAD calculation and MC filtering. In each horizontal/vertical direction, the extra 8 pixels are required for fractional pixel interpolation and the extra 2M pixels are used to confine the memory access bandwidth for candidate MVs in SDME process for SAD calculation. The confinement of candidate MVs works as followed. Consider the window centers (centerL0.x, centerL0.y) and (centerL1.x, centerL1.y) in the reference picture of LIST_0 and LIST_1, the candidate MVs, (MvL0.x, MvL0.y) and (MvL1.x, MvL1.y), are marked available (confined successfully) if its MVs satisfy the below equation [1].

−a≤MvL0.x−centerL0.x≤b−a≤MvL0.y−centerL0.y≤b−a≤MvL1.x−centerL1.x≤b                          [1]

Where a= M and b = M+0.75 for the case of no-MV-refinement, and a= M-0.75 and b=M for the case of 8-point-MV-refinement.

The two window centers are selected based on the ranking of the candidate MV as the potential integer window center to contain the maximum number of candidate MVs to satisfy the equation [1]. The SDME techniques consider only the candidate MVs marked as available and therefore the memory access bandwidth is constrained.

# Performance

The encoder is complied in Linux OS and is tested in a compute farm consisting of hundred of CPU cores. The decoder was tested on a single workstation with the following configurations: Windows Server 2003 Enterprise x64 Edition, Intel Xeon CPU E5335 @ 2.00GHz, 8GB RAM, and Visual Studio 2008. The 9 candidates introduced in [2] are used, and the SDME window size parameter M is set to 8.

## Skip-mode only SDME without MV refinement

The Skip-mode only SDME techniques are implemented onto HM2.0 Reference Software and are evaluated under the test conditions defined by the CE group [3]. The data is provided for the test case of random access given the current SDME techniques are applied only on B picture.

Table 1 shows the performance of Skip-mode only 9-candidate set based SDME compared to the performance of HM2.0 anchors. In high efficiency case, the average BD bitrate reduction for Luma component is about 0.7% (up to 1.4% for clip BlowingBubbles) with about 1% increase in encoding time and about 4% increase in decoding time. In low complexity case, the average BD bitrate reduction for Luma component is about 0.5% with about 1% increase in encoding time and about 3% increase in decoding time.

Table 1 - The performance of Skip-mode only 9-candidate set based SDME without MV refinement

|  | Random access |  |  | Random access LoCo |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | -0.5 | -0.5 | -0.8 | -0.4 | -0.4 | -0.4 |
| Class B | -0.5 | -0.4 | -0.5 | -0.4 | -0.3 | -0.4 |
| Class C | -0.6 | -0.6 | -0.6 | -0.4 | -0.3 | -0.3 |
| Class D | -1.1 | -1.0 | -1.1 | -0.7 | -0.7 | -0.7 |
| Class E |  |  |  |  |  |  |
| All | -0.7 | -0.6 | -0.7 | -0.5 | -0.4 | -0.5 |
| Enc Time[%] | 101% |  |  | 101% |  |  |
| Dec Time[%] | 104% |  |  | 103% |  |  |

## Skip-mode only SDME with MV refinement

Table 2 shows the performance of Skip-mode only 9-candidate set based SDME with 8-point-MV-refinement compared with the CE anchors. In high efficiency case, the average BD bitrate reduction for Luma component is about 1.3% (up to 2.4% for clip PeopleOnStreet) with about 3% increase in encoding time and about 11% increase in decoding time. In low complexity case, the average BD bitrate reduction for Luma component is about 1.1% with about 3% increase in encoding time and about 15% increase in decoding time.

Table 2 - The performance of Skip-mode only 9-candidate set based SDME with 8-point MV refinement

|  | Random access |  |  | Random access LoCo |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | -1.0 | -1.1 | -1.4 | -0.8 | -0.9 | -0.8 |
| Class B | -1.1 | -0.9 | -0.9 | -1.1 | -0.8 | -0.7 |
| Class C | -1.1 | -1.0 | -1.1 | -0.8 | -0.7 | -0.7 |
| Class D | -1.9 | -1.7 | -1.7 | -1.5 | -1.4 | -1.5 |
| Class E |  |  |  |  |  |  |
| All | -1.3 | -1.2 | -1.3 | -1.1 | -0.9 | -0.9 |
| Enc Time[%] | 103% |  |  | 103% |  |  |
| Dec Time[%] | 111% |  |  | 115% |  |  |

# Conclusions

This contribution provides test result for CE1 topic of Self Derivation of Motion Estimation (SDME) techniques on HM 2.0 reference software. The skip-mode only rounded-candidate based SDME are tested for bi-prediction mode on HM2.0 reference software. To quantize the impact of the memory access bandwidth in checking multiple candidate motion vectors, the tests of SDME techniques only consider the candidate motion vectors within a predefined range to confine the required pixels used for the process of sum-of-absolute-difference (SAD) and of motion compensation filtering. Two test points of Skip-mode only rounded-candidate based SDME with 8-point MV refinement has reported that an overall 1.3% BD bitrate reduction (up to2.4% for clip PeopleOnStreet) for the test of random access (high efficiency) category under the common test conditions with an overall 3% increase in encoding time and an overall 11% increase in decoding time increase. Two test points of Skip-mode rounded-candidate based SDME without MV refinement has reported an overall 0.7% BD bitrate reduction (up to 1.4% for clip BlowingBubbles) for the test of random access (high efficiency) category with an overall 1% increase in encoding time and an overall 4% increase in decoding time.

# References

- Y. Chiu, L. Xu, W. Zhang, H. Jiang, “CE1: Report of self derivation of motion estimation in TMuC 0.9”, Meeting Document JCTVC-D167, January 2011.

- Y. Chiu, L. Xu, W. Zhang, H. Jiang, M. Yang, S. Lin, H. Yu, “CE1 Subtest1: A joint proposal of candidate-based decoder-side motion vector derivation”, Meeting Document JCTVC-D448, January 2011.

- Yi-Jen Chiu, Haoping Yu, Yu-Wen Huang, and Shun-ichi Sekiguchi, “Description of Core Experiment 1: Decoder-Side Motion Vector Derivation”, Meeting Document JCTVC-D601, January 2010.

# Patent rights declaration(s)

(NOTE – Activities in the JCT-VC and contributions to the JCT-VC are subject to the common patent policy for ITU-T/ITU-R/ISO/IEC. A statement of that policy can be found at
http://www.itu.int/ITU-T/dbase/patent/patent-policy.html, with further information available at http://www.itu.int/ITU-T/ipr/index.html and in the ISO/IEC Directives. The form to be used for the formal reporting of patent rights to ITU-T/ITU-R/ISO/IEC can be found at http://www.itu.int/ITU-T/ipr/index.html. Contributions to the JCT-VC proposing normative technical content shall contain a non-binding informal notice of whether the submitter may have patent rights that would be necessary for implementation of the resulting standard. The provided informal notice shall indicate the category of anticipated licensing terms according to the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form. This obligation to provide an informal notice is supplemental to, and does not replace, any existing obligations of parties with technology included in a final or draft standard to submit formal IPR declarations to ITU-T/ITU-R/ISO/IEC. An example of an informal IPR notification statement for a contribution is provided below.)

Intel Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).