| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 5th Meeting: Geneva, CH, 16-23 March, 2011 | Document: JCTVC-E085 WG11 Number: m19599 |
| --- | --- |

| Title: | Method and syntax for partial CU merge |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Shan Liu Ximin Zhang Shawmin Lei 2860 Junction Ave. San Jose, CA95134, USA | Emails: | shan.liu@mediatek.com ximin.zhang@mediatek.com shawmin.lei@mediatek.com |
| Source: | MediaTek USA |  |  |

_____________________________

# Abstract

This contribution proposes a method and syntax for partial CU merge, in specific related to the 2nd PU of an Nx2N or 2NxN CU that is greater than SCU. Experimental results report average encoding time reduction 13% compared with HM2.0 High Efficiency anchors; 18% compared with HM2.0 Low Complexity anchors, with minor average BD-rate increase, less than 0.1% for HE and 0.2-0.3% for LC. Decoding time remains the same.

- Introduction

The current HEVC (High Efficiency Video Coding) defines coding unit (CU) and prediction unit (PU), where a CU may consists of one or multiple PUs. The four partition types and PU sizes are shown in Figure 1. Among them, an INTRA coded 2Nx2N CU may select 2Nx2N or NxN partition type if it is a smallest CU (SCU); otherwise, it can only select 2Nx2N partition type. For an INTER coded 2Nx2N SCU, all four partition types may be selected if is an SCU; otherwise, excluding NxN.The partition types that are allowed for a CU are listed in Table 1.

Figure 1 Various partition types for a 2Nx2N CU.

Table 1	 Partition types in HEVC.

| Partition Types | INTER CU > SCU | INTER CU = SCU | INTRA CU > SCU | INTRA CU = SCU |
| --- | --- | --- | --- | --- |
| 2Nx2N | Yes | Yes | Yes | Yes |
| Nx2N | Yes | Yes | No | No |
| 2NxN | Yes | Yes | No | No |
| NxN | No | Yes | No | Yes |

In current HM (version 2.0) each CU can be merged as a whole (i.e. 2Nx2N merge) or partially merged. If partition types Nx2N or 2NxN is selected for an INTER predicted CU, the first partition (i.e. PU) of this CU is forced to merge. That is, the first PU of an Nx2N or 2NxN CU must not have its own motion vectors; instead, it has to share one of its neighbor motion vectors. The merging candidates for the first PU of a partially merged CU are shown in Figure 2.

Figure 2 Candidates for the first PU in a partial merge CU in HEVC.

In the meantime, the second PU of this Nx2N or 2NxN CU can be either merged or self-coded (i.e. having its own motion vectors, etc.) Hence, for the first PU of a selected Nx2N or 2NxN CU, there is no syntax bit needed for signaling merge; while for the second PU, 1 bit “merge_flag” is required to indicate whether it is merged or self-coded. Note that this partial merge only applies to CUs that are greater than SCU (Smallest CU) in size in current HEVC.

# Proposed Methods

In this contribution we first propose to force merge both PUs in a CU that consists of two PUs. Hence, no syntax bits (i.e. merge_flag) are needed to signal partial merge for neither PU. Second, we propose an alternative set of merge candidates for the 2nd PU. Figure 3 and 4 show the merging candidates for the 2nd PU of a CU in the current HEVC HM2.0. The candidate indexes are coded using variable length table, with priorities from 1 the highest to the 4 lowest. The codeword for the highest priority candidate consists of 1 bit, followed by 2, 3, and 4 bits for the lower priority candidate indexes. In the figures, candidate 1 is denoted as “top” (or “left”) candidate 2 is “collocated”, candidate 3 is “corner” and candidate 4 is “corner bottom-left”.

PU_1

PU_2

1

PU_2

3

PU_2

4

PU_2

PU_1

PU_2

2

PU_2

Collocated

Figure 3 Candidates for the second PU in a partial merge CU consists of two Nx2N PUs in HEVC.

PU_1

PU_1

PU_2

PU_2

1

PU_2

3

PU_2

4

PU_2

PU_1

PU_2

2

PU_2

Collocated

Figure 4 Candidates for the second PU in a partial merge CU consists of two 2NxN PUs in HEVC.

In this contribution we propose that for each forced merged 2nd PU, the maximum number of merge candidates is 2. Thus, only 1 bit syntax is needed to specify the merge candidate (or merge candidate index) that is used for prediction. As shown in Figure 5, the 2nd PU of a CU with partition type Nx2N may have four potential merging candidates, i.e. “collocated”, “top”, “corner” and “corner bottom left” with priority index 1, 2, 3, and 4. As shown in Figure 6, the 2nd PU of a CU with partition type 2NxN may have four potential merging candidates, i.e. “collocated”, “left”, “corner” and “corner bottom left” with priority index 1, 2, 3, and 4.

If the collocated candidate exists, then it must be one of the merging candidates, referred as “temporal” candidate, one of the three remaining candidates (i.e. “top”/”left” “corner” and “corner bottom-left”) will be selected as “spatial” candidate, depending on their existence and priority order. That is, if candidate 2 exists, then it is selected as the “spatial” candidate; otherwise if candidate 3 exists, then it is selected as the “spatial” candidate; otherwise candidate 4 is selected as the “spatial” candidate, if available. If the temporal candidate (i.e. candidate 1) does not exist, then the 1 bit syntax is used to specify which spatial candidate is used for prediction. Based on the priority order, the first two available spatial candidates of the three will be selected as the merging candidates.

PU_1

PU_2

2

PU_2

4

PU_2

PU_1

PU_2

1

PU_2

Collocated

3

PU_2

Figure 5 Proposed candidates for the second PU in a partial merge CU consists of two Nx2N PUs.

Collocated

PU_1

PU_1

PU_2

PU_2

2

PU_2

3

PU_2

4

PU_2

PU_1

PU_2

1

PU_2

Figure 6 Proposed candidates for the second PU in a partial merge CU consists of two 2NxN PUs.

In the case that less than two merging candidates exist, no syntax bit is needed to specify the selected merging candidate.

# Simulation results

Simulation were generated following common test conditions defined in JCTVC-600Error: Reference source not found. Results are shown in table 2.

Table 2	 Results for proposed partial CU merge method compared with anchor.

|  | Random access |  |  | Random access LoCo |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | -0.1 | 0.2 | -0.2 | 0.1 | 0.0 | 0.0 |
| Class B | 0.1 | 0.2 | 0.2 | 0.3 | 0.3 | 0.3 |
| Class C | 0.1 | 0.1 | 0.1 | 0.3 | 0.2 | 0.3 |
| Class D | 0.1 | 0.1 | -0.1 | 0.2 | 0.2 | 0.1 |
| Class E |  |  |  |  |  |  |
| All | 0.1 | 0.2 | 0.0 | 0.2 | 0.2 | 0.2 |
| Enc Time[%] | 88% |  |  | 83% |  |  |
| Dec Time[%] | 101% |  |  | 101% |  |  |
|  | Low delay |  |  | Low delay LoCo |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A |  |  |  |  |  |  |
| Class B | -0.1 | 0.2 | 0.2 | 0.3 | 0.3 | 0.4 |
| Class C | 0.1 | 0.3 | 0.2 | 0.3 | 0.2 | 0.0 |
| Class D | 0.2 | 0.4 | 0.2 | 0.4 | 0.1 | 0.3 |
| Class E | -0.2 | -0.7 | 0.1 | 0.3 | 0.0 | 0.5 |
| All | 0.0 | 0.1 | 0.2 | 0.3 | 0.1 | 0.3 |
| Enc Time[%] | 87% |  |  | 81% |  |  |
| Dec Time[%] | 101% |  |  | 101% |  |  |

# Conclusions

This contribution proposed a method for partial CU merge, in specific related to the merge of the 2nd PU in an Nx2N or 2NxN CU. Experimental results reportedly show encoding time reduction (average 13% for HE, 18% for LC) with BD-rate increase less than 0.1% for HE, 0.2-0.3% for LC. We recommend the committee to consider adopting this method and including it in HM.

# Patent rights declaration(s)

MediaTek Inc. may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

- Frank Bossen, “Common test conditions and software reference configurations”, JCTVC-D600, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T VCEG and ISO/IEC MPEG, Daegu, Korea, Jan. 2011.

- Gisle Bjontegaard, “Improvements of the BD-PSNR model”, ITU-T Q.6/SG16 VCEG, VCEG-AI11, Berlin, Germany, July 2008.