| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 5th Meeting: Geneva, CH, 16-23 March, 2011 | Document: JCTVC-E086 |
| --- | --- |

| Title: | [AHG: Complexity Assessment] Summary of Complexity Assessment for CEs and Three-Level Assessment method |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposed Draft |  |  |
| Author(s) or Contact(s): | Xing Wen, Oscar C. Au Dept. of ECE, Hong Kong University of Science and Technology Clearwater Bay, Hong Kong | Tel: Email: | 852-66261845 wxxab@ust.hk eeau@ust.hk |
|  | Mona MATHUR STMicroelectronics Pvt Ltd, Plot No-1, Knowledge Park-III, Greater Noida, UP, India | Tel: Email: | +91 1204003907 mona.mathur@st.com |
| Source: | AHG of Complexity Assessment |  |  |

_____________________________

# Abstract

In Daegu meeting, most of CEs were form their own complexity assessment methods, especially CE7 and CE 10. This contribution is a supplementary document for AHG report (JCTVC-E012), it first reviews existing complexity assessment methods proposed by all CEs. And based these, a drafted Three-Level Complexity Assessment method is proposed by AHG as a start point to form a more general complexity assessment method, which can give a standard complexity analysis for all kinds of tool/algorithm. BoG is needed by AHG for further discussion.

# Introduction

Since Guangzhou meeting, complexity of both software and hardware aspects become another critical criteria to measure an algorithm except only consider about BD-Bitrate reduction. In Daegu meeting, most of CEs have established their own complexity evaluations, as shown in following Table 1.

Table 1. Different complexity assessment methods proposed by different CEs

| CE num | Title | Complexity assessment |
| --- | --- | --- |
| CE1 JCT-VC D601 | DMVD | For assessment of the complexity impact, the following numbers should be provided: - Algorithm description including tool configuration - Number of operations (adds, multiplications, …) on number of reference frames - Memory bandwidth consumption (One option is to consider the use of a measurement tool provided by frame memory compression AHG) - Encoding time (relative to the tool switched off) - Decoding time (relative to the tool switched off) |
| CE2 JCT-VC D602 | Flexible Motion Partitioning | Not specified |
| CE3 JCT-VC D603 | Interpolation for MC (Luma) | - encoding/decoding times - Excel sheet that counts number of operations and instruction cycles compared to another |
| CE4 JCT-VC D604 | Slice Boundary Processing and Fine Granularity | Not specified |
| CE5 JCT-VC D605 | Low Complexity Entropy Coding Improvements | Not specified |
| CE6 JCT-VC D606 | Intra Prediction Improvement | For the complexity measurement, the anchor, the reference software and the reference software with the tool implemented will be executed on the same machine and the computational time will be measured for each software. Then, a time factor is calculated which compares the reference software including the sub-CE test tool and the reference software without the tool. |
| CE7 JCT-VC D607 | Alternative transforms | - Tool description In the JCT-VC text document associated with each proposed tool, the abstract shall include a brief text description of the proposed alternative transform, along with a brief summary of the compression performance and complexity evaluation. - Alternative transform description The alternative transforms for each transform size shall be described in such a way that makes it clear what operations are being performed. Pseudo-code or data-flow diagrams are preferred. - Arithmetic operations For each transform size of each alternative transform, the following statistics associated with multiplies, adds, and shifts shall be reported. These counts shall be based upon the pseudo-code or data-flow diagrams. - Average number of arithmetic operations per block - Average number of arithmetic operations per pixel - Worst-case number of arithmetic operations per block - Worst-case number of arithmetic operations per pixel - Table showing: Mults, Adds, Shifts, Total per block and per pixel, for 4x4, 8x8, … - Bit precision Report the maximum bit precision needed for each transform size. - Memory requirements For each transform size, report any additional memory requirements in bytes. - Quantization Report the presence and number of additional quantization matrices, for each transform size. Any additional bit-depth required by use of these additional quantization matrices shall also be reported. - Scan orders The number of additional scan orders shall be reported for each transform size. The Intra prediction mode mapping table, if applicable, shall be reported for each transform size. Report any additional complexity related to adaptive scanning that is introduced by the alternative transform. - R-D optimization If any additional rate-distortion loops or iterations are being performed in the encoder or decoder, their presence shall be indicated in the spreadsheet as “yes” or “no”. The reasons for doing this is that some participants may not be able to report encode or decode runtimes due to variations in computing platforms. - Encoder/Decoder run times Encoder and decoder run times shall be reported as described above. - Other statistics If present, memory transfers, reordering, and other data operations not included above shall be reported. |
| CE8 JCT-VC D608 | Non-deblocking loop filtering | - Maximum number of vertical taps - Number of operations, if a proposal introduces a non-linear feature, such branch operation shall also be reported |
| CE9 JCT-VC D609 | MV Coding and Skip/Merge operations | Run time |
| CE10 JCT-VC D610 | Core transform design | Complexity Analyze the complexity of candidate transform designs using the following parameters: • Arithmetic operations • Software execution speed • Dynamic range • Parallelizability on SIMD • Number of sequential operations • HW Proponents are encouraged to provide additional complexity analyses voluntarily. B.2.1. Arithmetic Operations Report the number of arithmetic operations needed for implementing the 2D inverse transform, in terms of multiplications, additions, shifts, compares. If the design allows flexibility in the way it is implemented, report the range of arithmetic operations available (for example, methods with and without multiplications). Include in the count TPE and final rounding if used. [Paper study] B.2.2. Software Execution Speed Develop a software model of candidate (transform + quantization modules) in single-thread C++ compatible with current HM software. Develop a common test environment for testing all candidates. In software tests below, run the tests N times, for N sufficiently large to smooth variations (e.g., N = 100K). Report total execution times. B.2.2.1. IBDI Off Develop a test input frame of a random noise (9-bit, [-255, 255]), and run N trials of the modules: [forward transform, quantization, dequantization, inverse transform]. B.2.2.2. IBDI On Develop a test input frame of a random noise (11-bit, [-1020, 1020]), and run N trials of the modules: [forward transform, quantization, dequantization, inverse transform]. (TI/Cisco volunteer to develop a common test environment.) B.2.3. Dynamic Range Analysis B.2.3.1. (Optional) Report if the candidate transform design is exactly invertible in fixed-point arithmetic, and if so, at what bitdepth. [Paper study] B.2.3.2. In the software module developed for B.2.2., test the dynamic range of the data at four points: prior to dequantization, prior to inverse transform, after first-stage inverse transform, and after second-stage inverse transform. Example input may be data consisting of +/- 255, either random, or Hadamard-like. (TBD) B.2.4. Parallelizability on SIMD Develop an optimized implementation for a target SIMD architecture (e.g., Intel Core i7 platform) for candidate designs [forward transform, quantization, dequantization, inverse transform]. Combine source codes into one environment (machine, OS, compiler, SW package) for comparative testing. Volunteers: David Flynn (BBC) volunteers to conduct this test - which is greatly appreciated by the group. Recommend that this be done in a common environment so anyone can test, crosscheck. B.2.5. Number of sequential operations Report the number of cascaded operators (multipliers/adders) in the longest execution path. [Paper study] B.2.6. HW [Optional] B.2.6.1. Develop optimized hardware models (RTL) of candidate transform designs, and determine the silicon area required, where optimization is for a common hardware condition. Details to be determined. B.2.6.2. Determine the hardware speed. From area and speed, power consumption can be estimated. This subtask is intended to be a paper study, given B.2.6.1. TBD: specific tasks, common hardware conditions, distribution of task responsibilities. Remarks: TI / Samsung plan to implement three proposals (Cisco/TI, Samsung/FastVDO, Qualcomm) in hardware, and cross-verify, for this test. |
| CE11 JCT-VC D611 | Coefficient scanning and coding | For the complexity measurement, the anchor, the reference software and the reference software with the tool implemented will be executed on the same machine and the computational time will be measured. A time factor will then be calculated between the reference software and the reference software with the tool implemented. However, the complexity analysis does not solely rely on the encoding and decoding time measurements of a particular implementation (reference software) of the working draft. The impact on optimized implementations in hardware and software are taken into account. Core experiment participants who investigate parallelization aspects are requested to provide a description showing how the parallelization is improved and which architecture(s) would see the benefit of the parallelization. Average and worst case throughput gain from parallelized methods should be considered. As the number of bin impact overall throughput, the following needs to be reported - the total number of bins and bin reduction/increase due to proposed technique - a breakdown of the number of bins coded in regular and bypass mode for CABAC - a breakdown of the number of bins for sig flag, last flag and absolute coefficient level The size of the context memory (i.e. number of contexts used) should be reported. In addition, for sub experiment CE.B Scheme of selecting the scan pattern during encoding needs to be described. Scheme of updating the scan pattern during parsing needs to be described. |
| CE12 JCT-VC D612 | Deblocking filtering | Complexity (encoding time measurement, decoding time measurement, memory storage, etc.) |
| CE13 JCT-VC D613 | Sample Adaptive Offset | For assessment of the complexity impact, the following numbers should be provided: 1. Algorithm description including tool configuration. 2. Encoding time (relative to the tool switched off) 3. Decoding time (relative to the tool switched off) 4. Number of line buffers on the decoder side |
| CE14 JCT-VC D614 | Intra Mode Coding | Running time |

However, their measurements are suffering from two common problems. Firstly, some of the CEs only pay attention to running time, and use this as the only criteria to measure the complexity of proposed algorithms. Secondly, although some of CEs proposed their own complexity measurement, they are only designed for certain CE and cannot become a general guideline for other CE or future use, or a standard measurement for JCT-VC. So in this proposal, we propose a three level complexity assessment method, as a start point to build a standard measurement which can become a guideline for future use.

# Three-Level Complexity Assessment (TLCA)

In this section, we propose our Three-Level complexity assessment (TLCA) method. Complexity is multidimensional, and it is important to capture the essence of this complexity space, but at the same time, keep the analysis space limited in size so it could be done in an efficient way. For different kind of method, it needs to emphasis different sense of complexity. So it is hard to adopt certain complexity assessment method for all kinds of tools/algorithms. So this contribution describes a Three-Level complexity assessment scheme. This scheme consists of three level assessment method to measure different aspects of certain algorithm. Numbers of complexity metrics are listed below, and it was the sense of the group to do more, rather than less, testing. These tests are constructed to explore suitability of designs in both software and hardware. And each Level, some of the tests are considered required, while other are optional and can be added as complementary results based on different tools/algorithms.

## TLCA-Level1 Assessment

Level 1 assessment is the basic level for proposed 3-level complexity assessment scheme. This level is recommended for all proposed tools/algorithms since most of the results can be obtained with software simulation and does not require extra experiments. This level assessment consists of three parts. All of them are recommended for all kinds of tools/algorithms.

### TLCA-Level1-1 Tool description

In the JCT-VC text document associated with each proposed tools/algorithms, at first, the abstract shall include a brief text description of the proposed tools/algorithms, along with a brief summary of the compression performance and complexity evaluation.

Furthermore, the description should also contain a detailed behavior description of proposed tool/algorithm, as follows:

- Mathematical background and description, not only in simple equations, but also detailed explanation and necessary extra information

- System level figures, block diagram to illustrate proposed tools/algorithms

- Word description of step/procedures to implement proposed tools/algorithms. Note that all necessary steps/procedures should be reported.

- Pseudo-code and data-flow diagrams of proposed tools/algorithms

These descriptions are all required for all kinds of tools/algorithms to help to understand the behavior of the tools/algorithms and can be used as a basis of other Levels of our proposed CA method.

### TLCA-Level1-2 Run time

The computational complexity of an application is often estimated by its overall execution time, which is very easy to be measured by the C function gettimeoftheday(), part of the standard library sys/time.h Although run time is not always precise [JCT-VC C200], it is still be adopted by JCT-VC and most of CE as a very important measurement of complexity. So Level1-2 is also required (but not all parts) for all the proposed tools/algorithms.

#### Run time factor

Currently, most of the proposals in JCT-VC use run time factor as complexity measurement. In order to get the run time factor, the anchor, the reference software and the reference software with the tool implemented will be executed on the same machine and the computational time will be measured for each of them. Then, a time factor is calculated which compares the reference software including the proposed tool/algorithm and the reference software without the tool/algorithm. The proposed tools/algorithms should report as follows:

- Simulation environment, including hardware configurations (CPU, Memory, etc.), OS (32bit or 64bit of Windows or Linux)

- Excel files contained detailed time obtained by same environment of each sequences at each QP point

- Time factor calculated based on simulation results with the environment mentioned above

#### Run time pie-chart for different coding components (optional)

Besides normal Run time factor, run time pie-chart of different coding components can be offered as complementary results. For example, the run time for intra/inter prediction, transform, entropy coding with proposed tool/algorithm on and off. This is because some of the tools/algorithms have different impact to different coding components, while total run time is not change.

### TLCA-Level1-3 Arithmetic Operations

The operations such as multiplies, adds, and shifts for certain tools/algorithms shall be counted. These counts shall be based upon the pseudo-code or data-flow diagrams from Level 1.

- The required operations of proposed tools/algorithm based on the descriptions in TLCA-Level 1-1

- Table show number of operations per block/pixel/operation unit for worst/average case for proposed tools/algorithms

- Tables/pie-charts show number of operations for each step/procedure in proposed tools/algorithms at worst/average case (optional)

- The bit-depth for each operation required by proposed tools/algorithm (optional) for example, 8-bit addition and 12-bit addition operation numbers should be reported separately

## TLCA-Level2 Assessment

This level is focus on memory bandwidth and bit-depth impact, also with the parallelizability.

### TLCA-Level2-1 Memory bandwidth

Tables contained memory bandwidth to show the worst or average case memory bandwidth of the reference software (the anchor) and the reference software with the tool/algorithm implemented. This can be done by paper work and sometimes need experiments. One option is to consider the use of a measurement tool provided by frame memory compression AHG.

- Table show average memory bandwidth of the reference software (the anchor) for all settings (Intra, Random access and low delay at HE and LC)

- Table show worst/average case memory bandwidth of the proposed tools/algorithms (the anchor) for all settings (Intra, Random access and low delay at HE and LC)

- Equations and steps based on Level 1-1 and 1-3 to show how to calculate the memory bandwidth of the proposed tools/algorithms

#### Bit precision/Bit-depth

Report bit precision (maximum) for proposed tool/algorithm.

#### Extra storage

Any extra storage required by the proposed tools/algorithm should be reported.

### TLCA-Level2-2 Parallelizability of tools/algorithms

Higher resolutions (up to 4x HD), higher frame rates (up to 240fps) and low power requirements are being projected as among the key requirements of future video coding applications. This has lead to an increased focus on using parallelization as a tool in the video coding pipeline to achieve the desired goals of high coding efficiency at higher throughput and lower latency. This is also evident from the many proposals in HEVC highlighting solutions aimed at exploiting parallelizability of the algorithm/method as a tool to enhance the performance. Various forms of parallelism have been proposed for HEVC at the prediction level (eg. Intra), bin level, syntax element level and slice level.

While the aim of achieving the desired coding goals brings into focus parallelization of the adopted approaches, here in we bring into focus how a parallel model is quite different form a sequential one and what could be the possible metrics on which the parallelism of proposed models can be judged.

A parallel implementation brings with it the additional overheads of context switching, communication and synchronization which can drastically affect the performance of a parallel program. Here in we list some of the modeling issues that affect the performance of a parallel program/algo, Amount of Parallelism possible in the tools/algorithms being targeted.

- Communication Overhead - the cost paid in the terms of additional communication required between processors

- Granularity of Operation --- parallelism is being targeted at a coarse or a fine level of granularity—Parallelism at finer granularity comes at the cost of additional communication overhead.

- Balanced Pipelines – Unless the tasks are balanced properly the throughput will get affected

- Synchronization Overhead --- The parallel tasks need to be synchronized at key points and this can affect the decision on how the parallelization will be designed in the targeted algo.

- Multiple Instances of Processors --- The number of processors in a system can adversely affect the synchronization requirements. Many a times the performance of a parallel program is adversely affected with increase in the number of available processors beyond a point.

- Memory Usage – There is a performance trade-off between the size of on-chip memory and size of parallelism that can be achieved.

- Scalability --- judges if more parallelism be achieved by simply adding more processors.

While it is not possible to judge and compare the various algorithms against each of these modeling issues, it is important to understand the impact of these modeling parameters on the performance of parallel models/algorithms. Here we define some measurable parameters which can allow us to judge the usefulness of a parallel algorithm,

- Implementation complexity ---- Implementation complexity can be judged on many lines. The memory communication overhead is one of the more important parameters to judge the optimality of a parallel implementation. A tool has been developed by Memory Compression AHG for judging the memory bandwidth of the Motion Compensation (MC) at the decoder. A similar tool needs to be deployed for judging the memory requirements of a parallel program.

- Scalability ---- How does the algorithm scale with the number of processors --- usually the impact on communication overhead does not scale linearly with an increase in the number of processors --- so it would beneficial if the proposals are accompanied by simulation results showing the implementation of algos with eg – 2, 4, 6 and 8 processors.

- Balanced pipelines ---- For a video encoder/decoder a parallel algorithm may be marred by unbalanced pipelines. Eg. For slice level parallelism --- say the algo proposes that slices can be decoded in parallel – considering a hypothetical case --- that the number of processors is equal to the half (or 1/4th) the number of slices being decoded. The amount of data handled by any two slices can vary drastically and if the algo does not take care of a judicious distribution than these can lead to synchronization delays and affect the throughput. So the suggestion is that the implementation should be tested on a wide variety of data sets.

- Throughput --- Average and worst case throughput of the parallel implementation must be stated.

- Coding efficiency --- Any gain/loss of the coding efficiency as compared to a sequential implementation

- Latency --- Average and worst case latency of the parallel implementation must be stated.

Parallel Architecture(s) Supported:  TBD

Parallel Programming Languages:  TBD --- OpenMP, OpenCL, POSIX threads.

## TLCA-Level3 Assessment (optional)

Since it is also important whether the proposed tools/algorithms can be efficiently implemented by hardware or not, it is necessary to review the complexity of proposed tools/algorithms from hardware implementation aspect. However, it is unreasonable to make this Level as requirement for all tools/algorithms and it is not possible for all participants to optimize the hardware design and finish implementation. So we recommend this Level as optional.

### TLCA-Level3-1 Hardware description/design

Each tool/algorithm should design hardware architecture, and offer sufficient description for it, including:

- System level overview which illustrate basic idea of proposed architecture based Level 1

- Conceptual diagram, hardware block diagram of designed architecture

- Tables or chart for detailed data-flow

Note that if the proposed tools/algorithms are modified to fit hardware implementation, extra simulation results should be offered.

### TLCA-Level3-2 Hardware Implementation

Develop optimized hardware models (RTL) of candidate transform designs and after synthesis, report:

- Silicon area (equivalent gate count) required (optimized for a common hardware condition)

- Technology (e.g. 0.18 um or 0.13 um)

- Maximum working frequency, and working frequency at certain data throughput (e.g. 1080P@30fps real-time processing)

- Power consumption at maximum working frequency and certain frequency for real-time processing