| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 5th Meeting: Geneva, CH, 16-23 March, 2011 | Document: JCTVC-E087 |
| --- | --- |

| Title: | CE4-subset1: Cross-check result of Huawei's fine granularity slice partition (JCTVC-Exxx) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Masato Shima Canon Inc. 30-2, Shimomaruko 3-chome, Ohta-ku, Tokyo 146-8501, Japan | Tel: Email: | +81-3-3758-2111 shima.masato@canon.co.jp |
| Source: | Canon Inc. |  |  |

_____________________________

# Abstract

This contribution contains the cross check result of JCTVC-Exxx, CE4 subtest 1: Fine Granularity Slice Partition by Huawei.

# Cross Check Procedure

The source code corresponds to CE4 subtest 1 case 6 was provided by Huawei, and it was compiled with GCC 3.4.6.  All the simulation was done on a Linux cluster.  The reference data were generated by disabling “FINE_GRANULARITY_SLICES” macro while the test data were generated by enabling it.  The test condition defined in JCTVC-D604 subtest1 was used for the crosscheck.

# Results

Cross check results are summarized in Table 1, where proposed fine granularity slices are compared with LCU aligned slices.

|  | Intra |  |  | Intra LoCo |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | 0.4 | 0.4 | 0.4 | 0.2 | 0.3 | 0.4 |
| Class B | 0.5 | 0.7 | 0.6 | 0.3 | 0.4 | 0.4 |
| Class C | 0.7 | 0.9 | 0.8 | 0.4 | 0.4 | 0.4 |
| Class D | 0.6 | 0.7 | 0.7 | 0.3 | 0.4 | 0.5 |
| Class E | 0.5 | 0.6 | 0.7 | 0.4 | 0.5 | 0.4 |
| All | 0.6 | 0.7 | 0.6 | 0.3 | 0.4 | 0.4 |
| Enc Time[%] | 92% |  |  | 90% |  |  |
| Dec Time[%] | 107% |  |  | 114% |  |  |
|  |  | Random access |  |  | Random access LoCo |  |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | 0.1 | -0.1 | 0.0 | 0.1 | 0.1 | 0.3 |
| Class B | 0.5 | 0.6 | 0.4 | 0.2 | 0.4 | 0.3 |
| Class C | 0.4 | 0.3 | 0.2 | 0.2 | 0.2 | 0.2 |
| Class D | 0.3 | 0.4 | 0.6 | 0.1 | 0.4 | 0.2 |
| Class E |  |  |  |  |  |  |
| All | 0.3 | 0.3 | 0.3 | 0.2 | 0.3 | 0.2 |
| Enc Time[%] | 97% |  |  | 98% |  |  |
| Dec Time[%] | 98% |  |  | 95% |  |  |
|  | Low delay |  |  | Low delay LoCo |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A |  |  |  |  |  |  |
| Class B | 0.3 | 0.3 | 0.2 | 0.1 | 0.3 | 0.3 |
| Class C | 0.3 | 0.1 | 0.1 | 0.0 | 0.1 | 0.0 |
| Class D | 0.2 | 0.6 | 0.6 | 0.0 | 0.3 | -0.5 |
| Class E | 0.1 | 0.7 | 0.2 | 0.0 | 0.5 | -0.3 |
| All | 0.2 | 0.4 | 0.3 | 0.0 | 0.3 | -0.1 |
| Enc Time[%] | 100% |  |  | 98% |  |  |
| Dec Time[%] | 98% |  |  | 105% |  |  |

Table 1: BD-rate results for proposed fine granularity slices vs. LCU-aligned slices

The BD-rate results obtained in our simulation match the results provided by Huawei.  Accordingly, bitrate and PSNR results provided by Huawei were verified.  On the other hand, because results on encoder/decoder time were not provided by Huawei, only our simulation results on encoder/decoder time are presented in Table 1.

# Conclusion

It has been verified that the bitrate and PSNR results provided by Huawei on LCU-aligned slices and proposed fine granularity slices are accurate.