| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 5th Meeting: Geneva, CH, 16-23 March, 2011 | Document: JCTVC-E089 |
| --- | --- |

| Title: | CE4-subset2: Cross-check result of Huawei's intra coding improvements for slice boundary blocks (JCTVC-Exxx) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Masato Shima Canon Inc. 30-2, Shimomaruko 3-chome, Ohta-ku, Tokyo 146-8501, Japan | Tel: Email: | +81-3-3758-2111 shima.masato@canon.co.jp |
| Source: | Canon Inc. |  |  |

_____________________________

# Abstract

This contribution contains the cross check result of JCTVC-Exxx, CE4 subtest 2: intra coding improvements for slice boundary blocks by Huawei.

# Cross Check Procedure

The source code corresponds to CE4 subtest 2 cases 5/6 was provided by Huawei, and it was compiled with GCC 3.4.6.  All the simulation was done on a Linux cluster.  The reference data were generated by disabling both “HS_PADDING” and “HS_PADDING_POSITIVE_ANG” macros while the test data were generated by enabling them.  The test condition defined in JCTVC-D604 subtest2 was used for the crosscheck.

# Results

Cross check results are summarized in Table 1, where proposed intra coding improvements on/off are compared.

|  | Intra |  |  | Intra LoCo |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | -0.4 | 0.1 | 0.3 | -0.5 | -0.2 | -0.1 |
| Class B | -0.5 | -0.4 | -0.5 | -0.8 | -0.6 | -0.5 |
| Class C | -0.7 | -0.6 | -0.5 | -0.8 | -1.0 | -0.9 |
| Class D | -0.5 | -0.5 | -0.5 | -0.6 | -0.6 | -0.8 |
| Class E | -0.8 | -0.3 | -0.3 | -1.0 | -0.2 | -0.6 |
| All | -0.6 | -0.3 | -0.3 | -0.7 | -0.5 | -0.5 |
| Enc Time[%] | 102% |  |  | 104% |  |  |
| Dec Time[%] | 101% |  |  | 101% |  |  |
|  |  | Random access |  |  | Random access LoCo |  |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | -0.2 | 0.0 | -0.2 | -0.2 | 0.2 | 0.0 |
| Class B | -0.2 | 0.0 | -0.4 | -0.3 | -0.2 | -0.1 |
| Class C | -0.3 | -0.3 | -0.3 | -0.3 | -0.4 | -0.3 |
| Class D | -0.2 | -0.4 | -0.1 | -0.2 | 0.1 | -0.3 |
| Class E |  |  |  |  |  |  |
| All | -0.2 | -0.2 | -0.2 | -0.3 | -0.1 | -0.2 |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |
|  | Low delay |  |  | Low delay LoCo |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A |  |  |  |  |  |  |
| Class B | -0.1 | 0.0 | -0.2 | -0.1 | -0.1 | 0.0 |
| Class C | -0.1 | -0.2 | -0.2 | -0.2 | -0.1 | -0.4 |
| Class D | -0.1 | 0.9 | -0.1 | 0.0 | 0.1 | -0.2 |
| Class E | -0.2 | 0.5 | 0.0 | 0.0 | 0.6 | -0.8 |
| All | -0.1 | 0.3 | -0.2 | -0.1 | 0.1 | -0.3 |
| Enc Time[%] | 100% |  |  | 101% |  |  |
| Dec Time[%] | 100% |  |  | 101% |  |  |

Table 1: BD-rate results for proposed intra coding improvements on/off

The BD-rate results obtained in our simulation match the results provided by Huawei.  Accordingly, bitrate and PSNR results provided by Huawei were verified.  For the encoding/decoding time, our simulation results are quite similar to the results provided by Huawei. (mostly identical, 1%~2% gap found in some cases)

# Conclusion

It has been verified that the bitrate and PSNR results provided by Huawei on intra coding improvements on slice boundary blocks are accurate.