| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 5th Meeting: Geneva, CH, 16-23 March, 2011 | Document: JCTVC-E090 |
| --- | --- |

| Title: | Adaptive CU Depth Range |  |  |  |
| --- | --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |  |
| Purpose: | Proposal |  |  |  |
| Author(s) or Contact(s): | *Xiang Li *Jicheng An *Xun Guo #Shawmin Lei *North Building 10F, Raycom Infotech Park Tower C, No. 2 Kexueyuan South Rd., Haidian District, Beijing, China 100190 #No. 1, Dusing Rd. 1, Hsinchu Science Park, Hsinchu, Taiwan 30078 | Email: | x.li@mediatek.com jicheng.an@mediatek.com xun.guo@mediatek.com shawmin.lei@mediatek.com |  |
| Source: | MediaTek Inc. |  |  |  |

_____________________________

# Abstract

In current HEVC (HM 2.0), CU depth has a fixed range at sequence level. In this contribution, it is proposed to adaptively select CU depth range to avoid potential encoding redundancies while reducing encoding complexity. Based on CU depth information of previously coded slices and CUs, CU depth range is adapted at slice or CU level. Accordingly, CU splitting flag and bi-prediction flag of CU depth outside the depth range are removed from corresponding entropy coding tables so that redundancies are removed. Moreover, encoding time is reduced since CU depths beyond the depth range and bi-prediction at maximal CU depth are not tested during encoding. It is observed that the encoding process is speeded up by 9%, 8%, 21%, 27%, 26% and 32% on average for IHE, ILC, RAHE, RALC, LDHE and LDLC configurations, respectively, while the resulting coding loss is kept within 0.3% (the average of six cases is 0.15%) in terms of BD-rate increase.

# Introduction

In current HEVC [1] (HM 2.0), the global maximal CU depth  (the superscript G denotes global) is signaled at sequence level in SPS. Without explicitly signaling, minimal CU depth  is always set to zero. Consequently, the range of CU depth [,] will be used for all frames covered by the SPS. Considering that optimal CU depth range is highly content dependent, it is not efficient to use a fixed CU depth range at sequence level.

Moreover, there would be encoding redundancies if the actual depth range [,] is smaller than [,]. First, there is potential redundancy with split_coding_unit_flag. When actual  is larger than , split_coding_unit_flags have to be coded until the current CU depth reaches  even though these flags are always “true” (always further splitting). Similarly, when , split_coding_unit_flags is always “false” at depth . Clearly, such redundancies can be avoided if  and  information is included in bitstreams.

Second, there is potential redundancy with inter prediction direction. As observed in the literature [2-3], bi-prediction is not efficient for small block sizes. Consequently, bi-prediction is seldom selected at high CU depths. In this case, the codeword to indicate bi-prediction can be removed (only forward and backward prediction are allowed) so that the average code length of inter prediction direction can be reduced.

More importantly, coding complexity will be reduced if only the actual depth range [,] is tested during encoding, assuming that the actual range [,] is smaller than [,]. Moreover, the encoding complexity can be further reduced if bi-prediction is disabled at the maximum CU depth .

# Proposed Method

To avoid potential redundancies while reducing encoding complexity, it is proposed to signal CU depth range at slice level or derive CU depth range at CU level.

## CU Depth Range Determination

### Determining CU Depth Range at Slice Level

The CU depth range [,] at slice level can be determined as follows.

where the superscript S indicates the current slice,  represents the average CU depth of the previously coded same-type frame, a and b are two constants. In this proposal, a and b are set to 1 and 2.75, respectively.

### Determining CU Depth at CU Level

The CU depth range [,] at CU level can be determined as follows.

where the superscript C indicates the current CU,  and  denote the CU depths of the left and top PUs, respectively.

## Avoiding Potential Redundancies

The aforementioned potential redundancies can be avoided by following conditions.

### Condition of Forcing Further Split

When the current CU depth  is smaller than current  (i.e.,), split_coding_unit_flag of the current CU depth is determined as true. In this case, split_coding_unit_flag is skipped in bitstream so that bit saving is obtained.

### Condition of Disabling Further Split

When the current CU depth is no less than current  (i.e.,), split_coding_unit_flag of the current CU depth is determined as false. In this case, split_coding_unit_flag is skipped in bitstream so that bit saving is obtained.

### Condition of Disabling Bi-Prediction

When the current CU depth  is no less than current  (i.e.,), bi-prediction is disabled. In this case, only forward-prediction and backward-prediction are allowed. Accordingly, the average length of the codeword for inter prediction direction is reduced.

# Related Syntax Modifications

## Syntax Definitions

To enable or disable the proposed method, HEVC syntax modifications for SPS, slice header, coding tree structure and prediction direction are proposed as in Table 1 to 4, respectively. In Table 1, seq_adaptive_CU_depth_idc equaling 1 or 2 indicates that the feature of adaptive CU depth range is enabled at slice or CU level. When seq_adaptive_CU_depth_idc is set to 0, the feature is disabled.

Table 1. Syntax modification for SPS

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| …… |  |
| log2_min_coding_block_size_minus3 | ue(v) |
| log2_diff_max_min_coding_block_size | ue(v) |
| seq_adaptive_CU_depth_idc | ue(v) |
| log2_min_transform_block_size_minus2 | ue(v) |
| } |  |

In Table 2, slice_CU_depth_min and slice_CU_depth_max indicate the CU depth range at slice level. These two parameters are determined with  in this proposal.

Table 2. Syntax modification for slice header

| slice_header( ) { | Descriptor |
| --- | --- |
| …… |  |
| slice_qp_delta | se(v) |
| if(seq_adaptive_CU_depth_idc==1) { |  |
| slice_CU_depth_min | ue(v) |
| slice_CU_depth_max | ue(v) |
| } |  |
| alf_param() |  |
| …… |  |
| } |  |

In table 3, additional conditions in section 2.2.1 and 2.2.2 are added to control the coding process of split_coding_unit_flag.

Table 3. Syntax modification for coding tree structure

| coding_tree( ) { | Descriptor |
| --- | --- |
| if( x0 + ( 1 << log2CUSize ) <= PicWidthInSamplesL && y0 + ( 1 << log2CUSize ) <= PicHeightInSamplesL && log2CUSize > Log2MinCUSize && CurCUDepth >= MinCUDepth && CurCUDepth < MaxCUDepth) |  |
| split_coding_unit_flag[ x0 ][ y0 ] | u(1) ¦ ae(v) |
| …… |  |

In table 4, additional condition in section 2.2.3 is added to control the coding process of inter_pred_idc. When the condition in section 2.2.3 is met, Bi-prediction is not allowd.

Table 4. Syntax modification for prediction direction

| prediction_unit(x0, y0, log2PUWidth, log2PUHeight, PartIdx ) { | Descriptor |
| --- | --- |
| …… |  |
| if( slice_type == B) |  |
| inter_pred_idc[ x0 ][ y0 ] | ue(v)¦ ae(v) |
| …… |  |

## Modified LCEC Tables

### Modified LCEC Tables for split_coding_unit_flag

In the current LCEC (HM 2.0), split_coding_unit_flag flag is jointly coded with other flags. Table 5 shows the initial joint coding table in HM 2.0. When split_coding_unit_flag can be determined by the proposed method, the codeword is saved accordingly. When split_coding_unit_flag is determined as true, other modes in Table 5 will be never selected. Consequently, no codeword needs to be sent. When split_coding_unit_flag is determined as false, it is proposed to use another table (Table 6) to code the other modes. When split_coding_unit_flag cannot be determined by the proposed method, the original table (Table 5) is used.

### Modified LCEC Table for inter_pred_idc

In the current LCEC, inter prediction direction flag, inter_pred_idc, is jointly coded with reference index. When bi-prediction is disabled by the proposed method (only uni-prediction is allowed in this case), it is proposed to use a new coding table (Table 7) to code reference index.

Table 5. Initial Joint Coding Table in HM2.0

Table 6. Proposed Initial Joint Coding Table

Table 7. Proposed Initial Coding Table for Reference Index

# Simulation Results

Simulations defined in [4] were conducted to verify the performance of the proposed adaptive CU depth range (ACUD). The results of ACUD at slice and CU level are summarized in Table 8. Please refer to attached Excel files for detailed results. Note that YUV files were output during the decoding.

On average, slice level ACUD improves encoding speed by 3% and 5% for IHE and ILC, 16% and 23% for RAHE and RALC, 23% and 28% for LDHE and LDLC, respectively. The resulting average coding loss of six cases is all within 0.3% in terms of BD-rate increase.

Compared to slice level ACUD, CU level ACUD provides an even faster speed while keeping a similar coding performance: Compared to the anchor, the encoding speed is improved by 9% and 8% for IHE and ILC, 21% and 27% for RAHE and RALC, 26% and 32% for LDHE and LDLC, respectively.

Table 8. Experimental Results for Slice Level ACUD (left) and CU Level ACUD (right)

# Conclusions

In this contribution, CU depth range is proposed to be adaptively determined at slice or CU level. The proposed method avoids potential encoding redundancies while reducing encoding complexity. Simulations show that encoding time is saved by 9%, 8%, 21%, 27%, 26% and 32% on average for IHE, ILC, RAHE, RALC, LDHE and LDLC configurations, respectively. More importantly, the average coding loss of all test cases is kept within 0.3% (the average of six cases is 0.15%) in terms of BD-rate increase.

# References

[1] T. Wiegand, W.-J. Han, B. Bross, J.-R. Ohm, G. J. Sullivan, “WD1: Working Draft 1 of High-Efficiency Video Coding”, JCT-VC meeting contribution JCTVC-C403, Guangzhou, China, Oct. 2010.

[2] H.-C. Lin, H.-M. Hang, and W.-H. Peng, “Fast Temporal Prediction Selection for H.264/AVC Scalable Video Coding,” IEEE Int’l Conf. on Image Processing, pp. 3425–3428, 2009.

[3] H.-C. Lin and H.-M. Hang, “Fast Algorithm on Selecting Bi-directional Prediction Type in H.264/AVC Scalable Video Coding,” IEEE Int’l Symposium on Circuits and Systems, pp. 113–116, 2010.

[4] F. Bossen, “Common test conditions and software reference configurations”, JCT-VC meeting contribution JCTVC-D600, Daegu, South Korea, Jan. 2011.

# Patent rights declaration(s)

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).