| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 5th Meeting: Geneva, CH, 16-23 March, 2011 | Document: JCTVC-E091 |
| --- | --- |

| Title: | Adaptive De-Quantization Offset |  |  |  |
| --- | --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |  |
| Purpose: | Proposal |  |  |  |
| Author(s) or Contact(s): | *Xiang Li *Xun Guo #Shawmin Lei *North Building 10F, Raycom Infotech Park Tower C, No. 2 Kexueyuan South Rd., Haidian District, Beijing, China 100190 #No. 1, Dusing Rd. 1, Hsinchu Science Park, Hsinchu, Taiwan 30078 | Email: | x.li@mediatek.com xun.guo@mediatek.com shawmin.lei@mediatek.com |  |
| Source: | MediaTek Inc. |  |  |  |

_____________________________

# Abstract

In current HEVC (HM 2.0), de-quantization offset is always set to zero. In this contribution, it is proposed to adaptively select de-quantization offset based on the distribution of previously coded transformed coefficients. It is reported that 0.9%, 0.7%, 0.5%, and 0.5% luminance BD-rate reductions were obtained by the proposed method for RAHE, RALC, LDHE, and LDLC configurations, respectively. In addition, higher coding gains (above 2% RD-rate reduction on average) were observed for chrominance components. The average encoding and decoding time for the proposed method were noted as 98% and 99% as those of the anchor.

# Introduction

In current HEVC (HM 2.0), the de-quantization process is defined as

where  and  are reconstructed transformed coefficient and quantized transformed coefficient, respectively, and Q denotes quantization interval.

In order to minimize the distortion between  and un-quantized transformed coefficient , it is proposed to use the de-quantization described in .

where b is called de-quantization offset. Clearly, how to determine b is a key problem in this approach.

# Proposed Method

## Optimal De-Quantization Offset

Based on the distribution of transformed coefficients, optimal b in  can be derived as follows.

Assuming that transformed coefficients submit to zero-mean Laplace distribution, i.e.,

where  is the distribution parameter which can be derived from the standard deviation  of transformed coefficients, namely

For transformed coefficients inside quantization dead zone  whereindicates quantization rounding offset, the reconstructed distortion can be calculated as

Note that here in the quantization dead zone, the de-quantization offset b is set to zero. If b is non-zero, additional flag has to be coded to indicate whether a coefficient is to be reconstructed to positive or negative value, which normally costs much.

Similar to , reconstructed distortion  for quantization interval () can be derived as

Consequently, the total distortion D is obtained as

Then the optimal offset  can be solved by minimizing total distortion D, namely

In HM 2.0 software, RDOQ is turned on by default. In this case, the rounding offset  can be regarded as zero. Let  be the percentage of transformed coefficients which are quantized to -1, 0, or 1, i.e.,

Plugging  into ,  is solved as

Defining  as normalized de-quantization offset, namely

total distortion D is obtained by putting ,  into . Setting , optimal  is solved as

Accordingly,

With  and , the proposed adaptive de-quantization offset can be calculated. In practice, a lookup table may be used to save computational complexity.

## De-Quantization Offset Determination

### Determining De-Quantization Offset at Slice Level

At the beginning of each slice, three base de-quantization offsets are calculated for Y, U and V components, respectively. Assuming  information of Y, U, V of the previously coded same-type-same-temporal-level frame is , , , the slice base offsets ,  and  (the subscript S indicates slice level) are obtained by taking , ,  into . Please note that for intra frames, the offsets are always set to zero. Moreover, when the condition in  is met, the base offsets are set to zero as well.

where  represents the percentage of quantized zeros in previously coded same-type-same-temporal-level frame. In fact,  is a simple method to check whether transformed coefficients are Laplacian distributed.

### De-Quantization Offset Adaptation at LCU Level

De-quantization offsets are also adjusted at LCU level according to the statistics of neighboring LCUs.

Supposing the  information of left and top LCU is  and , respectively, the offset  of the current LCU is derived by taking the average of  and  into . Please note that when neighboring CU is intra coded, the related  information is marked as unavailable so that it will not be used in the offset calculation.

Finally, the LCU offset is further clipped according to the related base offset at slice level, namely

## Refinement based on De-Quantization Offset

In HM 2.0 software,  in RDO process is directly dependent on quantization parameters. When quantization process is modified,  should be changed accordingly. In this proposal,  is adjusted as follows.

# Related Syntax Definitions

To enable or disable the proposed method, HEVC syntax definitions in SPS and slice header are proposed as in Table 1 and Table 2, respectively.

Table 1 Syntax definition in SPS

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| …… |  |
| interpolation_filter_flag | u(1) |
| seq_adaptive_dequantization_offset_flag | u(1) |
| rbsp_trailing_bits( ) |  |
| } |  |

In Table 1, when seq_adaptive_dequantization_offset_flag is 1, the adaptive de-quantization offset is enabled. Otherwise, the feature is disabled.

Table 2 Syntax definitions in slice header

| slice_header( ) { | Descriptor |
| --- | --- |
| …… |  |
| slice_qp_delta | se(v) |
| if(seq_adaptive_dequantization_offset_flag) { |  |
| slice_adaptive_dequantization_offset_idc | u(2) |
| if(slice_adaptive_dequantization_offset_idc) { |  |
| slice_adaptive_dequantization_offset_luma_base | se(v) |
| slice_adaptive_dequantization_offset_chroma1_base | se(v) |
| slice_adaptive_dequantization_offset_chroma2_base | se(v) |
| } |  |
| } |  |
| alf_param() |  |
| …… |  |
| } |  |

In Table 2, slice_adaptive_dequantization_offset_idc indicates the type of the de-quantization offset. The detailed defines are depicted in Table 3.

Table 3 Definition of slice_adaptive_dequantization_offset_idc

| slice_adaptive_dequantization_offset_idc | Meaning |
| --- | --- |
| 0 | Adaptive de-quantization offset is disabled |
| 1 | The range information of de-quantization offset will be signaled at slice level and de-quantization offset will be implicitly derived at LCU level. |

When slice_adaptive_dequantization_offset_idc is 1, slice_adaptive_dequantization_offset_luma_base, slice_adaptive_dequantization_offset_chroma1_base, and slice_adaptive_dequantization_offset_chro-ma2_base indicate the related lower and upper bounds (according to ) of LCU level de-quantization offset adaptation.

# Implementation Details

In this section, implementation details of this proposal are provided based on HM 2.0. Note that no changes are made for modules not mentioned in this section.

## Encoding Process

### Base Offset Calculation at Slice Level

During slice initialization at encoder, the following steps are performed to derive base offsets at slice level.

Step1: Get  information (,  and ) of previously coded same-type-same-temporal-level frame.

Step2: If  information is not available, or current frame is an intra frame, or the condition in  is met, the slice base offsets ,  and  are all set to zero, then end.

Step3: According to ,  and , get related slice base offsets ,  and  from lookup tables defined in Table 4 - Table 7 which are derived from . Note that when , the related  shall be set to 0.48.

Table 4 Lookup table of  ( 0.99991 )

|  | 1.00000 | 0.99999 | 0.99998 | 0.99997 | 0.99996 | 0.99995 | 0.99994 | 0.99993 | 0.99992 | 0.99991 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0.00 | 0.17 | 0.18 | 0.19 | 0.19 | 0.19 | 0.20 | 0.20 | 0.20 | 0.21 |

Table 5 Lookup table of  (0.9999  0.9991 )

|  | 0.9999 | 0.9998 | 0.9997 | 0.9996 | 0.9995 | 0.9994 | 0.9993 | 0.9992 | 0.9991 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0.21 | 0.22 | 0.23 | 0.24 | 0.24 | 0.24 | 0.25 | 0.25 | 0.25 |

Table 6 Lookup table of  (0.999  0.981 )

|  | -- | 0.999 | 0.998 | 0.997 | 0.996 | 0.995 | 0.994 | 0.993 | 0.992 | 0.991 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | -- | 0.26 | 0.28 | 0.29 | 0.29 | 0.30 | 0.31 | 0.31 | 0.32 | 0.32 |
|  | 0.990 | 0.989 | 0.988 | 0.987 | 0.986 | 0.985 | 0.984 | 0.983 | 0.982 | 0.981 |
|  | 0.32 | 0.33 | 0.33 | 0.33 | 0.33 | 0.34 | 0.34 | 0.34 | 0.34 | 0.34 |

Table 7 Lookup table of  (0.98  0.40 )

|  | -- | 0.98 | 0.97 | 0.96 | 0.95 | 0.94 | 0.93 | 0.92 | 0.91 | 0.90 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | -- | 0.35 | 0.36 | 0.37 | 0.38 | 0.39 | 0.39 | 0.40 | 0.40 | 0.41 |
|  | 0.89 | 0.88 | 0.87 | 0.86 | 0.85 | 0.84 | 0.83 | 0.82 | 0.81 | 0.80 |
|  | 0.41 | 0.41 | 0.42 | 0.42 | 0.42 | 0.42 | 0.43 | 0.43 | 0.43 | 0.43 |
|  | 0.79 | 0.78 | 0.77 | 0.76 | 0.75 | 0.74 | 0.73 | 0.72 | 0.71 | 0.70 |
|  | 0.44 | 0.44 | 0.44 | 0.44 | 0.44 | 0.44 | 0.45 | 0.45 | 0.45 | 0.45 |
|  | 0.69 | 0.68 | 0.67 | 0.66 | 0.65 | 0.64 | 0.63 | 0.62 | 0.61 | 0.60 |
|  | 0.45 | 0.45 | 0.45 | 0.46 | 0.46 | 0.46 | 0.46 | 0.46 | 0.46 | 0.46 |
|  | 0.59 | 0.58 | 0.57 | 0.56 | 0.55 | 0.54 | 0.53 | 0.52 | 0.51 | 0.50 |
|  | 0.46 | 0.46 | 0.46 | 0.47 | 0.47 | 0.47 | 0.47 | 0.47 | 0.47 | 0.47 |
|  | 0.49 | 0.48 | 0.47 | 0.46 | 0.45 | 0.44 | 0.43 | 0.42 | 0.41 | 0.40 |
|  | 0.47 | 0.47 | 0.47 | 0.47 | 0.48 | 0.48 | 0.48 | 0.48 | 0.48 | 0.48 |

### Calculation at Slice Level

During slice initialization at encoder, the following steps are performed to determine  at slice level.

Step1: Default  calculation in HM2.0.

Step2:  refinement with .

Please note that the refined  will not be changed within a frame/slice as long as QP is not changed.

### Offset Calculation at LCU Level

During LCU initialization, the following steps are performed to derive offsets at LCU level.

Step1: Calculate the average  information of left and top LCU. When left/top CU is intra coded, the related  information is marked as unavailable.

Step2: According to the average  information, get related LCU offsets ,  and  from lookup tables defined in Table 4 - Table 7.

Step3: According to the related base offsets at slice level, clip LCU offsets ,  and  with .

### Modified De-Quantization

The proposed offset is included in de-quantization process as follows

### Modified RDOQ

Since the de-quantization process is modified when  is non-zero, the distortion calculation in RDOQ has to be modified accordingly. Without de-quantization offset  (or =0), the distortion in RDOQ is calculated as

where c is transformed coefficient before quantization.

When including a non-zero , the distortion shall be calculated as

## Decoding Process

### Base Offset Decoding at Slice Level

Base offsets ,  and  at slice level are signaled in slice header. The related syntax and decoding method are depicted in Table 2.

### Offset Calculation at LCU Level

This part of decoding is the same as that in 4.1.3

### Modified De-Quantization

This part of decoding is the same as that in 4.1.4.

# Simulation Results

Simulation results of four inter test configurations [1] are summarized in Table 8. Please refer to attached xls file for detailed results. Note that YUV files were output during the decoding.

On average, 0.9%, 0.7%, 0.5%, and 0.5% luminance BD-rate reduction are observed for RAHE, RALC, LDHE, LDLC configurations, respectively. Higher coding gains are also noticed for chrominance components (above 2% BD-rate reduction on average). Particularly, up to 5.1% luminance BD-Rate reduction is found for NebutaFestival in RAHE. More important, the complexity introduced by the proposed method is quite low. The encoding and decoding time is almost the same as that of the anchor [1].

Table 8 Experimental Results of Adaptive De-Quantization Offset

# Conclusions

Based on the distribution of previously coded transformed coefficients, de-quantization offset is proposed to be adaptively calculated at slice level and further adjusted at LCU level. Simulations show that luminance BD-rate was reduced by 0.9%, 0.7%, 0.5% and 0.5% for RAHE, RALC, LDHE and LDLC, respectively. Higher coding gains (above 2% BD-rate reduction on average) were observed for chrominance components. More importantly, the computational complexity introduced by the proposed method is quite low. The encoding and decoding time of HM 2.0 with the proposed method is almost the same as that of the anchor.

# References

[1] F. Bossen, “Common test conditions and software reference configurations”, JCT-VC meeting contribution JCTVC-D600, Daegu, South Korea, Jan. 2011.

# Patent rights declaration(s)

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).