| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 5th Meeting: Geneva, CH, 16-23 March, 2011 | Document: JCTVC-E092 |
| --- | --- |

| Title: | Motion Vector Decimation for Temporal Prediction |  |  |  |
| --- | --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |  |
| Purpose: | Proposal |  |  |  |
| Author(s) or Contact(s): | *Xun Guo, #JianLiang Lin, #Yu-Wen Huang, #Shawmin Lei *North Building 10F, Raycom Infotech Park Tower C, No. 2 Kexueyuan South Rd., Haidian District, Beijing, China 100190 #No. 1, Dusing Rd. 1, Hsinchu Science Park, Hsinchu, Taiwan 30078 | Email: | xun.guo@mediatek.com jl.lin@mediatek.com yuwen.huang@mediatek.com shawmin.lei@mediatek.com |  |
| Source: | MediaTek Inc. |  |  |  |

_____________________________

# Abstract

This contribution proposes a motion vector (MV) decimation method for storing the MV data at lower resolution. Specifically, the MVs from the bottom right 4x4 block in a MV compression unit are used as the MVs of the whole unit instead of the top left one in HM2.0. Moreover, the reference indices of the bottom right block are also used for the whole unit to avoid the mismatch between MVs and reference indices. It is reported that average BD-rate reductions of 0.3%, 0.3%, 0.5% and 0.4% are observed by using the proposed method for RA-HE, RA-LC, LD-HE and LD-LC configurations, respectively, with almost no change of encoding and decoding time.

# Introduction

In HEVC, the MVs of reference frames are used in the MV prediction process of the current frame as temporal MV candidates, such as in AMVP and PU-level Merge. Therefore, the MV data needs to be stored. In order to save the memory for storing MV data, a MV memory compression method was adopted by HM2.0 [1]. In this method, a lower resolution MV buffer is used, and the MVs are compressed by using a decimation method. Fig. 1 shows an example of the decimation. In this example, the MV compression is conducted for each 16x16 unit, which is the default setting in the current HM2.0, and the MVs of the top left 4x4 block are used as the MVs of the whole 16x16 unit.

Fig. 1. Illustration of the motion vector decimation in HM2.0

# Proposed Method

.In the temporal MV derivation method of AMVP and PU-level Merge, the temporal co-located position of the central block in the current prediction unit (PU) is used to derive the temporal MV candidate. Considering the PU sizes may be different to the MV compression unit, it is straightforward to evaluate whether the current MV decimation method is the most efficient one. The simplest way is to evaluate other positions in the MV compression unit. After some evaluations, we propose to use the bottom right position instead of the current top left position. Fig. 2 shows an example of the proposed MV decimation method. In this figure, two kinds of MV compression unit with compression ratio of 16 and 4 are illustrated. The yellow 4x4 block in each compression unit means the decimation position in the MV compression process. That is, for 16x16 unit (left one), position 15 in Zig-zag order is used, and for 8x8 unit (right one), position 3 in Zig-zag order is used.

Fig. 2. Proposed MV decimation method for 16x16 unit and 8x8 unit

However, the current decimation method may lead to a mismatch between MVs and reference indices. That is, the blocks other than the decimation block use their own reference indices which may be different to that of the decimation block. To avoid this, we also use the reference indices of the bottom right block for the whole MV compression unit.

# Simulation Results

We tested the proposed MV decimation method in HM2.0 based on the recommended test conditions in JCTVC-D600 [2]. We use the default setting for the compression ratio in HM2.0 software, i.e. 16. Two tests were conducted: one is using the MVs of the bottom right block, the other is using both the MVs and reference indices of the bottom right block. The results are shown in Table 1 and Table2.

In order to show the gap between with and without MV buffer compression, we also tested the HM2.0 software with turning off the macro AMVP_BUFFERCOMPRESS, and the results are shown in table 3. It can be seen from the results that the performance loss taken by the MV buffer compression can be mostly compensated by simply changing the decimation position.

Table 1. Experimental results for using the MVs of the bottom right block

|  |  | Random access |  |  | Random access LoCo |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | -0.3 | -0.3 | -0.3 | -0.4 | -0.4 | -0.4 |
| Class B | -0.2 | -0.1 | -0.1 | -0.1 | -0.1 | -0.1 |
| Class C | -0.3 | -0.3 | -0.3 | -0.3 | -0.2 | -0.3 |
| Class D | -0.5 | -0.4 | -0.4 | -0.5 | -0.5 | -0.4 |
| Class E |  |  |  |  |  |  |
| All | -0.3 | -0.3 | -0.3 | -0.3 | -0.3 | -0.3 |
| Enc Time[%] | 100% |  |  | 99% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |
|  | Low delay |  |  | Low delay LoCo |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A |  |  |  |  |  |  |
| Class B | -0.3 | -0.2 | -0.4 | -0.2 | -0.3 | 0.0 |
| Class C | -0.5 | -0.3 | -0.5 | -0.5 | -0.5 | -0.7 |
| Class D | -0.6 | -0.7 | -0.7 | -0.5 | -0.5 | -0.9 |
| Class E | -0.7 | -1.0 | -0.3 | -0.4 | -0.9 | -0.5 |
| All | -0.5 | -0.5 | -0.5 | -0.4 | -0.5 | -0.5 |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 99% |  |  | 100% |  |  |

Table 2. Experimental results for using the MVs and reference indices of bottom right block

|  |  | Random access |  |  | Random access LoCo |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | -0.4 | -0.2 | -0.5 | -0.5 | -0.4 | -0.3 |
| Class B | -0.3 | -0.2 | -0.2 | -0.2 | -0.1 | -0.1 |
| Class C | -0.5 | -0.5 | -0.5 | -0.5 | -0.4 | -0.3 |
| Class D | -0.7 | -0.7 | -0.7 | -0.8 | -0.7 | -0.7 |
| Class E |  |  |  |  |  |  |
| All | -0.5 | -0.4 | -0.5 | -0.5 | -0.4 | -0.4 |
| Enc Time[%] | 100% |  |  | 102% |  |  |
| Dec Time[%] | 101% |  |  | 101% |  |  |
|  | Low delay |  |  | Low delay LoCo |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A |  |  |  |  |  |  |
| Class B | -0.4 | -0.2 | -0.4 | -0.4 | -0.3 | -0.1 |
| Class C | -0.6 | -0.5 | -0.5 | -0.7 | -0.7 | -0.8 |
| Class D | -0.8 | -0.4 | -0.8 | -0.8 | -0.7 | -0.7 |
| Class E | -0.7 | -0.4 | -0.2 | -0.6 | -0.9 | -0.8 |
| All | -0.6 | -0.4 | -0.5 | -0.6 | -0.6 | -0.6 |
| Enc Time[%] | 100% |  |  | 98% |  |  |
| Dec Time[%] | 99% |  |  | 100% |  |  |

Table 3. The experimental results by turning off the MV memory compression

|  |  | Random access |  |  | Random access LoCo |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | -0.7 | -0.7 | -0.6 | -0.7 | -0.6 | -0.5 |
| Class B | -0.3 | -0.1 | -0.2 | -0.2 | -0.1 | -0.1 |
| Class C | -0.6 | -0.5 | -0.5 | -0.6 | -0.5 | -0.5 |
| Class D | -1.0 | -1.2 | -1.0 | -1.2 | -1.1 | -0.9 |
| Class E |  |  |  |  |  |  |
| All | -0.6 | -0.6 | -0.6 | -0.7 | -0.6 | -0.5 |
| Enc Time[%] | 99% |  |  | 100% |  |  |
| Dec Time[%] | 98% |  |  | 99% |  |  |
|  | Low delay |  |  | Low delay LoCo |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A |  |  |  |  |  |  |
| Class B | -0.3 | -0.2 | -0.2 | -0.4 | -0.3 | -0.3 |
| Class C | -1.0 | -1.2 | -1.1 | -1.1 | -1.2 | -1.3 |
| Class D | -1.3 | -1.3 | -1.3 | -1.3 | -1.1 | -1.5 |
| Class E | -0.7 | -0.8 | -0.6 | -0.5 | -0.3 | -0.3 |
| All | -0.8 | -0.8 | -0.8 | -0.8 | -0.8 | -0.9 |
| Enc Time[%] | 100% |  |  | 99% |  |  |
| Dec Time[%] | 97% |  |  | 99% |  |  |

# Conclusions

This contribution proposed a motion vector decimation method, which uses the motion vectors of the bottom right block in a storing granularity instead of the top left one. Average BD-rate reductions of 0.3%, 0.2%, 0.4% and 0.4% are observed by using the proposed method for RA-HE, RA-LC, LD-HE and LD-LC configurations, respectively. The encoding and decoding time is essentially the same with the anchor. Moreover, the proposed method is very easy to be integrated into the current HM2.0 software.

# References

[1] Y. Su and A. Segall, “CE9: Reduced resolution storage of motion vector data,” JCTVC-D072, JCTVC meeting, Daegu, KR. Jan. 2011.

[2] F. Bossen, “Common test conditions and software reference configurations”, JCT-VC meeting contribution JCTVC-D600, Daegu, South Korea, Jan. 2011.

# Patent rights declaration(s)

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).