| 1Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 5th Meeting: Geneva, CH, 16-23 March, 2011 | Document: JCTVC-E095 |
| --- | --- |

| Title: | Unified motion vector removal process for AMVP |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Shigeru Fukushima Masayoshi Nishitani Toru Kumakura Motoharu Ueda JVC KENWOOD Holdings, Inc. 58-7, Shinmei-cho, Yokosuka, Kanagawa 239-8550, Japan | Tel: Email: | +81-46-836-6597 fukushima.shigeru@jk-holdings.com nishitani.masayoshi@jk-holdings.com kumakura.toru@jk-holdings.com ueda.motoharu@jk-holdings.com |
| Source: | JVC KENWOOD Holdings, Inc. |  |  |

_____________________________

# Abstract

In the HM2.0, the different motion vector removal process was removed for simplification. However, removing the different motion vector removal process always brings coding loss because the different motion vector removal process can remove useless more MVP candidates.

This proposal aims to unify the different motion vector removal process and the identical motion vector removal process to obtain coding gain resulted from both of the processes with maintaining simplified algorithm.

It can remove a ‘different’ motion vector and an ‘identical’ motion vector at the same process.

The proposed algorithm needs a few changes from HM2.0. At first, the identical motion vector removal process is removed. Second, the different motion vector removal process is restored. Finally, the different motion vector removal process is modified to consider the index cost of MVP for removing more MVP candidates when the cost of MVD is the same.

The simulation results show that the proposed technique provides overall 0.2% BD-rate gain without the increase of the complexity.

# Introduction

AMVP (Advanced Motion Vector Coding) [1] [2] is a technique to improve the accuracy of MVP (Motion Vector Predictor) by using the index that specifies one efficient MVP. In order to reduce the index cost of MVP, AMVP has the process which removes useless MVP candidates. The process is classified into two components, an identical motion vector removal process and a different motion vector removal process. In the HM2.0 [3], the different motion vector removal process was deleted for simplification. However, deleting the different motion vector removal process always brings coding loss because the different motion vector removal process can remove useless more MVP candidates.

This proposal aims to unify the different motion vector removal process and the identical motion vector removal process to obtain coding gain resulted from both of the processes with maintaining simplified algorithm.

Figure 1 illustrates the comparison of motion vector removal process in the HM1.0, HM2.0, and this proposal.

The HM1.0 includes both of an identical motion vector removal process and a different motion vector removal process.

The HM2.0 only includes an identical motion vector removal process. Different motion vector removal process is not included.

This proposal includes a unified motion vector removal process which is modified from the different motion vector removal process in the HM1.0. It can remove a ‘different’ motion vector and an ‘identical’ motion vector at the same process.

Figure 1 Comparison of motion vector removal process

# Algorithm Description

## Different Motion Vector Removal process in HM1.0 (previous algorithm)

Figure 2 illustrates the domain which is covered by each MVP in the previous algorithm (HM1.0).

Figure 2 Domain covered by each mvp in the previous algorithm

Under the previous algorithm, it is insufficient to remove useless MVP candidates because non priority domain that shows same cost of MVD (Motion Vector Difference) exists among the MVP candidates.

### Decoding procedure

Decoding procedure of the different motion vector removal process implemented in HM1.0 is as follows.

- Calculate mvCandidate as assumed that mvp(i) is mvpCandidate

where i is the index of mvp with mvdOriginal

- Calculate mvdOtherCandidate

- Remove the mvp(i) if the cost of mvdOtherCandidate is smaller than the cost of mvdOriginal

Figure 3 illustrates the calculation of mvCandidate and mvdOtherCandidate.

mvCandidate = mvpCandidate + mvdOriginal

mvdOtherCandidate = mvCandidate - mvpOtherCandidate

Figure 3 Calculation of mvCandidate and mvdOtherCandidate

Table 1 presents an example of the different motion vector removal process when the cost of mvdOriginal is 8bit. In this example, mvp2 is removed from the MVP candidate list. (See Figure 2)

Table 1 Example of the different motion vector removal process

| mvpCandidate | result | mvpOtherCandidate | mvdOtherCandidate |
| --- | --- | --- | --- |
| mvp1 | Keep | mvp2 | 14bit |
|  |  | mvp3 | 16bit |
| mvp2 | Removed | mvp1 | 6bit |
|  |  | mvp3 | 12bit |
| mvp3 | Keep | mvp1 | 8bit |
|  |  | mvp2 | 12bit |

### Encoder restriction

Encoder should select the MVP with the smallest cost of MVD. If the MVD with non smallest cost is selected, it will be removed on the different motion vector removal process.

## Proposed Algorithm

Figure 4 illustrates the domain which is covered by each MVP in the proposed algorithm.

Figure 4 Domain covered by each mvp in the proposed algorithm

The proposed technique needs a minor change to the different motion vector removal process. The modification is to consider the index cost of MVP for removing more MVP candidates when the cost of MVD is the same. An identical motion vectors can be removed in this process.

### Decoding procedure

Decoding procedure of proposed algorithm is as follows.

- Calculate mvCandidate as assumed that mvp(i) is mvpCandidate (Same as previous algorithm)

where i is the index of mvp with mvdOriginal

- Calculate mvdOtherCandidate (Same as previous algorithm)

- Remove the mvp(i)

- if the cost of mvdOtherCandidate is smaller than the cost of mvdOriginal (Same as previous algorithm)

or

- else if the cost of mvdOtherCandidate is the same as the cost of mvdOriginal and j is smaller than i

where j is an index of mvp with mvdOtherCandidate

The calculation of mvCandidate and mvdOtherCandidate is the same as the previous algorithm. (See Figure 3)

Table 2 presents an example of the unified motion vector removal process when the cost of mvdOriginal is 8bit. (The situation is the same as Table 1.) In this example, mvp2 and mvp3 are removed from the MVP candidate list. (See Figure 4) This means that the proposed algorithm has possibility to be able to remove more MVP candidates including the identical motion vector candidates compared to the previous algorithm.

Table 2 Example of the unified motion vector removal process

| mvpCandidate | result | mvpOtherCandidate | mvdOtherCandidate | mvpIdx |
| --- | --- | --- | --- | --- |
| mvp1 | Keep | mvp2 | 14bit | big |
|  |  | mvp3 | 16bit | big |
| mvp2 | Removed | mvp1 | 6bit | small |
|  |  | mvp3 | 12bit | big |
| mvp3 | Removed | mvp1 | 8bit | small |
|  |  | mvp2 | 12bit | small |

### Encoder restriction

Encoder should select the MVP with the smallest cost of MVD. If the MVD with non smallest cost is selected, it will be removed on the unified motion vector removal process. (Same as the previous algorithm)

Encoder should select the MVP with smaller index when the cost of MVD is same. If the MVP with bigger index is selected, it will be removed on the unified motion vector removal process.

Figure 5 illustrates the situation with the same cost of MVD. In this situation, Encoder should select mvp1 as MVP because mvp1 has cheaper cost than mvp2.

Figure 5 Situation with same cost of mvd

## Comparison of the algorithm

Table 3 presents the number of candidates required to be checked for removing.

Table 3 Number of candidates required to be checked

| Algorithm | Identical Motion Vector Removal Process | Unified (or Different) Motion Vector Removal Process | Sum of the maximum number required to be checked |
| --- | --- | --- | --- |
| HM1.0 | N | M (M<=N) | 2N |
| HM2.0 | N | - | N |
| Proposal | - | N | N |

In the HM1.0, the identical motion vector removal process needs to check N candidates and different motion vector removal process also needs to check M candidates (N is the number of initial MVP candidates and M is the number of MVP candidates reduced by the identical motion vector removal process). The different motion vector removal process needs to prepare to check N candidates because the maximum value of M is N (--- the case which NO candidates are removed).

In the HM2.0, the identical motion vector removal process needs to check N candidates with the same procedure in the HM1.0. The different motion vector removal process is not included in the HM2.0.

In this proposal, unified motion vector removal process needs to check only N candidates. The maximum number required to be checked in this proposal is the same as the one in the HM2.0.

Table 4 presents the summarized effectiveness of motion vector removal process.

Table 4 Summarized effectiveness of motion vector removal process

| Algorithm | Coding efficiency | Complexity |
| --- | --- | --- |
| HM1.0 | ○ | △ |
| HM2.0 | △ | ◎ |
| Proposal | ◎ | ○ |

## Implementation

### Software

This proposed technique is implemented into HM2.0 software.

### Syntax structure

The syntax of this proposal is not changed from HM2.0.

### Notes

The identical motion vector removal process still remains on encoder to maintain the current HM encoding algorithm.

# Experiments

## Test conditions

The common test conditions [4] are used.

## Experimental environment

The experimental environment used is presented in Table 5 and Table 6.

Table 5 Experimental environment for encoding

| CPU | Intel Core i7 870 2.93GHz |
| --- | --- |
| Memory | 16GB (DDR3) |
| OS | Windows 7 Professional 64bit |
| Compiler | Microsoft Visual C++ 2008 Professional edition SP1 |
| Executable | x64 |

Table 6 Experimental environment for decoding

| CPU | Intel Core i7 860 2.80GHz |
| --- | --- |
| Memory | 8GB (DDR3) |
| OS | Windows 7 Professional 64bit |
| Compiler | Microsoft Visual C++ 2008 Professional edition SP1 |
| Executable | x64 |

Notes: The decoding time is measured with the writing of yuv file.

## Simulation results

Table 7 presents the simulation results for random access.

Table 8 presents the simulation results for low delay.

Table 7 Simulation results for random access

|  | Random access |  |  | Random access LoCo |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | -0.1 | 0.0 | -0.2 | -0.2 | -0.2 | 0.0 |
| Class B | -0.1 | 0.0 | 0.0 | -0.1 | -0.1 | -0.1 |
| Class C | -0.2 | -0.1 | -0.2 | -0.2 | -0.1 | -0.1 |
| Class D | -0.1 | -0.3 | -0.1 | -0.2 | -0.3 | -0.2 |
| Class E |  |  |  |  |  |  |
| All | -0.1 | -0.1 | -0.1 | -0.2 | -0.2 | -0.1 |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 99% |  |  | 100% |  |  |

Table 8 Simulation results for low delay

|  | Low delay |  |  | Low delay LoCo |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A |  |  |  |  |  |  |
| Class B | -0.1 | 0.2 | -0.1 | -0.1 | -0.1 | 0.1 |
| Class C | -0.1 | 0.1 | -0.1 | -0.2 | -0.2 | -0.3 |
| Class D | -0.3 | -0.1 | -0.5 | -0.2 | -0.3 | -0.2 |
| Class E | -0.2 | 0.0 | 0.0 | -0.1 | 0.1 | -0.3 |
| All | -0.2 | 0.1 | -0.2 | -0.2 | -0.1 | -0.1 |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |

The simulation results show that the proposed technique provides overall 0.2% BD-rate [5] gain compared to the HM2.0 anchor. The complexity is not increased. Furthermore, the coding gain is obtained steadily for all test condition. (See the attached excel sheet)

# Conclusions

The simulation results show that the proposed technique provides overall 0.2% BD-rate gain without the increase of the complexity. The proposed technique has no impact for complexity but brings steady coding gain.

It can remove a ‘different’ motion vector and an ‘identical’ motion vector at the same process.

Therefore, we suggest that the proposed technique be adopted into HM for HEVC standardization.

# References

[1] “Samsung’s Response to the Call for Proposals on Video Compression Technology”,

JCTVC-A124, Dresden, April 2010

[2] “WD1: Working Draft 1 of High-Efficiency Video Coding”,

JCTVC-C403, Guangzhou, October 2010

[3] “WD2: Working Draft 2 of High-Efficiency Video Coding”,

JCTVC-D503, Daegu, January 2011

[4] Frank Bossen “Common test conditions and software reference configurations”,

JCTVC-D600, Daegu, January 2011

[5] G. Bjøntegaard, “Improvements of the BD-PSNR model,” ITU-T SG16 Q.6 Document,

VCEG-AI11, Berlin, July 2008

# Patent rights declaration(s)

JVC KENWOOD Holdings, Inc. and Victor Company of Japan, Limited may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).