| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 5th Meeting: Geneva, CH, 16-23 March, 2011 | Document: JCTVC-E097 |
| --- | --- |

| Title: | Temporal predictor restriction harmonized with motion vector compression |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Hideki Takehara Shigeru Fukushima JVC KENWOOD Holdings, Inc. 58-7, Shinmei-cho, Yokosuka, Kanagawa 239-8550, Japan | Tel: Email: | +81-46-836-5729 takehara.hideki@jk-holdings.com fukushima.shigeru@jk-holdings.com |
| Source: | JVC KENWOOD Holdings, Inc. |  |  |

_____________________________

# Abstract

The maximum number of access times to motion vector data of reference pictures depends on the minimum partition size allowed to use temporal predictor. Furthermore allowance of using the temporal predictor for any block size causes the increase of both encoder and decoder runtime.

This proposal aims to reduce the maximum number of access times to the temporal predictor in parsing process on decoder by harmonizing the partition size of using the temporal predictor with the partition size of the motion vector storage.

The number of access times to the temporal predictor in this proposal is reduced to 1/16 compared with the HM2.0 anchor.

The simulation results show that the proposed technique only causes overall 0.1% BD-rate loss. Overall encoder runtime is 101%, decoder runtime is 98-99% compared with the HM2.0 anchor, respectively.

# Introduction

Temporal predictor is useful to reduce motion vector difference to be transmitted. However, it requires the access to the temporal predictor for AMVP and Merge in parsing process. The maximum number of access times to motion vector data of reference pictures depends on the minimum partition size allowed to use the temporal predictor. Furthermore allowance of using the temporal predictor for any block size causes the increase of both encoder and decoder runtime.

In the HM2.0 [1], reduced resolution storage of motion vector data is adopted [2]. It means that the temporal predictor can be used for any block size although the motion vector of reference pictures is not stored by the size of 4x4 block anymore (i.e. 16x16 block).

Figure 1 illustrates an example of storing the motion vector in HM2.0

Figure 1 Storing motion vector in HM2.0

This proposal aims to reduce the maximum number of access times to the temporal predictor in parsing process on decoder by harmonizing the partition size of using the temporal predictor with the partition size of the motion vector storage.

# Algorithm Description

## Proposed algorithm

The partition size of using the temporal predictor is restricted to more than or equal to the partition size of the motion vector storage.

Figure 2 illustrates the allowed size of the temporal predictor in this proposal.

Figure 2 Allowed size of temporal predictor

The number of access times to the temporal predictor in parsing process on decoder can be reduced to 1/16 compared with the HM2.0 anchor.

Furthermore, it may bring coding gain depending on test condition because the index cost of the temporal predictor is no need to be indicated any more in AMVP and Merge.

## Implementation

### Software

This proposed technique is implemented into HM2.0 software.

### Syntax structure

The syntax of this proposal is not changed because the reduced ratio of motion vector data is defined by the macro “AMVP_DECIMATION_FACTOR” in HM2.0.

# Experiments

## Test conditions

The common test conditions [3] are used.

## Experimental environment

The experimental environment used is presented in Table 1 and Table 2.

Table 1 Experimental environment for encoding

| CPU | Intel Core i7 870 2.93GHz |
| --- | --- |
| Memory | 16GB (DDR3) |
| OS | Windows 7 Professional 64bit |
| Compiler | Microsoft Visual C++ 2008 Professional edition SP1 |
| Executable | x64 |

Table 2 Experimental environment for decoding

| CPU | Intel Core i7 860 2.80GHz |
| --- | --- |
| Memory | 8GB (DDR3) |
| OS | Windows 7 Professional 64bit |
| Compiler | Microsoft Visual C++ 2008 Professional edition SP1 |
| Executable | x64 |

Notes: The decoding time is measured with the writing of yuv file.

## Simulation results

Table 3 presents the simulation results for random access.

Table 4 presents the simulation results for low delay.

Table 3 Simulation results for random access

|  | Random access |  |  | Random access LoCo |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | 0.1 | 0.1 | 0.0 | 0.1 | -0.1 | 0.2 |
| Class B | 0.1 | 0.0 | -0.1 | -0.1 | -0.1 | -0.1 |
| Class C | 0.2 | 0.1 | 0.1 | 0.0 | 0.0 | 0.0 |
| Class D | 0.4 | 0.2 | 0.1 | 0.3 | -0.1 | -0.1 |
| Class E |  |  |  |  |  |  |
| All | 0.2 | 0.1 | 0.0 | 0.1 | -0.1 | 0.0 |
| Enc Time[%] | 99% |  |  | 101% |  |  |
| Dec Time[%] | 98% |  |  | 99% |  |  |

Table 4 Simulation results for low delay

|  | Low delay |  |  | Low delay LoCo |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A |  |  |  |  |  |  |
| Class B | 0.1 | -0.2 | -0.3 | -0.2 | -0.3 | -0.1 |
| Class C | 0.4 | 0.4 | 0.1 | 0.2 | 0.1 | 0.0 |
| Class D | 0.2 | -0.2 | -0.6 | 0.2 | -0.1 | -0.2 |
| Class E | -0.2 | -0.2 | 0.0 | -0.4 | -0.3 | -0.2 |
| All | 0.1 | 0.0 | -0.2 | 0.0 | -0.1 | -0.1 |
| Enc Time[%] | 100% |  |  | 104% |  |  |
| Dec Time[%] | 98% |  |  | 99% |  |  |

The simulation results show that the proposed technique provides overall 0.1% BD-rate [4] loss compared to the HM2.0 anchor. Average encoder runtime is 101%, decoder runtime is 98-99% compared with the HM2.0 anchor, respectively. (Please see the attached excel sheet.)

# Conclusions

The simulation results show that the proposed technique only causes overall 0.1% BD-rate loss. Average encoder runtime is 101%, decoder runtime is 98-99% compared with the HM2.0 anchor, respectively.

The number of access times to the temporal predictor in parsing process on decoder can be reduced to 1/16 compared with the HM2.0 anchor.

It may bring coding gain depending on test condition because the index cost of the temporal predictor is no need to be indicated any more in AMVP and Merge.

Therefore, we suggest that the proposed technique be adopted into HM for HEVC standardization.

# References

[1] “WD2: Working Draft 2 of High-Efficiency Video Coding”,

JCTVC-D503, Daegu, January 2011

[2] Yeping Su and Andrew Segall “CE9: Reduced resolution storage of motion vector data”,

JCTVC-D072, Daegu, January 2011

[3] Frank Bossen “Common test conditions and software reference configurations”,

JCTVC-D600, Daegu, January 2011

[4] G. Bjøntegaard, “Improvements of the BD-PSNR model,” ITU-T SG16 Q.6 Document,

VCEG-AI11, Berlin, July 2008.

# Patent rights declaration(s)

JVC KENWOOD Holdings, Inc. and Victor Company of Japan, Limited may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).