| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 5th Meeting: Geneva, CH, 16-23 March, 2011 | Document: JCTVC-E099 WG11 Number: m19614 |
| --- | --- |

| Title: | CE7: Cross-check of Samsung’s proposal on alternative transforms (E125) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Chuohao Yeo Yih Han Tan 1 Fusionopolis Way #21-01 Connexis (South Tower) Singapore 138632 | Tel: Email: | +65 6408 2000 chyeo@i2r.a-star.edu.sg |
| Source: | Institute for Infocomm Research |  |  |

_____________________________

# Abstract

This contribution provides a summary of the cross-checks performed by the Institute for Infocomm Research on Samsung’s proposal on alternative transforms. The cross-check rate-distortion performance results match those provided by Samsung. The implemented algorithm is also in line with Samsung’s proposal.

# Introduction

Samsung has proposed the use of DST (Odd-Type DST-III) and DCT for performing mode-dependent transforms for intra residual coding [4] [5]. The table below summarizes their choice of transforms, which is used only for 4x4 and 8x8 blocks.

| IntraPredMode | IntraPredType | IntraPredAngleID | Column DST | Row DST |
| --- | --- | --- | --- | --- |
| 0 | Intra_Vertical | 0 | 1 | 0 |
| 1 | Intra_Horizontal | 0 | 0 | 1 |
| 2 | Intra_DC | - | 0 | 0 |
| 3 | Intra_Vertical | -8 | 1 | 1 |
| 4 | Intra_Vertical | -4 | 1 | 1 |
| 5 | Intra_Vertical | +4 | 1 | 0 |
| 6 | Intra_Vertical | +8 | 1 | 0 |
| 7 | Intra_Horizontal | -4 | 1 | 1 |
| 8 | Intra_Horizontal | +4 | 0 | 1 |
| 9 | Intra_Horizontal | +8 | 0 | 1 |
| 10 | Intra_Vertical | -6 | 1 | 1 |
| 11 | Intra_Vertical | -2 | 1 | 1 |
| 12 | Intra_Vertical | +2 | 1 | 0 |
| 13 | Intra_Vertical | +6 | 1 | 0 |
| 14 | Intra_Horizontal | -6 | 1 | 1 |
| 15 | Intra_Horizontal | -2 | 1 | 1 |
| 16 | Intra_Horizontal | +2 | 0 | 1 |
| 17 | Intra_Horizontal | +6 | 0 | 1 |
| 18 | Intra_Vertical | -7 | 1 | 1 |
| 19 | Intra_Vertical | -5 | 1 | 1 |
| 20 | Intra_Vertical | -3 | 1 | 1 |
| 21 | Intra_Vertical | -1 | 1 | 1 |
| 22 | Intra_Vertical | +1 | 1 | 0 |
| 23 | Intra_Vertical | +3 | 1 | 0 |
| 24 | Intra_Vertical | +5 | 1 | 0 |
| 25 | Intra_Vertical | +7 | 1 | 0 |
| 26 | Intra_Horizontal | -7 | 1 | 1 |
| 27 | Intra_Horizontal | -5 | 1 | 1 |
| 28 | Intra_Horizontal | -3 | 1 | 1 |
| 29 | Intra_Horizontal | -1 | 1 | 1 |
| 30 | Intra_Horizontal | +1 | 0 | 1 |
| 31 | Intra_Horizontal | +3 | 0 | 1 |
| 32 | Intra_Horizontal | +5 | 0 | 1 |
| 33 | Intra_Horizontal | +7 | 0 | 1 |

The 4-point DST is implemented using a fast implementation that avoids full matrix multiply. The 8-point DST is implemented using full matrix multiply.

The default coefficient scans in HM2 are used.

# Simulation Results

The test conditions as specified in CE7 [1] are followed. The implementation is on HM 2.0. Tests were conducted on Intra and Random Access configurations for both high efficiency and low-complexity settings [2].

## RD results

Table 1 summarizes the BD-Rate [3] results for Samsung’s proposal.

Table 1. BD-Rate results for Samsung’s proposal

|  | Intra |  |  | Intra LoCo |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | -1.2 | -1.7 | -1.4 | -2.1 | -0.3 | -0.2 |
| Class B | -0.7 | -1.2 | -1.2 | -1.3 | -0.9 | -0.8 |
| Class C | -0.9 | -1.1 | -1.1 | -1.7 | -1.1 | -1.0 |
| Class D | -1.0 | -1.0 | -1.1 | -1.7 | -1.0 | -1.0 |
| Class E | -1.2 | -2.0 | -1.8 | -2.3 | -0.7 | -1.0 |
| All | -1.0 | -1.4 | -1.3 | -1.8 | -0.8 | -0.8 |
| Enc Time[%] | 101% |  |  | 103% |  |  |
| Dec Time[%] | 102% |  |  | 104% |  |  |
|  | Random access |  |  | Random access LoCo |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | -0.4 | -0.7 | -1.0 | -0.7 | -0.1 | 0.1 |
| Class B | -0.3 | -0.4 | -0.1 | -0.6 | -0.2 | -0.2 |
| Class C | -0.5 | -0.5 | -0.4 | -0.8 | -0.5 | -0.3 |
| Class D | -0.5 | -0.4 | -0.3 | -0.7 | -0.1 | -0.5 |
| Class E |  |  |  |  |  |  |
| All | -0.4 | -0.5 | -0.4 | -0.7 | -0.2 | -0.2 |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 101% |  |  | 101% |  |  |

A variant of the implementation was also tested, where the bit-shifts necessary for the 1-D transform (i.e., in the case of the DST), is performed at the end of 1st stage of the inverse transform, instead of performing all the shifts (including that for the de-quantization) at the end of the inverse transform. In addition, when the 8-point transform is used, the first implementation combined the rounding operations of the de-quantization and from TPE, which differs from HM2. In the variant, the two operations are separated when the DCT is performed, to match the behavior of HM2. Table 2 shows the BD-Rate results for this variant. There is no significant difference in coding performance from the above.

Table 2. BD-Rate results for variant

|  | Intra |  |  | Intra LoCo |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | -1.2 | -1.7 | -1.4 | -2.1 | -0.4 | -0.2 |
| Class B | -0.7 | -1.2 | -1.2 | -1.3 | -0.9 | -0.8 |
| Class C | -0.9 | -1.1 | -1.1 | -1.7 | -1.1 | -1.1 |
| Class D | -1.0 | -1.0 | -1.0 | -1.7 | -1.0 | -1.0 |
| Class E | -1.2 | -2.0 | -1.8 | -2.3 | -0.7 | -1.0 |
| All | -1.0 | -1.4 | -1.3 | -1.8 | -0.8 | -0.8 |
| Enc Time[%] | 101% |  |  | 103% |  |  |
| Dec Time[%] | 101% |  |  | 104% |  |  |
|  | Random access |  |  | Random access LoCo |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | -0.4 | -0.5 | -0.7 | -0.7 | 0.0 | 0.2 |
| Class B | -0.3 | -0.3 | -0.2 | -0.6 | -0.1 | -0.2 |
| Class C | -0.5 | -0.6 | -0.5 | -0.8 | -0.5 | -0.3 |
| Class D | -0.4 | -0.4 | -0.6 | -0.7 | -0.2 | -0.2 |
| Class E |  |  |  |  |  |  |
| All | -0.4 | -0.5 | -0.5 | -0.7 | -0.2 | -0.1 |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 101% |  |  | 100% |  |  |

More detailed coding results can be found in the Excel attachment.

## Qualitative results

We have not noticed any visible visual artifacts in the decoded frames.

## Complexity

Samsung’s proposal uses a combination of DCT and ODST-3, so requires one additional transform matrix per block size, i.e. for 4-point and 8-point.

The 4x4 transform used a fast implementation that does not require full matrix multiply. The 8x8 transform is implemented using a matrix multiply.

The table below summarizes the operation counts and other aspects. More details are provided in the attached excel spreadsheet.

| Evaluation Metrics |  |  | Core HM transform | Samsung's Proposal (E125) | Samsung's Proposal v2 (E125) | Comments |
| --- | --- | --- | --- | --- | --- | --- |
| 4x4 | Average Transform Arithmetic Operations (ops/pixel) | Mults | 0.0 | 2.3 | 2.3 |  |
|  |  | Adds | 5.0 | 7.0 | 7.5 |  |
|  |  | Shifts | 2.0 | 3.0 | 3.5 |  |
|  | Worst-Case Transform Arithmetic Operations (ops/pixel) | Mults | 0.0 | 4.5 | 4.5 |  |
|  |  | Adds | 5.0 | 9.0 | 10.0 |  |
|  |  | Shifts | 2.0 | 4.0 | 5.0 |  |
|  | Bit Precision | Max. required (bits) | 32 | 32 | 32 |  |
|  | Memory requirements | Additional for transform (bytes) | 0 | 5 | 5 |  |
| 8x8 | Average Transform Arithmetic Operations (ops/pixel) | Mults | 0.0 | 8 | 8 | In v1, HM2 core inverse transforms were modified to perform all the bit shifts in one stage instead of 2 stages |
|  |  | Adds | 10.0 | 12.5 | 13 |  |
|  |  | Shifts | 5.5 | 2.75 | 3.75 |  |
|  | Worst-Case Transform Arithmetic Operations (ops/pixel) | Mults | 0.00 | 16 | 16 |  |
|  |  | Adds | 10.00 | 16 | 16 |  |
|  |  | Shifts | 5.50 | 4.5 | 5.5 |  |
|  | Bit Precision | Max. required (bits) | 32 | 32 | 32 |  |
|  | Memory requirements | Additional for transform (bytes) | 0 | 64 | 64 |  |
| Misc | Quantization | Additional quantization matrices | 0 | 0 | 0 |  |
|  |  | Size of additional quantization matrices (bytes) | 0 | 0 | 0 |  |
|  | Scan orders | Additional scan orders | 0 | 0 | 0 |  |
|  |  | Mode mapping table size (bytes) | 0 | 0 | 0 |  |
|  | Additional R-D loops? (yes/no) | encoder | No | No | No |  |
|  |  | decoder | No | No | No |  |

## Training

The DCT/ODST-3 combination does not require any training.

# Conclusion

We have cross-checked Samsung’s proposal [4] [5], studied the algorithm and verified their coding simulation results.

# References

[1] R. Cohen et al., “CE7: Alternative Transforms,” JCTVC-D607, Daegu, Korea, Jan 2011.

[2] Frank Bossen, “Common test conditions and software reference configurations,” JCTVC-D600, Daegu, Korea, Jan 2011.

[3] Gisle Bjontegaard, “Calculation of Average PSNR Differences between RD curves,” ITU-T SC16/Q6, VCEG-M33, Austin, USA, Apr 2001.

[4] A. Saxena, F. C. Fenandes, “CE7:  Mode-dependent DCT/DST for intra prediction in video coding,” JCTVC-D033, Daegu, Korea, Jan 2011.

[5] A. Saxena, F. C. Fenandes, “CE7: Mode-dependent DCT/DST without 4*4 full matrix multiplication for intra prediction,” JCTVC-E125, Geneva, Switzerland, Mar 2011.

For avoidance of doubt, positive BD-Rate implies that the proposed method uses more rate, while a negative BD-Rate implies that the proposed method improves coding performance.