| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 5th Meeting: Geneva, CH, 16-23 March, 2011 | Document: JCTVC-E100 WG11 Number: m19615 |
| --- | --- |

| Title: | CE11: Cross-check of Qualcomm’s proposal on adaptive scans |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Chuohao Yeo 1 Fusionopolis Way #21-01 Connexis (South Tower) Singapore 138632 | Tel: Email: | +65 6408 2000 chyeo@i2r.a-star.edu.sg |
| Source: | Institute for Infocomm Research |  |  |

_____________________________

# Abstract

This contribution reports the cross-checking activity performed by the Institute for Infocomm Research on Qualcomm’s proposal on adaptive scans, which was part of CE11. The cross-check rate-distortion performance results match those provided by Qualcomm. The implemented algorithm is also in line with Qualcomm’s proposal.

# Introduction

In CE11 Sub Experiment CE.B, different coefficient scanning methods are to be tested [1]. Qualcomm has proposed an adaptive scan where the scan order for the first few coefficients (up to 64) is updated based on previously coded coefficients. For transform blocks sizes larger than 16x16, the scan order for the 16x16 block is used.

In HM2, the scan selection is mode-dependent, and one of zig-zag scan, horizontal scan and vertical scan is used for intra luma and chroma coding. The table below shows the ScanIdx (0=Zig-Zag, 1=Horizontal, 2=Vertical) used for each Intra luma prediction mode. Note that this remains unchanged from HM2. However, for 16x16 and 32x32 blocks, “horizontal” and “vertical” scan is also used, besides Zig-Zag scan.

| Mode | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ModScanIdx | 1 | 2 | 0 | 0 | 1 | 1 | 0 | 2 |
| Mode | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 |
| ModScanIdx | 2 | 0 | 0 | 1 | 1 | 0 | 0 | 2 |
| Mode | 16 | 17 | 18 | 19 | 20 | 21 | 22 | 23 |
| ModScanIdx | 2 | 0 | 0 | 0 | 1 | 1 | 1 | 1 |
| Mode | 24 | 25 | 26 | 27 | 28 | 29 | 30 | 31 |
| ModScanIdx | 0 | 0 | 0 | 0 | 2 | 2 | 2 | 2 |
| Mode | 32 | 33 |  |  |  |  |  |  |
| ModScanIdx | 0 | 0 |  |  |  |  |  |  |

The proposed adaptive scanning is used only for Intra luma coding when LCEC is used as the entropy coder. For each block size, 2 arrays of scan statistics and adaptive scan orders are updated; as mentioned, 32x32 and 16x16 share the same scans for coding their 64 coefficients. It is noted that the Horizontal and Vertical scans share the same scan statistics and adaptive scan orders; when ScanIdx=2, the transpose of the scan order for ScanIdx=1 is used.

At the beginning of parsing each slice, the scans are initialized to zig-zag scans and horizontal scans for ScanIdx=0 and 1 respectively, while the scan statistics are initialized such that the first value is 16, the next two values are 8, the next four values are 4, and so on. However, for the block size of 16x16, the scan order for ScanIdx=1 is also initialized to zig-zag scan. During parsing, the current adaptive scan corresponding to the selected ScanIdx is used; again, it is noted that when ScanIdx=2, the transpose of the scan order for ScanIdx=1 is used. At the same time, for each non-zero coefficient, a count for that coefficient location is incremented. If the count for that index is greater than that of the previous index, then the statistics and order of the two are swapped. If the count after incrementing reaches 255 (i.e., the 8-bit limit), then all the scan counts are renormalized by shifting them down by 1 bit. Therefore, the counts can be implemented using just 8-bit counters.

The encoding process is very similar to the decoding process, except that the scanning of the coefficients and statistics updating is done before the LCEC encoding of the scanned coefficients. The adaptive scans are also used in the RDOQ process.

# Simulation Results

The test conditions as specified in CE11 [1] are followed. The implementation is on HM 2.0. Tests were conducted on all low-complexity settings (Intra, Random Access, Low-Delay) [2].

## RD results

Table 1 summarizes the BD-Rate [3] results for Qualcomm’s proposal.

Table 1. Coding Performance of Qualcomm's adaptive scan proposal

|  | Intra LoCo |  |  |
| --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate |
| Class A | -2.0 | -1.4 | -1.3 |
| Class B | -1.0 | -0.8 | -0.8 |
| Class C | -0.6 | -0.3 | -0.3 |
| Class D | -0.2 | -0.1 | -0.1 |
| Class E | -1.2 | -0.9 | -0.9 |
| All | -1.0 | -0.7 | -0.7 |
| Enc Time[%] | 104% |  |  |
| Dec Time[%] | 103% |  |  |
|  | Random access LoCo |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate |
| Class A | -1.0 | -0.7 | -0.4 |
| Class B | -0.4 | -0.3 | -0.4 |
| Class C | -0.3 | -0.2 | -0.1 |
| Class D | 0.0 | 0.2 | 0.1 |
| Class E |  |  |  |
| All | -0.4 | -0.3 | -0.2 |
| Enc Time[%] | 101% |  |  |
| Dec Time[%] | 102% |  |  |
|  | Low delay LoCo |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate |
| Class A |  |  |  |
| Class B | -0.1 | -0.1 | -0.2 |
| Class C | -0.1 | 0.0 | -0.1 |
| Class D | 0.1 | 0.2 | -0.1 |
| Class E | 0.1 | -0.5 | 0.3 |
| All | 0.0 | -0.1 | -0.1 |
| Enc Time[%] | 101% |  |  |
| Dec Time[%] | 101% |  |  |

## Selection and update of scans

This has been described in detail in the Introduction.

In summary, for each block size of 4x4, 8x8 and 16x16/32x32 (which shares the information with 16x16), 2 8-bit arrays of scan counts and 2 arrays of scan orders are kept.

During encoding/decoding, for each non-zero coefficient, its count is updated, and one comparison is needed to see if any change to the scan order is necessary. In addition, if the incremented count reaches 255, than all the counts need to be renormalized by using bit-shift operations.

# Conclusions

We have cross-checked Qualcomm’s proposal [4], studied the algorithm and verified their coding simulation results.

# References

[1] V. Sze et al., “Description of Core Experiment (CE11): Coefficient scanning and coding,” JCTVC-D611, Daegu, Korea, Jan 2011.

[2] Frank Bossen, “Common test conditions and software reference configurations,” JCTVC-D600, Daegu, Korea, Jan 2011.

[3] Gisle Bjontegaard, “Calculation of Average PSNR Differences between RD curves,” ITU-T SC16/Q6, VCEG-M33, Austin, USA, Apr 2001.

[4] M. Karczewicz, X. Wang, W.-J. Chen, “CE5: Improved coefficient coding with LCEC,” JCTVC-D374, Daegu, Korea, Jan 2011.

For avoidance of doubt, positive BD-Rate implies that the proposed method uses more rate, while a negative BD-Rate implies that the proposed method improves coding performance.