| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 5th Meeting: Geneva, CH, 16-23 March, 2011 | Document: JCTVC-E101 WG11 Number: m19616 |
| --- | --- |

| Title: | Simplified AMVP candidate derivation for Inter and Merge modes |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Chuohao Yeo Yih Han Tan Zhengguo Li 1 Fusionopolis Way #21-01 Connexis (South Tower) Singapore 138632 | Tel: Email: | +65 6408 2000 chyeo@i2r.a-star.edu.sg |
| Source: | Institute for Infocomm Research |  |  |

_____________________________

# Abstract

In this contribution, a simplification of the selection of motion vector predictor candidates when performing Inter coding is presented. For large prediction units, existing methods for deriving motion vector predictor candidates may require checking a large number of neighboring partitions within a loop. The worst-case number of checks depends on the size of the prediction unit. In CE9, two methods are tested for limiting the worst-case number of partitions to be checked to a constant when deriving motion vector predictor candidates. This contribution test different configurations from what was tested within CE9. Experimental results report that compared to the configurations tested in CE9, the proposed configurations achieve similar or slightly better compression performance while maintaining the search complexity.

# Introduction

In HM2, the motion vector coding scheme used is Advanced Motion Vector Prediction (AMVP). Instead of always using the median predictor as in H.264/AVC, motion vectors can be predicted by any of 3 possible motion vector predictors (MVP): “Left”, “Above” and “Co-located”.

Consider the Prediction Unit (PU) shown in Figure 1. Given a particular reference index (ref_idx) for the motion vector, the MVPs are derived in the following manner. All the 4x4 partitions along the left edge of the PU are first searched, i.e., “Lx” in Figure 1, from the lower left to the upper left. Each partition is checked to see if it is Inter coded and has the same ref_idx as the current PU. If so, the search stops, and the motion vector of that partition is used as the “Left” MVP. If there are no suitable partitions, then there is no “Left” MVP. Next, all the 4x4 partitions along the top edge of the PU are searched, i.e., “Ax” in Figure 1, from the upper right to the upper left. Again, each partition is checked to see if it is Inter coded and has the same ref_idx has the current PU. In addition, it is also checked to see if it is different from the “Left” MVP (if that exists). If all the conditions are matched, the search stops, and the motion vector of that partition is used as the “Above” MVP. If there are no suitable partitions, then there is no “Above” MVP.

Figure 1. Illustration of AMVP candidate derivation in HM2 (HEVC WD2)

In the example shown in Figure 1, the PU is 32x32 pixels, each shaded block is 4x4 pixels, and so there are 8 4x4 partitions along each edge. To search for the “Left” MVP, partitions “L0” to “L8” are searched in order until the first useful partition is found. To search for the “Above” MVP, partitions “A0” to “A9” are searched in order until the first useful partition is found. Finally, the co-located partition in the reference frame corresponding to ref_idx = 0 is checked. Therefore, in the worst case, a total of 20 partitions need to be checked.

The table below lists the total number of partitions that need to be checked in the worst case for different partition sizes.

Table 1. Worst case number of checks to derive AMVP candidates

| PU Dimensions | Worst case number of checks |
| --- | --- |
| 4 x 4 | 6 |
| 4 x 8 | 7 |
| 8 x 4 | 7 |
| 8 x 8 | 8 |
| 8 x 16 | 10 |
| 16 x 8 | 10 |
| 16 x 16 | 12 |
| 16 x 32 | 16 |
| 32 x 16 | 16 |
| 32 x 32 | 20 |
| 32 x 64 | 28 |
| 64 x 32 | 28 |
| 64 x 64 | 36 |

In CE9, TI has proposed two simplified variants of the AMVP derivation to be tested [4], and they are shown below [1].

Figure 2. CE9 Experiment E

Figure 3. CE9 Experiment F

Essentially, only a subset of the neighbouring partitions is checked. In CE9 experiment E, up to 8 partitions need to be checked, while in CE9 experiment F, up to 6 partitions need to be checked.

# Proposed Approach

We propose a slight variant of the configurations in CE9 experiments E/F. The key differences are that:

- Partitions to be checked should be as far as possible, while overlapping with the source PU as much as possible.

- The top-left partition should be checked if either the “Left” or “Above” MVP has not been set.

Based on these, we propose the following two configurations (Figure 4 and Figure 5):

Figure 4. Proposed configuration - 8 partition checks (E’)

Figure 5. Proposed configuration - 6 partition checks (F’)

For the configuration shown in Figure 4, first, “L0”, “L1”, and “L2” are searched in that order to find the first partition that is Inter coded and has the same ref_idx as the source PU; the motion vector of that partition, if any, would be the “Left” MVP. Then, “A0”, “A1” and “A2” are searched in that order to find the first partition that is Inter coded, has the same ref_idx as the source PU, and does not have the same motion vector as the “Left” MVP; the motion vector of that partition, if any, would be the “Above” MVP.

Finally, if either the “Left” MVP or the “Above” MVP has not been found yet, then “M” would be checked if it is Inter coded, has the same ref_idx as the source PU, and does not have the same motion vector as the previously found MVPs; if so, its motion vector would be added as an additional MVP. The “Co-located” MVP is derived as in HM2; there is no change in the proposed approach. Therefore, at most 8 partitions would need to be checked for all PU sizes.

The configuration shown in Figure 5 is very similar to that in Figure 4, except that the partitions “L2” and “A2” are not considered. Therefore, at most 6 partitions would need to be checked for all PU sizes.

In addition, any of the above derivation process can be followed for deriving candidates for Merge modes. Consider F’ shown in Figure 5. The “Left” merge candidate can be derived by checking if “L0” is Inter coded; if it is, its motion information would be copied for the merge candidate. If it is not, the “L1” is next checked if it is Inter coded. Similarly, “A0” and “A1” are checked for deriving the “Above” merge candidate, and “Co” is checked for the “Co-located” merge candidate. If either of the “Left” or “Above” merge candidate is not available, then “M” checked to fill in the “Corner” merge candidate; if both are available, this would not be done. Note that unlike the design in HM2, there can only be at most 3 merge candidates, since the “Corner” candidate is only added if either the “Left” or “Above” merge candidate is not available. In HM2, there can be up to 5 merge candidates (Left, Above, Co-located, Corner1, Corner2).

# Experimental Results

We have independently implemented the two cases tested in CE9 Experiments E and F [1] in HM2. We have also implemented the proposed AMVP configurations in HM2.

In the experiments, both low-delay and random access configuration are used, as defined in the common test conditions [2]. The coding performances of HM2 with anchor conditions and with each of the AMVP configurations are compared. Coding performance is measured using BD-Rate with 4 operating points [3].

Table 2 and Table 3 show the coding performance for High Efficiency and Low Complexity respectively.

Table 2. Coding performance for High Efficiency cases

|  | CE9 Experiment E |  |  | CE9 Experiment F |  |  | Proposed E' |  |  | Proposed F' |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Random access |  |  | Random access |  |  | Random access |  |  | Random access |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | 0.0 | 0.1 | -0.1 | 0.1 | 0.0 | 0.0 | 0.0 | -0.1 | -0.1 | 0.0 | 0.1 | 0.0 |
| Class B | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.1 | 0.0 | 0.1 | 0.1 | 0.0 | 0.0 | 0.0 |
| Class C | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | -0.1 | -0.1 | 0.0 | 0.0 | -0.1 |
| Class D | 0.0 | 0.0 | -0.1 | 0.0 | 0.0 | 0.1 | 0.0 | -0.1 | 0.0 | 0.0 | -0.2 | -0.1 |
| Class E |  |  |  |  |  |  |  |  |  |  |  |  |
| All | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| Enc Time[%] | 99% |  |  | 100% |  |  | 100% |  |  | 99% |  |  |
| Dec Time[%] | 100% |  |  | 101% |  |  | 101% |  |  | 100% |  |  |
|  | Low delay |  |  | Low delay |  |  | Low delay |  |  | Low delay |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A |  |  |  |  |  |  |  |  |  |  |  |  |
| Class B | 0.0 | 0.0 | -0.2 | 0.0 | 0.1 | 0.0 | 0.0 | -0.1 | 0.0 | 0.0 | 0.0 | -0.2 |
| Class C | 0.0 | 0.1 | 0.0 | 0.0 | 0.1 | 0.1 | 0.0 | 0.1 | 0.2 | 0.0 | 0.0 | 0.1 |
| Class D | -0.1 | -0.4 | 0.0 | -0.1 | -0.2 | -0.2 | -0.1 | -0.1 | -0.4 | -0.1 | -0.4 | -0.2 |
| Class E | -0.1 | 0.1 | 0.0 | 0.1 | 0.0 | 1.0 | 0.0 | -0.4 | 0.0 | -0.1 | -0.4 | 0.3 |
| All | 0.0 | -0.1 | 0.0 | 0.0 | 0.0 | 0.2 | 0.0 | -0.1 | -0.1 | -0.1 | -0.2 | 0.0 |
| Enc Time[%] | 99% |  |  | 99% |  |  | 99% |  |  | 99% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  | 100% |  |  | 99% |  |  |

Table 3. Coding performance for low-complexity cases.

|  | CE9 Experiment E |  |  | CE9 Experiment F |  |  | Proposed E' |  |  | Proposed F' |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Random access LoCo |  |  | Random access LoCo |  |  | Random access LoCo |  |  | Random access LoCo |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | 0.0 | -0.1 | 0.0 | 0.0 | 0.0 | 0.1 | 0.0 | 0.0 | 0.0 | 0.0 | 0.1 | 0.1 |
| Class B | 0.0 | 0.0 | 0.1 | 0.0 | 0.1 | 0.1 | 0.0 | -0.1 | -0.1 | 0.0 | 0.0 | 0.0 |
| Class C | 0.0 | 0.0 | 0.0 | 0.0 | 0.1 | 0.1 | -0.1 | -0.1 | 0.0 | 0.0 | 0.0 | 0.0 |
| Class D | 0.0 | 0.0 | 0.0 | 0.0 | 0.1 | -0.1 | -0.1 | -0.2 | -0.1 | -0.1 | -0.2 | -0.2 |
| Class E |  |  |  |  |  |  |  |  |  |  |  |  |
| All | 0.0 | 0.0 | 0.0 | 0.0 | 0.1 | 0.0 | 0.0 | -0.1 | 0.0 | 0.0 | 0.0 | 0.0 |
| Enc Time[%] | 100% |  |  | 100% |  |  | 100% |  |  | 100% |  |  |
| Dec Time[%] | 100% |  |  | 101% |  |  | 100% |  |  | 100% |  |  |
|  | Low delay LoCo |  |  | Low delay LoCo |  |  | Low delay LoCo |  |  | Low delay LoCo |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A |  |  |  |  |  |  |  |  |  |  |  |  |
| Class B | 0.0 | 0.0 | 0.2 | 0.0 | -0.2 | 0.1 | 0.0 | -0.2 | 0.1 | 0.0 | -0.2 | 0.0 |
| Class C | 0.0 | 0.0 | 0.0 | 0.0 | -0.1 | -0.3 | 0.0 | -0.2 | -0.4 | 0.0 | 0.0 | -0.2 |
| Class D | 0.1 | 0.2 | 0.3 | 0.1 | -0.2 | 0.2 | 0.0 | 0.0 | -0.2 | 0.1 | -0.1 | 0.0 |
| Class E | -0.1 | -0.5 | 0.2 | 0.0 | -0.4 | 0.0 | -0.3 | 0.0 | -0.3 | -0.2 | -0.3 | -0.2 |
| All | 0.0 | 0.0 | 0.2 | 0.0 | -0.1 | 0.0 | -0.1 | -0.1 | -0.2 | 0.0 | -0.2 | 0.0 |
| Enc Time[%] | 100% |  |  | 100% |  |  | 100% |  |  | 100% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  | 100% |  |  | 99% |  |  |

The results show that all the configurations give about the same coding performance. However, F and F’ are more attractive because they require a smaller number of worst-case checks. Comparing F and F’, we see that F’ results in less loss of coding performance than F.

Table 4 shows the coding performance when the design F’ is used for coding Merge modes. Here, the AMVP candidates are derived as in HM2, i.e., no change is made to the derivation process for Inter modes. It is observed that there is no significant coding loss.

Table 4. Results for using design F’ for derivation of Merge candidates

|  | Random access |  |  | Random access LoCo |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | 0.1 | 0.2 | 0.1 | 0.2 | 0.1 | 0.1 |
| Class B | 0.1 | 0.0 | 0.1 | 0.2 | 0.2 | 0.1 |
| Class C | 0.1 | 0.0 | 0.1 | 0.1 | 0.1 | 0.1 |
| Class D | 0.1 | 0.0 | 0.1 | 0.1 | -0.1 | 0.0 |
| Class E |  |  |  |  |  |  |
| All | 0.1 | 0.1 | 0.1 | 0.2 | 0.1 | 0.1 |
| Enc Time[%] | 98% |  |  | 99% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |
|  | Low delay |  |  | Low delay LoCo |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A |  |  |  |  |  |  |
| Class B | 0.1 | 0.1 | -0.2 | 0.3 | 0.2 | 0.6 |
| Class C | 0.1 | 0.1 | 0.1 | 0.2 | 0.2 | 0.0 |
| Class D | 0.0 | -0.1 | -0.4 | 0.4 | 0.3 | 0.3 |
| Class E | 0.1 | -0.1 | -0.1 | 0.3 | 0.3 | 0.3 |
| All | 0.1 | 0.0 | -0.2 | 0.3 | 0.3 | 0.3 |
| Enc Time[%] | 98% |  |  | 99% |  |  |
| Dec Time[%] | 100% |  |  | 99% |  |  |

# Conclusions

We have carried out an independent implementation of Experiments E and F of CE9 and shown results for them. We have also proposed two variants of the two configurations and tested them. Based on the results we obtained, we would recommend adopting configuration F’ for motion vector predictor derivation, since it results in a fixed number of worst-case partitions checks regardless of PU size, and it have coding results that are slightly better than F with the same worst-case number of partition checks. The draft text for the derivation process is included in the Annex. We would like to acknowledge Qualcomm’s assistance with cross-checking E’ and F’.

We have also carried out experiments extending design F’ to coding merge modes. We recommend further study of this aspect of the contribution.

# References

[1] J. Jung et al., “Description of Core Experiment 9: MV Coding and Skip/Merge operations,” in JCTVC-D609, Daegu, Korea, Jan 2011.

[2] F. Bossen, “Common test conditions and software reference configurations,” in JCTVC-D600, Daegu, Korea, Jan. 2011.

[3] G. Bjøntegaard, “Calculation of Average PSNR Differences between RD curves,” in ITU-T SC16/Q6, VCEG-M33, Austin, USA, Jan. 2004.

[4] M. Zhou, “Scalable  motion vector competition and simplified MVP calculation,” in JCTVC-D055, Daegu, Korea, Jan. 2011.

# Patent rights declaration(s)

Institute for Infocomm Research may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# Annex – Derivation process for motion vector predictor candidates (F’)

##### Derivation process for motion vector predictor candidates

Inputs to this process are

- a luma location ( xP, yP ) specifying the top-left luma sample of the current prediction unit relative to the top-left sample of the current picture,

- variables specifying the width and the height of the prediction unit for luma, nPSW and nPSH,

- the reference index of the current prediction unit partition refIdxLX (with X being 0 or 1).

Outputs of this process are (with N being replaced by A or B)

- the motion vectors mvLXN of the neighbouring prediction units,

- the availability flags availableFlagLXN of the neighbouring prediction units.

Figure 8-6 – Spatial motion vector neighbours

The motion vector mvLXA and the availability flag availableFlagLXA are derived in the following ordered steps:

- Let a set of 2 sample locations be (xAk, yAk), with k = nA-1 .. nA, specifies sample locations with xAk = xP – 1, yAnA = yP + nPSH, and yAnA-1 = yAnA - 1. The set of sample locations ( xAk, yAk ) represent the sample locations immediately to the left side of the left partition boundary and it’s extended line. [Ed.: (WJ) MinPuSize should be defined somewhere, nA = nPSH / MinPuSize]

- Let the availability flag availableFlagLXA be initially set equal to 0 and the both components of mvLXA are set equal to 0.

- For ( xAk, yAk ) from ( xAnA-1, yAnA-1 ) to ( xAnA, yAnA ), the following applies repeatedly until availableFlagLXA is equal to 1:

- If the prediction unit covering luma location ( xAk, yAk ) is available, PredMode is not MODE_INTRA, predFlagLX[ xAk ][ yAk ] is equal to 1 and the reference index refIdxLX[ xAk ][ yAk ] is equal to the reference index of the current prediction unit refIdxLX, then availableFlagLXA is set equal to 1 and the motion vector mvLXA is set equal to the motion vector mvLX[ xAk ][ yAk ].

The motion vector mvLXB and the availability flag availableFlagLXB are derived in the following ordered steps:

- Let a set of 2 sample location (xBk, yBk), with k = nB-1 .. nB, specifies sample locations with xBnB = xP + nPSW, yBk = yP – 1, and xBnB-1 = xBnB - 1. The set of sample locations ( xBk, yBk ) represent the sample locations immediately to the upper side of the above partition boundary and it’s extended line. [Ed.: (WJ) MinPuSize should be defined somewhere, nB = nPSW / MinPuSize]

- Let the availability flag availableFlagLXB be initially set equal to 0 and the both components of mvLXB are set equal to 0.

- For ( xBk, yBk ) from ( xBnB-1, yBnB-1 ) to ( xBnB, yBnB ), the following applies repeatedly until availableFlagLXB is equal to 1:

- If the prediction unit covering luma location ( xBk, yBk ) is available, PredMode is not MODE_INTRA, predFlagLX[ xBk ][ yBk ] is equal to 1, the reference index refIdxLX[ xBk ][ yBk ] is equal to the reference index of the current prediction unit refIdxLX and availableFlagLXA is equal to 0 or mvLX[ xBk ][ yBk ] is not identical to mvLXA, then availableFlagLXB is set equal to 1 and the motion vector mvLXB is set equal to the motion vector mvLX[ xBk ][ yBk ].

If availableFlagLXA is equal to 0 or availableFlagLXB is equal to 0, then motion vectors mvLXA and mvLXB and availability flags availableFlagLXA and availableFlagLXB are updated in the following ordered steps:

- If the prediction unit covering above-left luma location ( xP – 1, yP – 1 ) is available, PredMode is not MODE_INTRA, predFlagLX[ xP – 1 ][ yP – 1 ] is equal to 1 and the reference index refIdxLX[ xP – 1 ][ yP – 1 ] is equal to the reference index of the current prediction unit refIdxLX, then the following steps are performed:

  - If availableFlagLXA is equal to 1,

    - If the motion vector mvLX[ xP – 1 ][ yP – 1 ] is not equal to mvLXA, then availableFlagLXB is set equal to 1 and the motion vector mvLXB is set equal to the motion vector mvLX[ xP – 1 ][ yP – 1 ].

  - Otherwise, if availableFlagLXB is equal to 1,

    - If the motion vector mvLX[ xP – 1 ][ yP – 1 ] is not equal to mvLXB, then availableFlagLXA is set equal to availableFlagLXB, the motion vector mvLXA is set equal to the motion vector mvLXB, availableFlagLXB is set equal to 1 and the motion vector mvLXB is set equal to the motion vector mvLX[ xP – 1 ][ yP – 1 ].

  - Otherwise, availableFlagLXA is set equal to 1 and the motion vector mvLXA is set equal to the motion vector mvLX[ xP – 1 ][ yP – 1 ].