| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 5th Meeting: Geneva, CH, 16-23 March, 2011 | Document: JCTVC-E102 |
| --- | --- |

| Title: | CE9: Skip/Merge Simplification with Reduced Candidate Set (Test L) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yih Han Tan Chuohao Yeo Zhengguo Li 1 Fusionopolis Way #21-01 Connexis (South Tower) Singapore 138632 | Tel: Email: | +65 6408 2000 yhtan@i2r.a-star.edu.sg |
| Source: | Institute for Infocomm Research |  |  |

_____________________________

# Abstract

One of the major goals of CE9 is to evaluate possible modifications to the motion information derivation process during inter coding. In this test, the Coding unit (CU) level Skip mode is first modified such that the motion vector derivation process of the Merge modes is used. On top of this Skip-Merge mode, the effect of reducing the number of possible Skip-Merge candidates is studied.

# Introduction

HM2 currently supports both a CU-level Skip mode and a Prediction Unit (PU) -level Merge mode. While the motion derivation processes of the two mechanisms differ, both modes allow low cost representations of a CU without coding motion vector differences and transform coefficients. Though experiments have shown that both the Skip and Merge modes improve coding performance, a redesign of the coding structure of an Inter CU could help simplify design and improve coding performance. One of the major goals of CE9 [1] is to evaluate possible modifications to the motion information derivation process during inter coding.

In this test, the Merge/Skip mechanism of a CU is first modified such that only the motion vector derivation process of the Merge modes is used. The effect of reducing the set of possible Merge candidates is then studied.

# Approach

In the current design of HM2, a CU-level symbol (QT-Root flag in the case of CABAC, and CBF symbol in the case of LCEC) signals to the decoder whether any coefficients are coded for a CU. In previous experiments on designs that only use the merge mode, header information such as flags that signal partition type and QT-Root flag have to be coded. This could potentially make the 2Nx2N Merge mode more costly than the CU-level Skip mode even when no residual need to be coded. In contribution JCTVC-D051[2], the Skip mode was replaced by a modified Merge mode that does not code residual data. This modified Merge mode was signaled by the flag that previously signaled the Skip mode.

For Test L, the following modifications were made to HM2:

- AMVP Skip mode is disabled.

- A 2Nx2N Skip-Merge mode is signaled by the flag that previously signaled the Skip mode. The prediction mechanism of the Skip-Merge mode is the same as a 2Nx2N PU-Merge mode except that no residual data is coded.

- If the Skip-Merge mode is not used and a 2Nx2N PU-Merge mode is subsequently signaled, residual data is always present. For CABAC, QT-Root flag is inferred to be 1 and not decoded, while for LCEC, an additional sorting table for CBF coding and parsing is used to handle this case where CBF is always non-zero.

- For the experiment on reducing the number of Skip-Merge candidates, the candidate set is reduced to just the ‘Top’ and ‘Left’ Merge candidates when the Skip-Merge mode is signaled. Furthermore, if the Skip-Merge mode is not used and a 2Nx2N PU-Merge mode using the ‘Top’ or ‘Left’ Merge candidate is subsequently signaled, for CABAC, QT-Root flag is inferred to be 1 and not decoded, while for LCEC, as mentioned in point (c), an additional sorting table for CBF coding is used to handle this case where CBF is always non-zero.

# Results

We made the modifications for the experiments in HM2. In the experiments, both low-delay and random access configuration are used, as defined in the common test conditions [3]. Coding performance is measured using BD-Rate with 4 operating points [4].

Table 1 shows the coding performance for the Skip-Merge design (without the reduced candidate set for Skip-Merge mode) and Table 2 shows the performance of the same design with a reduced candidate set for the Skip-Merge mode (Test L).

Table 1. Coding Performance of Skip-Merge

|  |  | Random access |  |  | Random access LoCo |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | 0.2 | 0.1 | -0.1 | -0.2 | -0.4 | -0.4 |
| Class B | -0.1 | -0.3 | -0.4 | -0.7 | -0.7 | -0.6 |
| Class C | -0.1 | 0.0 | 0.0 | -0.3 | -0.3 | -0.3 |
| Class D | -0.1 | 0.0 | 0.1 | -0.3 | -0.2 | -0.1 |
| Class E |  |  |  |  |  |  |
| All | 0.0 | -0.1 | -0.1 | -0.4 | -0.4 | -0.3 |
| Enc Time[%] | 99% |  |  | 100% |  |  |
| Dec Time[%] | 100% |  |  | 101% |  |  |

|  | Low delay |  |  | Low delay LoCo |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A |  |  |  |  |  |  |
| Class B | -0.3 | 0.1 | -0.2 | 1.3 | 0.5 | 0.7 |
| Class C | -0.1 | 0.5 | 0.2 | 0.6 | 0.4 | 0.3 |
| Class D | -0.3 | 0.2 | 0.1 | 0.3 | 0.3 | -0.3 |
| Class E | -1.6 | -1.1 | -0.2 | 0.5 | -1.0 | 0.0 |
| All | -0.5 | 0.0 | 0.0 | 0.7 | 0.2 | 0.2 |
| Enc Time[%] | 99% |  |  | 100% |  |  |
| Dec Time[%] | 100% |  |  | 99% |  |  |

The results in Table 1 show that the Skip mode can be replace with the Merge-Skip mode leading to coding performance improvement in most test cases. Some loss in coding performance is observed in the Low-delay, Low complexity test cases. The results in Table 1 can be used for cross-check of CE9 Test B, since the Skip-Merge implementation in this contribution was developed independently of the Test B proponent.

Table 2. Coding Performance of Skip-Merge with reduced candidate set

|  |  | Random access |  |  | Random access LoCo |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | 0.3 | 0.1 | 0.1 | 0.0 | -0.2 | -0.2 |
| Class B | 0.0 | -0.1 | -0.2 | -0.8 | -0.9 | -0.8 |
| Class C | 0.0 | 0.2 | 0.1 | -0.1 | -0.1 | -0.1 |
| Class D | -0.1 | 0.1 | 0.2 | -0.2 | -0.1 | -0.1 |
| Class E |  |  |  |  |  |  |
| All | 0.0 | 0.1 | 0.0 | -0.3 | -0.3 | -0.3 |
| Enc Time[%] | 99% |  |  | 99% |  |  |
| Dec Time[%] | 100% |  |  | 99% |  |  |

|  | Low delay |  |  | Low delay LoCo |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A |  |  |  |  |  |  |
| Class B | -0.2 | 0.2 | 0.2 | 1.1 | 0.2 | 0.2 |
| Class C | -0.1 | 0.6 | 0.3 | 0.7 | 0.5 | 0.5 |
| Class D | -0.3 | 0.2 | -0.5 | 0.2 | -0.2 | -0.5 |
| Class E | -1.6 | -0.3 | 0.0 | 0.4 | -1.0 | -0.5 |
| All | -0.5 | 0.2 | 0.0 | 0.7 | 0.0 | 0.0 |
| Enc Time[%] | 99% |  |  | 99% |  |  |
| Dec Time[%] | 99% |  |  | 96% |  |  |

The results in Table 2 show that the candidate set of the Skip-Merge mode can be reduced to just the ‘Top’ and ‘Left’ candidates with little impact to the coding performance. Though the reduced candidate set should reduce the number of rate-distortion checks, the reduced complexity did not lead to noticeable encoding time decrease.

# Conclusions

The experiments in this contribution show that with a redesign of how Merge modes are coded, the Skip mode in the current HM2 design can be effectively replaced by a Skip-Merge mode. The candidate set of the Skip-Merge mode can also be reduced without hurting coding performance. We recommend adopting the Skip-Merge mode with reduced candidate set in place of the current Skip mode.

# References

[1] J. Jung et al., “Description of Core Experiment 9: MV Coding and Skip/Merge operations,” in JCTVC-D609, Daegu, Korea, Jan 2011.

[2] Y. H. Tan, C. Yeo, Z. Li, “Merge/Skip/Direct Simplification,” in JCTVC-D051, Daegu, Korea, Jan 2011.

[3] F. Bossen, “Common test conditions and software reference configurations,” in JCTVC-D600, Daegu, Korea, Jan. 2011.

[4] G. Bjøntegaard, “Calculation of Average PSNR Differences between RD curves,” in ITU-T SC16/Q6, VCEG-M33, Austin, USA, Jan. 2004.

# Patent rights declaration(s)

Institute for Infocomm Research may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).