| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 5th Meeting: Geneva, CH, 16-23 March, 2011 | Document: JCTVC-E103 |
| --- | --- |

| Title: | CE9: Cross-check of Test D from HHI |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Yih Han Tan Chuohao Yeo 1 Fusionopolis Way #21-01 Connexis (South Tower) Singapore 138632 | Tel: Email: | +65 6408 2000 yhtan@i2r.a-star.edu.sg |
| Source: | Institute for Infocomm Research |  |  |

_____________________________

# Abstract

One of the major goals of CE9 is to evaluate possible modifications to the motion information derivation process during inter coding. In this test, an alternative method of deriving the set of motion vector predictor candidates for motion vector prediction is evaluated.

# Introduction

In HM2, the motion vector coding scheme used is Advanced Motion Vector Prediction (AMVP), where motion vectors can be predicted by any of 3 possible motion vector predictors (MVP): “Left”, “Above” and “Co-located” (JCTVC-D231).

In this test, the Merge/Skip mechanism of a CU is first modified such that only the motion vector derivation process of the Merge modes is used. A 2Nx2N Skip-Merge mode is signaled by the flag that was previously used to signal the Skip mode. The prediction mechanism of the Skip-Merge mode is the same as a 2Nx2N PU-Merge mode except that no residual coefficients are coded.

Consider the Prediction Unit (PU) shown in Figure 1. In the current design of HM2, given a particular reference index (ref_idx) for the motion vector, the MVPs are derived in the following manner. All the 4x4 partitions along the left edge of the PU are first searched, i.e., “Lx” in Figure 1, from the lower left to the upper left. Each partition is checked to see if it is Inter coded and has the same ref_idx as the current PU. If so, the search stops, and the motion vector of that partition is used as the “Left” MVP. If there are no suitable partitions, then there is no “Left” MVP. Next, all the 4x4 partitions along the top edge of the PU are searched, i.e., “Ax” in Figure 1, from the upper right to the upper left. Again, each partition is checked to see if it is Inter coded and has the same ref_idx has the current PU. In addition, it is also checked to see if it is different from the “Left” MVP (if that exists). If all the conditions are matched, the search stops, and the motion vector of that partition is used as the “Above” MVP. If there are no suitable partitions, then there is no “Above” MVP.

Figure 1. Illustration of AMVP candidate derivation in HM2 (HEVC WD2)

When HHI_MRG_CAND_FOR_AMVP is set to 1 in this test, DCM_SIMPLIFIED_MVP is set to 0, disabling the motion vector predictor derivation method (D231) adopted at the last meeting. The motion vector predictor candidate derivation process for Inter coding differs from that in HM2 in the following ways:

- The “Above” MVP is A8.

- The “Left” MVP is L8.

- Up to two “Corner” MVPs can be added (A0, L0). As a result, up to 2 more candidates can be added compared to the motion vector derivation scheme in HM2.

# Simulation Results

The test conditions as specified in CE9 [1] are followed.

| Table 1. Coding performance for Test B |  | Random access |  |  | Random access LoCo |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | 0.2 | 0.0 | 0.1 | 0.0 | -0.1 | -0.1 |
| Class B | 0.0 | -0.2 | -0.2 | -0.6 | -0.4 | -0.4 |
| Class C | 0.0 | 0.0 | 0.1 | -0.2 | -0.2 | 0.0 |
| Class D | -0.1 | 0.0 | 0.0 | -0.3 | -0.2 | -0.1 |
| Class E |  |  |  |  |  |  |
| All | 0.0 | -0.1 | 0.0 | -0.3 | -0.2 | -0.2 |
| Enc Time[%] | 99% |  |  | 98% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |

|  | Low delay |  |  | Low delay LoCo |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A |  |  |  |  |  |  |
| Class B | -0.3 | 0.0 | -0.2 | 1.4 | 0.8 | 0.8 |
| Class C | -0.1 | 0.4 | 0.2 | 0.7 | 0.8 | 0.8 |
| Class D | -0.4 | 0.4 | -0.4 | 0.3 | 0.4 | 0.3 |
| Class E | -2.0 | -1.2 | -0.7 | 0.4 | -0.5 | -0.2 |
| All | -0.6 | 0.0 | -0.2 | 0.8 | 0.5 | 0.5 |
| Enc Time[%] | 98% |  |  | 98% |  |  |
| Dec Time[%] | 100% |  |  | 98% |  |  |

# Conclusions

We have conducted Test D of CE9 and verified the results provided by HHI.

# References

[1] J. Jung et al., “Description of Core Experiment 9: MV Coding and Skip/Merge operations,” in JCTVC-D609, Daegu, Korea, Jan 2011.

# Patent rights declaration(s)

Institute for Infocomm Research may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).