| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 5th Meeting: Geneva, CH, 16-23 March, 2011 | Document: JCTVC-E104 |
| --- | --- |

| Title: | RQT depth selection |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yih Han Tan Chuohao Yeo Hui Li Tan Zhengguo Li 1 Fusionopolis Way #21-01 Connexis (South Tower) Singapore 138632 | Tel: Email: | +65 6408 2000 yhtan@i2r.a-star.edu.sg |
| Source: | Institute for Infocomm Research |  |  |

_____________________________

# Abstract

In this proposal, a different configuration of the transform quad-tree depth is evaluated. In the default configuration of HM 2.0, the maximum depth of the transform quad-tree is different for different efficiency and complexity settings. This contribution reports experimental results on setting the maximum level of the transform quad-tree level to two for all experimental settings. It also presents some modifications to the intra encoding mode decision process that results in a complexity reduction while maintaining coding performance.

# Introduction

In HM 2.0, the definition of transform units (TU), nested within coding units (CU), allows a TU’s size to vary independent of the size of CU in which it resides. During the encoding of an N x N CU, the encoder can signal the use of a transform that is of the same size as the CU. Alternatively, the CU can be coded with 4 transforms, each of size N/2 x N/2. It is possible for this transform quad-tree to grow further; the maximum depth of a transform tree is dependent on a sequence level residual quad-tree depth (RQT-Depth) signal that can be controlled by the encoder. The default RQT configuration for each experimental setting is shown in Table 1.

Table 1. RQT Configuration in HM 2.0

| Settings | RQT-Depth (Intra/Inter) |
| --- | --- |
| Intra | (3/-) |
| Intra (lo-co) | (1/-) |
| Low delay | (3/3) |
| Low delay (lo-co) | (1/2) |
| Random access | (3/3) |
| Random access (lo-co) | (1/2) |

# Proposed Approach

For the experiments in this contribution, RQT Depth was set to two for all complexity and efficiency settings. On top of this change, the rate-distortion (R-D) decision process during Intra encoding is also modified.

During Intra encoding in HM 2.0, full R-D checks are done only on a subset of all defined Intra prediction modes. While the most probable mode (MPM) is always checked to determine its R-D performance, low cost Hadamard checks are used to determine the rest of the subset that will undergo full R-D checks. Hadamard costs are computed for all possible modes when coding a PU. Based on the Hadamard costs, a (PU-size dependent) number of top performing modes are then chosen for full R-D checks. The top performing mode (R-D wise) is then selected for coding the PU.

Currently, the RQT check (to find the optimal transform tree) is done only after the Intra direction mode decision is done and the check is done only on the intra direction mode that resulted in the lowest RD cost. That is, the Intra mode is decided and finalized before the RQT check.

In the proposed approach, the Intra encoding process is modified in the following ways:

- RQT checking is always done for all Intra-prediction modes that undergo full R-D checks (HHI_RQT_INTRA_SPEEDUP 0).

- The number of full R-D checks for Intra prediction mode decision is reduced (to 3 best-performing modes based on Hadamard costs + MPM).

- The Hadamard cost is computed without considering the bits required to code the Intra prediction modes.

# Experimental Results

In the experiments, Intra, Low-delay and Random access configuration are used, as defined in the common test conditions [1]. The coding performances of HM 2.0 with and without the proposed modifications are compared. Coding performance is measured using BD-Rate with 4 operating points [2]. Table 2 shows the coding performance of the proposed modifications.

Table 2. Coding Performance of proposed modifications

|  | Intra |  |  | Intra LoCo |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | 0.02 | -0.05 | 0.01 | -0.24 | 0.99 | 1.00 |
| Class B | 0.07 | -0.13 | -0.10 | -0.49 | 0.23 | 0.32 |
| Class C | 0.06 | -0.33 | -0.29 | -0.32 | -0.35 | -0.25 |
| Class D | 0.14 | -0.41 | -0.36 | -0.10 | -0.31 | -0.45 |
| Class E | 0.01 | -0.01 | -0.04 | -0.75 | 2.03 | 1.53 |
| All | 0.06 | -0.19 | -0.16 | -0.37 | 0.43 | 0.37 |
| Enc Time[%] | 87% |  |  | 94% |  |  |
| Dec Time[%] | 100% |  |  | 101% |  |  |

|  | Random access |  |  | Random access LoCo |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | 0.21 | 0.43 | -0.33 | -0.27 | 1.55 | 1.44 |
| Class B | 0.29 | 0.17 | 0.04 | -0.26 | 0.77 | 0.57 |
| Class C | 0.18 | -0.07 | 0.07 | -0.20 | -0.21 | 0.14 |
| Class D | 0.30 | -0.23 | -0.01 | -0.05 | 0.07 | -0.18 |
| Class E |  |  |  |  |  |  |
| All | 0.25 | 0.08 | -0.05 | -0.20 | 0.56 | 0.50 |
| Enc Time[%] | 91% |  |  | 100% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |

|  | Low delay |  |  | Low delay LoCo |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A |  |  |  |  |  |  |
| Class B | 0.32 | 0.90 | 0.89 | -0.20 | 0.33 | 0.46 |
| Class C | 0.20 | 0.44 | 0.37 | -0.21 | 0.06 | 0.05 |
| Class D | 0.21 | 0.46 | 0.03 | -0.03 | 0.40 | 0.04 |
| Class E | 0.00 | -0.25 | 1.32 | -0.17 | 1.41 | 0.78 |
| All | 0.20 | 0.46 | 0.63 | -0.15 | 0.48 | 0.31 |
| Enc Time[%] | 92% |  |  | 100% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |

For the Intra test cases, the proposed modification decreases the encoding complexity in the high efficiency setting while maintaining coding performance. In the low complexity setting, the coding performance is also improved.

For the Low delay and Random access test cases, encoder complexity decrease comes at a cost of some coding performance drop in the high efficiency setting. In the low complexity setting, some coding performance improvement is observed with no change in encoder complexity.

# Conclusions

Experimental results in this contribution showed that setting RQT depth to two and modifying the encoding process can help achieve improved complexity-coding performance trade-off during Intra coding. Most of the gains of RQT can also be achieved by setting RQT depth to two for Inter coding. We recommend using the default RQT depth of 2 for all test settings and also adopting the proposed modifications to the Intra R-D decision process.

# References

[1] F. Bossen, “Common test conditions and software reference configurations,” in JCTVC-D600, Daegu, Korea, Jan. 2011.

[2] G. Bjøntegaard, “Calculation of Average PSNR Differences between RD curves,” in ITU-T SC16/Q6, VCEG-M33, Austin, USA, Jan. 2004.

# Patent rights declaration(s)

Institute for Infocomm Research may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).