| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 5th Meeting: Geneva, CH, 16-23 March, 2011 | Document: JCTVC-E105 WG11 Number: m19620 |
| --- | --- |

| Title: | CE14 Subtest 1: Cross-check report from Institute for Infocomm Research |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Hui Li Tan Yih Han Tan Chuohao Yeo 1 Fusionopolis Way #21-01 Connexis (South Tower) Singapore 138632 | Tel: Email: | +65 6408 2000 chyeo@i2r.a-star.edu.sg |
| Source: | Institute for Infocomm Research |  |  |

_____________________________

# Abstract

CE14 subtest 1 aims to investigate the coding performance and complexity when using two possible candidate prediction modes during intra mode coding. This report documents the results and observations from the experiments done by Institute for Infocomm Research for the cross-checks of three proposals (JCTVC-D166, JCTVC-D235, JCTVC-D253). The coding results obtained match what was provided by the proponents.

# Introduction

In HM2, the minimum of the modes of the prediction unit (PU) on the left of the current PU (Mode A) and that of the PU above the current PU (Mode B) is used as the most probable mode (MPM) during intra mode coding of the current block. In the proposals tested in CE14 subtest 1 [4], either Mode A or Mode B can be used as the MPM during the coding of the intra prediction mode (Mode C) of the current PU and the choice is signaled to the decoder.

Mode A and Mode B are stored in a candidate mode list candModeList[x], where x∈{0,1}. If a candidate prediction mode is not available, it is replaced with the DC prediction mode. If both candidate prediction modes are the same, then the candidate prediction mode will be added to candModeList[0], otherwise, in the implementation based on JCTVC-D166 and JCTVC-D235, the smaller candidate prediction mode will be stored at candModeList[0] while the larger candidate prediction mode will be stored at candModeList[1]. In the implementation based on JCTVC-D253, the above mode (Mode B) is always stored in candModeList[0] while the left mode (Mode A) is always stored in candModeList[1].

If MostProbableMode is decoded to be one, Mode C is one of the candidates in candModeList and MostProbableIndex∈{0,1} is then decoded to identify Mode C. MostProbableIndex is decoded only when more than one candidate exist in candModeList. If MostProbableMode is decoded to be zero, remMod has to be decoded to determine the intra prediction mode. In JCTVC-D166 and JCTVC-D235, remMod is derived by subtracting the candidate prediction mode(s) from the list of possible modes to be represented; while for JCTVC-D253, remMod is derived by only subtracting the minimum of the candidate prediction modes.

We observed that when LCEC is used, JCTVC-D166 and JCTVC-D235 share the same coding tables and is based on the LCEC intra mode coding adopted in HM2, while JCTVC-D253 uses a previous version of LCEC intra mode coding.

It is also noted that when Mode A and Mode B are different, JCTV-D166 and JCTV-D235 adds both candidate prediction modes to the set of prediction modes for which intra RD checks are carried out during intra mode encoding. JCTVC-D253 only adds min(Mode A, Mode B).

One additional observation is that for PUs of size 64x64, there are only three luma intra prediction modes. Hence, if Mode A and Mode B are different, it could potentially cost more to code a most probable mode than to code the remaining mode. Moreover, there is a redundant bin/bit signaled when the remaining mode is coded in this case.

# Simulation Results

The proposed modification were made in HM 2.0. Tests were conducted using the two Intra configurations, i.e. high-efficiency and low-complexity, following the common testing conditions [5].

Tables 1-3 show the results of the experiments. From the experimental results, it can be observed that the use of additional candidate prediction modes resulted in coding gains with no significant change in encoding and decoding complexity.

|  | Intra |  |  | Intra LoCo |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | -0.3 | -0.2 | -0.1 | -0.3 | -0.2 | -0.2 |
| Class B | -0.5 | -0.4 | -0.4 | -0.6 | -0.4 | -0.5 |
| Class C | -0.5 | -0.4 | -0.4 | -0.5 | -0.5 | -0.5 |
| Class D | -0.3 | -0.3 | -0.3 | -0.4 | -0.4 | -0.4 |
| Class E | -0.8 | -0.5 | -0.5 | -1.0 | -0.6 | -0.6 |
| All | -0.5 | -0.4 | -0.3 | -0.5 | -0.4 | -0.4 |
| Enc Time[%] | 102% |  |  | 103% |  |  |
| Dec Time[%] | 101% |  |  | 100% |  |  |

Table 1. Cross-check results for JCTVC-D166

|  | Intra |  |  | Intra LoCo |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | -0.3 | -0.2 | -0.1 | -0.3 | -0.2 | -0.2 |
| Class B | -0.5 | -0.4 | -0.4 | -0.6 | -0.4 | -0.5 |
| Class C | -0.5 | -0.4 | -0.4 | -0.5 | -0.5 | -0.5 |
| Class D | -0.3 | -0.3 | -0.3 | -0.4 | -0.4 | -0.4 |
| Class E | -0.8 | -0.5 | -0.5 | -1.0 | -0.6 | -0.6 |
| All | -0.5 | -0.4 | -0.3 | -0.5 | -0.4 | -0.4 |
| Enc Time[%] | 101% |  |  | 105% |  |  |
| Dec Time[%] | 100% |  |  | 101% |  |  |

Table 2. Cross-check results for JCTVC-D235

|  | Intra |  |  | Intra LoCo |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | -0.2 | -0.2 | -0.1 | 0.1 | 0.0 | 0.0 |
| Class B | -0.4 | -0.3 | -0.3 | -0.1 | -0.2 | -0.2 |
| Class C | -0.3 | -0.3 | -0.3 | -0.2 | -0.3 | -0.3 |
| Class D | -0.2 | -0.2 | -0.2 | -0.2 | -0.3 | -0.3 |
| Class E | -0.6 | -0.5 | -0.4 | -0.4 | -0.3 | -0.4 |
| All | -0.3 | -0.3 | -0.3 | -0.1 | -0.2 | -0.2 |
| Enc Time[%] | 100% |  |  | 102% |  |  |
| Dec Time[%] | 101% |  |  | 101% |  |  |

Table 3. Cross-check results for JCTVC-D253

# Conclusions

These CE14 cross-check experiments report results which are consistent with those provided by the proponents.  The proposed modification brings some coding gains with no significant increase in both encoding and decoding complexity.

# References

[1] M. Guo, X. Guo and S. Lei., “Improved Intra Mode Coding,” JCTVC-D166, Daegu, Korea, 20-28 January, 2011.

[2] S. Kanumuri, TK Tan and Frank Bossen, "Enhancements to Intra Coding,” JCTVC-D235, Daegu, Korea, 20-28 January, 2011.

[3] W. Ding, X. Che, Y. Shi, B. Yin, “Improved Intra Mode Coding by Multiple Mode Candidates,” JCTVC-D253, Daegu, Korea, 20-28 January, 2011.

[4] F. Bossen, “Common conditions and software reference configurations,” Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JCTVC-D600, 4th Meeting: Daegu, KR, Jan., 2011.

[5] S. Lei et al., “CE14: Intra Mode Coding,” JCTVC-D614, Daegu, Korea, 20-28 January, 2011.