| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 5th Meeting: Geneva, CH, 16-23 March, 2011 | Document: JCTVC-E107r2 |
| --- | --- |

| Title: | CE7.6: Simplified adaptive transform selection |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Atsuro Ichigaya Yasuko Sugito Shinichi Sakaida NHK Science & Technology Research Laboratories 1-10-11, Kinuta, Setagaya-ku, Tokyo, Japan | Tel: Email: | +81-3-5494-3354 ichigaya.a-go@nhk.or.jp sugitou.y-gy@nhk.or.jp sakaida.s-gq@nhk.or.jp |
| Source: | NHK (Japan Broadcasting Corporation) |  |  |

_____________________________

# Abstract

Current HM 2.0 adopts 4x4 to 32x32 discrete cosine transform (DCT) as transformation for residue signals. When DCT encodes signals like sine wave shapes, the encoding performance are decreased. This is because that DCT bases are composed of cosine functions. On the other hand, discrete sine transform (DST) encodes such signals effectively because the DST bases are composed of sine function. DST is complementally related to DCT regarding phase information. In short, the combination of both transforms is minimum set for covering variety signals.

Both DCT and DST can be designed and performed fast by using Chen’s butterfly computation. However, encoder requires twice processing for two transforms. This contribution describes simplified calculation process for these transforms by using the 1-D DCT-DST shared butterfly, 1D-DCT butterfly and additional inverters. The proposed method shares a part of butterfly each other and reduces total number of operations. It was implemented to HM2.0 and evaluated the encoding performance. This proposal used RD-Optimization for selecting either DCT or DST and added transform_type_flag for the signalling. The coding results reportedly show that the BD-rate reduction are 0.6% on average for random access HE and low delay HE conditions. The encoding complexity increasing of the condition is 126% and 123%, while there’s no complexity increase in decoding process. This proposal work well for inter slices rather than intra slices, because inter residue does not have the image signal specific characteristics unlike intra residue. Though the software complexity is large, this proposal requires small additional hardware resources by sharing a part of conventional core transform processes.

# Introduction

Alternative transforms are discussed in Core Experiment 7, which was established as JCTVC-C507 and D607 [1, 2]. We proposed adaptive transform (DCT/DST) selection in JCT-VC D182 [3]. This proposal adopts DST type II as an additional transform because this type transform is suitable for hardware implementation. The DST has similar bases as DCT bases. These bases are symmetry shapes and the phase is shifted 90 degrees each other. Though DCT work well for video coding, it is not easy for DCT to clearly separate amplitude-frequency and phase-frequency characteristics. Therefore, quantized DCT deteriorates phase information of signals. Especially, it is the most difficult for DCT to encode the sine wave shape signals. It is one of block distortion factors. For such problems, DST type II works well as an additional transform with DCT.

DCT and DST are defined by the following equations.

DCT

C(u)=2Nγ(u)x=0N−1f(x)cos(2x+1)uπ2N　　u=1, 2,…, N	(1)

DST

S(u)=2Nγ(u)x=0N−1g(x)sin(2x+1)uπ2N　　u=1, 2,…, N	(2)

S(N−u)=2Nγ(u)x=0N−1(−1)xg(x)cos(2x+1)uπ2N　　u=0, 1,…, N-1	(3)

In short, N-point DST can be designed as follows.

Figure 1  4-point DST by using 4-point DCT     Figure 2  8-point DST by using 8-point DCT

Though these figures are for 4- and 8-point DST modules, the 16- and 32-point DST ones are also composed by using 16- and 32 point DCT modules. It means encoder can shares a DCT module for additional DST and needs to add only N/2-plex inverters.

4- and 8-points inverse DST modules are also shown by using 4- and 8-points IDCT modules as bellow.

Figure 3  4-point IDST by using 4-point IDCT     Figure 4  8-point IDST by using 8-point IDCT

Similarly, 16- and 32-point inverse DST can be composed by 16- and 32-point IDCT modules. The inverse transform modules can be share the IDCT modules and decoders only add N/2 inverters.

# Proposed method

1. DCT and DST shared butterfly

Current HM software is implemented Chen’s butterfly for processing N-points DCT. DST-II is also described Chen’s butterfly as follows.

Figure 5  4-point DCT butterfly.          Figure 6  4-point DST butterfly

These butterflies have same calculation in the first step. Thus they can be combined as bellow. It is named “DCT- DST shared butterfly”.

Figure 7  4-point DCT-DST shared butterfly

This shared butterfly decreases 25% operations than the total number of operations of two butterflies. This proposal used this shared butterfly module for horizontal 1-D transform.

In the case of vertical transform, a DST consists of a DCT module and N/2-plex inverters as figure 1 and 2. It means the additional hardware resources are only N/2-plex inverters.

Figure 8  4-point DST by sharing DCT butterfly

The 8-point DCT-DST shared butterfly can be described as follows. The 16- and 32-point butterflies can be designed similarly.

Figure 8 8-point DCT-DST shared butterfly

2. Quantization

DST can be described by using DCT with some inverters. It means that the Multiplication Factor (MF) and Post-scaling Factor (PF) of HM are also shared for DST quantization process. Therefore any other additional quantization matrices are not required for DST.

# Pseudo codes and complexity evaluation

i) Pseudo codes of transforms and inverse transforms

The pseudo codes of 4-point DCT butterfly and 4-point DCT-DST shared butterfly are as bellows.

// 4-point 1-D DCT in HM2.0

tmp1 = in[0] + in[3];

tmp2 = in[1] + in[2];

aai[0] = tmp1 + tmp2;

aai[2] = tmp1 - tmp2;

tmp1 = in[0] - in[3];

tmp2 = in[1] - in[2];

aai[1] = 2*tmp1 + tmp2;

aai[3] = tmp1 -2* tmp2;

// 4-point shared 1-D DCT-DST in proposal

tmp1 = in[0] + in[3];

tmp2 = in[1] + in[2];

aai[0] = tmp1 + tmp2;

aai[2] = tmp1 - tmp2;

bbi[3] = tmp1 + 2*tmp2;

bbi[1] = 2*tmp1 - tmp2;

tmp1 = in[0] - in[3];

tmp2 = in[1] - in[2];

aai[1] = 2*tmp1 + tmp2;

aai[3] = tmp1 -2* tmp2;

bbi[2] = tmp1 + tmp2;

bbi[0] = tmp1 - tmp2;

Comparing both pseudo codes for 4-point 1-D transformations, conventional transform requires 8 adds and 2 shifts (mults) and proposed transform requires only 4 additional adds and 2 additional shifts.

This proposal is applied to horizontal signals. The transforms, which is applied to vertical signals, is the same as conventional transform. However, when the transform is used as DST, the input data are changed the sign. The pseudo-code of proposed 4x4 DCT-DST function is described as bellow. xTn (n= 8, 16, 32) functions are described as same as xT4. The pseudo codes of n-th DCT-DST shared butterfly are in appendix.

// 4x4 DCT-DST in proposal

xT4(in)

{

//horizontal

xT4-1DShared(in,aai,bbi)

//vertical for DCT

xT4-1D(aai,out,DCT);

//vertical for DST

xT4-1D(bbi,out,DST);

}

//DCT-DST shared butterfly

xT4-1DShared(in, aai, bbi)

{

for(y=0;y<4;y++,in+=4)

{

tmp1 = in[0] + in[3];

tmp2 = in[1] + in[2];

aai[0][y] = tmp1 + tmp2;

aai[2][y] = tmp1 - tmp2;

bbi[1][y] = 2*tmp1 - tmp2;

bbi[3][y] = tmp1 + 2*tmp2;

tmp1 = in[0] - in[3];

tmp2 = in[1] - in[2];

aai[1][y] = 2*tmp1 + tmp2;

aai[3][y] = tmp1 -2* tmp2;

bbi[0][y] = tmp1 - tmp2;

bbi[2][y] = tmp1 + tmp2;

}

}

//Conventinal DCT butterfly

xT4-1D(cci, out, flag)

{

if(isDST(flag))

{

for(x=0;x<4;x++)

{

for(y=1;y<4;y++)

{

if((x+y)%2)

{

cci[x][y]-=cci[x][y];

}

}

}

}

for(x=0;x<4;x++, out++)

{

tmp1 = cci[x][0] + cci[x][3];

tmp2 = cci[x][1] + cci[x][2];

out[0] = tmp1 + tmp2;

out[8] = tmp1 - tmp2;

tmp1 = cci[x][0] - cci[x][3];

tmp2 = cci[x][1] - cci[x][2];

out[4]  = tmp1 * 2 + tmp2;

out[12] = tmp1 - tmp2 * 2;

}

}

This proposal shares IDCT module, xIT4, as IDST modules according to figure 3 and 4. The 4x4 inverse transform is described as follows. The 8x8, 16x16 and 32x32 inverse transform are described as same as 4x4.

// 4x4 IDCT in proposal

#define N 4

xIT ( coef, out, flag )

{

if(isDST(flag))

{

for(Int i = 0; i < N; i++)

{

for(Int j = 0; j < N; j++)

{

temp[N*N - 1 - (i*N + j)] = Coef[i*N + j];

}

}

//conventional IDCT

xIT4(temp, out);

for(Int i = 0; i < N; i++)

{

for(Int j = 0; j < N; j++)

{

if((i+j)%2)

{

out[i*N + j] = -out[i*N+j];

}

}

}

}

else

{

//conventional IDCT

xIT4(coeff, out);

}

}

According to this pseudo code, inverse transform does not require any other arithmetic operations. Additional processes are only a pre-process and a post-process of inverse transform. The pre-process is re-ordering coefficients and the post-process is changing the sign of alternative outputs. Therefore, this proposal does not such impact to decoder side complexity.

ii) The number of operations

- Coding results

i) Coding condition

The evaluation tests were performed under the common condition [4]. The proposed method is based on HM2.0 and implemented DST modules. In HM 2.0, the block size is defined as 4 x 4 to 32 x 32. Our DST is used for all block sizes. The test sequences were the same as current formal test sequences.

ii) Coding results

Table 1  Coding results of Intra HE and LC

Table 2  Coding results of Random access HE and LC

Table 3  Coding results of Low delay HE and LC

Table 4  Coding results of each sequences

- Conclusion

According to the coding results, this proposal achieves 0.4% bit-rate reduction on Intra HE condition and 0.6% BD-rate improved on Random access HE and Low delay HE conditions. The encoding complexity is a large on Intra only conditions. This large complexity is due to increase of RD-Optimization procedure. It is an encoding issue and it can be improved eventually. The main feature of the proposal is small decoding complexity. This proposal does not require modifying decoder design. In addition, other tools for improvement of transform method which are discussed in CE10 and CE11, e.g. complexity reducing, coefficients scanning and coding coefficients, are also applicable to our transform method. Our proposal can be improved by introducing the results of these CEs.

By adaptively using two transforms which have different energy compaction characteristics, higher coding efficiency is achieved over wide variety of images.

- References

| [1] | R. Cohen et al., “Description of Core Experiment 7: Alternative Transforms,” Doc. JCTVC-C507, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T VCEG and ISO/IEC MPEG, Guangzhou, CN, October 2010 |
| --- | --- |
| [2] | R. Cohen et al., “Description of Core Experiment 7: Alternative Transforms,” Doc. JCTVC-D507, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T VCEG and ISO/IEC MPEG, Daegu, KR, January 2011 |
| [3] | A. Ichigaya et al., “Performance report of adaptive DCT/DST selection.” Doc. JCTVC-D182, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T VCEG and ISO/IEC MPEG, Daegu, KR, January 2011 |
| [4] | F. Bossen, “Common Test conditions and software reference configurations,” Doc. JCTVC-D600, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T VCEG and ISO/IEC MPEG, Daegu, KR, January 2011 |

# Patent rights declaration(s)

NHK (Japan Broadcasting corporation) may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# Appendix

i) The pseudo code of 8-point DCT-DST shared butterfly

The pseudo codes of 8-point DCT butterfly and 8-point DCT-DST shared butterfly are as bellows.

//8-point DCT in HM2.0

ai1[0] = in[0] + in[7];

ai1[1] = in[1] + in[6];

ai1[2] = in[2] + in[5];

ai1[3] = in[3] + in[4];

ai1[4] = in[0] - in[7];

ai1[5] = in[1] - in[6];

ai1[6] = in[2] - in[5];

ai1[7] = in[3] - in[4];

ai2[0] = ai1[0] + ai1[3];

ai2[1] = ai1[1] + ai1[2];

ai2[2] = ai1[0] - ai1[3];

ai2[3] = ai1[1] - ai1[2];

ai2[4] = ai1[5] + ai1[6] + ((ai1[4]>>1) + ai1[4]);

ai2[5] = ai1[4] - ai1[7] - ((ai1[6]>>1) + ai1[6]);

ai2[6] = ai1[4] + ai1[7] - ((ai1[5]>>1) + ai1[5]);

ai2[7] = ai1[5] - ai1[6] + ((ai1[7]>>1) + ai1[7]);

aai[0] =  ai2[0]     +  ai2[1];

aai[2] =  ai2[2]     + (ai2[3]>>1);

aai[4] =  ai2[0]     -  ai2[1];

aai[6] = (ai2[2]>>1) -  ai2[3];

aai[1] =  ai2[4]     + (ai2[7]>>2);

aai[3] =  ai2[5]     + (ai2[6]>>2);

aai[5] =  ai2[6]     - (ai2[5]>>2);

aai[7] = (ai2[4]>>2) -  ai2[7];

// 8-point shared DCT-DST in proposal

ai1[0] = in[0] + in[7];

ai1[1] = in[1] + in[6];

ai1[2] = in[2] + in[5];

ai1[3] = in[3] + in[4];

ai1[4] = in[0] - in[7];

ai1[5] = in[1] - in[6];

ai1[6] = in[2] - in[5];

ai1[7] = in[3] - in[4];

ai2[0] = ai1[0] + ai1[3];

ai2[1] = ai1[1] + ai1[2];

ai2[2] = ai1[0] - ai1[3];

ai2[3] = ai1[1] - ai1[2];

bi2[0] =  ai1[4]-ai1[7];

bi2[1] = -ai1[5]+ai1[6];

bi2[2] =  ai1[4]+ai1[7];

bi2[3] = -ai1[5]-ai1[6];

ai2[4] = -bi2[3] + ((ai1[4]>>1) + ai1[4]);

ai2[5] =  bi2[0] - ((ai1[6]>>1) + ai1[6]);

ai2[6] =  bi2[2] - ((ai1[5]>>1) + ai1[5]);

ai2[7] = -bi2[1] + ((ai1[7]>>1) + ai1[7]);

bi2[4] = -ai2[3] + ((ai1[0]>>1) + ai1[0]);

bi2[5] =  ai2[0] - ((ai1[2]>>1) + ai1[2]);

bi2[6] =  ai2[2] + ((ai1[1]>>1) + ai1[1]);

bi2[7] = -ai2[1] - ((ai1[3]>>1) + ai1[3]);

aai[0] =  ai2[0]     +  ai2[1];

aai[2] =  ai2[2]     + (ai2[3]>>1);

aai[4] =  ai2[0]     -  ai2[1];

aai[6] = (ai2[2]>>1) -  ai2[3];

aai[1] =  ai2[4]     + (ai2[7]>>2);

aai[3] =  ai2[5]     + (ai2[6]>>2);

aai[5] =  ai2[6]     - (ai2[5]>>2);

aai[7] = (ai2[4]>>2) -  ai2[7];

bbi[0] =  bi2[0]     +  bi2[1];

bbi[2] =  bi2[2]     + (bi2[3]>>1);

bbi[4] =  bi2[0]     -  bi2[1];

bbi[6] = (bi2[2]>>1) -  bi2[3];

bbi[1] =  bi2[4]     + (bi2[7]>>2);

bbi[3] =  bi2[5]     + (bi2[6]>>2);

bbi[5] =  bi2[6]     - (bi2[5]>>2);

bbi[7] = (bi2[4]>>2) -  bi2[7];

ii) The pseudo code of 16-point DCT-DST shared butterfly

Pseudo codes of 16-point DCT butterfly and 16-point DCT-DST shared butterfly are as bellows.

//16-point DCT in HM2.0

B0 = pSrc[0] + pSrc[15];

B15 = pSrc[0] - pSrc[15];

B1 = pSrc[1] + pSrc[14];

B14 = pSrc[1] - pSrc[14];

B2 = pSrc[2] + pSrc[13];

B13 = pSrc[2] - pSrc[13];

B3 = pSrc[3] + pSrc[12];

B12 = pSrc[3] - pSrc[12];

B4 = pSrc[4] + pSrc[11];

B11 = pSrc[4] - pSrc[11];

B5 = pSrc[5] + pSrc[10];

B10 = pSrc[5] - pSrc[10];

B6 = pSrc[6] + pSrc[9];

B9 = pSrc[6] - pSrc[9];

B7 = pSrc[7] + pSrc[8];

B8 = pSrc[7] - pSrc[8];

C0 = B0 + B7;

C7 = B0 - B7;

C1 = B1 + B6;

C6 = B1 - B6;

C2 = B2 + B5;

C5 = B2 - B5;

C3 = B3 + B4;

C4 = B3 - B4;

C10 = Round( 45 * ( B13 - B10 ) );

C13 = Round( 45 * ( B13 + B10 ));

C11 = Round( 45 * ( B12 - B11 ));

C12 = Round( 45 * ( B12 + B11 ));

D0 = C0 + C3;

D3 = C0 - C3;

D8 = B8 + C11;

D11 = B8 - C11;

D12 = B15 - C12;

D15 = B15 + C12;

D1 = C1 + C2;

D2 = C1 - C2;

D9 = B9 + C10;

D10 = B9 - C10;

D13 = B14 - C13;

D14 = B14 + C13;

D5 = Round( 45 * ( C6 - C5 ));

D6 =Round( 45 * ( C6 + C5 ));

// 16-point shared DCT-DST in proposal

B0  = pSrc[0] + pSrc[15];

B15 = pSrc[0] - pSrc[15];

B1  = pSrc[1] + pSrc[14];

B14 = pSrc[1] - pSrc[14];

B2  = pSrc[2] + pSrc[13];

B13 = pSrc[2] - pSrc[13];

B3  = pSrc[3] + pSrc[12];

B12 = pSrc[3] - pSrc[12];

B4  = pSrc[4] + pSrc[11];

B11 = pSrc[4] - pSrc[11];

B5  = pSrc[5] + pSrc[10];

B10 = pSrc[5] - pSrc[10];

B6  = pSrc[6] + pSrc[9];

B9  = pSrc[6] - pSrc[9];

B7  = pSrc[7] + pSrc[8];

B8  = pSrc[7] - pSrc[8];

C0 = B0 + B7;

C7 = B0 - B7;

C1 = B1 + B6;

C6 = B1 - B6;

C2 = B2 + B5;

C5 = B2 - B5;

C3 = B3 + B4;

C4 = B3 - B4;

H0 = B15 - B8;

H7 = B15 + B8;

H1 =-B14 + B9;

H6 =-B14 - B9;

H2 = B13 - B10;

H5 = B13 + B10;

H3 =-B12 + B11;

H4 =-B12 - B11;

C10 = Round( 45 * (  H2 ));

C13 = Round( 45 * (  H5 ));

C11 = Round( 45 * ( -H3 ));

C12 = Round( 45 * ( -H4 ));

H10 = Round( 45 * (  C2 ));

H13 = Round( 45 * (  C5 ));

H11 = Round( 45 * ( -C3 ));

H12 = Round( 45 * ( -C4 ));

D0 = C0 + C3;

D3 = C0 - C3;

D8 = B8 + C11;

D11 = B8 - C11;

D12 = B15 - C12;

D15 = B15 + C12;

D1 = C1 + C2;

D2 = C1 - C2;

D9 = B9 + C10;

D10 = B9 - C10;

D13 = B14 - C13;

D14 = B14 + C13;

D5 = Round( 45 * ( C6 - C5 ));

D6 = Round( 45 * ( C6 + C5 ));

aaiTemp[0] = Round( 45 * ( D0 + D1 ));

aaiTemp[8] = Round( 45 * ( D0 - D1 ));

aaiTemp[4] = Round( 59 * D3 + 24 * D2);

aaiTemp[12] = Round( 24 * D3 - 59 * D2);

E4 = C4 + D5;

E5 = C4 - D5;

E6 = C7 - D6;

E7 = C7 + D6;

E9 = Round( 24 * D14 - 59 * D9 );

E10 = Round( -59 * D13 - 24 * D10);

E13 = Round( 24 * D13 - 59 * D10);

E14 = Round( 59 * D14 + 24 * D9);

aaiTemp[2] = Round( 12 * E4 + 62 * E7);

aaiTemp[10] = Round( 53 * E5 + 35 * E6);

aaiTemp[6] = Round( 53 * E6 - 35 * E5);

aaiTemp[14] = Round( 12 * E7 - 62 * E4);

F8 = D8 + E9;

F9 = D8 - E9;

F10 = D11 - E10;

F11 = D11 + E10;

F12 = D12 + E13;

F13 = D12 - E13;

F14 = D15 - E14;

F15 = D15 + E14;

aaiTemp[0] = Round( 45 * ( D0 + D1 ));

aaiTemp[8] = Round( 45 * ( D0 - D1 ));

aaiTemp[4] = Round( 59 * D3 + 24 * D2);

aaiTemp[12] = Round( 24 * D3 - 59 * D2);

I0  = H0 + H3;

I3  = H0 - H3;

I8  = -B7 + H11;

I11 = -B7 - H11;

I12 = B0 - H12;

I15 = B0 + H12;

I1  = H1 + H2;

I2  = H1 - H2;

I9  = B6 + H10;

I10 = B6 - H10;

I13 = -B1 - H13;

I14 = -B1 + H13;

I5  = Round( 45 * ( H6 - H5 ));

I6  = Round( 45 * ( H6 + H5 ));

bbiTemp[0] = Round( 45 * ( I0 + I1 ));

bbiTemp[8] = Round( 45 * ( I0 - I1 ));

bbiTemp[4] = Round( 59 * I3 + 24 * I2);

bbiTemp[12] = Round( 24 * I3 - 59 * I2);

E4 = C4 + D5;

E5 = C4 - D5;

E6 = C7 - D6;

E7 = C7 + D6;

E9 = Round( 24 * D14 - 59 * D9 );

E10 = Round(  -59 * D13 - 24 * D10 );

E13 = Round( 24 * D13 - 59 * D10 );

E14 = Round( 59 * D14 + 24 * D9 );

aaiTemp[2] = Round( 12 * E4 + 62 * E7 );

aaiTemp[10] = Round( 53 * E5 + 35 * E6 );

aaiTemp[6] = Round( 53 * E6 - 35 * E5);

aaiTemp[14] = Round( 12 * E7 - 62 * E4);

J4 = H4 + I5;

J5 = H4 - I5;

J6 = H7 - I6;

J7 = H7 + I6;

J9 = Round( 24 * I14 - 59 * I9);

J10 = Round(  -59 * I13 - 24 * I10);

J13 = Round( 24 * I13 - 59 * I10);

J14 = Round( 59 * I14 + 24 * I9);

bbiTemp[2] = Round( 12 * J4 + 62 * J7);

bbiTemp[10] = Round( 53 * J5 + 35 * J6);

bbiTemp[6] = Round( 53 * J6 - 35 * J5);

bbiTemp[14] = Round( 12 * J7 - 62 * J4);

F8  = D8 + E9;

F9  = D8 - E9;

F10 = D11 - E10;

F11 = D11 + E10;

F12 = D12 + E13;

F13 = D12 - E13;

F14 = D15 - E14;

F15 = D15 + E14;

aaiTemp[1] = Round( 6 * F8 + 63 * F15);

aaiTemp[9] = Round( 49 * F9 + 40 * F14);

aaiTemp[5] = Round( 30 * F10 + 56 * F13);

aaiTemp[13] = Round( 61 * F11 + 18 * F12);

aaiTemp[3] = Round( 61 * F12 - 18 * F11);

aaiTemp[11] = Round( 30 * F13 - 56 * F10);

aaiTemp[7] = Round( 49 * F14 - 40 * F9);

aaiTemp[15] = Round( 6 * F15 - 63 * F8 );

aaiTemp[1] = Round( 6 * F8 + 63 * F15);

aaiTemp[9] = Round( 49 * F9 + 40 * F14);

aaiTemp[5] = Round( 30 * F10 + 56 * F13);

aaiTemp[13] = Round( 61 * F11 + 18 * F12);

aaiTemp[3] = Round( 61 * F12 - 18 * F11);

aaiTemp[11] = Round( 30 * F13 - 56 * F10);

aaiTemp[7] = Round( 49 * F14 - 40 * F9);

aaiTemp[15] = Round( 6 * F15 - 63 * F8);

K8  = I8 + J9;

K9  = I8 - J9;

K10 = I11 - J10;

K11 = I11 + J10;

K12 = I12 + J13;

K13 = I12 - J13;

K14 = I15 - J14;

K15 = I15 + J14;

bbiTemp[1] = Round( 6 * K8 + 63 * K15);

bbiTemp[9] = Round( 49 * K9 + 40 * K14);

bbiTemp[5] = Round( 30 * K10 + 56 * K13);

bbiTemp[13] = Round( 61 * K11 + 18 * K12);

bbiTemp[3] = Round( 61 * K12 - 18 * K11);

bbiTemp[11] = Round( 30 * K13 - 56 * K10);

bbiTemp[7] = Round( 49 * K14 - 40 * K9);

bbiTemp[15] = Round( 6 * K15 - 63 * K8);

iv) The pseudo code of 32-point DCT-DST shared butterfly

Pseudo codes of 32-point DCT butterfly and 32-point DCT-DST shared butterfly are as bellows.

//32-point DCT in HM2.0

A0 = pSrc[0] + pSrc[31];

A31 = pSrc[0] - pSrc[31];

A1 = pSrc[1] + pSrc[30];

A30 = pSrc[1] - pSrc[30];

A2 = pSrc[2] + pSrc[29];

A29 = pSrc[2] - pSrc[29];

A3 = pSrc[3] + pSrc[28];

A28 = pSrc[3] - pSrc[28];

A4 = pSrc[4] + pSrc[27];

A27 = pSrc[4] - pSrc[27];

A5 = pSrc[5] + pSrc[26];

A26 = pSrc[5] - pSrc[26];

A6 = pSrc[6] + pSrc[25];

A25 = pSrc[6] - pSrc[25];

A7 = pSrc[7] + pSrc[24];

A24 = pSrc[7] - pSrc[24];

A8 = pSrc[8] + pSrc[23];

A23 = pSrc[8] - pSrc[23];

A9 = pSrc[9] + pSrc[22];

A22 = pSrc[9] - pSrc[22];

A10 = pSrc[10] + pSrc[21];

A21 = pSrc[10] - pSrc[21];

A11 = pSrc[11] + pSrc[20];

A20 = pSrc[11] - pSrc[20];

A12 = pSrc[12] + pSrc[19];

A19 = pSrc[12] - pSrc[19];

A13 = pSrc[13] + pSrc[18];

A18 = pSrc[13] - pSrc[18];

A14 = pSrc[14] + pSrc[17];

A17 = pSrc[14] - pSrc[17];

A15 = pSrc[15] + pSrc[16];

A16 = pSrc[15] - pSrc[16];

B0 = A0 + A15;

B15 = A0 - A15;

B1 = A1 + A14;

B14 = A1 - A14;

B2 = A2 + A13;

B13 = A2 - A13;

B3 = A3 + A12;

B12 = A3 - A12;

B4 = A4 + A11;

B11 = A4 - A11;

B5 = A5 + A10;

B10 = A5 - A10;

B6 = A6 + A9;

B9 = A6 - A9;

B7 = A7 + A8;

B8 = A7 - A8;

// 32-point shared DCT-DST in proposal

A0  = pSrc[0] + pSrc[31];

A31 = pSrc[0] - pSrc[31];

A1  = pSrc[1] + pSrc[30];

A30 = pSrc[1] - pSrc[30];

A2  = pSrc[2] + pSrc[29];

A29 = pSrc[2] - pSrc[29];

A3  = pSrc[3] + pSrc[28];

A28 = pSrc[3] - pSrc[28];

A4  = pSrc[4] + pSrc[27];

A27 = pSrc[4] - pSrc[27];

A5  = pSrc[5] + pSrc[26];

A26 = pSrc[5] - pSrc[26];

A6  = pSrc[6] + pSrc[25];

A25 = pSrc[6] - pSrc[25];

A7  = pSrc[7] + pSrc[24];

A24 = pSrc[7] - pSrc[24];

A8  = pSrc[8] + pSrc[23];

A23 = pSrc[8] - pSrc[23];

A9  = pSrc[9] + pSrc[22];

A22 = pSrc[9] - pSrc[22];

A10 = pSrc[10] + pSrc[21];

A21 = pSrc[10] - pSrc[21];

A11 = pSrc[11] + pSrc[20];

A20 = pSrc[11] - pSrc[20];

A12 = pSrc[12] + pSrc[19];

A19 = pSrc[12] - pSrc[19];

A13 = pSrc[13] + pSrc[18];

A18 = pSrc[13] - pSrc[18];

A14 = pSrc[14] + pSrc[17];

A17 = pSrc[14] - pSrc[17];

A15 = pSrc[15] + pSrc[16];

A16 = pSrc[15] - pSrc[16];

B0 = A0 + A15;

B15 = A0 - A15;

B1 = A1 + A14;

B14 = A1 - A14;

B2 = A2 + A13;

B13 = A2 - A13;

B3 = A3 + A12;

B12 = A3 - A12;

B4 = A4 + A11;

B11 = A4 - A11;

B5 = A5 + A10;

B10 = A5 - A10;

B6 = A6 + A9;

B9 = A6 - A9;

B7 = A7 + A8;

B8 = A7 - A8;

I0 = A31 - A16;

I15 = A31 + A16;

I1 =-A30 + A17;

I14 =-A30 - A17;

I2 = A29 - A18;

I13 = A29 + A18;

I3 =-A28 + A19;

I12 =-A28 - A19;

I4 = A27 - A20;

I11 = A27 + A20;

B20 = Round( 181 * ( A27 - A20 ) );

B27 = Round( 181 * ( A27 + A20 ) );

B21 = Round( 181 * ( A26 - A21 ) );

B26 = Round( 181 * ( A26 + A21 ) );

B22 = Round( 181 * ( A25 - A22 ));

B25 = Round( 181 * ( A25 + A22 ));

B23 = Round( 181 * ( A24 - A23 ));

B24 = Round( 181 * ( A24 + A23 ));;

C0 = B0 + B7;

C7 = B0 - B7;

C1 = B1 + B6;

C6 = B1 - B6;

C2 = B2 + B5;

C5 = B2 - B5;

C3 = B3 + B4;

C4 = B3 - B4;

C10 = Round( 181 * ( B13 - B10 ));

C13 = Round( 181 * ( B13 + B10 ));

C11 = Round( 181 * ( B12 - B11 ));

C12 = Round( 181 * ( B12 + B11 ));

C16 = A16 + B23;

C23 = A16 - B23;

C24 = A31 - B24;

C31 = A31 + B24;

C17 = A17 + B22;

C22 = A17 - B22;

C25 = A30 - B25;

C30 = A30 + B25;

C18 = A18 + B21;

C21 = A18 - B21;

C26 = A29 - B26;

C29 = A29 + B26;

C19 = A19 + B20;

C20 = A19 - B20;

C27 = A28 - B27;

C28 = A28 + B27;

I5 =-A26 + A21;

I10 =-A26 - A21;

I6 = A25 - A22;

I9 = A25 + A22;

I7 =-A24 + A23;

I8 =-A24 - A23;

B20 = Round( 181 * (  I4  ));

B27 = Round( 181 * (  I11 ));

B21 = Round( 181 * ( -I5  ));

B26 = Round( 181 * ( -I10 ));

B22 = Round( 181 * (  I6  ));

B25 = Round( 181 * (  I9  ));

B23 = Round( 181 * ( -I7  ));

B24 = Round( 181 * ( -I8  ));

I20 = Round( 181 * (  B4  ));

I27 = Round( 181 * (  B11 ));

I21 = Round( 181 * ( -B5  ));

I26 = Round( 181 * ( -B10 ));

I22 = Round( 181 * (  B6  ));

I25 = Round( 181 * (  B9  ));

I23 = Round( 181 * ( -B7  ));

I24 = Round( 181 * ( -B8  ));

C0 = B0 + B7;

C7 = B0 - B7;

C1 = B1 + B6;

C6 = B1 - B6;

C2 = B2 + B5;

C5 = B2 - B5;

C3 = B3 + B4;

C4 = B3 - B4;

C10 = Round( 181 * ( B13 - B10 ));

C13 = Round( 181 * ( B13 + B10 ));

C11 = Round( 181 * ( B12 - B11 ));

C12 = Round( 181 * ( B12 + B11 ));

C16 = A16 + B23;

C23 = A16 - B23;

C24 = A31 - B24;

C31 = A31 + B24;

C17 = A17 + B22;

C22 = A17 - B22;

C25 = A30 - B25;

C30 = A30 + B25;

C18 = A18 + B21;

C21 = A18 - B21;

C26 = A29 - B26;

C29 = A29 + B26;

C19 = A19 + B20;

C20 = A19 - B20;

C27 = A28 - B27;

C28 = A28 + B27;

J0 = I0 + I7;

J7 = I0 - I7;

J1 = I1 + I6;

J6 = I1 - I6;

J2 = I2 + I5;

J5 = I2 - I5;

J3 = I3 + I4;

J4 = I3 - I4;

D0 = C0 + C3;

D3 = C0 - C3;

D8 = B8 + C11;

D11 = B8 - C11;

D12 = B15 - C12;

D15 = B15 + C12;

D1 = C1 + C2;

D2 = C1 - C2;

D9 = B9 + C10;

D10 = B9 - C10;

D13 = B14 - C13;

D14 = B14 + C13;

D5 = Round( 181 * ( C6 - C5 ) );

D6 = Round( 181 * ( C6 + C5 ) );

D18 = Round( 97 * C29 - 236 * C18 );

D20 = Round(  - 236 * C27 - 97 * C20 );

D26 = Round(  - 236 * C21 + 97 * C26 );

D28 = Round( 97 * C19 + 236 * C28 );

D19 = Round( 97 * C28 - 236 * C19 );

D21 = Round(  - 236 * C26 - 97 * C21 );

D27 = Round(  - 236 * C20 + 97 * C27 );

D29 = Round( 97 * C18 + 236 * C29 );;

J10 = Round( 181 * ( I13 - I10 ));

J13 = Round( 181 * ( I13 + I10 ));

J11 = Round( 181 * ( I12 - I11 ));

J12 = Round( 181 * ( I12 + I11 ));

J16 =-A15 + I23;

J23 =-A15 - I23;

J24 = A0  - I24;

J31 = A0  + I24;

J17 = A14 + I22;

J22 = A14 - I22;

J25 =-A1  - I25;

J30 =-A1  + I25;

J18 =-A13 + I21;

J21 =-A13 - I21;

J26 = A2  - I26;

J29 = A2  + I26;

J19 = A12 + I20;

J20 = A12 - I20;

J27 =-A3  - I27;

J28 =-A3  + I27;

D0 = C0 + C3;

D3 = C0 - C3;

D8 = B8 + C11;

D11 = B8 - C11;

D12 = B15 - C12;

D15 = B15 + C12;

D1 = C1 + C2;

D2 = C1 - C2;

D9 = B9 + C10;

D10 = B9 - C10;

D13 = B14 - C13;

D14 = B14 + C13;

D5 = Round( 181 * ( C6 - C5 ));

D6 = Round( 181 * ( C6 + C5 ));

D18 = Round( 97 * C29 - 236 * C18);

D20 = Round(  - 236 * C27 - 97 * C20);

D26 = Round(  - 236 * C21 + 97 * C26);

D28 = Round( 97 * C19 + 236 * C28);

D19 = Round( 97 * C28 - 236 * C19);

D21 = Round(  - 236 * C26 - 97 * C21);

D27 = Round(  - 236 * C20 + 97 * C27);

D29 = Round( 97 * C18 + 236 * C29);;

K0 = J0 + J3;

K3 = J0 - J3;

K8 = B8 + J11;

K11 = I8 - J11;

K12 = I15 - J12;

K15 = I15 + J12;

K1 = J1 + J2;

K2 = J1 - J2;

K9 = I9 + J10;

K10 = I9 - J10;

K13 = I14 - J13;

K14 = I14 + J13;

K5 = Round( 181 * ( J6 - J5 ));

K6 = Round( 181 * ( J6 + J5 ));

K18 = Round( 97 * J29 - 236 * J18);

K20 = Round(  - 236 * J27 - 97 * J20);

K26 = Round(  - 236 * J21 + 97 * J26);

K28 = Round( 97 * J19 + 236 * J28);

K19 = Round( 97 * J28 - 236 * J19);

K21 = Round(  - 236 * J26 - 97 * J21);

K27 = Round(  - 236 * J20 + 97 * J27);

K29 = Round( 97 * J18 + 236 * J29);;

aaiTemp[0] = Round( 181 * ( D0 + D1 ) );

aaiTemp[16] = Round( 181 * ( D0 - D1 ) );

aaiTemp[8] = Round( 236 * D3 + 97 * D2 );

aaiTemp[24] = Round( 97 * D3 - 236 * D2 );

E4 = C4 + D5;

E5 = C4 - D5;

E6 = C7 - D6;

E7 = C7 + D6;

E9 = Round( 97 * D14 - 236 * D9 );

E10 = Round( - 236 * D13 - 97 * D10 );

E13 = Round( 97 * D13 - 236 * D10 );

E14 = Round( 236 * D14 + 97 * D9 );

E16 = C16 + D19;

E19 = C16 - D19;

E20 = C23 - D20;

E23 = C23 + D20;

E24 = C24 + D27;

E27 = C24 - D27;

E28 = C31 - D28;

E31 = C31 + D28;

E17 = C17 + D18;

E18 = C17 - D18;

E21 = C22 - D21;

E22 = C22 + D21;

E25 = C25 + D26;

E26 = C25 - D26;

E29 = C30 - D29;

E30 = C30 + D29;

aaiTemp[4] = Round( 49 * E4 + 251 * E7 );

aaiTemp[20] = Round( 212 * E5 + 142 * E6 );

aaiTemp[12] = Round( 212 * E6 - 142 * E5 );

aaiTemp[28] = Round( 49 * E7 - 251 * E4 );

aaiTemp[0] = Round( 181 * ( D0 + D1 ));

aaiTemp[16] = Round( 181 * ( D0 - D1 ));

aaiTemp[8] = Round( 236 * D3 + 97 * D2);

aaiTemp[24] = Round( 97 * D3 - 236 * D2);

bbiTemp[0] = Round( 181 * ( K0 + K1 ));

bbiTemp[16] = Round( 181 * ( K0 - K1 ));

bbiTemp[8] = Round( 236 * K3 + 97 * K2);

bbiTemp[24] = Round( 97 * K3 - 236 * K2);

E4 = C4 + D5;

E5 = C4 - D5;

E6 = C7 - D6;

E7 = C7 + D6;

E9 = Round( 97 * D14 - 236 * D9);

E10 = Round(  - 236 * D13 - 97 * D10);

E13 = Round( 97 * D13 - 236 * D10);

E14 = Round( 236 * D14 + 97 * D9);

E16 = C16 + D19;

E19 = C16 - D19;

E20 = C23 - D20;

E23 = C23 + D20;

E24 = C24 + D27;

E27 = C24 - D27;

E28 = C31 - D28;

E31 = C31 + D28;

E17 = C17 + D18;

E18 = C17 - D18;

E21 = C22 - D21;

E22 = C22 + D21;

E25 = C25 + D26;

E26 = C25 - D26;

E29 = C30 - D29;

E30 = C30 + D29;

L4 = J4 + J5;

L5 = J4 - J5;

L6 = J7 - J6;

L7 = J7 + J6;

L9 = Round( 97 * K14 - 236 * K9);

L10 = Round(  - 236 * K13 - 97 * K10);

L13 = Round( 97 * K13 - 236 * K10);

L14 = Round( 236 * K14 + 97 * K9);

L16 = J16 + K19;

L19 = J16 - K19;

L20 = J23 - K20;

L23 = J23 + K20;

L24 = J24 + K27;

L27 = J24 - K27;

L28 = J31 - K28;

L31 = J31 + K28;

L17 = J17 + K18;

L18 = J17 - K18;

L21 = J22 - K21;

L22 = J22 + K21;

L25 = J25 + K26;

L26 = J25 - K26;

L29 = J30 - K29;

L30 = J30 + K29;

aaiTemp[4] = Round( 49 * E4 + 251 * E7);

aaiTemp[20] = Round( 212 * E5 + 142 * E6);

aaiTemp[12] = Round( 212 * E6 - 142 * E5);

aaiTemp[28] = Round( 49 * E7 - 251 * E4);

F8 = D8 + E9;

F9 = D8 - E9;

F10 = D11 - E10;

F11 = D11 + E10;

F12 = D12 + E13;

F13 = D12 - E13;

F14 = D15 - E14;

F15 = D15 + E14;

F17 = Round( 49 * E30 - 251 * E17 );

F18 = Round(  - 251 * E29 - 49 * E18 );

F21 = Round( 212 * E26 - 142 * E21 );

F22 = Round(  - 142 * E25 - 212 * E22 );

F25 = Round( 212 * E25 - 142 * E22 );

F26 = Round( 142 * E26 + 212 * E21 );

F29 = Round( 49 * E29 - 251 * E18 );

F30 = Round( 251 * E30 + 49 * E17 );;

aaiTemp[2] = Round( 25 * F8 + 254 * F15 );

aaiTemp[18] = Round( 197 * F9 + 162 * F14 );

aaiTemp[10] = Round( 120 * F10 + 225 * F13 );

aaiTemp[26] = Round( 244 * F11 + 74 * F12 );

aaiTemp[6] = Round( 244 * F12 - 74 * F11 );

aaiTemp[22] = Round( 120 * F13 - 225 * F10 );

aaiTemp[14] = Round( 197 * F14 - 162 * F9 );

aaiTemp[30] = Round( 25 * F15 - 254 * F8 );

G16 = E16 + F17;

G17 = E16 - F17;

G18 = E19 - F18;

G19 = E19 + F18;

G20 = E20 + F21;

G21 = E20 - F21;

G22 = E23 - F22;

G23 = E23 + F22;

G24 = E24 + F25;

bbiTemp[4] = Round( 49 * L4 + 251 * L7);

bbiTemp[20] = Round( 212 * L5 + 142 * L6);

bbiTemp[12] = Round( 212 * L6 - 142 * L5);

bbiTemp[28] = Round( 49 * L7 - 251 * L4);

F8 = D8 + E9;

F9 = D8 - E9;

F10 = D11 - E10;

F11 = D11 + E10;

F12 = D12 + E13;

F13 = D12 - E13;

F14 = D15 - E14;

F15 = D15 + E14;

F17 = Round( 49 * E30 - 251 * E17);

F18 = Round(  - 251 * E29 - 49 * E18);

F21 = Round( 212 * E26 - 142 * E21);

F22 = Round(  - 142 * E25 - 212 * E22);

F25 = Round( 212 * E25 - 142 * E22);

F26 = Round( 142 * E26 + 212 * E21);

F29 = Round( 49 * E29 - 251 * E18);

F30 = Round( 251 * E30 + 49 * E17);;

M8 = K8 + L9;

M9 = K8 - L9;

M10 = K11 - L10;

M11 = K11 + L10;

M12 = K12 + L13;

M13 = K12 - L13;

M14 = K15 - L14;

M15 = K15 + L14;

M17 = Round( 49 * L30 - 251 * L17);

M18 = Round(  - 251 * L29 - 49 * L18);

M21 = Round( 212 * L26 - 142 * L21);

M22 = Round(  - 142 * L25 - 212 * L22);

M25 = Round( 212 * L25 - 142 * L22);

M26 = Round( 142 * L26 + 212 * L21);

M29 = Round( 49 * L29 - 251 * L18);

M30 = Round( 251 * L30 + 49 * L17);;

aaiTemp[2] = Round( 25 * F8 + 254 * F15);

aaiTemp[18] = Round( 197 * F9 + 162 * F14);

aaiTemp[10] = Round( 120 * F10 + 225 * F13);

aaiTemp[26] = Round( 244 * F11 + 74 * F12);

aaiTemp[6] = Round( 244 * F12 - 74 * F11);

aaiTemp[22] = Round( 120 * F13 - 225 * F10);

aaiTemp[14] = Round( 197 * F14 - 162 * F9);

aaiTemp[30] = Round( 25 * F15 - 254 * F8);

bbiTemp[2] = Round( 25 * M8 + 254 * M15);

bbiTemp[18] = Round( 197 * M9 + 162 * M14);

bbiTemp[10] = Round( 120 * M10 + 225 * M13);

bbiTemp[26] = Round( 244 * M11 + 74 * M12);

bbiTemp[6] = Round( 244 * M12 - 74 * M11);

bbiTemp[22] = Round( 120 * M13 - 225 * M10);

bbiTemp[14] = Round( 197 * M14 - 162 * M9);

bbiTemp[30] = Round( 25 * M15 - 254 * M8);

G16 = E16 + F17;

G17 = E16 - F17;

G18 = E19 - F18;

G19 = E19 + F18;

G20 = E20 + F21;

G21 = E20 - F21;

G22 = E23 - F22;

G23 = E23 + F22;

G24 = E24 + F25;

G25 = E24 - F25;

G26 = E27 - F26;

G27 = E27 + F26;

G28 = E28 + F29;

G29 = E28 - F29;

G30 = E31 - F30;

G31 = E31 + F30;

aaiTemp[1] = Round( 12 * G16 + 255 * G31 );

aaiTemp[17] = Round( 189 * G17 + 171 * G30 );

aaiTemp[9] = Round( 109 * G18 + 231 * G29 );

aaiTemp[25] = Round( 241 * G19 + 86 * G28 );

aaiTemp[5] = Round( 62 * G20 + 248 * G27 );

aaiTemp[21] = Round( 219 * G21 + 131 * G26 );

aaiTemp[13] = Round( 152 * G22 + 205 * G25 );

aaiTemp[29] = Round( 253 * G23 + 37 * G24 );

aaiTemp[3] = Round( 253 * G24 - 37 * G23 );

aaiTemp[19] = Round( 152 * G25 - 205 * G22 );

aaiTemp[11] = Round( 219 * G26 - 131 * G21 );

aaiTemp[27] = Round( 62 * G27 - 248 * G20 );

aaiTemp[7] = Round( 241 * G28 - 86 * G19 );

aaiTemp[23] = Round( 109 * G29 - 231 * G18 );

aaiTemp[15] = Round( 189 * G30 - 171 * G17 );

aaiTemp[31] = Round( 12 * G31 - 255 * G16 );

G25 = E24 - F25;

G26 = E27 - F26;

G27 = E27 + F26;

G28 = E28 + F29;

G29 = E28 - F29;

G30 = E31 - F30;

G31 = E31 + F30;

N16 = L16 + M17;

N17 = L16 - M17;

N18 = L19 - M18;

N19 = L19 + M18;

N20 = L20 + M21;

N21 = L20 - M21;

N22 = L23 - M22;

N23 = L23 + M22;

N24 = L24 + M25;

N25 = L24 - M25;

N26 = L27 - M26;

N27 = L27 + M26;

N28 = L28 + M29;

N29 = L28 - M29;

N30 = L31 - M30;

N31 = L31 + M30;

aaiTemp[1] = Round( 12 * G16 + 255 * G31);

aaiTemp[17] = Round( 189 * G17 + 171 * G30);

aaiTemp[9] = Round( 109 * G18 + 231 * G29);

aaiTemp[25] = Round( 241 * G19 + 86 * G28);

aaiTemp[5] = Round( 62 * G20 + 248 * G27);

aaiTemp[21] = Round( 219 * G21 + 131 * G26);

aaiTemp[13] = Round( 152 * G22 + 205 * G25);

aaiTemp[29] = Round( 253 * G23 + 37 * G24);

aaiTemp[3] = Round( 253 * G24 - 37 * G23);

aaiTemp[19] = Round( 152 * G25 - 205 * G22);

aaiTemp[11] = Round( 219 * G26 - 131 * G21);

aaiTemp[27] = Round( 62 * G27 - 248 * G20);

aaiTemp[7] = Round( 241 * G28 - 86 * G19);

aaiTemp[23] = Round( 109 * G29 - 231 * G18);

aaiTemp[15] = Round( 189 * G30 - 171 * G17);

aaiTemp[31] = Round( 12 * G31 - 255 * G16);

bbiTemp[1] = Round( 12 * N16 + 255 * N31);

bbiTemp[17] = Round( 189 * N17 + 171 * N30);

bbiTemp[9] = Round( 109 * N18 + 231 * N29);

bbiTemp[25] = Round( 241 * N19 + 86 * N28);

bbiTemp[5] = Round( 62 * N20 + 248 * N27);

bbiTemp[21] = Round( 219 * N21 + 131 * N26);

bbiTemp[13] = Round( 152 * N22 + 205 * N25);

bbiTemp[29] = Round( 253 * N23 + 37 * N24);

bbiTemp[3] = Round( 253 * N24 - 37 * N23);

bbiTemp[19] = Round( 152 * N25 - 205 * N22);

bbiTemp[11] = Round( 219 * N26 - 131 * N21);

bbiTemp[27] = Round( 62 * N27 - 248 * N20);

bbiTemp[7] = Round( 241 * N28 - 86 * N19);

bbiTemp[23] = Round( 109 * N29 - 231 * N18);

bbiTemp[15] = Round( 189 * N30 - 171 * N17);

bbiTemp[31] = Round( 12 * N31 - 255 * N16);