| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 5th Meeting: Geneva, CH, 16-23 March, 2011 | Document: JCTVC-E109 WG11 Number: m16924 |
| --- | --- |

| Title: | CE8 Subtest 3: Cross-check of MediaTek, Qualcomm, Toshiba’s proposal (JCTVC-E047) on adaptation between ALF and AO |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Wang (PoLin) Lai Felix C. A. Fernandes | Tel: Email: | wlai@sta.samsung.com +1 972 761 7456 ffernandes@sta.samsung.com +1 972 761 7427 |
| Source: | Samsung Electronics Co., Ltd. |  |  |

_____________________________

# Abstract

This report presents Samsung’s cross-check results for MQT’s proposal to CE8 Subtest 3, on adaptation between ALF and AO (JCTVC-E047). RD performance match exactly, and decoding time reduction is confirmed, which can attributed to the fact that now some slice can select AO instead of ALF. Analysis of code changes, such as filter adaptation and AO usage on non-referenced B frames, are also presented.

# Cross-check results

Summary of the results are shown in the table below and more detailed results can be found in the accompanying Excel sheet. The simulations were run in the 64-bit Linux cluster environment.

|  | CE8.3 results by MQT ( JCTVC-E047[1] ) |  |  | Cross-check results by Samsung |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | IO | RA | LD | IO | RA | LD |
| BD-rate | -0.2 | -0.7 | -1.0 | -0.2 | -0.7 | -1.0 |
| Enc. Time | 100% | 100% | 100% | 90% | 100% | 102% |
| Dec. time | 89% | 89% | 90% | 87% | 89% | 92% |
| Vert. taps | 7 |  |  | 7 |  |  |

# Findings and recommendations

- A unified pixel classification method is developed and used for ALF and AO, instead of the pixel classification method in HM 2.0.

In this unified method, for each pixel, the window to compute sum-modified Laplacian (SML) is changed from 7×7 in HM2.0 to 1×1.

- The reduction in decoding time is most likely the results of slices using AO instead of ALF.

# References

[1]	“CE8 Subtest 3: Adaptation between ALF and AO,” JCTVC-E047, Geneva, March 2011.

# Patent rights declaration(s)

Samsung may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).