| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 5th Meeting: Geneva, CH, 16-23 March, 2011 | Document: JCTVC-E111 WG11 Number: m19626 |
| --- | --- |

| Title: | CE8 Subtest 2: Cross-check of Qualcomm's proposal (JCTVC-E323) on block-based filter adaptation and directional features |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Wang (PoLin) Lai Felix C. A. Fernandes | Tel: Email: | wlai@sta.samsung.com +1 972 761 7456 ffernandes@sta.samsung.com +1 972 761 7427 |
| Source: | Samsung Electronics Co., Ltd. |  |  |

_____________________________

# Abstract

This report presents cross-check results for Qualcomm’s proposal JCTVC-E323 to CE8 Subtest 2, on block-based filter adaptation (BA) and directional features (DIR). Coding results match exactly to the proponent’s data. Run-time also follows closely to the results reported by the proponent: Increases in encoding time for BA+DIR of less than 5%; with decreases in decoding time for both BA and BA+DIR, partially due to BA computation rather than PA in HM2.0. Findings of code-study are also presented.

# Cross-check results

Summary of the results are shown in the table below and more detailed results can be found in the accompanying Excel sheet. The simulations were run in the 64-bit Linux cluster environment.

4x4 Block-based filter adaptation (BA)

|  | CE8.2 by Qualcomm ( JCTVC-E323[1] ) |  |  | Cross-check results by Samsung |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | IO | RA | LD | IO | RA | LD |
| Class A | 0.0 | 0.2 |  | 0.0 | 0.2 |  |
| Class B | 0.0 | 0.1 | 0.2 | 0.0 | 0.1 | 0.2 |
| Class C | 0.2 | 0.1 | 0.1 | 0.2 | 0.1 | 0.1 |
| Class D | 0.1 | 0.1 | -0.1 | 0.1 | 0.1 | -0.1 |
| Class E | 0.0 |  | -0.2 | 0.0 |  | -0.2 |
| BD-rate | 0.1 | 0.1 | 0.0 | 0.1 | 0.1 | 0.0 |
| Enc. Time | 100% | 100% | 99% | 91% | 110% | 95% |
| Dec. time | 95% | 95% | 93% | 88% | 98% | 80% |

4x4 Block-based filter adaptation + Directional features (BA+DIR)

|  | CE8.2 by Qualcomm ( JCTVC-E323[1] ) |  |  | Cross-check results by Samsung |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | IO | RA | LD | IO | RA | LD |
| Class A | -0.2 | -0.5 |  | -0.2 | -0.5 |  |
| Class B | -0.2 | -0.7 | -0.4 | -0.2 | -0.7 | -0.4 |
| Class C | -0.3 | -0.4 | -0.4 | -0.3 | -0.4 | -0.4 |
| Class D | -0.1 | -0.2 | -0.2 | -0.1 | -0.2 | -0.2 |
| Class E | -0.2 |  | -0.5 | -0.2 |  | -0.5 |
| BD-rate | -0.2 | -0.5 | -0.4 | -0.2 | -0.5 | -0.4 |
| Enc. Time | 102% | 102% | 102% | 99% | 104% | 105% |
| Dec. time | 93% | 92% | 94% | 98% | 90% | 96% |

# Findings and recommendations

## Block-based filter adaptation (BA)

To perform block-based filter adaptation, the following changes are observed:

- For each pixel, the neighborhood to compute sum-modified Laplacian (SML) is changed from ±3 × ±3 in HM2.0 to ±1 × ±1. Note this change can also be applied to pixel-based classification.

- For each 4×4 block, the average of the 16 SML values from all 16 pixels is computed as the feature to classifying all pixels in the 4×4 block into the same class.

In conclusion, the saving in computation is using smaller neighborhood to compute SML, but not in using fewer SML values (still 16 SML values for 16 pixels).

Recommend further study on using fewer than 1 SML per pixel (e.g. 2,4, or 8 SML values for each 4×4 block.), or on isolating the effect of having smaller neighborhood to compute SML.

  - Directional feature for filter adaptation (DIR)

To further perform filter adaptation with directional features, the following changes are observed:

- Directional features (vertical and horizontal) are calculated using [-1, 2, -1] weights, which can be computed as intermediate results for SML.

- To accommodate both directional feature and SML feature, initially the classification might produce up to 3×16 = 48 filters (set by NO_VAR_BINS). Then when computing filter coefficients and determining number of filters, the mergeFiltersGreedy funtion will return up to 16 filters (set by NO_FILTERS).

In conclusion, the reported increases in encoding time may be attributed to the computation of initially up to 48 filters, and the merging process to produce the resulting up to 16 filters

Recommend further study especially comparison against the JCTVC-E141 [2], in which directional features are computed as sum of vertical and horizontal differences using [1 -1] weights in each 4×4 block, up to 18 filters.

# References

- “CE8 Subtest 2: Block based adaptive loop filter,” JCTVC-E323, Geneva, March 2011.

- “CE8 Subtest 2: Region-based adaptive loop filter using two-dimensional feature,” JCTVC-E323, Geneva, March 2011.

# Patent rights declaration(s)

Samsung may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).