| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 5th Meeting: Geneva, CH, 16-23 March, 2011 | Document: JCTVC-E113 |
| --- | --- |

| Title: | Using MPS to encode extended UDI mode numbers for robustness |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Guichun Li (Santa Clara University), Lingzhi Liu (Hisilicon Technologies Co. Ltd.) , Yongbing Lin (Hisilicon Technologies Co. Ltd.), Nam Ling (Santa Clara University), Jianhua Zheng (Hisilicon Technologies Co. Ltd.), Philipp Zhang (Hisilicon Technologies Co. Ltd.), | Tel: Email: | +1-408-554-4794 g1li@scu.edu lzliu@huawei.com linyongbing@huawei.com nling@scu.edu zhengjianhua@huawei.com pzhang@huawei.com |
| Source: | Santa Clara University & Hisilicon Technologies Co. Ltd. |  |  |

_____________________________

# Abstract

Current intra prediction for HEVC uses unified directional intra (UDI) prediction. Using more non-directional prediction modes can improve not only the coding gain but also the subjective quality. In this contribution, multiple prediction sets (MPS) method is used to signal intra prediction modes when there is more than 1 non-directional mode. The method can lower the storage requirement of the system and improve the error resilience of intra prediction; the coding performance also can be improved.

# Introduction

HEVC UDI prediction uses 34 modes, in which there are 33 directional modes and only 1 non-directional mode. It has been reported [2] [5] [6] that use additional non-directional mode can further improve the coding gain and the subjective quality.

MPS was introduced in JCTVC-A022E [1], JCTVC-C111 [2]. The basic idea of MPS is that the prediction modes are grouped into multiple predictor sets in which each of the predictor set has similar subset divisions. The group number and group index of the current block can be predicted from the predictor group number and group index of neighboring blocks independently.

CE6.e of the 4th JCTVC meeting tested several different implementations of intra planar mode. In subtest e 3.b, Mode 2 is used to signal both the mode DC and planar. When mode 2 is signaled, an additional flag is sent to indicate DC or Planar mode. However this method has two problems.

First, in some application, e.g. storage and video surveillance, a decoder is only required to check whether the bit stream is corresponding to the standard. Because current intra prediction mode is predicted from its neighboring blocks, in order to decide whether the bit of mode number need to be decoded or not, the system has to store at least all the decoded intra prediction modes of a whole block-line. This causes additional storage requirement for the decoding system.

Moreover, in H.264, the number of bits that to be decoded for an intra prediction mode is not related to the decoded value of the modes of its neighbor blocks. In current implementation, because whether or not decoding this bit is related to the current decoded intra prediction mode and current decoded mode is related to the mode number of its top and left neighboring blocks, this signaling method has some error resilience problems. Suppose the mode of upper mode is wrongly decoded, e.g. it is mode 2 but decoded as mode 3, and the mode of left block is 4 and if the current mode is either DC or planar, then the decoder will decide not to decode another bit for DC and planar flag and treat it as the next syntax. This will cause serious problem as decoder crash.

# Implementation

To avoid problems described above, and to support even more intra prediction modes, MPS method is used to code the intra prediction mode when there are two or more non-directional modes, because the bit length of MPS syntax is not decided by any content of previous blocks.

## MPS Intra with Planar Mode

If there are more than 34 intra predictors, the mode numbers are grouped into 4 sets, 9 in each. Each mode is represented by Group number and Index number. The basic group, group 0, contains the 9 basic prediction modes as in H.264/AVC. The MPS table is shown in Table-1. Mode 2 is DC mode and mode 34 is the Planar mode as proposed in D235.

|  | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 |
| 1 | 11 | 16 | 34 | 14 | 10 | 12 | 13 | 15 | 17 |
| 2 | 22 | 30 | 35 | 18 | 19 | 23 | 25 | 28 | 9 |
| 3 | 21 | 29 | 36 | 26 | 20 | 24 | 33 | 27 | 31 |
| 4 |  |  |  |  |  |  |  |  | 32 |

Table-1 UDI mode numbers in Multiple Predictor Set (for 34 + Extra modes)

For 4x4 PUs, only 17 + extra modes are needed. In order to reduce the overhead, in 4x4 case, the predictors are grouped into 4 sets, 5 in each. Each mode is also represented by Group number and Index number, as shown in Table-2.

|  | 0 | 1 | 2 | 3 | 4 |
| --- | --- | --- | --- | --- | --- |
| 0 | 0 | 1 | 2 | 3 | 4 |
| 1 | 5 | 16 | 34 | 7 | 12 |
| 2 | 11 | 8 | 35 | 14 | 10 |
| 3 | 13 | 9 | 36 | 15 | 6 |

Table-2 UDI modes in Multiple Predictor Sets (for 17 + Extra modes)

## Sub-block based Intra mode as a second additional mode

The MPS intra signaling method supports up to 2 additional intra modes on UDI. In order to test the performance of utilizing multiple non-directional modes, a 2nd additional mode, 2x2 sub-block based intra mode is used, as proposed in JCTVC-E289. In MPS mode table, the mode number for this predictor is 35.

# Simulation results

Since the low complexity configuration is using a different entropy coding scheme, the MPS intra signaling method is only applied for CABAC, thus Intra High Efficiency is tested to evaluate the performance.

Simulation results of Intra-HE are shown in Table-3 and Table-4.

|  | Intra HE |  |  |
| --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate |
| Class A | -0.6 | -0.2 | -0.1 |
| Class B | -0.7 | -0.3 | -0.4 |
| Class C | -0.4 | -0.1 | -0.1 |
| Class D | -0.3 | 0.0 | 0.0 |
| Class E | -0.8 | -0.5 | -0.1 |
| All | -0.6 | -0.2 | -0.2 |
| Enc Time[%] | 102% |  |  |
| Dec Time[%] | 100% |  |  |

Table-3 MPS Intra Extended UDI with Planar

|  | Intra HE |  |  |
| --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate |
| Class A | -1.0 | -0.4 | -0.2 |
| Class B | -1.1 | -0.5 | -0.6 |
| Class C | -0.6 | -0.3 | -0.1 |
| Class D | -0.5 | -0.1 | -0.1 |
| Class E | -1.1 | -0.6 | -0.1 |
| All | -0.9 | -0.4 | -0.2 |
| Enc Time[%] | 103% |  |  |
| Dec Time[%] | 130% |  |  |

Table-4 MPS Intra Extended UDI with Planar and 2x2 Sub-block-based Mode

|  | Intra HE |  |  |
| --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate |
| Class A | -0.7 | 0.1 | 0.3 |
| Class B | -0.8 | -0.3 | -0.3 |
| Class C | -0.5 | -0.1 | 0.0 |
| Class D | -0.5 | 0.0 | 0.1 |
| Class E | -0.9 | -0.2 | 0.2 |
| All | -0.7 | -0.1 | 0.0 |
| Enc Time[%] | 107% |  |  |
| Dec Time[%] | 100% |  |  |

Table-5 D235 Planar with D235’s signaling method

|  | Intra HE |  |  |
| --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate |
| Class A | -0.6 | 0.0 | 0.3 |
| Class B | -0.6 | -0.1 | -0.2 |
| Class C | -0.5 | 0.0 | 0.1 |
| Class D | -0.5 | 0.1 | 0.2 |
| Class E | -0.5 | -0.1 | 0.4 |
| All | -0.5 | 0.0 | 0.1 |
| Enc Time[%] | 104% |  |  |
| Dec Time[%] | 99% |  |  |

Table-6 2x2 Sub-block Based Mode with D235’s signaling method

# Conclusion

MPS Intra performs slightly lower than CE6.e 1.c + 3.b (JCTVC-D235 Planar Mode with PU level signaling sharing with DC) implementations in terms of BD-rate. However, it does not have the mode storage requirement and error resistance problem of current implementation.

Table-4 has shown that by utilizing more additional intra modes with MPS signaling method, the performance can be further improved. Since the D235 Planar mode and the E289 sub-block based mode have some overlap, the performance of the combination is not exact additive. Also MPS Intra has potential of supporting up to 3 non-directional modes without increasing the cost.

Therefore it is recommended to use MPS when there are 2 or more non-directional intra modes, or there’s any extra intra modes used in UDI.

# Reference

- Lingzhi Liu, “Multiple Predictor Sets for Intra Coding”, ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JCTVC-A022E, April 2010.

- Guichun Li, Lingzhi Liu, Nam Ling, Jianhua Zheng, and Philipp Zhang, “Integration of Plane Mode in Unified Intra Prediction”, ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JCTVC-C111, Guangzhou, China, Oct. 2010.

- Guichun Li, Lingzhi Liu, Nam Ling, Jianhua Zheng, and Philipp Zhang, “Integration of Plane Mode in Unified Intra Prediction”, ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, 4th Meeting: Daegu, S. Korea, JCTVC-D026, January, 2011.

- Ali Tabatabai, Keiichi Chono, Muhammed Coban, Kawamura Kei, Kazuo Sugimoto, Akiyuki Tanizawa, “Core Experiment 6: Intra Prediction Improvement”, ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, 4th Meeting: Daegu, S. Korea, JCTVC-D606, January, 2011.

- Sandeep Kanumuri, TK Tan, Frank Bossen, “Enhancements to Intra Coding”, ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, 4th Meeting: Daegu, S. Korea, JCTVC-D235, January, 2011.

- Yongbing Lin, Lingzhi Liu, Changcai Lai, Jianhua Zheng, “Simplified Planar Intra Prediction”, ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, 5th Meeting: Geneva, Switzerland, JCTVC-E289, March, 2011.

# Patent rights declaration(s)

Santa Clara University and HiSilicon Technologies Co., Ltd. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).