| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 5th Meeting: Geneva, CH, 16-23 March, 2011 | Document: JCTVC-E114 WG11 Number: m19629 |
| --- | --- |

| Title: | Cross verification of LG proposal JCTVC-E059 on modifications of temporal mv compression and temporal mv predictor |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Minhua Zhou Texas Instruments Inc., USA | Tel: Email: | +1-214-480-3816 zhou@ti.com |
| Source: | Texas Instruments Inc. |  |  |

_____________________________

# Abstract

The document reports cross-check results of LG proposal JCTVC-E059 on Modifications of temporal mv compression and temporal mv predictor. It has been verified that the results provided by LG for the following tools are accurate: 1) centre block location based motion vector compression, 2) reference index compression and 3) disabling co-located intra block by replacing it with zero vector. The source code is also checked to verify the accuracy of LG proposal description.

# Introduction

The document reports cross-check results of LG proposal on modifications of temporal mv compression and temporal mv predictor [2].

# Algorithm description

Based on the source code provided by LG, the LG proposal basically contains three parts:

Figure 1: illustration of 16 4x4 motion vectors in a 16x16 region

- Motion vector compression, instead of picking upper-left corner of 4x4 motion vector (i.e. block 0 in Figure 1) as the vector for the 16x16 region, it searches through the centre four 4x4 block vectors in the order of block 12, 3, 6, 9, the motion vector of the first inter-coded block is selected as the motion vector for the entire 16x16 region.

- Reference index compression: the reference index of the first inter-coded block (i.e. 12, 3, 6, 9) is selected as reference index for entire 16x16 region.

- Avoiding co-located intra block: if none of the centre blocks is inter-coded block, the motion vector is set to zero and reference index is set to -1 for the entire 16x16 region region.

Note that if the part 3 (i.e. avoid co-located intra block) is disabled, only block 12 is used in the process of part 1 and part 2.

# Test Settings and Conditions

The following four settings are cross-checked:

| Setting | Reference index compression | Motion vector compression from centre positions | Avoid co-located intra block | Objective |
| --- | --- | --- | --- | --- |
| 0 |  | x |  | Test gain of MV compression from centre positions |
| 1 |  | x | x | Test gain of MV compression from centre positions plus avoid co-located intra-block |
| 2 | x |  |  | Test gain of reference index compression |
| 3 | x | x | x | Test total gain |

Table 1: Test settings and objectives

The simulations of this document have used HM2.0 software, the simulation platform is LSF equipped with Intel(R) Xeon(R) CPU X5570@2.93GHz 64 bits Linux machines, the common test conditions and reference configurations specified in [1] are followed.

# Experimental Results

A summery of test results is provided in Table 2.

| Settings | Random access (%) | Random access LoCo (%) | Low delay (%) | Low delay LoCo (%) |
| --- | --- | --- | --- | --- |
| MV compression from centre positions | -0.3 | -0.3 | -0.4 | -0.4 |
| MV compression from centre positions plus avoiding co-located intra-block | -0.4 | -0.4 | -0.5 | -0.5 |
| reference index compression | 0.0 | 0.0 | -0.1 | -0.1 |
| Test total gain | -0.5 | -0.5 | -0.6 | -0.6 |

Table 2: Summary of testing results

The detailed results of four settings are provided in Table 3 to Table 6. It has been verified the results presented in this section perfectly match the results provided by LG.

|  |  | Random access |  |  | Random access LoCo |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | -0.3 | -0.2 | -0.3 | -0.4 | -0.4 | -0.3 |
| Class B | -0.2 | -0.1 | -0.1 | -0.1 | -0.1 | -0.1 |
| Class C | -0.3 | -0.3 | -0.4 | -0.3 | -0.2 | -0.3 |
| Class D | -0.5 | -0.5 | -0.5 | -0.5 | -0.5 | -0.5 |
| Class E |  |  |  |  |  |  |
| All | -0.3 | -0.3 | -0.3 | -0.3 | -0.3 | -0.3 |
| Enc Time[%] | 102% |  |  | 97% |  |  |
| Dec Time[%] | 103% |  |  | 95% |  |  |
|  | Low delay |  |  | Low delay LoCo |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A |  |  |  |  |  |  |
| Class B | -0.3 | 0.0 | -0.2 | -0.2 | -0.2 | 0.3 |
| Class C | -0.5 | -0.3 | -0.4 | -0.5 | -0.6 | -0.8 |
| Class D | -0.6 | -0.7 | -0.5 | -0.5 | -0.9 | -0.9 |
| Class E | -0.4 | -0.2 | -0.3 | -0.5 | -0.6 | -0.2 |
| All | -0.4 | -0.3 | -0.4 | -0.4 | -0.6 | -0.4 |
| Enc Time[%] | 102% |  |  | 95% |  |  |
| Dec Time[%] | 104% |  |  | 94% |  |  |

Table 3: Coding gain of MV compression from centre positions

|  |  | Random access |  |  | Random access LoCo |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | -0.4 | -0.3 | -0.7 | -0.4 | -0.3 | -0.3 |
| Class B | -0.2 | -0.2 | -0.2 | -0.2 | -0.1 | -0.1 |
| Class C | -0.4 | -0.3 | -0.4 | -0.4 | -0.4 | -0.4 |
| Class D | -0.5 | -0.5 | -0.5 | -0.6 | -0.6 | -0.6 |
| Class E |  |  |  |  |  |  |
| All | -0.4 | -0.3 | -0.4 | -0.4 | -0.3 | -0.3 |
| Enc Time[%] | 104% |  |  | 97% |  |  |
| Dec Time[%] | 109% |  |  | 92% |  |  |
|  | Low delay |  |  | Low delay LoCo |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A |  |  |  |  |  |  |
| Class B | -0.3 | -0.2 | -0.6 | -0.2 | -0.3 | -0.2 |
| Class C | -0.6 | -0.4 | -0.6 | -0.6 | -0.7 | -0.7 |
| Class D | -0.6 | -0.4 | -0.8 | -0.6 | -0.8 | -0.5 |
| Class E | -0.7 | -1.1 | -0.2 | -0.4 | -0.4 | -0.5 |
| All | -0.5 | -0.4 | -0.5 | -0.5 | -0.5 | -0.5 |
| Enc Time[%] | 103% |  |  | 99% |  |  |
| Dec Time[%] | 107% |  |  | 98% |  |  |

Table 4: Coding gain of MV compression from centre positions plus avoiding intra co-located blocks

|  |  | Random access |  |  | Random access LoCo |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | 0.0 | 0.1 | -0.1 | 0.0 | 0.0 | 0.0 |
| Class B | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| Class C | -0.1 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| Class D | -0.1 | 0.0 | -0.1 | -0.1 | -0.1 | -0.2 |
| Class E |  |  |  |  |  |  |
| All | 0.0 | 0.0 | -0.1 | 0.0 | 0.0 | 0.0 |
| Enc Time[%] | 101% |  |  | 98% |  |  |
| Dec Time[%] | 103% |  |  | 93% |  |  |
|  | Low delay |  |  | Low delay LoCo |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A |  |  |  |  |  |  |
| Class B | 0.0 | 0.1 | 0.2 | -0.1 | 0.0 | 0.1 |
| Class C | -0.1 | 0.1 | 0.0 | -0.2 | -0.1 | -0.2 |
| Class D | -0.2 | -0.2 | -0.3 | -0.1 | -0.4 | -0.1 |
| Class E | -0.1 | -0.5 | 0.2 | 0.1 | -0.3 | -0.2 |
| All | -0.1 | -0.1 | 0.0 | -0.1 | -0.2 | -0.1 |
| Enc Time[%] | 99% |  |  | 100% |  |  |
| Dec Time[%] | 100% |  |  | 103% |  |  |

Table 5: Coding gain of reference index compression

|  |  | Random access |  |  | Random access LoCo |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | -0.4 | -0.3 | -0.6 | -0.5 | -0.4 | -0.4 |
| Class B | -0.3 | -0.3 | -0.2 | -0.3 | -0.1 | -0.2 |
| Class C | -0.6 | -0.5 | -0.5 | -0.6 | -0.5 | -0.5 |
| Class D | -0.8 | -0.8 | -0.7 | -0.9 | -0.9 | -0.9 |
| Class E |  |  |  |  |  |  |
| All | -0.5 | -0.4 | -0.5 | -0.5 | -0.5 | -0.5 |
| Enc Time[%] | 107% |  |  | 99% |  |  |
| Dec Time[%] | 115% |  |  | 95% |  |  |
|  | Low delay |  |  | Low delay LoCo |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A |  |  |  |  |  |  |
| Class B | -0.4 | -0.2 | -0.5 | -0.4 | -0.5 | -0.3 |
| Class C | -0.7 | -0.6 | -0.5 | -0.9 | -1.0 | -1.0 |
| Class D | -0.9 | -0.8 | -1.0 | -0.8 | -1.1 | -0.7 |
| Class E | -0.6 | -0.8 | -0.6 | -0.4 | -0.9 | -0.5 |
| All | -0.6 | -0.6 | -0.6 | -0.6 | -0.8 | -0.6 |
| Enc Time[%] | 102% |  |  | 99% |  |  |
| Dec Time[%] | 106% |  |  | 104% |  |  |

Table 6: Total Coding gain of LG proposal

# Conclusions

It has been cross-verified that the results provided by LG in [2] are accurate. It is recommended to adopt the ref_idx compression as a bug fix, and further evaluate rest parts of  this proposal with other proposals in the same category in core experiments.

# References

- JCT-VC, “Common test conditions and software reference configurations”, JCTVC-C600, 4th JCT-VC Meeting, Daegu, KR, 20-28 January, 2011

- JCT-VC, “Modifications of temporal mv compression and temporal mv predictor” JCTVC-E059, 5th JCT-VC Meeting, Geneva, CH, 16-23 March, 2011