| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 5th Meeting: Geneva, CH, 16-23 March, 2011 | Document: JCTVC-E117 WG11 Number: m19632 |
| --- | --- |

| Title: | Evaluation results on motion vector storage compression |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Minhua Zhou Texas Instruments Inc., USA | Tel: Email: | +1-214-480-3816 zhou@ti.com |
| Source: | Texas Instruments Inc. |  |  |

_____________________________

# Abstract

This document reports evaluation results of motion vector storage compression adopted in the HM2.0. The test results reveal that for 16x motion vector compression used in the HM2.0 the average coding loss is 0.6% (HE-RA), 0.7% (LC-RA), 0.9% (HE-LD) and 0.9% (LC-LD), respectively. If 4x motion vector compression is used, the coding loss is reduced to 0.1% (HE-RA), 0.2% (LC-RA), 0.1% (HE-LD) and 0.1% (LC-LD), respectively.  It is recommended to maintain 16x motion vector compression in the HEVC test model but to study other possibilities to reduce the quality loss.

# Introduction

In HM2.0 the 16x motion vector storage compression [2] is adopted. This document reports evaluation results of 16x and 4x motion vector storage compression by using the HM2.0 software. The purpose of this study is to quantify the quality loss of the motion vector storage compression in the context of HM2.0.

# Algorithm description

The motion vector storage compression method in HM2.0 is designed to reduce the memory bandwidth spent on storing and loading temporal motion vectors for merge/skip mode and motion vector predictive coding. The 16x compression method basically down-samples the motion vector field by factor 4 in both horizontal and vertical direction, resulting in a same motion vector for each 16x16 region. Likewise, the 4x compression method down-samples the motion vector field by factor 2 in both horizontal and vertical direction, it leads to a same motion vector for each 8x8 region. The current method implemented in the HM2.0 software simply picks upper-left corner 4x4 vector as the vector for the entire 16x16 or 8x8 region.

# Test Conditions

The simulations of this document have used HM2.0 software, the simulation platform is LSF equipped with Intel(R) Xeon(R) CPU X5570@2.93GHz 64 bits Linux machines, the common test conditions and reference configurations specified in [1] are followed.

# Experimental Results

Table 1 summarizes the testing results of 16x motion vector storage compression adopted in HM2.0, the average coding loss is 0.6% for random access configuration, 0.7% for random access LoCo configuration,  and 0.9% for low delay and low delay LoCo configurations, respectively.

|  | Random access |  |  | Random access LoCo |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | -0.5 | -0.5 | -0.6 | -0.6 | -0.5 | -0.4 |
| Class B | -0.4 | -0.3 | -0.2 | -0.4 | -0.3 | -0.3 |
| Class C | -0.6 | -0.5 | -0.6 | -0.6 | -0.5 | -0.5 |
| Class D | -1.1 | -1.0 | -1.0 | -1.2 | -1.0 | -1.0 |
| Class E |  |  |  |  |  |  |
| All | -0.6 | -0.5 | -0.6 | -0.7 | -0.5 | -0.5 |
| Enc Time[%] | 102% |  |  | 96% |  |  |
| Dec Time[%] | 108% |  |  | 90% |  |  |
|  | Low delay |  |  | Low delay LoCo |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A |  |  |  |  |  |  |
| Class B | -0.5 | -0.2 | -0.2 | -0.6 | -0.6 | -0.3 |
| Class C | -1.0 | -0.7 | -0.9 | -1.2 | -1.1 | -1.3 |
| Class D | -1.3 | -1.3 | -1.4 | -1.3 | -1.2 | -1.1 |
| Class E | -0.7 | -1.2 | -0.2 | -0.7 | -0.6 | -0.2 |
| All | -0.9 | -0.8 | -0.7 | -0.9 | -0.9 | -0.7 |
| Enc Time[%] | 100% |  |  | 96% |  |  |
| Dec Time[%] | 105% |  |  | 95% |  |  |

Table 1: Coding gain by disabling the 16x motion vector storage compression in HM2.0

Table 2 summarizes the testing results of 4x motion vector storage compression compared to the 16x one of  HM2.0, the average coding gain is 0.5% for random access and random access LoCo configurations,  and 0.8% for low delay and low delay LoCo configurations, respectively. Therefore, if 4x compression method was used in the HM2.0, the coding loss would be reduced to 0.2% in random access configuration and 0.1% in the rest of configurations, respectively.

|  | Random access |  |  | Random access LoCo |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | -0.4 | -0.3 | -0.5 | -0.4 | -0.4 | -0.3 |
| Class B | -0.3 | -0.2 | -0.2 | -0.3 | -0.2 | -0.2 |
| Class C | -0.6 | -0.5 | -0.5 | -0.5 | -0.4 | -0.4 |
| Class D | -0.9 | -0.7 | -0.9 | -0.9 | -0.8 | -0.8 |
| Class E |  |  |  |  |  |  |
| All | -0.5 | -0.5 | -0.5 | -0.5 | -0.4 | -0.4 |
| Enc Time[%] | 103% |  |  | 99% |  |  |
| Dec Time[%] | 108% |  |  | 98% |  |  |
|  | Low delay |  |  | Low delay LoCo |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A |  |  |  |  |  |  |
| Class B | -0.4 | -0.4 | -0.3 | -0.5 | -0.4 | -0.2 |
| Class C | -0.9 | -0.8 | -0.9 | -1.0 | -1.0 | -1.0 |
| Class D | -1.0 | -0.9 | -1.1 | -1.0 | -0.8 | -0.9 |
| Class E | -0.9 | -1.2 | -0.6 | -0.6 | -0.9 | -0.3 |
| All | -0.8 | -0.8 | -0.7 | -0.8 | -0.7 | -0.6 |
| Enc Time[%] | 100% |  |  | 97% |  |  |
| Dec Time[%] | 105% |  |  | 98% |  |  |

Table 2: Coding gain of 4x motion vector storage compression relative to the 16x one in HM2.0.

# Conclusions

The experimental results show that the quality loss resulted from 16x motion vector storage compression of the current HM2.0 software is relatively high, the loss from 4x motion vector storage compression is negligible but the compression ratio is not satisfactory. It is recommended to seek other possibilities to reduce the quality loss while maintaining the motion vector compression ratio at 16x.

# References

- JCT-VC, “Common test conditions and software reference configurations”, JCTVC-C600, 4th JCT-VC Meeting, Daegu, KR, 20-28 January, 2011

[2] Yeping Su, Andrew Segall, “CE9: Reduced resolution storage of motion vector data,” JCTVC-D072, Jan. 2011.