| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 5th Meeting: Geneva, CH, 16-23 March, 2011 | Document: JCTVC-E0118 WG11 Number: m19633 |
| --- | --- |

| Title: | A study on HM2.0 bitstream parsing and error resiliency issue |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Minhua Zhou Texas Instruments Inc., USA Vivienne Sze Texas Instruments Inc., USA | Tel: Email: Tel: Email: | +1-214-480-3816 zhou@ti.com +1-214-567-4005 sze@ti.com |
| Source: | Texas Instruments Inc. |  |  |

_____________________________

# Abstract

The current HM2.0 design is reported to have both bitstream parsing issues and error resiliency issues. This document presents the investigation results and recommends solutions.  Specifically 1) encode the merge flag independent of the merge mode MVP list size; 2) remove the temporal MVP from the merge mode; 3) start core experiments on adding high-level flags to allow encoder to disable temporal MVP in both skip mode and motion vector coding, or disable temporal MVP from motion vector coding but still keep in skip mode, for generation of more error resilient bitstreams; 4) continue to study solutions for resolving bitstream parsing issue caused by the MVP list size dependent MVP index coding.  Notable investigation results include:  1) MVP list size independent merge flag coding does not cause quality loss, in fact, it leads to small gain in low delay configurations; 2) The quality loss of disabling the temporal MVP in merge mode and motion vector coding is relatively small, 0.4% in RA, 0.3% in RALC, -0.2% in LD and 0.5% in LDLC, respectively; 3) Disabling the temporal MVP in the merge mode leads to quality gain: 0.0% in RA, -0.2% in RALC, -0.1% in LD and -0.4% in LDLC, respectively.

# Introduction

The current HM2.0 design is reported to have both bitstream parsing issues and error resiliency issues. This document presents the investigation results and recommends solutions.

# Problem statement

In HM2.0 even if video reconstruction is not needed, parsing bitstream on the decoder side would still need full reconstruction of CU overhead information such as motion vectors, prediction direction, partitioning information (2Nx2N, 2N x N, N x 2N, NxN) and intra/inter flag. This consumes processing power and memory bandwidth, and is extremely unfriendly to architectures on which bitstream might need to be pre-parsed before full decoding.  The causes of this bitstream parsing issue are:

- Conditional coding of merge flag, the merge flag is encoded into bitstream only if the merge mode MVP list size is larger than zero. Otherwise, it is not encoded. The decoder has to construct the merge mode MVP list even if the merge mode is not used. Also, the context selection for merge flag coding depends on the merge mode MVP list size.

- MVP index coding for merge mode depends on the merge mode MVP list size. If the merge mode MVP list size is zero, the MVP index is not present in the bitsteam. Otherwise, the binarization of the MVP index depends on the merge mode MVP list size

- MVP index coding for skip mode and motion vector coding depending on the skip mode / motion vector coding MVP list size. It operates in exactly the same manner as the merge mode MVP index coding.

The HM2.0 bitstream is more error sensitive due to the following facts:

  - to construct the merge/skip/MV MVP list, motion vectors, prediction direction, reference frame index, partitioning information (2Nx2N, 2N x N, N x 2N, NxN) and intra/inter flag are required, any error in those information will cause incorrect reconstruction of the MVP list and leads to bitstream parsing stop.

  - temporal co-located motion vectors are used as motion vector predictor in merge mode, skip mode and motion vector coding, if motion vector field of a frame is lost, the error cannot be recovered until next intra-coded frame

# Test Settings and Conditions

The following tests are carried out for the study of bitstrem parsing issue:

- Coding MVP index in merge, skip and MVD and merge flag independent of MVP list size:  this test is designed to quantify the total quality loss if the parsing issue is completely resolved. In this test the merge flag is always coded, the context selection for the merge flag and merge index are independent of the merge mode MVP list size (the context selection for merge flag is replaced with the approach used in HM-1.0), and the MVP index coding (binarizaton) for merge mode, skip mode and motion vector coding is independent of the MVP list size using truncated unary coding with a fixed maximum length of 5, 3 and 3, respectively, such that mvp_idx is sent even when there is only a single candidate.

- Coding the merge flag all the time with added zero vectors to the merge mode MVP list: this test is aimed to evaluate the overhead if the merge flag is encoded for every PU instead of conditionally encoded depending on the merge mode MVP list size. In this test the context selection for merge flag is replaced with the approach used in HM-1.0 to completely remove its coding dependency on the merge mode MVP list size, zero vector is added to the merge mode MVP list if it is empty to keep the minimum size of merge mode MVP list at one so that the merge flag is encoded all the time. The following rules are used for adding the zero motion vector when the merge mode MVP list is empty:

  - If current slice is a single list (forward prediction only) slice, the prediction direction is set to forward (list 0) and reference index and motion vector are set to zero.

  - If the current slice is bi-directional slice (list 0 and list 1 prediction), motion vectors and reference indices for both directions are set to zero, the prediction direction is set to bi-directional prediction (list0 plus list1).

The rest parts of HM2.0 software are unchanged.

The following tests are carried out for the study of error resiliency issue:

- disabling the temporal MVP from the merge, skip mode, motion vector coding  and from motion estimation to understand the total quality loss if the temporal MVP (TMVP) is disabled in all modes

- disabling the temporal MVP from the merge, skip mode, motion vector coding but keeping it in the motion estimation to understand quality impact of TMVP as initial MVP for motion vector refinement in motion estimation

- disabling the temporal MVP from the merge, motion vector coding but keeping it in skip mode and motion estimation to understand quality impact of TMVP as MVP for motion vector coding

- disabling the temporal MVP in the merge mode only to understand the quality impact of TMVP in the merge mode.

The simulations of this document have used HM2.0 software, the simulation platform is LSF equipped with Intel(R) Xeon(R) CPU X5570@2.93GHz 64 bits Linux machines, the common test conditions and reference configurations specified in [1] are followed.

# Experimental Results

Table 1 summarizes the results related to the bitstream parsing. The main observations are

- The total quality loss of MVP list size independent merge flag and MVP index coding is 1.1% in RA, 2.0% in LC-RA, 1.2% in LD and 3.5% in LDLC, respectively.

- MVP list size independent merge flag coding does not cause quality loss, in fact, it leads to small gain in low delay configurations

The detailed results are provided from Table 3 to Table 4.

| Settings | H | Random access LoCo (%) | Low delay (%) | Low delay LoCo (%) |
| --- | --- | --- | --- | --- |
| Coding MVP index in merge, skip and MVD and merge flag independent of MVP list size | 1.1 | 2.0 | 1.2 | 3.5 |
| Coding the merge flag all the time with added zero vectors to the merge mode MVP list | 0.0 | 0.0 | -0.1 | 0.0 |

Table 1: Summary of testing results for bitstream parsing related tests

The error resiliency related testing results are summarized in Table 2. The detailed results are listed in Table 5 to Table 8.  The main observations are:

- The total quality loss of disabling temporal MVP from merge, skip mode, motion vector coding and from motion estimation coding is 2.3% in HE-RA, 2.2% in LC-RA, 1.7% in HE-LD and 2.4% in LC-LD, respectively.

- When the TMVP is still enabled in the motion and disabled in merge, skip mode, and motion vector coding, the total loss is reduced to 1.9% in HE-RA, 1.9% in LC-RA, 1.6% in HE-LD and 2.4% in LC-LD, respectively.

- The quality loss of disabling the temporal MVP in merge mode and motion vector coding is relatively small, 0.4% in HE-RA, 0.3% in LC-RA, -0.2% in HE-LD and 0.5% in LC-LD, respectively.

- disabling the temporal MVP in the merge mode leads to quality gain: 0.0% in HE-RA, -0.2% in LC-RA, -0.1% in HE-LD and -0.4% in LC-LD, respectively.

| Settings | Random access (%) | Random access LoCo (%) | Low delay (%) | Low delay LoCo (%) |
| --- | --- | --- | --- | --- |
| Disabling temporal MVP in merge, skip, motion vector coding and in ME | 2.3 | 2.2 | 1.7 | 2.4 |
| Disabling temporal MVP in merge, skip, motion vector coding but enabling it in ME | 1.9 | 1.9 | 1.6 | 2.4 |
| Disabling temporal MVP in merge and motion vector coding but enabling it in skip mode and ME | 0.4 | 0.3 | -0.2 | 0.5 |
| Disabling temporal MVP in merge mode only | 0.0 | -0.2 | -0.1 | -0.4 |

Table 2: Summary of testing results for error resiliency related tests

|  |  | Random access |  |  | Random access LoCo |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | 0.9 | 0.6 | 0.1 | 1.8 | 1.4 | 1.3 |
| Class B | 0.9 | 0.1 | 0.1 | 2.1 | 1.6 | 1.4 |
| Class C | 1.1 | 0.7 | 0.7 | 2.0 | 1.6 | 1.6 |
| Class D | 1.3 | 0.8 | 0.8 | 2.2 | 1.6 | 1.7 |
| Class E |  |  |  |  |  |  |
| All | 1.1 | 0.5 | 0.4 | 2.0 | 1.6 | 1.5 |
| Enc Time[%] | 94% |  |  | 96% |  |  |
| Dec Time[%] | 99% |  |  | 91% |  |  |
|  | Low delay |  |  | Low delay LoCo |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A |  |  |  |  |  |  |
| Class B | 1.0 | 0.0 | -0.6 | 2.8 | 1.6 | 1.7 |
| Class C | 1.4 | 0.8 | 0.7 | 2.5 | 1.6 | 1.5 |
| Class D | 1.4 | 0.7 | 0.7 | 2.7 | 1.5 | 1.1 |
| Class E | 1.1 | -0.3 | 0.4 | 7.0 | 4.5 | 5.3 |
| All | 1.2 | 0.3 | 0.2 | 3.5 | 2.1 | 2.2 |
| Enc Time[%] | 93% |  |  | 98% |  |  |
| Dec Time[%] | 101% |  |  | 96% |  |  |

Table 3: Coding loss by coding the merge flag and MVP index independent of the MVP list size (HM2_anchor_fixedMergeFlag&MVPListCoding_keepTMVP.xls)

|  |  | Random access |  |  | Random access LoCo |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | 0.1 | -0.1 | -0.2 | 0.0 | -0.1 | 0.0 |
| Class B | 0.0 | -0.5 | -0.5 | 0.0 | 0.0 | 0.0 |
| Class C | 0.0 | -0.4 | -0.3 | -0.1 | -0.1 | 0.0 |
| Class D | 0.0 | -0.4 | -0.3 | 0.0 | -0.1 | 0.0 |
| Class E |  |  |  |  |  |  |
| All | 0.0 | -0.3 | -0.4 | 0.0 | -0.1 | 0.0 |
| Enc Time[%] | 96% |  |  | 101% |  |  |
| Dec Time[%] | 99% |  |  | 100% |  |  |
|  | Low delay |  |  | Low delay LoCo |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A |  |  |  |  |  |  |
| Class B | -0.1 | -1.0 | -0.9 | 0.0 | -0.1 | 0.0 |
| Class C | 0.0 | -0.3 | -0.5 | -0.1 | -0.1 | -0.3 |
| Class D | -0.1 | -0.9 | -0.7 | 0.0 | 0.2 | 0.3 |
| Class E | -0.1 | -1.3 | -1.0 | 0.0 | -0.2 | 0.4 |
| All | -0.1 | -0.8 | -0.8 | 0.0 | 0.0 | 0.1 |
| Enc Time[%] | 92% |  |  | 100% |  |  |
| Dec Time[%] | 98% |  |  | 104% |  |  |

Table 4: Results for merge flag coding independent of merge mode MVP list size (HM2_anchor_fixedMergeFlagCoding_only.xls)

|  |  | Random access |  |  | Random access LoCo |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | 3.1 | 2.6 | 2.3 | 3.4 | 2.8 | 3.0 |
| Class B | 2.2 | 1.3 | 1.3 | 1.8 | 0.7 | 0.7 |
| Class C | 1.9 | 1.6 | 1.7 | 1.9 | 1.5 | 1.6 |
| Class D | 2.1 | 1.5 | 1.6 | 2.0 | 1.1 | 1.3 |
| Class E |  |  |  |  |  |  |
| All | 2.3 | 1.7 | 1.7 | 2.2 | 1.5 | 1.6 |
| Enc Time[%] | 95% |  |  | 95% |  |  |
| Dec Time[%] | 98% |  |  | 88% |  |  |
|  | Low delay |  |  | Low delay LoCo |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A |  |  |  |  |  |  |
| Class B | 2.1 | 1.5 | 1.4 | 3.0 | 1.5 | 1.7 |
| Class C | 1.8 | 2.0 | 1.9 | 2.7 | 1.9 | 1.8 |
| Class D | 1.4 | 0.9 | 0.8 | 1.9 | 1.0 | 0.8 |
| Class E | 1.3 | 1.2 | 1.8 | 2.0 | 0.4 | 1.5 |
| All | 1.7 | 1.4 | 1.4 | 2.4 | 1.3 | 1.4 |
| Enc Time[%] | 97% |  |  | 97% |  |  |
| Dec Time[%] | 95% |  |  | 85% |  |  |

Table 5: Coding loss by disabling temporal MVP in merge, skip, motion vector coding and in ME (HM2_anchor_DynMVPListCoding_noTMVP.xls)

|  |  | Random access |  |  | Random access LoCo |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | 2.3 | 1.8 | 1.7 | 2.6 | 1.9 | 2.2 |
| Class B | 2.0 | 1.2 | 1.2 | 1.5 | 0.6 | 0.6 |
| Class C | 1.6 | 1.4 | 1.5 | 1.6 | 1.2 | 1.4 |
| Class D | 1.9 | 1.4 | 1.4 | 1.9 | 1.0 | 1.2 |
| Class E |  |  |  |  |  |  |
| All | 1.9 | 1.4 | 1.4 | 1.9 | 1.1 | 1.3 |
| Enc Time[%] | 96% |  |  | 101% |  |  |
| Dec Time[%] | 102% |  |  | 101% |  |  |
|  | Low delay |  |  | Low delay LoCo |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A |  |  |  |  |  |  |
| Class B | 1.9 | 1.7 | 1.5 | 3.0 | 1.8 | 2.0 |
| Class C | 1.6 | 1.8 | 1.7 | 2.5 | 1.8 | 1.7 |
| Class D | 1.3 | 1.0 | 0.8 | 1.8 | 0.8 | 0.5 |
| Class E | 1.3 | 0.6 | 1.6 | 2.0 | 0.8 | 1.5 |
| All | 1.6 | 1.3 | 1.4 | 2.4 | 1.3 | 1.5 |
| Enc Time[%] | 97% |  |  | 100% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |

Table 6: Coding loss by disabling temporal MVP in merge, skip and motion vector coding but enabling it in ME (HM2_anchor_DynMVPListCoding_noTMVP_keepTMVPinME.xls)

|  |  | Random access |  |  | Random access LoCo |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | 0.4 | 0.2 | 0.0 | 0.6 | 0.4 | 0.5 |
| Class B | 0.4 | 0.0 | 0.0 | 0.3 | 0.0 | 0.0 |
| Class C | 0.2 | -0.1 | 0.0 | 0.1 | -0.1 | 0.0 |
| Class D | 0.6 | 0.0 | 0.0 | 0.4 | 0.1 | 0.1 |
| Class E |  |  |  |  |  |  |
| All | 0.4 | 0.0 | 0.0 | 0.3 | 0.1 | 0.2 |
| Enc Time[%] | 94% |  |  | 95% |  |  |
| Dec Time[%] | 97% |  |  | 89% |  |  |
|  | Low delay |  |  | Low delay LoCo |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A |  |  |  |  |  |  |
| Class B | -0.1 | -0.4 | -0.6 | 0.8 | 1.8 | 2.4 |
| Class C | 0.0 | -0.1 | -0.4 | 0.5 | 0.6 | 0.6 |
| Class D | -0.1 | -0.4 | -0.6 | 0.8 | 1.2 | 1.0 |
| Class E | -0.5 | -1.3 | -0.3 | -0.4 | 0.1 | 0.4 |
| All | -0.2 | -0.5 | -0.5 | 0.5 | 1.0 | 1.2 |
| Enc Time[%] | 101% |  |  | 96% |  |  |
| Dec Time[%] | 107% |  |  | 91% |  |  |

Table 7: Coding loss by disabling temporal MVP in merge and motion vector coding but enabling it in skip and ME (HM2_anchor_DynMVPListCoding_noTMVPinMVD_keepTMVPinMESkip.xls)

|  |  | Random access |  |  | Random access LoCo |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | 0.0 | -0.1 | -0.2 | -0.1 | -0.1 | 0.0 |
| Class B | 0.0 | 0.0 | 0.0 | -0.2 | -0.2 | -0.1 |
| Class C | -0.1 | -0.1 | -0.1 | -0.2 | -0.2 | -0.2 |
| Class D | 0.0 | 0.0 | 0.0 | -0.2 | -0.2 | -0.1 |
| Class E |  |  |  |  |  |  |
| All | 0.0 | -0.1 | -0.1 | -0.2 | -0.2 | -0.1 |
| Enc Time[%] | 97% |  |  | 92% |  |  |
| Dec Time[%] | 104% |  |  | 90% |  |  |
|  | Low delay |  |  | Low delay LoCo |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A |  |  |  |  |  |  |
| Class B | 0.0 | -0.2 | -0.5 | -0.5 | -0.5 | -0.1 |
| Class C | 0.0 | 0.1 | -0.1 | -0.3 | -0.2 | -0.4 |
| Class D | -0.3 | -0.5 | -0.9 | -0.4 | -0.5 | -0.6 |
| Class E | -0.3 | -1.0 | -0.1 | -0.4 | -0.7 | -0.3 |
| All | -0.1 | -0.3 | -0.4 | -0.4 | -0.4 | -0.3 |
| Enc Time[%] | 97% |  |  | 94% |  |  |
| Dec Time[%] | 103% |  |  | 91% |  |  |

Table 8: Coding gain by disabling temporal MVP in merge mode of HM2.0 (HM2_anchor_DynMVPListCoding_noTMVPinMRG.xls)

# Conclusions & Recommendations

In this document the HM2.0 bitstream parsing and error resiliency issues are studied. Results show that there is no coding loss by removing dependency between the merge flag coding and the merge mode MVP list size, and the loss caused by disabling the temporal MVP from the motion vector coding is relatively small. Based on the evaluation results it is recommended

- encode the merge flag independent of the merge mode MVP list size as proposed in the document to partially resolve the bitstream parsing issue.

- remove the temporal MVP from the merge mode.

- start core experiments on adding high-level flags to allow encoder to disable temporal MVP in both skip mode and motion vector coding, or disable temporal MVP from motion vector coding but still keep in skip mode, as error resiliency tools for generation of more error resilient bitstreams

- continue to study solutions for resolving bitstream parsing issues caused by the MVP list size dependent MVP index coding

# References

- JCT-VC, “Common test conditions and software reference configurations”, JCTVC-C600, 4th JCT-VC Meeting, Daegu, KR, 20-28 January, 2011

- JCT-VC, “CE9: 3.1 PU Merge & Skip tools and proposed improvements” JCTVC-D0233, 4th JCT-VC Meeting, Daegu, KR, 20-28 January, 2011

# Patent rights declaration(s)

Texas Instruments may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).