| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 5th Meeting: Geneva, CH, 16-23 March, 2011 | Document: JCTVC-E120 WG11 Number: m19635 |
| --- | --- |

| Title: | Cross verification of Microsoft proposal JCTVC-E148 on “an investigation on robust parsing” |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Minhua Zhou Texas Instruments Inc., USA | Tel: Email: | +1-214-480-3816 zhou@ti.com |
| Source: | Texas Instruments Inc. |  |  |

_____________________________

# Abstract

The document reports cross-check results of Microsoft proposal JCTVC-E148 on “an investigation on robust parsing”. It has been verified that the results provided by Microsoft for the following settings are accurate: 1) disable temporal motion vector predictor in merge, skip and motion vector coding MVP list (noAllTMVP), 2) encode the MVP index for merge, skip mode and motion vector coding independent of the MVP list size, and encode the merge mode flag all the time to avoid bitstream parsing issue (codeFull), 3) disable temporal motion vector predictor in merge mode (noMergeTMVP), and 4) define a new NAL unit type to signal whether temporal MVP is disabled in the merge mode , skip mode and motion vector coding MVP  list (Proposed method). The source code is also checked to verify the accuracy of proposal description from Microsoft.

# Introduction

The document reports cross-check results of Microsoft proposal JCTVC-E148 on “An investigation on robust parsing”.

# Test Settings and Conditions

The following four settings recommended by Microsoft are cross-verified:

- disable temporal motion vector predictor in merge, skip and motion vector coding MVP list (noAllTMVP)

- encode the MVP index for merge, skip mode and motion vector coding independent of the MVP list size, and encode the merge mode flag all the time to avoid bitstream parsing issue (codeFull)

- disable temporal motion vector predictor in merge mode. (noMergeTMVP)

- define a new NAL unit type to signal whether temporal MVP is disabled in the merge mode, skip mode and motion vector coding MVP  list, this NAL unit type requires marking motion vector fields of all the reference frames in DPB not available for derivation process of temporal co-located MVPs.   In the test the temporal TMVP is disabled every eight frames (Proposed)

The source code is checked to verify that the above settings match the source code.

The simulations of this document have used HM2.0 software, the simulation platform is LSF equipped with Intel(R) Xeon(R) CPU X5570@2.93GHz 64 bits Linux machines, the common test conditions and reference configurations specified in [1] are followed.

# Experimental Results

A summary of test results is provided in Table 1.

| Settings | Random access (%) | Random access LoCo (%) | Low delay (%) | Low delay LoCo (%) |
| --- | --- | --- | --- | --- |
| noAllTMVP | 2.3 | 2.2 | 1.7 | 2.4 |
| codeFull | 1.2 | 1.7 | 1.4 | 3.0 |
| noMergeTMVP | 0.0 | -0.2 | -0.1 | -0.4 |
| proposed | 0.5 | 0.4 | 0.3 | 0.2 |

Table 1: Summary of testing results

The detailed results of four settings are provided in Table 2 to Table 5. It has been verified the results presented in this section perfectly match the results provided by Microsoft.

|  |  | Random access |  |  | Random access LoCo |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | 3.1 | 2.6 | 2.3 | 3.4 | 2.8 | 3.0 |
| Class B | 2.2 | 1.3 | 1.3 | 1.8 | 0.7 | 0.7 |
| Class C | 1.9 | 1.6 | 1.7 | 1.9 | 1.5 | 1.6 |
| Class D | 2.1 | 1.5 | 1.6 | 2.0 | 1.1 | 1.3 |
| Class E |  |  |  |  |  |  |
| All | 2.3 | 1.7 | 1.7 | 2.2 | 1.5 | 1.6 |
| Enc Time[%] | 98% |  |  | 97% |  |  |
| Dec Time[%] | 101% |  |  | 90% |  |  |
|  | Low delay |  |  | Low delay LoCo |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A |  |  |  |  |  |  |
| Class B | 2.1 | 1.5 | 1.4 | 3.0 | 1.5 | 1.7 |
| Class C | 1.8 | 2.0 | 1.9 | 2.7 | 1.9 | 1.8 |
| Class D | 1.4 | 0.8 | 0.7 | 1.9 | 1.0 | 0.8 |
| Class E | 1.3 | 1.2 | 1.8 | 1.9 | 0.4 | 1.5 |
| All | 1.7 | 1.4 | 1.4 | 2.4 | 1.3 | 1.4 |
| Enc Time[%] | 102% |  |  | 103% |  |  |
| Dec Time[%] | 100% |  |  | 94% |  |  |

Table 2: Coding loss when TMVP is disabled for merge and AMVP (noAllTMVP)

|  |  | Random access |  |  | Random access LoCo |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | 1.0 | 0.5 | 0.2 | 1.6 | 1.3 | 1.1 |
| Class B | 1.0 | 0.3 | 0.2 | 1.8 | 1.3 | 1.2 |
| Class C | 1.3 | 0.7 | 0.8 | 1.7 | 1.3 | 1.3 |
| Class D | 1.5 | 0.8 | 1.0 | 1.8 | 1.3 | 1.3 |
| Class E |  |  |  |  |  |  |
| All | 1.2 | 0.6 | 0.5 | 1.7 | 1.3 | 1.2 |
| Enc Time[%] | 99% |  |  | 97% |  |  |
| Dec Time[%] | 98% |  |  | 92% |  |  |
|  | Low delay |  |  | Low delay LoCo |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A |  |  |  |  |  |  |
| Class B | 1.1 | -0.1 | -0.4 | 2.3 | 1.2 | 1.2 |
| Class C | 1.4 | 0.6 | 0.6 | 2.1 | 1.3 | 1.1 |
| Class D | 1.6 | 0.8 | 0.7 | 2.2 | 0.8 | 1.2 |
| Class E | 1.5 | -0.1 | 0.9 | 6.2 | 3.9 | 4.1 |
| All | 1.4 | 0.3 | 0.4 | 3.0 | 1.6 | 1.7 |
| Enc Time[%] | 99% |  |  | 95% |  |  |
| Dec Time[%] | 103% |  |  | 90% |  |  |

Table 3: Coding loss when MVP index coding is independent of the MVP list size and merge flag is always encoded. (codeFull)

|  |  | Random access |  |  | Random access LoCo |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | 0.0 | -0.1 | -0.2 | -0.1 | -0.1 | 0.0 |
| Class B | 0.0 | 0.0 | 0.0 | -0.2 | -0.2 | -0.1 |
| Class C | -0.1 | -0.1 | -0.1 | -0.2 | -0.2 | -0.2 |
| Class D | 0.0 | 0.0 | 0.0 | -0.2 | -0.2 | -0.1 |
| Class E |  |  |  |  |  |  |
| All | 0.0 | -0.1 | -0.1 | -0.2 | -0.2 | -0.1 |
| Enc Time[%] | 97% |  |  | 92% |  |  |
| Dec Time[%] | 104% |  |  | 90% |  |  |
|  | Low delay |  |  | Low delay LoCo |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A |  |  |  |  |  |  |
| Class B | 0.0 | -0.2 | -0.5 | -0.5 | -0.5 | -0.1 |
| Class C | 0.0 | 0.1 | -0.1 | -0.3 | -0.2 | -0.4 |
| Class D | -0.3 | -0.5 | -0.9 | -0.4 | -0.5 | -0.6 |
| Class E | -0.3 | -1.0 | -0.1 | -0.4 | -0.7 | -0.3 |
| All | -0.1 | -0.3 | -0.4 | -0.4 | -0.4 | -0.3 |
| Enc Time[%] | 97% |  |  | 94% |  |  |
| Dec Time[%] | 103% |  |  | 91% |  |  |

Table 4: Coding gain when TMVP is disabled for merge mode (noMergeTMVP)

|  |  | Random access |  |  | Random access LoCo |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | 0.8 | 0.7 | 0.6 | 0.7 | 0.7 | 0.8 |
| Class B | 0.3 | 0.1 | 0.1 | 0.2 | 0.1 | 0.2 |
| Class C | 0.3 | 0.2 | 0.2 | 0.2 | 0.1 | 0.2 |
| Class D | 0.6 | 0.3 | 0.3 | 0.5 | 0.1 | 0.1 |
| Class E |  |  |  |  |  |  |
| All | 0.5 | 0.3 | 0.3 | 0.4 | 0.3 | 0.3 |
| Enc Time[%] | 100% |  |  | 95% |  |  |
| Dec Time[%] | 100% |  |  | 90% |  |  |
|  | Low delay |  |  | Low delay LoCo |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A |  |  |  |  |  |  |
| Class B | 0.3 | 0.2 | 0.2 | 0.3 | 0.0 | 0.1 |
| Class C | 0.3 | 0.4 | 0.2 | 0.3 | 0.0 | -0.1 |
| Class D | 0.2 | -0.1 | 0.1 | 0.3 | 0.0 | 0.1 |
| Class E | 0.1 | 0.1 | 0.6 | -0.1 | -0.2 | -0.1 |
| All | 0.3 | 0.2 | 0.3 | 0.2 | -0.1 | 0.0 |
| Enc Time[%] | 100% |  |  | 97% |  |  |
| Dec Time[%] | 101% |  |  | 91% |  |  |

Table 5: Coding loss when TMVP is disabled for merge and AMVP every 8 frames (proposed)

# Conclusions

It has been cross-verified that the results provided by Microsoft in [2] are accurate. It is recommended to adopt the removal of  TMVP from the merge mode, and adding a high-level flag (or a new NAL unit type) to disable the TMVP in the skip and motion vector coding MVP list for error resiliency purpose.

# References

- JCT-VC, “Common test conditions and software reference configurations”, JCTVC-C600, 4th JCT-VC Meeting, Daegu, KR, 20-28 January, 2011

- JCT-VC, “An investigation on robust parsing” JCTVC-E148, 5th JCT-VC Meeting, 5th Meeting: Geneva, CH, 16-23 March, 2011