| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 5th Meeting: Geneva, CH, 16-23 March, 2011 | Document: JCTVC-E122 WG11 Number: m 19637 |
| --- | --- |

| Title: | Cross-verification of Mitsubishi proposal JVCTV-E064 on Improvement to AMVP/Merge process |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Minhua Zhou Texas Instruments Inc., USA | Tel: Email: | +1-214-480-3816 zhou@ti.com |
| Source: | Texas Instruments Inc. |  |  |

_____________________________

# Abstract

The document reports cross-check results of Mitsubishi proposal JCTVC-E064 on Improvement to AMVP/Merge process. It has been verified that the results provided by Mitsubishi for the following tools are accurate: 1) use scaled spatial MVP for the AMVP MVP list if the spatial MVP of same reference index is unavailable, 2) disable the use of temporally collocated motion information in both AMVP and merge mode when PU size is smaller than or equal to 8x8. The total gain measured is -0.1% in HE-RA, -0.3% in LC-RA, -0.1% in HE-LD and -0.3% in LC-LD, respectively.  The source code is also checked to verify the accuracy of Mitsubishi proposal description.

# Introduction

The document reports cross-check results of Mitsubishi proposal JCTVC-E064 on Improvement to AMVP/Merge process [2]

# Algorithm description

Mitsubishi proposal has two parts:

- derivation of spatial MVPs for he skip/motion vector coding (AMVP) MVP list generation: if the left spatial MVP of same reference index is not available, use scaled spatial MVP of different reference index from the bottom left neighbouring PU(if available); likewise, if the upper spatial MVP of same reference index is not available, use scaled spatial MVP of different reference index from the upper right neighbouring PU  (if available).

- disable the use of temporally collocated motion information in both AMVP and merge mode when  PU size is smaller than or equal to 8x8.

# Test Settings and Conditions

The simulations of this document have used HM2.0 software, the simulation platform is LSF equipped with Intel(R) Xeon(R) CPU X5570@2.93GHz 64 bits Linux machines, the common test conditions and reference configurations specified in [1] are followed.

# Experimental Results

As shown in Table 1, the total gain of Mitsubishi proposal JVCTV-E064 is -0.1% in HE-RA, -0.3% in LC-RA, -0.1% in HE-LD and -0.3% in LC-LD, respectively. This result perfectly match the one provided by Mitsubishi.

|  | Random access |  |  | Random access LoCo |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | -0.1 | -0.2 | -0.4 | -0.3 | -0.5 | -0.4 |
| Class B | -0.1 | 0.0 | 0.0 | -0.4 | -0.3 | -0.3 |
| Class C | -0.2 | -0.2 | -0.2 | -0.4 | -0.2 | -0.3 |
| Class D | -0.1 | -0.2 | -0.2 | -0.2 | -0.4 | -0.3 |
| Class E |  |  |  |  |  |  |
| All | -0.1 | -0.1 | -0.2 | -0.3 | -0.3 | -0.3 |
| Enc Time[%] | 99% |  |  | 95% |  |  |
| Dec Time[%] | 102% |  |  | 90% |  |  |
|  | Low delay |  |  | Low delay LoCo |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A |  |  |  |  |  |  |
| Class B | -0.1 | -0.2 | -0.3 | -0.4 | -0.4 | -0.1 |
| Class C | 0.0 | 0.0 | -0.1 | -0.2 | 0.0 | -0.2 |
| Class D | -0.2 | -0.3 | -1.0 | -0.2 | 0.0 | 0.2 |
| Class E | -0.3 | -0.6 | -0.5 | -0.3 | -0.1 | 0.2 |
| All | -0.1 | -0.2 | -0.5 | -0.3 | -0.2 | 0.0 |
| Enc Time[%] | 100% |  |  | 97% |  |  |
| Dec Time[%] | 103% |  |  | 93% |  |  |

Table 1: Total coding gain of JCTVC-E064 proposal

# Conclusions

It has been cross-verified that the results provided by Mitsubishi in [2] are accurate. However it is unclear which part of proposal contributes the gain due to lack of breakdown numbers. It is recommended to conduct further study on complexity and coding efficiency trade-off of the scaled spatial MVP, and on whether disabling small block size temporal co-located motion vectors is truly beneficial under the consideration of motion information compression.

# References

- JCT-VC, “Common test conditions and software reference configurations”, JCTVC-C600, 4th JCT-VC Meeting, Daegu, KR, 20-28 January, 2011

- JCT-VC, “Improvement to AMVP/Merge process” JCTVC-E064, 5th JCT-VC Meeting, Geneva, CH, 16-23 March, 2011