| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 5th Meeting: Geneva, CH, 16-23 March, 2011 | Document: JCTVC-E123 WG 11Number :m19638 |
| --- | --- |

| Title: | CE7: Cross Check Report for JCTVC-E098 on Mode-Dependent Intra Residual Coding |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Ankur Saxena Felix C. Fernandes Elena Alshina Vadim Seregin | Email: | asaxena@sta.samsung.com felix.f@sta.samsung.com elena_a.alshina@samsung.com v.seregin@samsung.com |
| Source: | Samsung Electronics Co., Ltd |  |  |

_____________________________

# Abstract

This contribution summarizes Samsung’s cross-check for JCTVC-E098 proposal on Mode Dependent Intra Residual Coding for Intra HE, Intra Low Complexity, Random Access High Efficiency and Random Access Low Complexity settings, when alternate transforms are applied. Other results in JCTVC-E098 with Directional Scans have been verified by BBC in JCTVC-E126. The simulation results as verified by Samsung match closely with the results provided by the proponents in JCTVC-E098.

# Introduction

JCTVC-E098 [7] proposes a combination of tools for coding the intra prediction residuals by using the information in the intra prediction mode. Specifically, the proponents apply a mode-dependent DCT/DST spatial transform scheme for coding intra-prediction residues, and a Directional Scan. Mode-dependent DCT/DST has been previously proposed in literature in [1,2,3,4]. It was also shown in [3,5] that DST can utilize the same quantization matrices as DCT previously. Here is the brief summary of the proposed tool in JCTVC-E098:

Table 1 Summary for Tools in JCTVC-E098

| Applied for Transform Unit sizes | 4*4, 8*8 |
| --- | --- |
| Number of additional transform matrices at Size 4*4 and 8*8 | 1 |
| HM 2.0 Quantization matrix re-use | Yes |
| 4*4 Transform | Fast (Does not require full matrix multiplication) |
| 8*8 DST Implementation | Full Matrix Multiplication |
| Scan modification with HM 2.0 Anchor | Yes, only for Low Complexity Settings |

# Mode Mappings

The following mode-mappings are used in JCTVC-E098 for mapping the intra-prediction modes to DCT or DST.

Table 1: Mapping from intra prediction modes in to DCT/DST along the vertical and horizontal directions

|  | Mode | Vertical Transform | Horizontal Transform |
| --- | --- | --- | --- |
| 0 | VER | DST | DCT |
| 1 | HOR | DCT | DST |
| 2 | DC | DCT | DCT |
| 3 | VER-8 | DST | DST |
| 4 | VER-4 | DST | DST |
| 5 | VER+4 | DST | DCT |
| 6 | VER+8 | DST | DCT |
| 7 | HOR-4 | DST | DST |
| 8 | HOR+4 | DCT | DST |
| 9 | HOR+8 | DCT | DST |
| 10 | VER-6 | DST | DST |
| 11 | VER-2 | DST | DCT |
| 12 | VER+2 | DST | DCT |
| 13 | VER+6 | DST | DCT |
| 14 | HOR-6 | DST | DST |
| 15 | HOR-2 | DCT | DST |
| 16 | HOR+2 | DCT | DST |
| 17 | HOR+6 | DCT | DST |
| 18 | VER-7 | DST | DST |
| 19 | VER-5 | DST | DST |
| 20 | VER-3 | DST | DST |
| 21 | VER-1 | DST | DCT |
| 22 | VER+1 | DST | DCT |
| 23 | VER+3 | DST | DCT |
| 24 | VER+5 | DST | DCT |
| 25 | VER+7 | DST | DCT |
| 26 | HOR-7 | DST | DST |
| 27 | HOR-5 | DST | DST |
| 28 | HOR-3 | DST | DST |
| 29 | HOR-1 | DCT | DST |
| 30 | HOR+1 | DCT | DST |
| 31 | HOR+3 | DCT | DST |
| 32 | HOR+5 | DCT | DST |
| 33 | HOR+7 | DCT | DST |

# Simulation Results

The test conditions as stipulated in Core Experiment 7 [7] were followed for conducting the simulations. Tests were conducted on Intra High Efficiency, Intra Low Complexity, Random Access High Efficiency, and Random Access Low Complexity configurations, when the DCT/DST Transform was applied only. The proponents also provided results for Intra Low Complexity, and Random Access Low Complexity settings, when Directional Scans were additionally applied together with DCT/DST. Cross-verification for the Transform + Scan settings was performed by BBC in [8].

## R-D Results

Table 2 shows the simulation results for JCTVC-E098 proposal as cross-checked by Samsung. Detailed results are available in the attached Excel sheet. The test results matched closely with those provided by the proponents.

Table 1: Simulation Results for JCTVC-E098 proposal mode-dependent intra residual coding in Core Experiment, as cross-checked by Samsung

|  |  | Intra |  |  | Intra LoCo |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |  |
|  | Class A | -1.2 | -1.8 | -1.5 | -2.1 | -0.5 | -0.3 |  |
|  | Class B | -0.6 | -1.3 | -1.3 | -1.2 | -1.1 | -1.0 |  |
|  | Class C | -0.9 | -1.2 | -1.2 | -1.6 | -1.3 | -1.2 |  |
|  | Class D | -1.0 | -1.2 | -1.2 | -1.6 | -1.1 | -1.2 |  |
|  | Class E | -1.1 | -2.1 | -1.8 | -2.2 | -0.8 | -1.1 |  |
|  | All | -0.9 | -1.5 | -1.4 | -1.7 | -1.0 | -1.0 |  |
|  | Enc Time[%] | 102% |  |  | 102% |  |  |  |
|  | Dec Time[%] | 102% |  |  | 101% |  |  |  |
|  |  | Random access |  |  | Random access LoCo |  |  |  |
|  |  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |  |
|  | Class A | -0.4 | -0.9 | -0.6 | -0.7 | -0.2 | 0.2 |  |
|  | Class B | -0.3 | -0.4 | -0.1 | -0.5 | -0.3 | -0.3 |  |
|  | Class C | -0.5 | -0.6 | -0.5 | -0.8 | -0.6 | -0.5 |  |
|  | Class D | -0.4 | -0.5 | -0.4 | -0.7 | -0.5 | -0.6 |  |
|  | Class E |  |  |  |  |  |  |  |
|  | All | -0.4 | -0.6 | -0.4 | -0.7 | -0.4 | -0.3 |  |
|  | Enc Time[%] | 104% |  |  | 102% |  |  |  |
|  | Dec Time[%] | 102% |  |  | 103% |  |  |  |

## Qualitative Results

We did not notice any subjective differences in the visual quality of the coded sequences.

## Training of Matrices

The proposed transforms at size 4*4 and 8*8 seem to be closely related to DST Type 7. Specifically, to utilize the same quantization core in HM 2.0, the DST Transforms were appropriately scaled in [5]. For the 4*4 DST, the coefficient in the proposed matrices in JCTVC-E098 differ to those proposed in [5] by +/- 2.

## Complexity

The 4*4 DST matrix is implemented via a fast transform. The 8*8 transform is implemented via full matrix multiplication.

Bit Shifts: The DST transform for both 4*4 and 8*8 case is shifted by 6 bits.

## Scanning

The directional scans are introduced for block-sizes 16*16 and 32*32 for the Low Complexity Entropy Coding setting only. These scans use the same mapping table as the conventional HM 2.0 scans at size 4*4 and 8*8. For High Efficiency settings, the conventional HM 2.0 scans are retained.

## Operation Counts

Detailed results for operation counts are in the attached file JCTVC-E123-OperationCounts.xls.

# Conclusions

We have cross-checked JCTVC-E098 as part of Core Experiment 7, studied their algorithms and verified the simulation results.

# References

1. J. Han, A. Saxena and K. Rose, “Towards jointly optimal spatial prediction and adaptive transform in video/image coding,” IEEE International Conference on Acoustics, Speech and Signal Processing (ICASSP), March 2010, pp. 726-729.

2. C. Yeo, Y. H. Tan, Z. Li and S. Rahardja,  “Mode-dependent fast separable KLT for block-based intra coding,” ITU-T JCTVC-B024, Geneva, Switzerland, July 2010.

3. A. Saxena and F. Fernandes, “Jointly optimal intra prediction and adaptive primary transform,” ITU-T JCTVC-C108, Guangzhou, China, October 2010.

4.C. Yeo, Y. H. Tan, Z. Li and S. Rahardja,  “Results for mode-dependent fast separable KLT for block-based intra coding,” ITU-T JCTVC-C037, Guangzhou, China, October 2010.

5. A. Saxena and F. Fernandes, “CE7: Mode-dependent DCT/DST for intra prediction in video coding," JCTVC-D033, Daegu, Korea, January 2011.

6. R. Cohen et al., “CE7: Alternative Transforms,” JCTVC-D607, Daegu, Korea, Jan 2011.

7. R. Joshi et al., “Mode dependent intra residual coding”, JCTVC-E098, Geneva, Switzerland, March 2011.

8. A. Gabriellini and M. Mrak, “Cross Check Report for Mode Dependent Intra Residual Coding (JCTVC-E098) by BBC,” JCTVC-E126, Geneva, Switzerland, March 2011.