| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 5th Meeting: Geneva, CH, 16-23 March, 2011 | Document: JCTVC-E124 WG 11Number :m19639 |
| --- | --- |

| Title: | CE 6e : Cross Check for DoCoMo's Proposal JCTVC-E321 on Planar mode experiments and results |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Ankur Saxena, Felix C. Fernandes 1301 E. Lookout Drive, Richardson, TX – 75082, USA. | Tel: Email: | 1-9727617761, 1-9727617427 asaxena@sta.samsung.com, felix.f@sta.samsung.com |
| Source: | Samsung Electronics Co., Ltd |  |  |

_____________________________

# Abstract

This contribution summarizes Samsung’s cross-check for JCTVC-E321 proposal on Planar mode experiments and Experiments by DoCoMo in Core Experiment 6 for a particular setting CE_6_1c_2_a_3b. The purpose of this cross-check is to comment on the visual quality of Kimono sequence encoded in Random Access High Efficiency and Low Complexity settings at low QP values between the HM 2.0 Anchors and the proposed technique in JCTVC-E321 (CE 6_1c_2_a_3b). The results from Samsung’s simulations for BD rates closely match with the results provided by the proponents. The visual quality of the decoded video sequences is observed to show improvements, especially at QP=37 for both the Random Access High Efficiency and Low Complexity cases.

# Introduction

JCTVC-E321 [1] proposes various experiments in the context of Planar modes and the combination of DST Type 4. The simulations were performed on experiment CE 6_1c_2a_3b, with the following macro definitions:

Macro settings for 1.c + 2.a + 3.b

#define REPLACE_DC_MODE_WITH_PLANAR           1

#define CODE_PREDERROR_DIFFERENTLY            0

#define ADD_PLANAR_MODE                       1

# Simulation Results

## R-D Results

The Rate, and PSNR values for the Kimono sequence for the Random Access Settings HE and LC are given in the attached excel sheet. These match exactly with those provided by the proponents.

## Qualitative Results

The visual quality of the decoded Kimono sequence was observed to show improvements, especially at QP=37 for both the Random Access High Efficiency and Low Complexity cases in the sky region.

# Conclusions

We have verified the simulation results: Rate and PSNR values for Kimono sequence in Random Access High Efficiency and Low Complexity settings for JCTVC-E321 as part of Core Experiment 6_1c_2a_3b.

# References

1. S. Kanumuri and F. Bossen, “CE6.e/f: Planar mode experiments and results” JCTVC-E321, Geneva, Switzerland, March 2011.