| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 5th Meeting: Geneva, CH, 16-23 March, 2011 | Document: JCTVC-E125 WG 11Number :m19640 |
| --- | --- |

| Title: | CE7: Mode-dependent DCT/DST without 4*4 full matrix multiplication for intra prediction |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Ankur Saxena, Felix C. Fernandes 1301 E. Lookout Drive, Richardson, TX – 75082, USA. | Tel: Email: | 1-9727617761, 1-9727617427 asaxena@sta.samsung.com, felix.f@sta.samsung.com |
| Source: | Samsung Electronics Co., Ltd |  |  |

_____________________________

# Abstract

It was previously shown in JCTVC-C108 and JCTVC-D033 that following intra prediction, the optimal transform is DST Type-7 with performance close to KLT, along the direction of prediction. In this contribution, the conventional DCT or DST-Type 7 (separably along the vertical or horizontal directions) is proposed based on the intra prediction direction. The proposed DCT/DST transform scheme is applied only at block sizes 4*4 and 8*8, while the conventional DCT is retained at larger block sizes. No additional signaling information or R-D search is required during the encoding, and the algorithm works in a single-pass. Further, a fast DST at size 4*4 is proposed. The conventional quantization tables for HM 2.0 are retained and no changes have been made to the scanning order. Experimental results are provided with HM 2.0 as anchor for the test conditions as stipulated in Core Experiment 7, and average BD Rate gains of 1.0 %, 1.8 %, 0.4 % and 0.7 % are respectively obtained for Intra High Efficiency, Intra Low Complexity, Random Access High Efficiency and Random Access Low Complexity settings.

# Introduction

In our previous proposals [1] at Guangzhou and [2] in Daegu, we proposed using either the conventional DCT or a DST for the intra prediction residue based on the prediction mode. It was originally shown by Han et al. in [3], that following intra prediction, the optimal transform for vertical (respectively horizontal) modes along the vertical (respectively horizontal) direction of prediction is the following Type -7 DST:

with performance close to KLT (In the above DST equation, the block size is N*N, and i,j are the row and column indices of the 2-d DST matrix). In [1], we showed that the above DST is the optimal transform for a class of oblique modes in unified intra prediction. We also showed how to choose between the transforms: the conventional DCT and the DST per direction based on the intra prediction mode in unified intra prediction [4]. An independent study, where DST was shown to be the optimal transform for intra-prediction residues was also performed in [5,6].

In this contribution, we present our experimental results for the stipulated test conditions in [7].

# Mapping from Unified Intra Directions to DCT/DST

We use the following mapping for choosing the transforms: DCT or DST along the vertical or horizontal direction, based on the mode in unified intra prediction:

Table 1: Mapping from intra prediction modes in to DCT/DST along the vertical and horizontal directions

|  | Mode | Vertical Transform | Horizontal Transform |
| --- | --- | --- | --- |
| 0 | VER | DST | DCT |
| 1 | HOR | DCT | DST |
| 2 | DC | DCT | DCT |
| 3 | VER-8 | DST | DST |
| 4 | VER-4 | DST | DST |
| 5 | VER+4 | DST | DCT |
| 6 | VER+8 | DST | DCT |
| 7 | HOR-4 | DST | DST |
| 8 | HOR+4 | DCT | DST |
| 9 | HOR+8 | DCT | DST |
| 10 | VER-6 | DST | DST |
| 11 | VER-2 | DST | DCT |
| 12 | VER+2 | DST | DCT |
| 13 | VER+6 | DST | DCT |
| 14 | HOR-6 | DST | DST |
| 15 | HOR-2 | DCT | DST |
| 16 | HOR+2 | DCT | DST |
| 17 | HOR+6 | DCT | DST |
| 18 | VER-7 | DST | DST |
| 19 | VER-5 | DST | DST |
| 20 | VER-3 | DST | DST |
| 21 | VER-1 | DST | DCT |
| 22 | VER+1 | DST | DCT |
| 23 | VER+3 | DST | DCT |
| 24 | VER+5 | DST | DCT |
| 25 | VER+7 | DST | DCT |
| 26 | HOR-7 | DST | DST |
| 27 | HOR-5 | DST | DST |
| 28 | HOR-3 | DST | DST |
| 29 | HOR-1 | DCT | DST |
| 30 | HOR+1 | DCT | DST |
| 31 | HOR+3 | DCT | DST |
| 32 | HOR+5 | DCT | DST |
| 33 | HOR+7 | DCT | DST |

# Simulation Results

The test conditions as stipulated in Core Experiment 7 [7] were followed for conducting the simulations. The anchor was HM 2.0 with the conventional DCT, and the performance of the proposed mode-dependent DCT/DST transform scheme was evaluated for the following 4 settings: Intra High Efficiency, Intra Low Complexity, Random Access High Efficiency, and Random Access Low Complexity. The simulations were performed in a Linux 64 bit environment.

Table 2 shows the simulation results when our proposed scheme is applied at sizes 4*4 and 8*8. Detailed results are available in the attached Excel sheet.

Table 2: Simulation Results when mode-dependent DCT/DST applied at block sizes 4*4, 8*8

| Intra; Intra LoCo. Y BD-rate; U BD-rate; V BD-rate; Y BD-rate; U BD-rate; V BD-rate. Class A; -1.2; -1.7; -1.4; -2.1; -0.3; -0.2. Class B; -0.7; -1.2; -1.2; -1.3; -0.9; -0.8. Class C; -0.9; -1.1; -1.1; -1.7; -1.1; -1.0. Class D; -1.0; -1.0; -1.1; -1.7; -1.0; -1.0. Class E; -1.2; -2.0; -1.8; -2.3; -0.7; -1.0. All; -1.0; -1.4; -1.3; -1.8; -0.8; -0.8. Enc Time[%]; 102%; 104%. Dec Time[%]; 100%; 101%. Random access; Random access LoCo. Y BD-rate; U BD-rate; V BD-rate; Y BD-rate; U BD-rate; V BD-rate. Class A; -0.4; -0.7; -1.0; -0.7; -0.1; 0.1. Class B; -0.3; -0.4; -0.1; -0.6; -0.2; -0.2. Class C; -0.5; -0.5; -0.4; -0.8; -0.5; -0.3. Class D; -0.5; -0.4; -0.3; -0.7; -0.1; -0.5. Class E. All; -0.4; -0.5; -0.4; -0.7; -0.2; -0.2. Enc Time[%]; 102%; 102%. Dec Time[%]; 102%; 100%. |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

## Qualitative Results

We did not notice any subjective differences in the visual quality of the coded sequences.

## Training of Matrices

The proposed DST was derived analytically. No training was performed on any video sequence.

## Complexity

Quantization & Scaling Matrices: We retain the same quantization and inverse quantization matrices as used in HM 2.0.

Transform Matrices We only require the storage of one additional matrix corresponding to DST at block size 8*8. The 4*4 matrix is implemented via a fast transform. The 8*8 transform is implemented via full matrix multiplication.

Bit Precision: In our implementation, we have used the following number of bits for DST precision.

1. 4*4 Forward and Backward DST: 6-bit shift.

2. 8*8 DST: 6-bit shift.

Scanning: In this proposal we have retained the conventional HM 2.0 scans

# Conclusions

In this contribution, we have presented the results for our earlier proposals JCTVC-C108 and JCTVC-D033 in Core Experiment 7 for using mode-dependent DCT or DST transform [1,2] for intra-prediction residues instead of the conventional DCT. The proposed mode-dependent DCT/DST scheme requires the storage of only 1 additional DST matrix at a block-size, is based on the intra prediction mode, and does not require any explicit signaling. There is no requirement of introducing additional quantization matrices in our proposal. Simulation results show significant gains in compression performance as compared to HM 2.0 anchors. We recommend adopting this proposal into HM.

# References

1.  A. Saxena and F. Fernandes, “Jointly optimal intra prediction and adaptive primary transform,” ITU-T JCTVC-C108, Guangzhou, China, October 2010.

2. A. Saxena and F. Fernandes, “CE7: Mode-dependent DCT/DST for intra prediction in video coding," JCTVC-D033, Daegu, Korea, January 2011.

3. J. Han, A. Saxena and K. Rose, “Towards jointly optimal spatial prediction and adaptive transform in video/image coding,” IEEE International Conference on Acoustics, Speech and Signal Processing (ICASSP), March 2010, pp. 726-729.

4. Min et al., “Unification of the Directional Intra Prediction Methods in TMuC,” ITU-T JCTVC-B100_revision02, Geneva, Switzerland, July 2010

5. C. Yeo, Y. H. Tan, Z. Li and S. Rahardja,  “Mode-dependent fast separable KLT for block-based intra coding,” ITU-T JCTVC-B024, Geneva, Switzerland, July 2010.

6. C. Yeo, Y. H. Tan, Z. Li and S. Rahardja,  “Results for mode-dependent fast separable KLT for block-based intra coding,” ITU-T JCTVC-C037, Guangzhou, China, October 2010.

7. R. Cohen et al., “CE7: Alternative Transforms,” JCTVC-D607, Daegu, Korea, Jan 2011.

# Patent rights declaration(s)

Samsung Electronics Co., Ltd., may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).