### 8.5.4 Transformation process for scaled transform coefficients

Inputs of this process are:

–	a variable nS specifying the size of the current transform unit,

–	a (nS)x(nS) array d of scaled transform coefficients with elements dij.

–	a variable cIdx specifying the chroma component of the current block,

–	a variable intraPredMode specifying the intra prediction mode.

Output of this process is residual samples as a (nS)x(nS) array r with elements rij.

The variable trafoPrecisionExt is derived as follows:

–	If nS is greater than 4,

- If cIdx is equal to 0 and bit_depth_luma_minus8 is equal to 0, trafoPrecisionExt is set to 4.

- Otherwise, if cIdx is not equal to 0 and bit_depth_chroma_minus8 is equal to 0, trafoPrecisionExt is set to 4.

- Otherwise, trafoPrecisionExt is set equal to 0.

–	Otherwise, trafoPrecisionExt is set equal to 0.

[Ed: (WJ) derivation of trafoPrecisionExt is duplicated to that of the scaling process. Better to remove one of two.]

The constructed residual samples are derived as specified in the following ordered steps.

- Each (horizontal) row of scaled transform coefficients dij (i, j=0..nS-1) is transformed to eij (i, j=0..nS-1) by invoking the one-dimensional transformation process as specified in subclause  to Error: Reference source not found according to the size of the transform unit nS, with the (nS)x(nS) array d as the input and the output is the (nS)x(nS) array e.

- Each (vertical) column of the resulting matrix eij (i, j=0..nS-1) is transformed to fij (i, j=0..nS-1) by invoking the one-dimensional transformation process as specified in subclause  to Error: Reference source not found according to the size of the transform unit nS, with the (nS)x(nS) array e as the input and the output is the (nS)x(nS) array f.

- If nS is equal to 4 or 8, the residual sample value rij is derived by

rij = ( fij + 25+trafoPrecisionExt ) >> ( 6 + trafoPrecisionExt ), with i, j=0.. (nS)x(nS)-1	(8-1)

- Otherwise (nS is equal to 16 or 32), the residual sample value rij is derived by

rij = ( fij + 29+trafoPrecisionExt ) >> ( 10 + trafoPrecisionExt ), with i, j=0.. (nS)x(nS)-1	(8-2)

#### 8.5.4.1  Transformation process for 4 samples

8.5.4.1.1 Transformation process for horizontal row at size 4

Input of this process is:

-  an array of scaled transform coefficients y with elements yi, with i = 0..3.

–	a variable cIdx specifying the chroma component of the current block,

- - a variable IntraPredMode specifying the intra prediction mode of unified intra prediction of the Luma component of the current block,

- Output of this process is an array z with elements zi, with i = 0.. 3.

4-point one-dimensional inverse transformation process along the horizontal direction is defined as follows:

if cIdx = = 1

[same as AVC]

else

- - a variable isHorizontalTransformDCTDST specifying if the horizontal transform is DCT or DST Type 7 (0 implies DCT, 1 implies DST) is derived from IntraPredMode from the Table 1 below:

- if isHorizontalTransformDCTDST == 0

- [same as AVC]

- else

The 4 -point fast DST inverse transform along the horizontal direction is implemented as follows: for the input [y0 y1, y2 y3]

- First a set of intermediate values  c0, c1, and c2 is calculated as follows:

  - c0 = y0+ y2;

  - c1 = 58 * y1;

  - c2 = y2 + y3;

- The output of the horizontal transform zi  (i=0..3) is then computed as:

  - z0 =  (( 29 * c0 + c1  +  55 *  c2   - ((y3<<3)  + (y3<<2))  ) + rnd_factor ) >> shift_bits

  - z1 = (( 55 * c0 + c1  - 84  *  c2   +  (y3<<4)  + (y3<<1)       +  rnd_factor) >> shift_bits

  - z2 =  (( 74 * (y0  - y2) + 58*y3                                            +  rnd_factor ) >> shift_bits

  - z3 =  (( 84 * c0 - c1 -29 *c2      + (y3<<2) + (y3<< 1)           + rnd_factor ) >> shift_bits

where shift_bits = 6 bits

and  rnd_factor = (1<<shift_bits)>>1

8.5.4.1.2 Transformation process for vertical column at size 4

Input of this process is:

-  an array of samples after the horizontal transform, m with elements mi, i = 0..3.

-  a variable cIdx specifying the chroma component of the current block

- - a variable IntraPredMode specifying the intra prediction mode of unified intra prediction of the Luma component of the current block,

- Output of this process is an array x with elements xi, i = 0.. 3.

4-point one-dimensional inverse transformation process along the vertical direction is defined as follows:

if cIdx = = 1

[same as AVC]

else

- - a variable isVerticalTransformDCTDST specifying if the vertical transform is DCT or DST Type 7 (0 implies DCT, 1 implies DST) is derived from IntraPredMode from the Table 1 below:

- if isVerticalTransformDCTDST == 0

- [same as AVC]

- else:

The 4 -point fast DST inverse transform along the vertical direction is implemented as follows: for the input [m0 m1, m2, m3]

First an intermediate set of variables is calculated as:

c0 = m0 + m2

c1 = 58 * m1

c2 = m2 + m3

- The output of the vertical transform xi  (i=0..3) is then computed as:

x0  =  ( 29 * c0  +  c1   +  55 *  c2  - (m3<<3) - (m3<<2)     +  rnd_factor)   >>  shift_bits  ;

x1  =  ( 55 * c0  +  c1   -   84 * c2  + (m3<<4) + (m3<<1)    +  rnd_factor)   >>  shift_bits  ;

x2    =  ( 74 *  c0  - c2 ) +  58 *  c3                                     +  rnd_factor)   >>  shift_bits  ;

x3  =  ( 84 * c0   - c1    -   29 * c2  + (m3<<2) + (m3<<1)    +  rnd_factor)   >>  shift_bits  ;

where shift_bits = 6 bits

and  rnd_factor = (1<<shift_bits)>>1

Table 1 Mapping from intra prediction modes to DCT or DST Type 7 along the vertical and horizontal directions

// 0 implies use DCT, 1 implies DST

| IntraPredMode [ puPartIdx ] | IntraPredType [ puPartIdx ] | IntraPredAngleID [ puPartIdx ] | isVerticalTransformDCTDST | isHorizontalTransformDCTDST |
| --- | --- | --- | --- | --- |
| 0 | Intra_Vertical | 0 | 1 | 0 |
| 1 | Intra_Horizontal | 0 | 0 | 1 |
| 2 | Intra_DC | - | 0 | 0 |
| 3 | Intra_Vertical | -8 | 1 | 0 |
| 4 | Intra_Vertical | -4 | 1 | 0 |
| 5 | Intra_Vertical | +4 | 1 | 0 |
| 6 | Intra_Vertical | +8 | 1 | 0 |
| 7 | Intra_Horizontal | -4 | 0 | 1 |
| 8 | Intra_Horizontal | +4 | 0 | 1 |
| 9 | Intra_Horizontal | +8 | 0 | 1 |
| 10 | Intra_Vertical | -6 | 1 | 0 |
| 11 | Intra_Vertical | -2 | 1 | 0 |
| 12 | Intra_Vertical | +2 | 1 | 0 |
| 13 | Intra_Vertical | +6 | 1 | 0 |
| 14 | Intra_Horizontal | -6 | 0 | 1 |
| 15 | Intra_Horizontal | -2 | 0 | 1 |
| 16 | Intra_Horizontal | +2 | 0 | 1 |
| 17 | Intra_Horizontal | +6 | 0 | 1 |
| 18 | Intra_Vertical | -7 | 1 | 0 |
| 19 | Intra_Vertical | -5 | 1 | 0 |
| 20 | Intra_Vertical | -3 | 1 | 0 |
| 21 | Intra_Vertical | -1 | 1 | 0 |
| 22 | Intra_Vertical | +1 | 1 | 0 |
| 23 | Intra_Vertical | +3 | 1 | 0 |
| 24 | Intra_Vertical | +5 | 1 | 0 |
| 25 | Intra_Vertical | +7 | 1 | 0 |
| 26 | Intra_Horizontal | -7 | 0 | 1 |
| 27 | Intra_Horizontal | -5 | 0 | 1 |
| 28 | Intra_Horizontal | -3 | 0 | 1 |
| 29 | Intra_Horizontal | -1 | 0 | 1 |
| 30 | Intra_Horizontal | +1 | 0 | 1 |
| 31 | Intra_Horizontal | +3 | 0 | 1 |
| 32 | Intra_Horizontal | +5 | 0 | 1 |
| 33 | Intra_Horizontal | +7 | 0 | 1 |