### HM Encoder Description

### 5.5.1 Transform

Transforms of sizes from 4x4 to 32x32 are supported. For transforms of size 4x4, the sub-clause 5.5.1.1 is invoked. For size 8x8, the same method as that specified in the H.264/AVC standard is used. For transforms of 16x16 and 32x32, fast integer transforms using a butterfly structure are used.

5.5.1.1 Transformation process for scaled transform coefficients at size 4*4:

Inputs of this process are:

–	a variable nS specifying the size of the current transform unit with n=4,

–	a (nS)x(nS) array d of residual samples with elements dij.

–	a variable cIdx specifying the chroma component of the current block,

–	a variable intraPredMode specifying the intra prediction mode.

Output of this process is transform coefficient samples as a (nS)x(nS) array r with elements rij.

- Each (horizontal) row of residuals dij (i, j=0..nS-1) is transformed to eij (i, j=0..nS-1) by invoking the one-dimensional transformation process as specified in subclause 5.5.1.1.1 according to the size of the transform unit nS, with the (nS)x(nS) array d as the input and the output is the (nS)x(nS) array e.

- Each (vertical) column of the resulting matrix eij (i, j=0..nS-1) is transformed to fij (i, j=0..nS-1) by invoking the one-dimensional transformation process as specified in subclause 5.5.1.1.2 according to the size of the transform unit nS, with the (nS)x(nS) array e as the input and the output is the (nS)x(nS) array f.

5.5.1.1.1 Transformation process for horizontal row at size 4

Input of this process is:

-  an array of prediction residuals y with elements yi, with i = 0..3.

–  a variable cIdx specifying the chroma component of the current block,

- - a variable IntraPredMode specifying the intra prediction mode of unified intra prediction of the Luma component of the current block,

- Output of this process is an array z with elements zi, with i = 0.. 3.

4-point one-dimensional transformation process along the horizontal direction is defined as follows:

if cIdx = = 1

[same as AVC]

else

- - a variable isHorizontalTransformDCTDST specifying if the horizontal transform is DCT or DST Type 7 (0 implies DCT, 1 implies DST) is derived from IntraPredMode from the Table 1 below:

- if isHorizontalTransformDCTDST == 0

- [same as AVC]

- else

The 4 -point fast DST transform along the horizontal direction is implemented as follows: for the input [y0 y1, y2 y3]

- First a set of intermediate values  c0, c1, and c2 is calculated as follows:

c0 =  y0 + y3

c1 =  y1 + y3

c2 = 74* y2

- The output of the horizontal transform zi  (i=0..3) is then computed as:

z0 = ( 29* c0 + 55 *  c1 + c2                                          + rnd_factor) >> shift_bits ;

z1 = ( 117 * (y0  + y1  - y3  )                                          + rnd_factor) >> shift_bits ;

z2 = ( 84* c0 -  29 * c1 – c2                                          + rnd_factor) >> shift_bits ;

z3 = ( 87* c0 - 133 * c1 + 117* y2                                 + rnd_factor) >> shift_bits ;

where shift_bits = 6 bits

and  rnd_factor = (1<<shift_bits)>>1

5.5.1.1.2 Transformation process for horizontal row at size 4

Input of this process is:

-  an array of samples after the horizontal transform, m with elements mi, i = 0..3.

-  a variable cIdx specifying the chroma component of the current block

- - a variable IntraPredMode specifying the intra prediction mode of unified intra prediction of the Luma component of the current block,

- Output of this process is an array x with elements xi, i = 0.. 3.

4-point one-dimensional inverse transformation process along the vertical direction is defined as follows:

if cIdx = = 1

[same as AVC]

else

- - a variable isVerticalTransformDCTDST specifying if the vertical transform is DCT or DST Type 7 (0 implies DCT, 1 implies DST) is derived from IntraPredMode from the Table 1 below:

- if isVerticalTransformDCTDST == 0

- [same as AVC]

- else:

The 4 -point fast DST inverse transform along the vertical direction is implemented as follows: for the input [m0 m1, m2, m3]

First an intermediate set of variables is calculated as:

c0  =  m0 + m3

c1  =  m1 + m3

c2  = 74* m2

- The output of the vertical transform xi  (i=0..3) is then computed as:

x0 =  (29* c0 + 55 *  c1 + c2                                      +  rnd_factor)  >> shift_bits;

x1 =  (117 * (c0   + c1   -  c3  )                                     +  rnd_factor)  >> shift_bits;

x2 =  (84* c0 -  29 * c1  -  c2                                       +  rnd_factor)  >> shift_bits;

x3 =  (87* c0 - 133 * c1 +       117* c2                                   +  rnd_factor)  >> shift_bits;

where shift_bits = 6 bits

and  rnd_factor = (1<<shift_bits)>>1

Table 1 Mapping from intra prediction modes to DCT or DST Type 7 along the vertical and horizontal directions

// 0 implies use DCT, 1 implies DST

| IntraPredMode [ puPartIdx ] | IntraPredType [ puPartIdx ] | IntraPredAngleID [ puPartIdx ] | isVerticalTransformDCTDST | isHorizontalTransformDCTDST |
| --- | --- | --- | --- | --- |
| 0 | Intra_Vertical | 0 | 1 | 0 |
| 1 | Intra_Horizontal | 0 | 0 | 1 |
| 2 | Intra_DC | - | 0 | 0 |
| 3 | Intra_Vertical | -8 | 1 | 0 |
| 4 | Intra_Vertical | -4 | 1 | 0 |
| 5 | Intra_Vertical | +4 | 1 | 0 |
| 6 | Intra_Vertical | +8 | 1 | 0 |
| 7 | Intra_Horizontal | -4 | 0 | 1 |
| 8 | Intra_Horizontal | +4 | 0 | 1 |
| 9 | Intra_Horizontal | +8 | 0 | 1 |
| 10 | Intra_Vertical | -6 | 1 | 0 |
| 11 | Intra_Vertical | -2 | 1 | 0 |
| 12 | Intra_Vertical | +2 | 1 | 0 |
| 13 | Intra_Vertical | +6 | 1 | 0 |
| 14 | Intra_Horizontal | -6 | 0 | 1 |
| 15 | Intra_Horizontal | -2 | 0 | 1 |
| 16 | Intra_Horizontal | +2 | 0 | 1 |
| 17 | Intra_Horizontal | +6 | 0 | 1 |
| 18 | Intra_Vertical | -7 | 1 | 0 |
| 19 | Intra_Vertical | -5 | 1 | 0 |
| 20 | Intra_Vertical | -3 | 1 | 0 |
| 21 | Intra_Vertical | -1 | 1 | 0 |
| 22 | Intra_Vertical | +1 | 1 | 0 |
| 23 | Intra_Vertical | +3 | 1 | 0 |
| 24 | Intra_Vertical | +5 | 1 | 0 |
| 25 | Intra_Vertical | +7 | 1 | 0 |
| 26 | Intra_Horizontal | -7 | 0 | 1 |
| 27 | Intra_Horizontal | -5 | 0 | 1 |
| 28 | Intra_Horizontal | -3 | 0 | 1 |
| 29 | Intra_Horizontal | -1 | 0 | 1 |
| 30 | Intra_Horizontal | +1 | 0 | 1 |
| 31 | Intra_Horizontal | +3 | 0 | 1 |
| 32 | Intra_Horizontal | +5 | 0 | 1 |
| 33 | Intra_Horizontal | +7 | 0 | 1 |