| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 5th Meeting: Geneva, CH, 16-23 March, 2011 | Document: JCTVC-E125 WG 11Number :m19640 |
| --- | --- |

| Title: | CE7: Mode-dependent DCT/DST without 4*4 full matrix multiplication for intra prediction |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Ankur Saxena, Felix C. Fernandes 1301 E. Lookout Drive, Richardson, TX – 75082, USA. | Tel: Email: | 1-9727617761, 1-9727617427 asaxena@sta.samsung.com, felix.f@sta.samsung.com |
| Source: | Samsung Electronics Co., Ltd |  |  |

_____________________________

# Abstract

It was previously shown in JCTVC-C108 and JCTVC-D033 that following intra prediction, the optimal transform is DST Type-7 with performance close to KLT, along the direction of prediction. In this contribution, the conventional DCT or DST-Type 7 (separably along the vertical or horizontal directions) is proposed based on the intra prediction direction. The proposed DCT/DST transform scheme is applied only at block sizes 4*4 and 8*8, while the conventional DCT is retained at larger block sizes. No additional signaling information or R-D search is required during the encoding, and the algorithm works in a single-pass. Further, a fast DST at size 4*4 is proposed. The conventional quantization tables for HM 2.0 are retained and no changes have been made to the scanning order. Experimental results are provided with HM 2.0 as anchor for the test conditions as stipulated in Core Experiment 7, and average BD Rate gains of 1.0 %, 1.8 %, 0.4 % and 0.7 % are respectively obtained for Intra High Efficiency, Intra Low Complexity, Random Access High Efficiency and Random Access Low Complexity settings.

# Introduction

In our previous proposals [1] at Guangzhou and [2] in Daegu, we proposed using either the conventional DCT or a DST for the intra prediction residue based on the prediction mode. It was originally shown by Han et al. in [3], that following intra prediction, the optimal transform for vertical (respectively horizontal) modes along the vertical (respectively horizontal) direction of prediction is the following Type -7 DST:

[TS]i,j=22N+1sin(2i−1)jπ2N+1

with performance close to KLT (In the above DST equation, the block size is N*N, and i,j are the row and column indices of the 2-d DST matrix). In [1], we showed that the above DST is the optimal transform for a class of oblique modes in unified intra prediction. We also showed how to choose between the transforms: the conventional DCT and the DST per direction based on the intra prediction mode in unified intra prediction [4]. An independent study, where DST was shown to be the optimal transform for intra-prediction residues was also performed in [5,6].

In this contribution, we present our experimental results for the stipulated test conditions in [7].

# Mapping from Unified Intra Directions to DCT/DST

We use the following mapping for choosing the transforms: DCT or DST along the vertical or horizontal direction, based on the mode in unified intra prediction:

Table 1: Mapping from intra prediction modes in to DCT/DST along the vertical and horizontal directions

|  | Mode | Vertical Transform | Horizontal Transform |
| --- | --- | --- | --- |
| 0 | VER | DST | DCT |
| 1 | HOR | DCT | DST |
| 2 | DC | DCT | DCT |
| 3 | VER-8 | DST | DST |
| 4 | VER-4 | DST | DST |
| 5 | VER+4 | DST | DCT |
| 6 | VER+8 | DST | DCT |
| 7 | HOR-4 | DST | DST |
| 8 | HOR+4 | DCT | DST |
| 9 | HOR+8 | DCT | DST |
| 10 | VER-6 | DST | DST |
| 11 | VER-2 | DST | DST |
| 12 | VER+2 | DST | DCT |
| 13 | VER+6 | DST | DCT |
| 14 | HOR-6 | DST | DST |
| 15 | HOR-2 | DST | DST |
| 16 | HOR+2 | DCT | DST |
| 17 | HOR+6 | DCT | DST |
| 18 | VER-7 | DST | DST |
| 19 | VER-5 | DST | DST |
| 20 | VER-3 | DST | DST |
| 21 | VER-1 | DST | DST |
| 22 | VER+1 | DST | DCT |
| 23 | VER+3 | DST | DCT |
| 24 | VER+5 | DST | DCT |
| 25 | VER+7 | DST | DCT |
| 26 | HOR-7 | DST | DST |
| 27 | HOR-5 | DST | DST |
| 28 | HOR-3 | DST | DST |
| 29 | HOR-1 | DST | DST |
| 30 | HOR+1 | DCT | DST |
| 31 | HOR+3 | DCT | DST |
| 32 | HOR+5 | DCT | DST |
| 33 | HOR+7 | DCT | DST |

# Simulation Results

The test conditions as stipulated in Core Experiment 7 [7] were followed for conducting the simulations. The anchor was HM 2.0 with the conventional DCT, and the performance of the proposed mode-dependent DCT/DST transform scheme was evaluated for the following 4 settings: Intra High Efficiency, Intra Low Complexity, Random Access High Efficiency, and Random Access Low Complexity. The simulations were performed in a Linux 64 bit environment.

Experiment 1 (a): DCT/DST at 4*4 and 8*8 case (Slight Change to Inverse Core Transform)

Table 2 shows the simulation results when our proposed scheme is applied at sizes 4*4 and 8*8. Detailed results are available in the attached Excel sheet JCTVC-E125_DST_exp1_Size_4_8.xls. Here we had made minor changes in the Core Transform and Transform Precision Extension to combine the shifting and rounding operations in DCT at the same time. Note that this change in the inverse core transform does not affect the BD Rate performance of the proposed DCT/DST scheme, as can be seen from the next experiment. We would like to thank I2R for conducting the cross-check for this in JCTVC-E099.

Table 2: Simulation Results when mode-dependent DCT/DST applied at block sizes 4*4, 8*8

| Intra; Intra LoCo. Y BD-rate; U BD-rate; V BD-rate; Y BD-rate; U BD-rate; V BD-rate. Class A; -1.2; -1.7; -1.4; -2.1; -0.3; -0.2. Class B; -0.7; -1.2; -1.2; -1.3; -0.9; -0.8. Class C; -0.9; -1.1; -1.1; -1.7; -1.1; -1.0. Class D; -1.0; -1.0; -1.1; -1.7; -1.0; -1.0. Class E; -1.2; -2.0; -1.8; -2.3; -0.7; -1.0. All; -1.0; -1.4; -1.3; -1.8; -0.8; -0.8. Enc Time[%]; 102%; 104%. Dec Time[%]; 100%; 101%. Random access; Random access LoCo. Y BD-rate; U BD-rate; V BD-rate; Y BD-rate; U BD-rate; V BD-rate. Class A; -0.4; -0.7; -1.0; -0.7; -0.1; 0.1. Class B; -0.3; -0.4; -0.1; -0.6; -0.2; -0.2. Class C; -0.5; -0.5; -0.4; -0.8; -0.5; -0.3. Class D; -0.5; -0.4; -0.3; -0.7; -0.1; -0.5. Class E. All; -0.4; -0.5; -0.4; -0.7; -0.2; -0.2. Enc Time[%]; 102%; 102%. Dec Time[%]; 102%; 100%. |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

Experiment 1 (b): DCT/DST only at 4*4; DCT at size 8*8 and above (Slight Change to Inverse Core Transform)

Table 3 shows the simulation results when our proposed scheme is applied at size 4*4. Detailed results are available in the attached Excel sheet JCTVC-E125_DST_exp1_Size_4.xls. We would like to thank BBC for conducting the cross-check for this in JCTVC-E127.

Table 3: Simulation Results when mode-dependent DCT/DST is applied at block sizes 4*4

|  | Intra |  |  | Intra LoCo |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |  |
| Class A | -0.7 | -0.7 | -0.6 | -1.1 | -0.1 | 0.0 |  |
| Class B | -0.6 | -0.6 | -0.6 | -1.0 | -0.3 | -0.2 |  |
| Class C | -0.8 | -0.7 | -0.6 | -1.3 | -0.5 | -0.4 |  |
| Class D | -0.9 | -0.7 | -0.8 | -1.4 | -0.5 | -0.5 |  |
| Class E | -1.0 | -1.0 | -1.1 | -1.6 | -0.3 | -0.5 |  |
| All | -0.8 | -0.7 | -0.7 | -1.3 | -0.3 | -0.3 |  |
| Enc Time[%] | 100% |  |  | 100% |  |  |  |
| Dec Time[%] | 101% |  |  | 101% |  |  |  |
|  |  | Random access |  |  | Random access LoCo |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |  |
| Class A | -0.2 | -0.1 | 0.2 | -0.4 | 0.2 | 0.2 |  |
| Class B | -0.3 | 0.0 | 0.0 | -0.5 | 0.1 | 0.1 |  |
| Class C | -0.5 | -0.3 | -0.2 | -0.6 | -0.1 | -0.1 |  |
| Class D | -0.4 | -0.2 | -0.6 | -0.6 | -0.2 | 0.0 |  |
| Class E |  |  |  |  |  |  |  |
| All | -0.3 | -0.1 | -0.1 | -0.5 | 0.0 | 0.0 |  |
| Enc Time[%] | 103% |  |  | 102% |  |  |  |
| Dec Time[%] | 100% |  |  | 103% |  |  |  |

Experiment 2 (a): DCT/DST at 4*4 and 8*8 case (No Change to Inverse Core Transform)

Table 4 shows the simulation results when our proposed scheme is applied at sizes 4*4 and 8*8. Detailed results are available in the attached Excel sheet JCTVC-E125_DST_exp2_Size_4_8.xls. Here we have made no changes in the Core Transform and Transform Precision Extension so as to combine the shifting and rounding operations in DCT at the same time. We would like to thank I2R for conducting the cross-check for this in JCTVC-E099.

Table 4: Simulation Results when mode-dependent DCT/DST is applied at block sizes 4*4 and 8*8 (No changes to Core Transform)

|  |  |  | Intra |  |  |  |  | Intra LoCo |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Y BD-rate | U BD-rate |  | V BD-rate |  | Y BD-rate |  | U BD-rate |  | V BD-rate |  |  |  |
|  | Class A |  | -1.2 | -1.7 |  | -1.4 |  | -2.1 |  | -0.4 |  | -0.2 |  |  |  |
|  | Class B |  | -0.7 | -1.2 |  | -1.2 |  | -1.3 |  | -0.9 |  | -0.8 |  |  |  |
|  | Class C |  | -0.9 | -1.1 |  | -1.1 |  | -1.7 |  | -1.1 |  | -1.1 |  |  |  |
|  | Class D |  | -1.0 | -1.0 |  | -1.0 |  | -1.7 |  | -1.0 |  | -1.0 |  |  |  |
|  | Class E |  | -1.2 | -2.0 |  | -1.8 |  | -2.3 |  | -0.7 |  | -1.0 |  |  |  |
|  | All |  | -1.0 | -1.4 |  | -1.3 |  | -1.8 |  | -0.8 |  | -0.8 |  |  |  |
|  | Enc Time[%] |  | 102% |  |  |  |  | 100% |  |  |  |  |  |  |  |
|  | Dec Time[%] |  | 100% |  |  |  |  | 103% |  |  |  |  |  |  |  |
|  |  |  |  | Random access |  |  |  |  |  | Random access LoCo |  |  |  |  |  |
|  |  |  | Y BD-rate | U BD-rate |  | V BD-rate |  | Y BD-rate |  | U BD-rate |  | V BD-rate |  |  |  |
|  | Class A |  | -0.4 | -0.5 |  | -0.7 |  | -0.7 |  | 0.0 |  | 0.2 |  |  |  |
|  | Class B |  | -0.3 | -0.3 |  | -0.2 |  | -0.6 |  | -0.1 |  | -0.2 |  |  |  |
|  | Class C |  | -0.5 | -0.6 |  | -0.5 |  | -0.8 |  | -0.5 |  | -0.3 |  |  |  |
|  | Class D |  | -0.4 | -0.4 |  | -0.6 |  | -0.7 |  | -0.2 |  | -0.2 |  |  |  |
|  | Class E |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | All |  | -0.4 | -0.5 |  | -0.5 |  | -0.7 |  | -0.2 |  | -0.1 |  |  |  |
|  | Enc Time[%] |  | 101% |  |  |  |  | 104% |  |  |  |  |  |  |  |
|  | Dec Time[%] |  | 102% |  |  |  |  | 103% |  |  |  |  |  |  |  |

Experiment 2 (b): DCT/DST only at 4*4; DCT at size 8*8 and above (No Changes to Inverse Core Transform)

Table 5 shows the simulation results when mode-dependent DCT/DST is applied at size 4*4. Detailed results are available in the attached Excel sheet JCTVC-E125_DST_exp2_Size_4.xls. We would like tothank DoCoMo and Panasonic for conducting the cross-check for this in JCTVC-E460, and JCTVC-E461 respectively.

Table 5: Simulation Results when mode-dependent DCT/DST is applied at block sizes 4*4 only (No changes to Core Transform)

|  |  | Intra |  |  | Intra LoCo |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |  |
|  | Class A | -0.7 | -0.7 | -0.6 | -1.1 | -0.1 | 0.0 |  |
|  | Class B | -0.6 | -0.6 | -0.6 | -1.0 | -0.3 | -0.2 |  |
|  | Class C | -0.8 | -0.7 | -0.6 | -1.3 | -0.5 | -0.4 |  |
|  | Class D | -0.9 | -0.7 | -0.7 | -1.4 | -0.5 | -0.5 |  |
|  | Class E | -1.0 | -1.1 | -1.1 | -1.6 | -0.3 | -0.5 |  |
|  | All | -0.8 | -0.7 | -0.7 | -1.3 | -0.3 | -0.3 |  |
|  | Enc Time[%] | 102% |  |  | 102% |  |  |  |
|  | Dec Time[%] | 101% |  |  | 100% |  |  |  |
|  |  |  | Random access |  |  | Random access LoCo |  |  |
|  |  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |  |
|  | Class A | -0.2 | 0.1 | -0.4 | -0.4 | 0.1 | 0.2 |  |
|  | Class B | -0.3 | -0.1 | -0.2 | -0.5 | 0.1 | 0.1 |  |
|  | Class C | -0.5 | -0.2 | -0.2 | -0.6 | -0.2 | -0.2 |  |
|  | Class D | -0.4 | -0.4 | -0.4 | -0.6 | -0.1 | -0.4 |  |
|  | Class E |  |  |  |  |  |  |  |
|  | All | -0.3 | -0.2 | -0.3 | -0.5 | 0.0 | -0.1 |  |
|  | Enc Time[%] | 103% |  |  | 104% |  |  |  |
|  | Dec Time[%] | 100% |  |  | 100% |  |  |  |

## Qualitative Results

We did not notice any subjective differences in the visual quality of the coded sequences.

## Training of Matrices

The proposed DST was derived analytically.  No training was performed on any video sequence.

## Complexity

Quantization & Scaling Matrices: We retain the same quantization and inverse quantization matrices as used in HM 2.0.

Transform Matrices : We only require the storage of one additional matrix corresponding to DST at block size 8*8. The 8*8 transform is implemented via full matrix multiplication.

Storage : The 4*4 DST matrix is implemented via a fast algorithm and requires the storage of only 7 elements for the forward DST : {29, 55, 74, 84, 87, 117, and 133} while the inverse transform requires only 5 elements {29,55,58,74,84}. Note that some of the elements are common and the actual storage is only 8 distinct elements for both forward and inverse DST.

Bit Precision: In our implementation, we have used the following number of bits for DST precision.

1. 4*4 Forward and Backward DST: 6-bit shift.

2. 8*8 DST: 6-bit shift.

Scanning: In this proposal we have retained the conventional HM 2.0 scans

## 3.4 Fast 4*4 DST Transform Implementation

The forward 4*4 DST matrix with input [x(0),x(1),x(2), x(3)] and output [y(0),y(1),y(2), y(3)] is conceptually implemented via a fast algorithm as follows:

c[0] =	x[0] + x[3];

c[1]  = x[1] + x[3];

c[2]  = 74* x[2];

y[0] =  29* c[0] + 55 * c[1] + c[2]

y[1] =  117 * x[0]  + x[1] - x[3])

y[2] =  84* c[0] -  29 * c [1] - c[2]

y[3] =  87* c[0] - 133 * c[1] + 117* x[2]

The number of multiplications are 9, and the number of additions are 10, instead of full matrix multiplication, which will have 16 multiplications, and 12 adds.

Similarly, the backward DST with input [y(0),y(1),y(2), y(3)] and reconstructed output [xr(0),xr(1),xr(2), xr(3)] is conceptually implemented as follows:

c0 = y[0] + y[2];

c1 = 58 * y[1];

c2 = y[2] + y[3];

xr[0] =  29 * c0 + c1  +  55 *  c2   - ((y[3]<<3)  + (y[3]<<2))

xr [1] =  55 * c0 + c1  - 84  *  c2   +  (y[3]<<4)  + (y[3]<<1)

xr [2] =  74 * (y[0]  - y[2]) + 58*y[3]

xr[3] =  84 * c0 - c1 -29 *c2      + (y[3]<<2) + (y[3]<<1)

The number of multiplications are 9, and the number of shifts and additions are 6 and 16 respectively instead of full matrix multiplication, which will have 16 multiplications, and 12 adds. Note that in the above 4*4 DST description, we have only shown the concept of performing DST calculations, and ignored the bit-shifting rounding operations and addition of offsets that we have implemented as is generally performed in video coding. e.g., y[0] in actual implementation is evaluated as :

y[0] =  (29* c[0] + 55 * c[1] + c[2]  + offset) >> shift_bits

where shift_bits = 6, and offset = (1<<shift_bits)>1.

## 3.5  8*8 DST Transform Matrix

The same 8*8 DST matrix presented in [2] is retained in implementation in this proposal. In [2], the 8*8 DST matrix was shifted by 7 bits. Here we shift the DST matrix by 6 bits only. The loss in compression performance from going from 7 bits to 6 bits is negligible.

const short g_as_DST_MAT_8 [8][8] =

{

// Shifted by 6 bits

{    16,    49,    56,    90,    88,    84,    47,    34,},

{    32,    84,    67,    49,   -16,   -75,   -70,   -63,},

{    46,    93,    25,   -63,   -85,   -17,    56,    84,},

{    59,    75,   -37,   -84,    32,    90,   -13,   -93,},

{    70,    34,   -70,    17,    79,   -63,   -37,    90,},

{    79,   -17,   -47,    93,   -46,   -34,    67,   -75,},

{    85,   -63,    13,    34,   -70,    93,   -63,    49,},

{    88,   -90,    63,   -75,    59,   -49,    25,   -17,},

};

## 3.6 Operation Counts

Detailed operation counts together with encoding and decoding time for various experiments are given in the excel sheet JCTVC-E125-OperationCounts.xls and a brief summary of only operations counts is given in JCTVC-E125-OnlyOperationCountsSummary_NoEncDecTime.xls. We also provide this brief summary for the number of operations as follows:

| Evaluation Metrics |  |  | Core HM transform | Samsung Proposal E125 Experiment 1 | Samsung Proposal E125 Experiment 2 | Comments |
| --- | --- | --- | --- | --- | --- | --- |
| 4x4 | Average Transform Arithmetic Operations (ops/pixel) | Mults | 0.0 | 2.3 | 2.3 |  |
|  |  | Adds | 5.0 | 7.0 | 7.5 |  |
|  |  | Shifts | 2.0 | 3.0 | 3.5 |  |
|  | Worst-Case Transform Arithmetic Operations (ops/pixel) | Mults | 0.0 | 4.5 | 4.5 |  |
|  |  | Adds | 5.0 | 9.0 | 10.0 |  |
|  |  | Shifts | 2.0 | 4.0 | 5.0 |  |
|  | Bit Precision | Max. required (bits) | 32 | 32 | 32 |  |
|  | Memory requirements | Additional for transform (bytes) | 0 | 5 | 5 |  |
| 8x8 | Average Transform Arithmetic Operations (ops/pixel) | Mults | 0.0 | 8.0 | 8.0 |  |
|  |  | Adds | 10.0 | 12.5 | 13.0 |  |
|  |  | Shifts | 5.5 | 3.75 | 3.75 |  |
|  | Worst-Case Transform Arithmetic Operations (ops/pixel) | Mults | 0.0 | 16.0 | 16.0 |  |
|  |  | Adds | 10.0 | 16.0 | 16.0 |  |
|  |  | Shifts | 5.5 | 4.5 | 5.5 |  |
|  | Bit Precision | Max. required (bits) | 32 | 32 | 32 |  |
|  | Memory requirements | Additional for transform (bytes) | 0 | 64 | 64 |  |
| Misc. | Quantization | Additional quantization matrices | 0 | 0 | 0 |  |
|  |  | Size of additional quantization matrices (bytes) | 0 | 0 | 0 |  |
|  | Scan orders | Additional scan orders | 0 | 0 | 0 |  |
|  |  | Mode mapping table size (bytes) | 0 | 0 | 0 |  |
|  | Additional R-D loops? (yes/no) | encoder | No | No | No |  |
|  |  | decoder | No | No | No |  |

# Conclusions

In this contribution, we have presented the results for our earlier proposals JCTVC-C108 and JCTVC-D033 in Core Experiment 7 for using mode-dependent DCT or DST transform [1,2] for intra-prediction residues instead of the conventional DCT. The proposed mode-dependent DCT/DST scheme requires the storage of only 1 additional DST matrix at a block-size, is based on the intra prediction mode, and does not require any explicit signaling. There is no requirement of introducing additional quantization matrices in our proposal. Simulation results show significant gains in compression performance as compared to HM 2.0 anchors. We recommend adopting this proposal into HM.

# References

1.  A. Saxena and F. Fernandes, “Jointly optimal intra prediction and adaptive primary transform,” ITU-T JCTVC-C108, Guangzhou, China, October 2010.

2. A. Saxena and F. Fernandes, “CE7: Mode-dependent DCT/DST for intra prediction in video coding," JCTVC-D033, Daegu, Korea, January 2011.

3. J. Han, A. Saxena and K. Rose, “Towards jointly optimal spatial prediction and adaptive transform in video/image coding,” IEEE International Conference on Acoustics, Speech and Signal Processing (ICASSP), March 2010, pp. 726-729.

4. Min et al., “Unification of the Directional Intra Prediction Methods in TMuC,” ITU-T JCTVC-B100_revision02, Geneva, Switzerland, July 2010.

5. C. Yeo, Y. H. Tan, Z. Li and S. Rahardja,  “Mode-dependent fast separable KLT for block-based intra coding,” ITU-T JCTVC-B024, Geneva, Switzerland, July 2010.

6. C. Yeo, Y. H. Tan, Z. Li and S. Rahardja,  “Results for mode-dependent fast separable KLT for block-based intra coding,” ITU-T JCTVC-C037, Guangzhou, China, October 2010.

7. R. Cohen et al., “CE7: Alternative Transforms,” JCTVC-D607, Daegu, Korea, Jan 2011.

# Patent rights declaration(s)

Samsung Electronics Co., Ltd., may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

7. Annex

Text for HM Encoder Description, and for working draft of the decoder are attached as files JCTVC-E125_HMEncoderDescription.doc and JCTVC-E125_HMDecoderDescription.doc.