| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 5th Meeting: Geneva, 16-23 March, 2011 | Document: JCTVC-E126 WG11 Number: m19641 |
| --- | --- |

| Title: | CE7 Tool 1: Cross Check Report for Mode Dependent Intra Residual Coding (JCTVC-E098) by BBC |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Cross-Check |  |  |
| Author(s) or Contact(s): | Andrea Gabriellini Marta Mrak BBC Research & Development 56 Wood Lane London, W12 7SB, United Kingdom | Tel: Email: Tel: Email: | +44-3030409619 andrea.gabriellini@bbc.co.uk +44-3030409641 marta.mrak@bbc.co.uk |
| Source: | British Broadcasting Corporation |  |  |

_____________________________

# Abstract

This document reports the BBC’s cross-check results for Tool 1 of Core Experiment 7 presented in JCTVC-E098. Two separate tests are carried out investigating a mode-dependent transform and coefficient scanning for intra prediction residuals, respectively. The results for all configurations are consistent with those provided by the proponents in terms of bit rate and measured PSNR.

# Theoretical verification

The source code for the test has been analysed and it appears to work according to the tool’s proponents’ description.

# Experimental verification

Experimental verification has been conducted under the test conditions defined in revision 5 of [JCTVC-D607].

Two tests have been carried out:

- Mode-dependent transforms (DCT/DST) for block sizes 4x4 and 8x8 only, with default HM 2.0 coefficient scanning;

- Alternative coefficient scanning

Test A) has been carried out over 4 test points: intra high-efficiency, intra low-complexity, random access high-efficiency and random access low-complexity.

Test B) has been carried out over 2 test points: intra low-complexity and random access low-complexity.

## Simulation environment

The source code has been compiled and run on a Linux Debian 64bit cluster, with gcc version 4.4.5. The cluster nodes use uniform quad-core Intel X3450 2.66GHz processors, 8GB RAM, running a process per core on average.

# Summary results

The experimental cross-check results agree at least up to minor differences. The BD-rate values perfectly match the proponents’ while some of the running times differ slightly. The differences are compatible with the variations in running time observed on the cluster used for the experiments. These variations are due to the loading of the cluster and the associated file-server.

Negative values of BD-rate equates to reduction in bitrates for the same PSNR.

Table 1: CE7 Tool 1, mode-dependent transforms for intra coding

|  | Intra |  |  | Intra LoCo |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | -1.2 | -1.8 | -1.5 | -2.1 | -0.5 | -0.3 |
| Class B | -0.6 | -1.3 | -1.3 | -1.2 | -1.1 | -1.0 |
| Class C | -0.9 | -1.2 | -1.2 | -1.6 | -1.3 | -1.2 |
| Class D | -1.0 | -1.2 | -1.2 | -1.6 | -1.1 | -1.2 |
| Class E | -1.1 | -2.1 | -1.8 | -2.2 | -0.8 | -1.1 |
| All | -0.9 | -1.5 | -1.4 | -1.7 | -1.0 | -1.0 |
| Enc Time[%] | 99% |  |  | 104% |  |  |
| Dec Time[%] | 99% |  |  | 102% |  |  |

Table 2: CE7 Tool 1, mode-dependent transforms for intra coding

|  | Random access |  |  | Random access LoCo |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | -0.4 | -0.9 | -0.6 | -0.7 | -0.2 | 0.2 |
| Class B | -0.3 | -0.4 | -0.1 | -0.5 | -0.3 | -0.3 |
| Class C | -0.5 | -0.6 | -0.5 | -0.8 | -0.6 | -0.5 |
| Class D | -0.4 | -0.5 | -0.4 | -0.7 | -0.5 | -0.6 |
| Class E |  |  |  |  |  |  |
| All | -0.4 | -0.6 | -0.4 | -0.7 | -0.4 | -0.3 |
| Enc Time[%] | 100% |  |  | 107% |  |  |
| Dec Time[%] | 102% |  |  | 110% |  |  |

Table 3: CE7 Tool 1, mode-dependent coefficient scanning for intra coding

|  | Random access LoCo |  |  | Intra LoCo |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | -1.5 | -0.8 | -0.5 | -2.8 | -0.6 | -0.5 |
| Class B | -1.2 | -0.9 | -0.9 | -2.3 | -1.8 | -1.8 |
| Class C | -0.8 | -0.9 | -0.7 | -1.8 | -1.4 | -1.4 |
| Class D | -0.8 | -0.7 | -0.7 | -2.0 | -1.6 | -1.6 |
| Class E |  |  |  | -3.7 | -1.6 | -1.7 |
| All | -1.1 | -0.8 | -0.7 | -2.5 | -1.4 | -1.4 |
| Enc Time[%] | 111% |  |  | 113% |  |  |
| Dec Time[%] | 109% |  |  | 99% |  |  |

# Conclusions and recommendations

The results for all configurations are consistent with those provided by the proponents in terms of bit rate and measured PSNR.

# References

[JCTVC-E098] R. Joshi, P. Chen, M. Karczewicz, A. Tanizawa, J. Yamaguchi, C. Yeo, Y. H. Tan, H. Yang, H. Yu, “CE7: Mode dependent intra residual coding” JCTVC-E098, Geneva, Switzerland, March 2011.

[JCTVC-D607] R. Cohen, C. Yeo, and R. Joshi, “Description of Core Experiment 7: Alternative Transform”, JCTVC-D607, Daegu, KR, January 2011.