| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 5th Meeting: Geneva, CH, 16-23 March, 2011 | Document: JCTVC-E128 |
| --- | --- |

| Title: | CE3 : Results on Bi/Single MC interpolation filter |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Kenji Kondo and Teruhiko Suzuki Gate City Osaki West 12F 1-11-1 Osaki, Shinagawa-ku Tokyo, 141-0032, JAPAN | Tel: Email: | +81-3-5435-3308 kenji.kondo@jp.sony.com teruhikos@jp.sony.com |
| Source: | Sony Corporation |  |  |

_____________________________

# Abstract

This contribution reports results on enhanced MC filter, Bi/Single (BIS) switching interpolation filter [1, 3], in Core Experiment3 (CE3). In the test, the coding performance and complexity was measured under common test conditions in CE3, which is defined in JCTVC-D603r3 [2]. In this [1], the filter coefficients had 8 bit accuracy. In this contribution, it is changed from 8 to 6 bit in order to compare with HM 2.0. This BIS filter has different 2 sets of coefficients that are for Bi-prediction and Single-prediction. In [1], the filter length was 8 tap both bi-prediction and single-prediction. In this contribution, filter length of single-prediction is changed from 8 to 6 tap to reduce complexity. This BIS filter was implemented in HM 2.0 and experiment was carried out. The test results was 0.0% for RA HE, -0.6% for RA LC, 0.0% for LD HE, and -0.6% for LD LC. The encoding time was 96%, 95%, 97%, and 95% for RA HE, RA LC, LD HE, and LD LC. The decoding time was 99%, 98%, 97% and 97% for RA_HE, RA LC, LD HE, and LD LC.

# Introduction

The goal of CE3 is to further investigate interpolation filtering for motion compensation (Luma) technology. This contribution focuses on BIS filter switching that is one of investigation items.

The basic technique of BIS filter was introduced in [1, 3]. And, we changed bit accuracy and filter length. In [1], the filter coefficients had 8 bit accuracy. The coefficients are changed from 8 to 6 bit accuracy because current DCT-IF 8 tap in HM2.0 is implemented 6 bit. We show the filter coefficients in Table 1.

Furthermore, filter length for single prediction is reduced from 8 to 6 tap for half pel, and 5 tap for quarter pel as shown Table 1.

Table 1: Filter coefficients

| Prediction mode | Sub pel | Tap length | Filter coefficients |
| --- | --- | --- | --- |
| Single prediction | 1/2 pel | 6 | 2 , -7 , 37, 37, -7, 2 |
|  | 1/4 pel | 5 | -3 , 50 , 21 , -6 , 2 |
| Bi-prediction | 1/2 pel | 8 | -2 , 5 , -11 , 40 , 40 , -11 , 5 , -2 |
|  | 1/4 pel | 8 | -1 , 4 , -10 , 58 , 18 , -7 , 3 , -1 |

# Experiment

## Test condition and machine platform

Experiment was carried out based on HM software version 2.0 as follow CE3 mandate. And encoding time and decoding time were measured on same platform machines that spec is Core i7 CPU 2.93GHz, and have 8 GB memory. And the Linux 64 bit (called Cent OS) is installed in the platform machine.

For encode, cluster that consists of above identical platform was used. For decode, only 1 computer in the cluster was used, and decoding time was measured without yuv output.

## Coding condition

High coding efficiency anchor:

Random access configuration

Low delay configuration

Low complexity anchor:

Random access configuration

Low delay configuration

Common test condition is specified in JCTVC-D600 [4].

## Evaluation criteria

- Measure impact on bitrate/PSNR using provided data. Use 4-point BD-PSNR and BD-Rate [4].

- Complexity (encoding/decoding times, and use a common Excel sheet that counts number of operations instruction cycles and memory bandwidth compared to another)

# Test results

Simulation results are shown in Table 2 for random access case.

Table 2: Simulation result for random access case

|  |  | Random access |  |  | Random access LoCo |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | 0.1 | 0.2 | 0.3 | -0.9 | -0.1 | -0.1 |
| Class B | 0.0 | 0.0 | 0.0 | -0.6 | -0.3 | -0.4 |
| Class C | -0.1 | -0.3 | -0.3 | -0.4 | -0.3 | -0.3 |
| Class D | 0.1 | -0.4 | -0.3 | -0.3 | -0.6 | -0.4 |
| Class E |  |  |  |  |  |  |
| All | 0.0 | -0.1 | -0.1 | -0.6 | -0.3 | -0.3 |
| Enc Time[%] | 96% |  |  | 95% |  |  |
| Dec Time[%] | 99% |  |  | 98% |  |  |

Simulation results are shown in Table 3 for low delay case.

Table 3: Simulation result for low delay case

|  | Low delay |  |  | Low delay LoCo |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A |  |  |  |  |  |  |
| Class B | 0.2 | 0.3 | 0.3 | -0.7 | 0.1 | 0.0 |
| Class C | -0.2 | 0.1 | 0.0 | -0.9 | -0.2 | 0.1 |
| Class D | 0.1 | 0.3 | 0.6 | 0.0 | 0.1 | 0.6 |
| Class E | 0.0 | -0.1 | 0.5 | -0.7 | 0.7 | 1.1 |
| All | 0.0 | 0.2 | 0.3 | -0.6 | 0.1 | 0.4 |
| Enc Time[%] | 97% |  |  | 95% |  |  |
| Dec Time[%] | 97% |  |  | 97% |  |  |

# Complexity analysis

## Implementation into HM2.0

In software code, BIS switching is very simple for implementation. Current implementation has two functions which interpolate bi-prediction and single-prediction. It is just to change invoked function by either bi-predictive is used or not. When one block has two references (namely bi-predictive prediction), the function with filter coefficients of bi-prediction is invoked. When one block has only L0 or L1 reference, the function with filter coefficients of single-prediction is invoked. We expect that the time to change called function is negligible compared with FIR process.

## Theoretical complexity analysis

We investigated theoretical complexity analysis using CE3’s excel sheet. The summary of complexity analysis is shown in Table 4 and Table 5. For single prediction, we can see that complexity both operation and bandwidth is decreasing. This is consistent with reducing simulation time.

For bi-prediction, the complexity is almost same as DCTIF 8 tap.

Table 4: Summary of complexity analysis (multiplication case)

| Method |  |  | DCTIF 8-tap | Proposal |  |
| --- | --- | --- | --- | --- | --- |
| add/shift or multiple |  |  | multiple | multiple |  |
| Sheet |  |  | Anchor | Proposal | Ratio (%) |
| Single | Operations | Worst case / pixel | 48.75 | 33.75 | 69% |
|  |  | Average of operations / pixel | 24.08 | 17.39 | 72% |
|  | Bandwith | Worst case / pixel | 7.56 | 5.06 | 67% |
|  |  | Average of bytes / pixel | 2.48 | 1.95 | 79% |
| Bi | Operations | Worst case / pixel | 97.50 | 97.50 | 100% |
|  |  | Average of operations / pixel | 48.17 | 49.50 | 103% |
|  | Bandwith | Worst case / pixel | 15.13 | 15.13 | 100% |
|  |  | Average of bytes / pixel | 4.95 | 4.95 | 100% |
| Total | Operations | Worst case / pixel | 97.50 | 97.50 | 100% |
|  |  | Average of operations / pixel | 36.12 | 33.45 | 93% |
|  | Bandwith | Worst case / pixel | 15.13 | 15.13 | 100% |
|  |  | Average of bytes / pixel | 3.71 | 3.45 | 93% |

Table 5: Summary of complexity analysis (add/shift case)

| Method |  |  | DCTIF 8-tap | Proposal |  |
| --- | --- | --- | --- | --- | --- |
| add/shift or multiple |  |  | add/shift | add/shift |  |
| Sheet |  |  | Anchor | Proposal | Ratio (%) |
| Single | Operations | Worst case / pixel | 60.00 | 45.00 | 75% |
|  |  | Average of operations / pixel | 30.10 | 22.08 | 73% |
|  | Bandwith | Worst case / pixel | 7.56 | 5.06 | 67% |
|  |  | Average of bytes / pixel | 2.48 | 1.95 | 79% |
| Bi | Operations | Worst case / pixel | 120.00 | 127.50 | 106% |
|  |  | Average of operations / pixel | 60.21 | 60.21 | 100% |
|  | Bandwith | Worst case / pixel | 15.13 | 15.13 | 100% |
|  |  | Average of bytes / pixel | 4.95 | 4.95 | 100% |
| Total | Operations | Worst case / pixel | 120.00 | 127.50 | 106% |
|  |  | Average of operations / pixel | 45.15 | 41.14 | 91% |
|  | Bandwith | Worst case / pixel | 15.13 | 15.13 | 100% |
|  |  | Average of bytes / pixel | 3.71 | 3.45 | 93% |

# References

[1] K. Kondo and T. Suzuki, “CE3: Results on Bi/Single MC interpolation filter” JCTVC-D090, 4th. JCT-VC Meeting, Daegu, KR, 20-28, January, 2011.

[2]T.Chujoh and E.Alshina, “CE3: Interpolation for MC (Luma)” JCTVC-D603r3, 4th. JCT-VC Meeting, Daegu, KR, 20-28, January, 2011.

[3] K. Kondo and T. Suzuki, “TE3 subset4: Results on Bi/Single filter switching in FIF” JCTVC-C164, 3rd. JCT-VC Meeting, Guangzhou, CN, 7-15 Oct, 2010.

[4] F. Bossen, “Common TM Test Conditions,” JCTVC-D600, 4th. JCT-VC Meeting, Daegu, KR, 20-28 January, 2011.

[5] G. Bjøntegaard, “Calculation of Average PSNR Differences between RD curves,” VCEG-M33, 13th VCEG Meeting, Austin, USA, April 2001.

Sony Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).