| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 5th Meeting: Geneva, CH, 16-23 March, 2011 | Document: JCTVC-E131 WG11 Number: m19647 |
| --- | --- |

| Title: | CE14.1: Results for DOCOMO’s proposal and cross verification of MediaTek’s implementation for the most probable mode signalling for luma. |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal / Information |  |  |
| Author(s) or Contact(s): | TK Tan 3-6 Hikarinooka, Yokosuka-shi Kanagawa, 239 8536 Japan | Tel: Email: | +65 9799 3861 tktan.p@rd.nttdocomo.co.jp |
| Source: | NTT DOCOMO, Inc. |  |  |

_____________________________

# Abstract

This contribution reports the simulation results for Core Experiment 14.1 as described in JCTVC-D614r2.  In the current HM specification for the luma intra prediction mode coding, a single most probable mode is derived in the encoder and decoder.  The most probable mode is the smaller of the intra prediction modes of the partition above and to the left of the current partition.

In subtest 1 of core experiment CE14, a second candidate for the most probable mode is derived when available.  The encoder signals to the decoder if the intra prediction mode of the current partition is in the list of the most probable mode candidates. When the intra prediction mode is in the list of most probable mode candidates, an index is also signaled if the number of candidates is greater than 1.  Otherwise the intra prediction mode is not in the list of most probable mode candidates, so the remainder mode (intra prediction mode “excluding” the most probable mode candidates) is signaled to the decoder.

An overall average luma BD-rate of 0.5% improvement was achieved for both the high efficiency and low complexity intra only coding conditions.  There was no significant impact for the random access and low delay configurations with overall average luma BD-rate improvements of 0.1% to 0.2% recorded.

- Introduction

## Subtest1: The most probable mode signaling for luma

At the 4th JCT-VC meeting in Daegu, Korea, three proposals (JCTVC-D166 [2], JCTVC-D235 [3], JCTVC-D253 [4]) were essentially the same. The goal of this subtest is to verify the performance of this proposal under the new HM2.0 and to harmonize the proposed method with the newly adopted LCEC technique for intra mode coding described in JCTVC-D366 [5].

The following tasks were performed in this core experiment:

- Implemented and verified the proposed method on top of the HM2.0 software (revision 573).

- Compared the summary results in the excel sheets with that obtained by MediaTek.

- Cross verified the implementations by DOCOMO and MediaTek by decoding the bitstreams generated by the DOCOMO encoder with the MediaTek decoder and comparing the md5sum of the decoded sequences.

The following experimental conditions were used for the simulations.

- INTRA-only common conditions and software reference configurations as defined in JCTVC-D600. [6]

In addition, simulations were also conducted for the random access and low delay coding structures.

# Implementation details

Please refer to the core experiment description JCTVC-D614r2 [1] for the details of the proposal.  The proposed specification for the working draft and encoder description can be found in the Annex.

The implementation for the intra mode coding using CABAC entropy coding was essentially the same as was reported in JCTVC-D253.  The implementation of the intra mode coding using LCEC required some harmonization as the fixed length coding (FLC) in HM1.0 was replaced with additional ranking and variable length coding (VLC) tables in HM2.0.  The ranking tables and VLC tables were provided by MediaTek.  The tables are selected conditional on the number of intra prediction mode candidates and the number of intra prediction modes used for the intra coded partition.  Table 1 below illustrates the changes and additions of the tables.  The tables are listed in the Annex.

Table 1: Harmonization of the Intra prediction mode coding and the LCEC Intra mode coding

|  |  | Adaptive ranking tables |  | LCEC tables |  |
| --- | --- | --- | --- | --- | --- |
| Number of Intra prediction modes | Number of intra prediction mode candidates | Number of tables (entries in table) | Compared to HM2.0 | Number of tables (entries per table) | Compared to HM2.0 |
| 17 | 1 | 1x (16) | retrained | 1x (16) | same |
| 17 | 2 | 1x (15) | new table added | 1x (15) | retrained and entries reduce by 1 |
| 34 | 1 | 1x (33) | retrained | 1x (33) | retrained |
| 34 | 2 | 1x (32) | new table added | 1x (32) | retrained and entries reduce by 1 |

## Comparison to MediaTek’s implementatiom

Apart from the tables that were provided by MediaTek, the implementation of the proposal was done independently by DOCOMO.  The simulations were also conducted independently and results obtained were identical.  Furthermore, the bit streams generated by the DOCOMO encoder were decoded using the MediaTek decoder and the md5sum calculated.  These md5sum of decoded sequences are the same as that of the decoded sequences generated by the DOCOMO decoder.

# Experimental results

The experiments were conducted in accordance with the 6 common test conditions described in [6]. The proposed modifications were implemented in HM2.0 r573 of the common software.  The results are shown in Table 2. The excel sheet corresponding to this result is attached as part of the contribution.

Table 2: Results of Subtest1: The most probable mode signaling for luma (Ref: HM2.0 r573)

|  | Intra |  |  | Intra LoCo |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | -0.3 | -0.2 | -0.1 | -0.3 | -0.2 | -0.2 |
| Class B | -0.5 | -0.4 | -0.4 | -0.6 | -0.4 | -0.5 |
| Class C | -0.5 | -0.4 | -0.4 | -0.5 | -0.5 | -0.5 |
| Class D | -0.3 | -0.3 | -0.3 | -0.4 | -0.4 | -0.4 |
| Class E | -0.8 | -0.5 | -0.5 | -1.0 | -0.6 | -0.6 |
| All | -0.5 | -0.4 | -0.3 | -0.5 | -0.4 | -0.4 |
| Enc Time[%] | 100% |  |  | 104% |  |  |
| Dec Time[%] | 100% |  |  | 102% |  |  |
|  | Random access |  |  | Random access LoCo |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | -0.1 | 0.1 | -0.1 | -0.1 | 0.1 | 0.0 |
| Class B | -0.2 | -0.2 | -0.1 | -0.3 | -0.2 | -0.2 |
| Class C | -0.2 | -0.2 | -0.2 | -0.2 | -0.2 | -0.2 |
| Class D | 0.0 | 0.0 | 0.0 | -0.2 | 0.0 | 0.0 |
| Class E |  |  |  |  |  |  |
| All | -0.1 | -0.1 | -0.1 | -0.2 | -0.1 | -0.1 |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |
|  | Low delay |  |  | Low delay LoCo |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A |  |  |  |  |  |  |
| Class B | -0.1 | 0.0 | 0.1 | -0.1 | -0.2 | -0.1 |
| Class C | -0.1 | 0.0 | 0.0 | -0.2 | -0.2 | -0.2 |
| Class D | -0.1 | -0.1 | -0.4 | -0.1 | -0.2 | 0.0 |
| Class E | -0.2 | -1.3 | 1.1 | -0.1 | 0.1 | 0.1 |
| All | -0.1 | -0.3 | 0.1 | -0.1 | -0.2 | -0.1 |
| Enc Time[%] | 99% |  |  | 100% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |

For intra only coding, the proposal achieved 0.5% overall average luma BD-rate gains for both high efficiency and low complexity coding conditions. There was no significant impact on the coding performance for random access and low delay configurations.

# Conclusion

This report confirms the results of subtest1 on the most probable mode signaling for luma.  The proposal was successfully harmonized with the LCEC Intra mode coding.  Two separate implementations were tested and both produced the same results.

It is proposed that this contribution be adopted into the HM.  Please refer to the Annex for the proposed WD text.

# References

- Shawmin Lei et. al, “Description of Core Experiment 14: Intra Mode Coding”, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JCTVC-D614r2, 4th Meeting: Daegu, Korea, 20-28 January, 2011..

- M. Guo, X. Guo and S. Lei, “Improved Intra Mode Coding,” Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JCTVC-D166, 4th Meeting: Daegu, Korea, 20-28 January, 2011.

- S. Kanumuri, TK Tan and Frank Bossen, “Enhancements to Intra Coding,” Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JCTVC-D235, 4th Meeting: Daegu, Korea, 20-28 January, 2011.

- W. Ding, X. Che, Y. Shi, B. Yin, “Improved Intra Mode Coding by Multiple Mode Candidates,” Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JCTVC-D253, 4th Meeting: Daegu, Korea, 20-28 January, 2011.

- M. Karczewicz, X. Wang, W. Chen, and A. Fuldseth, “CE5: Improved intra prediction mode coding with LCEC,” Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JCTVC-D366, 4th Meeting: Daegu, Korea, 20-28 January, 2011.

- F. Bossen., “Common conditions and software reference configurations”, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JCTVC-D600, 4th Meeting: Daegu, Korea, 20-28 January, 2011.

# Patent rights declaration(s)

NTT DOCOMO, Inc. may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# Annex 1:  Proposed specification for the WD.

### Prediction unit syntax

| prediction_unit( x0, y0, log2PUWidth, log2PUHeight ) { | Descriptor |
| --- | --- |
| if( skip_flag[ x0 ][ y0 ] ) { |  |
| if( NumMVPCand( L0 ) > 1 ) |  |
| mvp_idx_l0[ x0 ][ y0 ] | ue(v) ¦ ae(v) |
| if( NumMVPCand( L1 ) > 1 ) |  |
| mvp_idx_l1[ x0 ][ y0 ] | ue(v) ¦ ae(v) |
| } else if( PredMode = = MODE_INTRA ) { |  |
| prev_intra_luma_pred_flag[ x0 ][ y0 ] | u(1) ¦ ae(v) |
| if( prev_intra_luma_pred_flag[ x0 ][ y0 ] ) |  |
| if(NumMPMCand > 1 ) |  |
| mpm_idx[ x0 ][ y0 ] | u(1) ¦ ae(v) |
| else |  |
| rem_intra_luma_pred_mode[ x0 ][ y0 ] | ue(v) ¦ ae(v) |
| intra_chroma_pred_mode[ x0 ][ y0 ] | ue(v) ¦ ae(v) |
| } else { /* MODE_INTER */ |  |
| . |  |
| . |  |
| . |  |

### 7.4.7 	Prediction unit semantics

< text skipped >

prev_intra_luma_pred_flag[ x0 ][ y0 ], mpm_idx[ x0 ][ y0 ] and rem_intra_luma_pred_mode[ x0 ][ y0 ] specifies the intra prediction mode of the ith prediction unit for luma samples with index i = 0 when intra_split_flag is equal to 0 or i =0..3 when intra_split_flag is equal to 1. When prev_intra_luma_pred_flag[ i ] is equal to 1, the intra prediction mode is inferred from a neighbouring intra-predicted prediction unit according to subclause 8.3.1.

When mpm_idx[ x0 ][ y0 ] is not present, it shall be inferred to have a value of 0.

The number of bins/bits indicating the rem_intra_luma_pred_mode[ i ] depends on the size of the prediction unit and limits the number of available prediction modes as follows:

< text skipped >

### 8.3.1	Derivation process for the IntraPredMode

< text skipped >

IntraPredMode[ xB ][ yB ] is derived as follows.

–	The derivation process for neighbouring treeblocks specified in subclause XXX with ( xB,  yB ) given as input and the output is assigned to tbAddrA and tbAddrB specifying the treeblock addresses of treeblocks covering ( xBA,  yBA ) and ( xBB, yBB ) respectively where ( xBA,  yBA ) is set to ( xB-1,  yB ) and ( xBB,  yBB ) is set to ( xB,  yB-1 ).

–	For N being either replaced A or B, the variables intraPredModeN are derived as follows.

- If the treeblock with address tbAddrN is not available, intraPredModeN is set equal to -1

- Otherwise, if the coding unit covering ( xBN,  yBN ) is not coded as intra mode, intraPredModeN is set equal to 2

- Otherwise, intraPredModeN is set equal to IntraPredMode[ xBN ][ yBN ], where IntraPredMode is the variable array assigned to the coding unit covering the luma location ( xBN, yBN ).

- For N being either replaced A or B, the variables candIntraPredModeN are derived as follows.

- If intraPredModeN is greater than or equal to intraPredModeNum

- If intraPredModeNum is equal to 5 then candIntraPredModeN is set to mapIntraPredMode5[ intraPredModeN ]

- If intraPredModeNum is equal to 3 then candIntraPredModeN is set to mapIntraPredMode3[ intraPredModeN ]

- Otherwise candIntraPredModeN is set to mapIntraPredMode9[ intraPredModeN ].

- Otherwise, candIntraPredModeN is set to intraPredModeN

- The candModeList[x] and NumMPMCand are derived as follows:

- If both candIntraPredModeN are not available, then the value 2 is assigned to candModeList[ 0 ] and NumMPMCand is set to 1

- Otherwise, if only one candIntraPredModeN is available or if both candIntraPredModeN are the same, then this candIntraPredModeN is assigned to candModeList[ 0 ] and NumMPMCand is set to 1

- Otherwise, both candIntraPredModeN are assigned to the candidate modes list with the smaller of the two candidates at candModeList[ 0 ] and the larger at candModeList[ 1 ] and NumMPMCand is set to 2.

–	IntraPredMode[ xB ][ yB ] is derived by applying the following procedure.

- If prev_intra_pred_flag[ xB ][ yB ] is true, the IntraPredMode[ xB ][ yB ] is set to  candModeList[mpm_idx[ xB ][ yB ]]

- Otherwise IntraPredMode[ xB ][ yB ] is derived by applying the following procedure

- IntraPredMode[ xB ][ yB ] = rem_intra_pred_mode

- for (cIdx = 0; cIdx < NumMPMCand; cIdx++)

if (IntraPredMode[ xB ][ yB ]>= candModeList[cIdx]) IntraPredMode[ xB ][ yB ]++

< text skipped >

### LCEC tables

Ranking Tables

const UInt  g_auiIntraModeTableD17[2][16]=

{

{ 0, 15, 11, 10, 13, 7, 9, 4, 14,  2, 3, 6,  8, 5, 12, 1},

{14, 10,  9,  0, 13, 7, 2, 8,  3, 12, 6, 4, 11, 1,  5, 0},

};

const UInt  g_auiIntraModeTableE17[2][16]=

{

{0, 15, 9, 10,  7, 13, 11, 5, 12, 6, 3,  2, 14, 4, 8, 1},

{3, 13, 6,  8, 11, 14, 10, 5,  7, 2, 1, 12,  9, 4, 0, 0},

};

const UInt  g_auiIntraModeTableD34[2][33]=

{

{2, 0, 29, 30, 20, 1, 21, 28, 15, 7, 16, 8, 11, 31, 22, 19, 32, 10, 27, 14, 4, 18, 12, 23, 17, 24, 26, 6, 25, 5, 9, 3, 13},

{2, 1, 28, 0, 29, 20, 27, 19, 15, 21, 7, 14, 10, 11, 30, 31, 18, 16, 26, 8, 17, 4, 22, 9, 23, 6, 25, 13, 24, 12, 3, 5,  0},

};

const UInt  g_auiIntraModeTableE34[2][33]=

{

{1, 5, 0, 31, 20, 29, 27, 9, 11, 30, 17, 12, 22, 32, 19, 8, 10, 24, 21, 15, 4, 6, 14, 23, 25, 28, 26, 18, 7, 2, 3, 13, 16},

{3, 1, 0, 30, 21, 31, 25, 10, 19, 23, 12, 13, 29, 27, 11, 8, 17, 20, 16, 7, 5, 9, 22, 24, 28, 26, 18, 6, 2, 4, 14, 15,  0},

};

LCEC Tables

const UInt huff17_2[2][17]=

{

{1, 7, 6, 11, 10, 9, 8, 7, 6, 5, 4, 3, 2, 3, 2, 1, 0},

{1, 0, 15, 14, 13, 12, 11, 10, 9, 8, 7, 6, 5, 4, 3, 2, 0},

};

const UInt lengthHuff17_2[2][17]=

{

{1, 4, 4, 5, 5, 5, 5, 5, 5, 5, 5, 5, 5, 6, 6, 6, 6},

{1, 4, 5, 5, 5, 5, 5, 5, 5, 5, 5, 5, 5, 5, 5, 5, 0},

};

const UInt huff34_2[2][34]=

{

{1, 0, 7, 6, 5, 4, 3, 2, 23, 22, 21, 20, 19, 18, 17, 16, 61, 60, 59, 58, 57, 56, 55, 54, 53, 52, 51, 50, 49, 48, 127, 126, 125, 124 },

{1, 0, 5, 4, 3, 2, 24, 23, 22, 21, 20, 19, 18, 17, 16, 15, 14, 13, 12, 63, 62, 61, 60, 59, 58, 57, 56, 55, 54, 53, 52, 51,50, 0 }

};

const UInt lengthHuff34_2[2][34]=

{

{1, 4,  5,  5,  5,  5,  5,  5,  6,  6,  6,  6,  6,  6,  6,  6,  7,  7,  7,  7,  7,  7,  7,  7,  7,  7,  7,  7,  7,  7,  8,  8,  8, 8 },

{1, 4,  5,  5,  5,  5,  6,  6,  6,  6,  6,  6,  6,  6,  6,  6,  6,  6,  6,  7,  7,  7,  7,  7,  7,  7,  7,  7,  7,  7,  7,  7,  7, 0 }

};

The table for the encoder was not counted.

One LCEC table comprises of 2 arrays: one array for the bit pattern and the other array for the length.