| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 5 th Meeting: Geneva, CH, 16-23 March, 2011 | Document: JCTVC-E132 WG11 Number: m19648 |
| --- | --- |

| Title: | CE11: Cross-check report of Qualcomm’s Adaptive Coefficient Scanning for LCEC (JCTVC-D374) by SHARP |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Yukinobu Yasugi Tomoyuki Yamamoto 1-9-2 Nakase, Mihama-ku, Chiba-shi, Chiba 261-8520 JAPAN | Tel: Email: | Tel: +81-43-299-8526 yasugi.yukinobu@sharp.co.jp, yamamoto.tomoyuki@sharp.co.jp |
| Source: | SHARP Corporation |  |  |

_____________________________

# Abstract

This contribution provides a cross-check result of Adaptive Coefficient Scanning for LCEC (JCTVC-D374) in CE11.

# Experimental condition

The evaluation was carried out using the HM-2.0 software with the proposed method [1] as defined in CE11 [2]. We tested both the first version (ver.1) and updated version (ver.2) of the software provided by the proponent. The updated version guarantees 8-bit counters for scan adaptation.

For each version, we evaluated two conditions:

Reference:	QC_ADAPT_SCAN= 0	(off)

Compared:	QC_ADAPT_SCAN=1	(on)

The computing platforms are summarized in Table 1. In the experiments, encoding and decoding were done using the grid system composed of variety of PCs, but it was configured so that the same-performance PC is used to encode/decode both anchor and the proposed method for each test point.

Table 1: Computing platforms

| OS | Linux (64bit) |
| --- | --- |
| CPU | Intel Core2 Quad 2.66GHz Intel Core2 Quad Q9550 2.83GHz Intel Xeon X5667 3.07GHz |
| Compiler | gcc 4.4.3 |

# Performance experiments

The experiments for both version 1and 2 have successfully done and the results are summarized in Table 2 and Table 3, respectively. A negative number means gain and a positive number means loss. Detailed results are included in the attachments JCTVC-E132_ver1.xls and JCTVC-E132_ver2.xls.

Table 2: Experimental results (ver.1)

|  | Intra LoCo |  |  | Random access LoCo |  |  | Low delay LoCo |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | -2.0 | -1.4 | -1.3 | -0.9 | -0.6 | -0.3 |  |  |  |
| Class B | -0.9 | -0.8 | -0.8 | -0.4 | -0.3 | -0.4 | -0.1 | -0.1 | 0.1 |
| Class C | -0.7 | -0.3 | -0.4 | -0.3 | -0.1 | 0.0 | -0.1 | 0.0 | -0.1 |
| Class D | -0.2 | -0.1 | -0.1 | -0.1 | -0.1 | -0.2 | 0.1 | 0.3 | 0.1 |
| Class E | -1.2 | -0.9 | -0.8 |  |  |  | -0.1 | -0.2 | 0.3 |
| All | -1.0 | -0.7 | -0.7 | -0.4 | -0.3 | -0.2 | 0.0 | 0.0 | 0.1 |
| Enc Time[%] | 104% |  |  | 100% |  |  | 101% |  |  |
| Dec Time[%] | 100% |  |  | 102% |  |  | 99% |  |  |

Table 3: Experimental results (ver.2)

|  | Intra LoCo |  |  | Random access LoCo |  |  | Low delay LoCo |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | -2.0 | -1.4 | -1.3 | -1.0 | -0.7 | -0.4 |  |  |  |
| Class B | -1.0 | -0.8 | -0.8 | -0.4 | -0.3 | -0.4 | -0.1 | -0.1 | -0.2 |
| Class C | -0.6 | -0.3 | -0.3 | -0.3 | -0.2 | -0.1 | -0.1 | 0.0 | -0.1 |
| Class D | -0.2 | -0.1 | -0.1 | 0.0 | 0.2 | 0.1 | 0.1 | 0.2 | -0.1 |
| Class E | -1.2 | -0.9 | -0.9 |  |  |  | 0.1 | -0.5 | 0.3 |
| All | -1.0 | -0.7 | -0.7 | -0.4 | -0.3 | -0.2 | 0.0 | -0.1 | -0.1 |
| Enc Time[%] | 103% |  |  | 100% |  |  | 100% |  |  |
| Dec Time[%] | 103% |  |  | 100% |  |  | 102% |  |  |

The result of version 2 has slight differences from the first version especially in the case of UV for RALC and LDLC. However, on average, two results are approximately same.

Note: Due to our grid environment, data of encoding/decoding time includes a few outliers.

# Conclusion

The simulation results of Qualcomm’s Adaptive Coefficient Scanning for LCEC were reported.

# Reference

- Marta Karczewicz, et al., “CE5: Improved coefficient coding with LCEC”, JCTVC-D374, 4th JCT-VC Meeting, Daegu, January 2011

- Vivienne Sze, et al., “Description of Core Experiment (CE11): Coefficient scanning and coding”, JCTVC-D611, 4th JCT-VC Meeting, Daegu, January 2010.