| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 5th Meeting: Geneva, CH, 16-23 January, 2011 | Document: JCTVC-E133 WG11 Number: m19650 |
| --- | --- |

| Title: | Adaptive scaling for reference pictures memory compression |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Takeshi Chujoh Tomoo Yamakage 1, Komukai-Toshiba-cho, Saiwai-ku, Kawasaki, 212-8582, Japan | Tel: Email: | +81-44-549-2288 takeshi.chujoh@toshiba.co.jp |
| Source: | TOSHIBA Corporation |  |  |

_____________________________

# Abstract

In this contribution, a reference pictures memory compression from 10-bit to 8-bit on high efficiency anchor and a definition for standardization are proposed.   This contribution shows a solution to improve coding efficiency by increasing internal process of video codec while minimizing reference frame memory access bandwidth.  There are two points.  Firstly, an adaptive scaling method with fixed length format is proposed and secondly, a definition by distortion for reference pictures memory compression is introduced.  As experimental results, the loss bitrate of adaptive scaling is average of 0.82% while the loss bitrate of fixed rounding is average of 2.00% and the low bitrate of internal 8-bit is average of 2.59%.

# Introduction

Although there are several discussion items in reference pictures memory compression Adhoc group, in this contribution, considering several items, we propose an adaptive scaling method for reference pictures memory compression from 10-bit to 8-bit on high efficiency anchor and its definition of compression distortion control for standardization.

# Adaptive scaling for reference pictures memory compression

By increasing internal bit depth, the accuracy of all internal processes is increased.  This intends to reduce the rounding error of intra-fame prediction, spatial transform, in-loop filtering in order to improve coding efficiency.  For example, by increasing from 8-bit to 10-bit, DPB (Decode Picture Buffer) will be required 1.25 times of memory capacity and 1.25 times of memory bandwidth compared to storing the 8-bit depth.  This is one of the most important problems about complexity.  Therefore, JCTVC-A117 [2], JCTVC-B044 [3], JCTVC-C075r1 [4] and JCTVC-D152 [5] introduced an adaptive scaling for bit depth compression.  The scaling algorithm in the case of 2-bit increase is as follows:

- obtain the minimum pixel value min and the maximum pixel value max of 4x4 block,

- calculate an 7-bit minimum pixel value M (= min >> 3),

- calculate a dynamic range R between max and (M<<3),

- decide the scaling value S so that R becomes less than (128 << S),

- if S is equal to 2, then if pixel_value[0] is less than 2, then P is set to 1 else calculate sixteen storing pixel values P (= (pixel_value[i]+2)>>2). If S is not equal to 2, then calculate sixteen storing pixel values P (= (pixel_value[i] - (M<<3))>>S) and store S, M, and P.

Although stored format on DPB is non-normative, for example, first 1 byte by 4x4 block indicates fixed scaling or adaptive scaling and if the value of first 1 byte is not equal to 0, sixteen 8-bit values P are described and if the value is equal to 0, a following 1-bit value S, an 7-bit minimum pixel value M and sixteen 7-bit value P are described.  As a result, it is possible to describe 128-bit format by 4x4 blocks. Figure 2 shows an example of fixed length format.

The de-scaling algorithm is as follows:

- read first 1 byte P[0] by 4x4 block.

- if P[0] is not equal to 0, then calculate sixteen de-scaling values D[i] (=P[i] << 2), else calculate sixteen de-scaling values D[i] (= (P[i] << S) + (M<<3) + ((S!=2 || S!=0)? (1<<(S-1)):0) ).

Figure 1 explains the relationship of above values.

Figure 1  Adaptive scaling process

Actually, this algorithm is a loss less compression on 8-bit depth level except value 0.  For example, according to BT.709, values 0 and 255 don’t appear.  Then, practically there is no problem.  This means that fixed scaling pixel value matches pixel value, which is adaptive de-scaled firstly and is fixed scaled secondary.  Therefore, adaptive scaling can get the results surely better than fixed scaling.

| adaptive_scaling_block( ) { |  |
| --- | --- |
| P[0] | u(8) |
| if (P[0]!=0) { |  |
| for (i=1;i<16;i++) { |  |
| P[i] | u(8) |
| } |  |
| } else { |  |
| S | u(1) |
| M | u(7) |
| for (i=0; i<16; i++) { |  |
| P[i] | u(7) |
| } |  |
| } |  |
| } |  |

Figure 2  An example of fixed length format (128-bit by 4x4 block)

# Specification for standardization

Reference pictures memory compression is a closed process into coding loop.  Therefore, if the part of loss less coding or bit stream format on frame memory is not specified by the standardization, any mismatch is not caused on both encoder and decoder.  The technical essential point is the distortion of reference pictures memory compression.

Figure 3  Definition of compression distortion control

In this contribution, a definition of adaptive scaring by controlling bit depth is shown.  Lower bits of each pixel are padded zero by following algorithm.  If the reference pictures memory compression is not needed on encoder side or decoder side, the following method is more effective and the same results as the adaptive scaling can be obtained.

(1) obtain the minimum pixel value min and the maximum pixel value max of 4x4 block

(2) decide the scaling value S from difference between max and min.

for (S=0; max – (min & ~0x7)>=(128<<S) && S<2; S++);

(3) control bit depth of pixels according to S.

if (S==2) {

D[0] = (pixel_value[0]<2)? 4: (pixel_value[0]+2) & ~0x3;

for (i=1;i<16;i++) D[i] = (pixel_value[i]+2) & ~0x3;

} else if (S==1) {

for (i=0;i<16;i++) D[i] = (pixel_value[i] & ~0x1) + 1;

} else {

for (i=0;i<16;i++) D[i] = pixel_value[i];

}

# Experimental results

The simulation was conducted for all sequences of WQVGA, WVGA, 720p, 1080p and cropped 4kx2k based on the recommended simulation common conditions, JCTVC-D600 [6].  The version of HM software is 2.0 plus to memory Adhoc group modification.  The OS is Windows XP SP2 64-bit, the CPU is Xeon® W3565, 3.2GHz and the compiler is Visual C++ 2008.  Error: Reference source not found indicates the results of loss coding efficiency of adaptive scaling and fixed scaling compared to high efficiency anchors.  Since there values show loss results from anchor condition, the smaller values mean better results.  While fixed scaling losses 2.00% on average, adaptive scaling only losses 0.82% on average.

Table 1  Loss BD-rate (%) on high efficiency anchor

|  | Random access |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Adaptive scaling |  |  | Fixed rounding |  |  |  |  | Internal 8-bit |  |  |
|  | Y | U | V | Y |  | U | V |  | Y | U | V |
| Class A | 0.48 | 1.59 | 1.50 | 1.12 |  | 8.04 | 12.70 |  | 3.50 | 21.29 | 28.97 |
| Class B | 0.57 | 1.24 | 1.12 | 1.33 |  | 3.62 | 4.51 |  | 2.48 | 7.17 | 7.64 |
| Class C | 0.36 | 0.61 | 0.48 | 0.78 |  | 1.88 | 1.88 |  | 1.40 | 3.15 | 3.50 |
| Class D | 0.31 | 0.41 | 0.14 | 0.63 |  | 1.87 | 1.91 |  | 0.99 | 2.75 | 3.09 |
| All | 0.44 | 0.98 | 0.83 | 0.99 |  | 3.84 | 5.21 |  | 2.12 | 8.51 | 10.62 |
|  | Low delay |  |  |  |  |  |  |  |  |  |  |
|  | Adaptive scaling |  |  | Fixed rounding |  |  |  |  | IBDI off |  |  |
|  | Y | U | V | Y | U |  |  | V | Y | U | V |
| Class B | 1.02 | 1.62 | 1.61 | 2.57 | 6.97 |  |  | 9.14 | 3.16 | 10.91 | 12.50 |
| Class C | 0.62 | 0.70 | 0.87 | 1.34 | 3.71 |  |  | 3.80 | 1.81 | 4.98 | 5.55 |
| Class D | 0.34 | 0.57 | 0.27 | 0.92 | 7.12 |  |  | 7.24 | 1.08 | 9.12 | 9.63 |
| Class E | 3.37 | 2.69 | 3.88 | 8.73 | 21.21 |  |  | 23.39 | 7.17 | 18.22 | 14.81 |
| All | 1.19 | 1.33 | 1.52 | 3.01 | 8.86 |  |  | 10.00 | 3.06 | 10.35 | 10.48 |

The complexity is measured by the encoding and decoding time.  The measurement values are output of encoder and decoder and the decoder does not output decoded image.  Table 3 indicates the results of increase rate of average encoding and decoding times. The results of adaptive scaling and fixed scaling are not significantly different.  As a result of distortion definition, both encoding and decoding time is a little reduced.

Table 2  Increase rate (%) of average encoding and decoding times

| Proponent | Adaptive scaling |  | Fixed rounding |  | Internal 8-bit |  |
| --- | --- | --- | --- | --- | --- | --- |
| Condition | Random access | Low delay | Random access | Low delay | Random access | Low delay |
| Encoding Time | 100.35 | 99.98 | 101.23 | 100.89 | 99.80 | 99.80 |
| Decoding Time | 104.53 | 102.89 | 108.05 | 108.38 | 98.04 | 99.36 |

Table 4 and 5 indicate average compression rate of MC read memory bandwidth on decoder side.  The module of measurement that is provided by Adhoc group of memory compression is used.  In this method, both luma and chroma block size is 4x4 and the compression ratio is defined 0.800.

Table 3  Compression rate (%) of MC read memory bandwidth on high efficiency anchor

|  | 8bit/ 8bit | 32bit/ 64bit | 32bit/ 128bit | 64bit/ 128bit | 64bit/ 256bit | 64bit/ 512bit | 64bit/ 256bit FIFO | 64bit/ 512bit FIFO |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Class A | 1.53 | -11.50 | -20.87 | -24.11 | -29.27 | -40.89 | -32.75 | -35.19 |
| Class B | 5.32 | -8.58 | -18.43 | -22.35 | -27.32 | -43.93 | -31.47 | -35.67 |
| Class C | 8.61 | -7.58 | -19.70 | -24.55 | -31.47 | -50.43 | -38.04 | -43.20 |
| Class D | 16.99 | -3.08 | -17.48 | -23.44 | -31.33 | -52.52 | -39.96 | -45.73 |
| All | 7.95 | -7.74 | -19.08 | -23.54 | -29.70 | -46.77 | -35.31 | -39.70 |

Table 4  Compression rate (%) of MC read memory bandwidth on high efficiency anchor

|  | 8bit/ 8bit | 32bit/ 64bit | 32bit/ 128bit | 64bit/ 128bit | 64bit/ 256bit | 64bit/ 512bit | 64bit/ 256bit FIFO | 64bit/ 512bit FIFO |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Class B | 9.46 | -6.13 | -17.47 | -21.98 | -27.49 | -45.05 | -34.28 | -38.79 |
| Class C | 11.24 | -6.85 | -20.12 | -25.29 | -32.56 | -52.44 | -42.14 | -47.66 |
| Class D | 20.95 | -2.76 | -18.36 | -24.38 | -33.36 | -55.19 | -43.52 | -50.02 |
| Class E | -8.18 | -15.17 | -22.75 | -25.04 | -35.26 | -50.57 | -37.61 | -41.02 |
| All | 9.47 | -7.16 | -19.34 | -23.98 | -31.68 | -50.47 | -39.18 | -44.23 |

In addition, as subjective quality checking of decoded sequences, the subjective quality difference caused by adaptive scaling is visually negligible.

# Conclusion

In this contribution, the experimental results of adaptive scaling for reference pictures memory compression were shown.  As experimental results, the bitrate loss by adaptive scaling was average of 0.82% and that value is definitely smaller than the value of fixed rounding.  Since additional complexity is negligible, it is suggested that the adaptive scaling should be introduce into the reference pictures memory compression on high efficiency anchor.

# Reference

[1]	T. Chujoh, “Tool Experiment 2 on IBDI and memory compression,” JCT-VC Document, JCTVC-A302r2, Geneva, July 2010.

[2]	T. Chujoh, A. Tanizawa and T. Yamakage, “Description of video coding technology proposal by TOSHIBA,” JCT-VC Document, JCTVC-A117, Dresden, April 2010.

[3]	T. Chujoh, T. Shiodera and T. Yamakage, “TE2: Adaptive scaling for bit depth compression on IBDI,” JCT-VC Document, JCTVC-B044, Geneva, July 2010.

[4]	T. Chujoh and T. Yamakage, “TE2: Adaptive scaling for bit depth compression on IBDI,” JCT-VC Document, JCTVC-C075r1, Guangzhou, October 2010.

[5]	T. Chujoh and T. Yamakage, “ Adaptive scaling for bit depth compression on IBDI,” JCT-VC Document, JCTVC-C152, Daegu, October 2011.

[6]	F. Bossen, “Common test conditions and software reference configurations,” JCT-VC Document, JCTVC-D500, Daegu, January 2011.

# Patent rights declaration(s)

TOSHIBA Corporation may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).