| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 5th Meeting: Geneva, CH, 16-23 March, 2011 | Document: JCTVC-E136 |
| --- | --- |

| Title: | CE14: Test Results for Intra Mode Coding by Multiple Mode Candidates |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Name Wenpeng Ding Xiaoyin Che Yunhui Shi Baocai Yin | Email: | wpding@bjut.edu.cn syhzm@bjut.edu.cn ybc@bjut.edu.cn |
| Source: | Beijing University of Technology |  |  |

_____________________________

# Abstract

In the Current TM software, a most probable mode is deduced to predict the intra modes. This contribution proposes to use multiple intra modes as prediction candidates. Several mode candidates will be deduced from neighboring intra modes to improve prediction accuracy of intra modes. Experimental results demonstrate the overall BDPSNR is reduced about 0.1% with intra_loco configuration, 0.3% with intra configuration.

# Introduction

In the Current TM software, a most probable mode is deduced to predict the intra modes. And one bit is used to signal whether the prediction is correct. If the prediction is correct, then only one bit is needed to represent the intra modes. Otherwise additional 2~5 bits will be used to indicate which intra mode is chosen for the current block. Several similar contributions are proposal to improve the coding effecieny of intra mode and CE14 was designed to verified the contributions [2]

# Improved intra mode coding

This contribution proposes to use multiply mode candidates to improve the accuracy of intra mode prediction. The intra modes of the above block and left block are chosen as the mode candidates. One bit is first used to indicate whether the intra mode of the current block is one of the mode candidates. And anther bit is used to indicate which mode candidate matches the current block. If no mode candidate matches the current block, the intra modes will be represented using 2~5 bits as that in the current TMUC software. If the two mode candidates are identical or only one mode candidates are available, the bit used to represent the mode candidates will be saved.

Fig.1: Mode candidates of current block

Table I: bits needed in different cases

| Num_of_Candidates | 1 |  | 2 |  |
| --- | --- | --- | --- | --- |
| Match current block | Yes | No | Yes | No |
| Bits needed | 1 | 1+log2(num_of_intra_mode-1) | 2 | 1+log2(num_of_intra_mode-1) |

The proposed Intra mode coding syntax is as follows

Table II: intra_pred_mode syntax

| read_intra_pred_mode(){ | Descriptor |
| --- | --- |
| use_candidate_dir | u(1)¦ae(v) |
| if (use_candidate_dir){ |  |
| If(num_of_ candidate_num = 2) |  |
| candidate_num | u(1)¦ae(v) |
| }else{ |  |
| rem_intra_pred_mode | ue(v)¦ae(v) |
| } |  |
| } |  |

# Experimental results

The proposed scheme has been implemented in the TM2.0 software. And LCEC_INTRA_MODE is disabled in intra_loco configuration. The proposed scheme is tested against the anchor data in terms of BDRate[1]. Table III Summaries the experimental results. The overall BDPSNR is reduced about 0.1% with intra_loco configuration and 0.3% with intra configuration. The BDPSNR is reduced up to 0.6% with intra_loco configuration and 0.8% with intra configuration.

Table III: Summary of BDPSNR VS anchor

|  | Intra |  |  | Intra LoCo |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | -0.2 | -0.2 | -0.1 | 0.1 | 0.0 | 0.0 |
| Class B | -0.4 | -0.3 | -0.3 | -0.1 | -0.2 | -0.2 |
| Class C | -0.3 | -0.3 | -0.3 | -0.2 | -0.3 | -0.3 |
| Class D | -0.2 | -0.2 | -0.2 | -0.2 | -0.3 | -0.3 |
| Class E | -0.6 | -0.5 | -0.4 | -0.4 | -0.3 | -0.4 |
| All | -0.3 | -0.3 | -0.3 | -0.1 | -0.2 | -0.2 |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |

# Reference

- Gisle Bjøntegaard, “Calculation of Average PSNR Differences between RD curves”, ITU-T SG16/Q6, 13th VCEG Meeting, Austin, Texas, USA, April 2001, Doc. VCEG-M33.

- S.Lei , “Description of Core Experiment 14: Intra Mode Coding”, JCTVC-D614, Daegu, Korea, 20-28 January, 2011.

Beijing University of Technology may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).