| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 5th Meeting: Geneva, Switzerland, 16-23 March, 2011 | Document: JCTVC-E138 WG11 Number: m19655 |
| --- | --- |

| Title: | CE8.3: Cross-check result of Samsung’s Loop filter with directional similarity mapping (JCTVC-E108) by Sharp |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Tomohiro Ikai, Takanori Yamazaki 1-9-2 Nakase, Mihama-ku, Chiba-shi, Chiba 261-8520 JAPAN | Tel: Email: | Tel: +81-43-299-8526 ikai.tomohiro@sharp.co.jp, |
| Source: | SHARP Corporation |  |  |

# Abstract

In this contribution, a CE8 subtest 3 cross-check result of Samsung’s loop filter is reported. The coding results perfectly match with those of proponents and confirmed the coding efficiency. The encoding and decoding time increase is also confirmed as the proponent reported.

# Performance experiments

The experimental results are summarized in Table 1. Detailed results are included in the attachment.

Table 1: Experimental results

|  | Intra |  |  |
| --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate |
| Class A | -0.1 | 0.0 | 0.0 |
| Class B | 0.0 | 0.0 | 0.0 |
| Class C | -0.5 | -0.1 | 0.0 |
| Class D | -0.3 | 0.5 | 0.5 |
| Class E | -0.4 | 0.0 | 0.0 |
| All | -0.2 | 0.1 | 0.1 |
| Enc Time[%] | 203% |  |  |
| Dec Time[%] | 160% |  |  |
|  | Random access |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate |
| Class A | -0.5 | 0.2 | -0.2 |
| Class B | -0.3 | 0.1 | 0.1 |
| Class C | -0.5 | 0.2 | 0.2 |
| Class D | 0.6 | 0.9 | 1.0 |
| Class E |  |  |  |
| All | -0.2 | 0.3 | 0.3 |
| Enc Time[%] | 146% |  |  |
| Dec Time[%] | 142% |  |  |
|  | Low delay |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate |
| Class A | 0.5 | 0.5 | 0.4 |
| Class B | -0.2 | 0.6 | 0.6 |
| Class C | 0.2 | 1.9 | 1.6 |
| Class D | 0.9 | 0.9 | 1.1 |
| Class E | 0.3 | 0.9 | 0.9 |
| All |  |  |  |
| Enc Time[%] | 131% |  |  |
| Dec Time[%] | 135% |  |  |

# Conclusion

In this contribution, a CE8 subtest 3[1][2] cross-check result of Samsung’s loop filter[3] is reported. The coding results perfectly match with those of proponents and confirmed the coding efficiency. The encoding and decoding time increase is also confirmed as the proponent reported.

# Reference

- T.Yamakage, Y.W. Huang, I.S. Chong, T. Ikai, M. Narroschke, Wang L. Lai, Ali Tabatabai, Madhukar Budagavi, Faouzi Kossentini, “Description of CE8: Non-deblocking loop filtering”, JCTVC-D608, 4th JCT-VC Meeting, Daegu, January 2011.

- T. Yamakage, I. S. Chong, M. Narroschke, “CE8: Non-deblocking Loop Filtering - Summary Report ,” JCTVC-E028, 5th JCT-VC Meeting, Geneva, Switzerland, March 2011.

- P. Lai, F. C. A. Fernandes, “CE8 Subtest 3: Loop filter with directional similarity mapping”, JCTVC-Dxx, JCTVC-E108, 5th JCT-VC Meeting, Geneva, Switzerland, March 2011.