| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 5th Meeting: Geneva, Switzerland, 16-23 March, 2011 | Document: JCTVC-E140 WG11 Number: m19657 |
| --- | --- |

| Title: | CE8.1: DF-combined adaptive loop filter |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Tomohiro Ikai, Takanori Yamazaki 1-9-2 Nakase, Mihama-ku, Chiba-shi, Chiba 261-8520 JAPAN | Tel: Email: | +81-43-299-8526 ikai.tomohiro@sharp.co.jp |
| Source: | SHARP Corporation |  |  |

_____________________________

# Abstract

This contribution is a CE8 subtest 1 (Single- and multi-input schemes in Non-deblocking loop filtering) proposal. In this contribution, the DF (Deblocking Filter) combined adaptive loop filter was evaluated. It is asserted the proposed technique provides coding efficiency improvement and the functionality to process the two inputs in parallel case. The coding gains of 1.1 %, 0.7 %, and 0.7 % in HE IO, HE RA, HE LD were reported. In parallel case, the coding gains of 1.3 %, 0.7 %, 0.5 % in HE IO, HE RA, HE LD were reported. The decoding time ratio between 104 % and 105 % and the encoding time ratio between 99 % and 103 % were reported in HE case. The WD2 base drafted text has been prepared..

## Algorithm

In the proposed technique, Wiener based in-loop filter is applied using both pre-DF signal and post-DF signal as inputs. (cf. the traditional ALF uses only post-DF signal.) The two-input system can process two cases, non-parallel case and parallel case, which are defined the Figure 1 and Figure 2 formulas respectively.

Figure 1: DF-combined loop filter (non-parallel case)

Figure 2: DF-combined loop filter (parallel case)

Where sout is ALF output, spre is pre-DF signal, and spost is post-DF signal. The values a, b, and c are Wiener filter coefficients, specifically a is ALF-spatial filter coefficients, b is a weighting value and c is dc-offset. In parallel case, the ALF-spatial filter is applied on pre-DF signal so that DF and ALF-spatial filter can be processed in parallel, while in non-parallel case the ALF-spatial filter is applied on post-DF signal.

## Filtering shapes and coefficients

The filter tap and shape of ALF spatial filter are the same as the anchor. The tap of another input is one so the operation is only weighting.

# Draft of proposal

The draft of this proposal is described with the additional file, where syntax, semantics and decoding process are included.

# Experimental results

## CE8 subtest 1 results

The experimental results of non-parallel case and parallel case are shown in Table 2 and Table 3.

Table 2: Experimental results (non-parallel case)

|  | Intra |  |  |
| --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate |
| Class A | -1.5 | 0.0 | 0.0 |
| Class B | -1.2 | 0.0 | 0.0 |
| Class C | -0.9 | 0.0 | 0.0 |
| Class D | -0.7 | 0.1 | 0.1 |
| Class E | -1.0 | 0.0 | 0.0 |
| All | -1.1 | 0.0 | 0.0 |
| Enc Time[%] | 103% |  |  |
| Dec Time[%] | 104% |  |  |
|  | Random access |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate |
| Class A | -1.1 | 0.0 | 0.0 |
| Class B | -0.9 | 0.0 | 0.1 |
| Class C | -0.5 | 0.0 | 0.0 |
| Class D | -0.3 | 0.0 | 0.2 |
| Class E |  |  |  |
| All | -0.7 | 0.0 | 0.1 |
| Enc Time[%] | 100% |  |  |
| Dec Time[%] | 104% |  |  |
|  | Low delay |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate |
| Class A |  |  |  |
| Class B | -0.9 | -0.1 | -0.1 |
| Class C | -0.4 | 0.1 | 0.1 |
| Class D | -0.4 | 0.2 | 0.0 |
| Class E | -1.5 | -0.4 | 0.3 |
| All | -0.7 | 0.0 | 0.0 |
| Enc Time[%] | 99% |  |  |
| Dec Time[%] | 104% |  |  |

Table 3: Experimental results (parallel case)

|  | Intra |  |  |
| --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate |
| Class A | -2.0 | 0.0 | 0.0 |
| Class B | -1.4 | 0.0 | 0.0 |
| Class C | -1.0 | 0.0 | 0.0 |
| Class D | -0.9 | 0.1 | 0.1 |
| Class E | -1.0 | 0.0 | 0.0 |
| All | -1.3 | 0.0 | 0.0 |
| Enc Time[%] | 103% |  |  |
| Dec Time[%] | 104% |  |  |
|  | Random access |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate |
| Class A | -1.1 | -0.1 | -0.3 |
| Class B | -0.8 | 0.0 | -0.1 |
| Class C | -0.6 | 0.0 | -0.1 |
| Class D | -0.4 | 0.0 | 0.0 |
| Class E |  |  |  |
| All | -0.7 | 0.0 | -0.1 |
| Enc Time[%] | 100% |  |  |
| Dec Time[%] | 105% |  |  |
|  | Low delay |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate |
| Class A |  |  |  |
| Class B | -0.6 | -0.2 | -0.6 |
| Class C | -0.3 | 0.0 | 0.0 |
| Class D | -0.5 | -0.5 | -0.1 |
| Class E | -0.9 | -0.5 | 0.0 |
| All | -0.5 | -0.3 | -0.2 |
| Enc Time[%] | 100% |  |  |
| Dec Time[%] | 104% |  |  |

The number of encoder passes is shown in Table 4.

Table 4: Number of encoding pass

|  | Num |
| --- | --- |
| Anchor | 16 |
| Proposal | 16 |

# Conclusion

This contribution is a CE8 subtest 1 [1][2] (Single- and multi-input schemes in Non-deblocking loop filtering) proposal. In this contribution, the DF (Deblocking Filter) combined adaptive loop filter [3][4] was evaluated. It is asserted the proposed technique provides coding efficiency improvement and the functionality to process the two inputs in parallel case. The coding gains of 1.1 %, 0.7 %, and 0.7 % in HE IO, HE RA, HE LD were reported. In parallel case, the coding gains of 1.3 %, 0.7 %, 0.5 % in HE IO, HE RA, HE LD were reported. The decoding time ratio between 104 % and 105 % and the encoding time ratio between 99 % and 103 % were reported in HE case. The WD2 base drafted text has been prepared. Thanks to Toshiba’s cross-check [5] coding results were confirmed.

# Reference

- T.Yamakage, Y.W. Huang, I.S. Chong, T. Ikai, M. Narroschke, Wang L. Lai, Ali Tabatabai, Madhukar Budagavi, Faouzi Kossentini, “Description of CE8: Non-deblocking loop filtering”, JCTVC-D608, 4th JCT-VC Meeting, Daegu, January 2011.

- T. Yamakage, I. S. Chong, M. Narroschke, “CE8: Non-deblocking Loop Filtering - Summary Report ,” JCTVC-E028, 5th JCT-VC Meeting, Geneva, Switzerland, March 2011.

- T. Ikai and T. Yamamoto, “Parallel adaptive loop filter”, JCTVC-C173, 3rd JCT-VC Meeting, Guangzhou, October 2010.

- T. Ikai and Y. Takanori, “CE8: DF-combined adaptive loop filter”, JCTVC-D115, 4th JCT-VC Meeting, Daegu, January 2011.

- T. Yamakage, “xxx” , 5th JCT-VC Meeting, Geneva, Switzerland, March 2011.

# Patent rights declaration(s)

SHARP Corporation may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).