| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 5th Meeting: Geneva, Switzerland, 16-23 March, 2011 | Document: JCTVC-E141 WG11 Number: m19658 |
| --- | --- |

| Title: | CE8.2: Region-based adaptive loop filter using two-dimensional feature |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Tomohiro Ikai, Takanori Yamazaki 1-9-2 Nakase, Mihama-ku, Chiba-shi, Chiba 261-8520 JAPAN | Tel: Email: | +81-43-299-8526 ikai.tomohiro@sharp.co.jp |
| Source: | SHARP Corporation |  |  |

_____________________________

# Abstract

In this contribution, the region-based adaptive loop filter using two-dimensional feature was evaluated as one of the CE8 subtest 2 proposals. The technique uses two-dimensional feature, activity and direction based on block level adaptation rather than pixel level adaptation, where up to eighteen sets of filter coefficients are sent  In the case of 4x4 block, the coding gains of 0.2 %, 0.5 % and 0.6 % in HE IO, HE RA, HE LD were reported. The decoding time ratio between 96 % and 99 % and the encoding time ratio between 83 % and 94 % were reported in HE case. In the case of 2x2 block, the coding gains of 0.4 %, 0.8 % and 1.0 % in HE IO, HE RA, HE LD, the decoding time ratio between 103 % to 106 % and the encoding time ratio between 94 % and 97 % were reported. On Windows platform, the decoding time ratio were reportedly 88% to 90% in 4x4 and 93% to 96% in 2x2. The WD2 base drafted text has been prepared.

# Introduction

In CE8 subtest 2, Filter adaptation schemes are evaluated. The current HM ALF utilizes the region-based ALF where region is divided by activity on pixel by pixel basis and up to sixteen sets of filter coefficients are sent. Compared to the HM ALF, the proposed technique utilizes two-dimensional feature activity and direction on block adaptation (BA) to catch image correlation more properly and up to eighteen sets of filter coefficients are sent. On top of this, hierarchical region division is used to achieve flexible division patterns and relatively small signal amount.

In the implementation, the proposed technique is compatible with the DF-combined ALF where the number of input signals is two (pre-DF signal and post-DF signal) to achieve further coding efficiency improvement which is disabled at default with #define macro.

# Proposed method

## Two-dimensional feature calculation

The two-dimensional feature are calculated by MxN pixel block basis.

First ActHoriz and ActVert are calculated for each MxN block with the following equations.

ActHoriz=yy=−1N+1xx=−1Ms(xx+1,yy)−s(xx,yy)ActVert=yy=−1Nxx=−1M+1s(xx,yy+1)−s(xx,yy)

where xx and yy are coordinates of the MxN block. s is input signal.

Second the two-dimensional features, Activity and Direction are calculated for each 2x2 block by the following equations.

Activity=ActHoriz+ActVertDirection=1(ActVert>2×ActHoriz)20(ActHoriz>2×ActVert)(otherwise)

## Hierarchical region division

The region division can be depicted in the 2-dimensional feature space. The maximum division is eighteen and is illustrated in Figure 1.

Figure 1. The maximum division in the 2-dimensional feature space

where the thresholds used in Activity’s division is not at even interval considering the Activity’s histogram which is in most cases concentrated in the low Activity. The thresholds are pre-determined and don’t need to be sent.

The region division is processed by two stages – first split and second split as illustrated in Figure 2 and Figure 3. In the first split, the image is divided with Activity into up to three regions. In the second split, each region divided in the first split is further divided with Activity and Direction into up to six regions where the second split pattern is eight, namely octave splitting. The number of first split regions is signaled as num_of_first_split_minus1. The type of second split is signaled as second_split_type.

Figure 2. Region division process (first split)

Figure 3. Region division process (second split)

Using the two-story hierarchical region division approach, image can be divided flexibly. Examples of flexible division are shown in Figure 4.

Figure 4. Examples of region division

## Filtering

Each divided region is filtered with the corresponding filter coefficients sent in the bitstream. The filter tap and shape of spatial filter is the same as the anchor.

# Draft of proposal

The draft of this proposal is described with the additional file, where syntax, semantics and decoding process are included.

# Experimental results

## Comparison with anchor

The proposed method was implemented on HM2.0 and simulated in accordance with the CE 8 subtest 2 test conditions. The experimental results are shown in Table 2.

Compared to the anchor ALF, the proposed method which combines Direction with Activity to divide regions, produces improvement.

Table 2 Experimental results (block size is 4x4)

|  | Intra |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y BD-rate |  | U BD-rate |  | V BD-rate |
| Class A | -0.2 |  | 0.0 |  | 0.0 |
| Class B | -0.3 |  | 0.0 |  | 0.0 |
| Class C | -0.3 |  | 0.1 |  | 0.1 |
| Class D | 0.0 |  | 0.2 |  | 0.2 |
| Class E | -0.3 |  | 0.1 |  | 0.1 |
| All | -0.2 |  | 0.1 |  | 0.1 |
| Enc Time[%] | 83 % |  |  |  |  |
| Dec Time[%] | 96 %(Linux) ¦ 88 %(Win) |  |  |  |  |
|  | Random access |  |  |  |  |
|  | Y BD-rate |  | U BD-rate |  | V BD-rate |
| Class A | -0.7 |  | 0.0 |  | 0.0 |
| Class B | -0.8 |  | 0.0 |  | 0.0 |
| Class C | -0.4 |  | 0.0 |  | 0.1 |
| Class D | -0.2 |  | -0.1 |  | 0.0 |
| Class E |  |  |  |  |  |
| All | -0.5 |  | 0.0 |  | 0.0 |
| Enc Time[%] | 94 % |  |  |  |  |
| Dec Time[%] | 98 %(Linux) ¦ 90 % (Win) |  |  |  |  |
|  | Low delay |  |  |  |  |
|  | Y BD-rate |  | U BD-rate |  | V BD-rate |
| Class A |  |  |  |  |  |
| Class B | -0.7 |  | 0.3 |  | 0.2 |
| Class C | -0.5 |  | 0.2 |  | 0.1 |
| Class D | -0.5 |  | -0.3 |  | 0.0 |
| Class E | -1.0 |  | 0.6 |  | 0.6 |
| All | -0.6 |  | 0.1 |  | 0.2 |
| Enc Time[%] | 94 % |  |  |  |  |
| Dec Time[%] | 99 %(Linux) ¦ 90 %(Win) |  |  |  |  |

Dec Time were measured both Linux and Windows. Linux Dec Time was confirmed by a cross-checker.

Enc Time was measured on Linux

Table 2 Experimental results (block size is 2x2)

|  | Intra |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y BD-rate |  | U BD-rate |  | V BD-rate |
| Class A | -0.4 |  | 0.0 |  | 0.0 |
| Class B | -0.4 |  | 0.0 |  | 0.0 |
| Class C | -0.5 |  | 0.1 |  | 0.1 |
| Class D | -0.3 |  | 0.2 |  | 0.2 |
| Class E | -0.6 |  | 0.1 |  | 0.1 |
| All | -0.4 |  | 0.1 |  | 0.1 |
| Enc Time[%] | 94 % |  |  |  |  |
| Dec Time[%] | 103 %(Linux) ¦ 93 %(Win) |  |  |  |  |
|  | Random access |  |  |  |  |
|  | Y BD-rate |  | U BD-rate |  | V BD-rate |
| Class A | -0.8 |  | -0.1 |  | -0.1 |
| Class B | -1.1 |  | -0.1 |  | -0.1 |
| Class C | -0.7 |  | 0.0 |  | -0.1 |
| Class D | -0.5 |  | 0.1 |  | -0.1 |
| Class E |  |  |  |  |  |
| All | -0.8 |  | 0.0 |  | -0.1 |
| Enc Time[%] | 97 % |  |  |  |  |
| Dec Time[%] | 104 %(Linux) ¦ 95 %(Win) |  |  |  |  |
|  | Low delay |  |  |  |  |
|  | Y BD-rate |  | U BD-rate |  | V BD-rate |
| Class A |  |  |  |  |  |
| Class B | -0.8 |  | 0.0 |  | 0.0 |
| Class C | -0.8 |  | 0.1 |  | 0.1 |
| Class D | -0.8 |  | 0.4 |  | 0.3 |
| Class E | -1.7 |  | 0.6 |  | 0.6 |
| All | -1.0 |  | 0.2 |  | 0.2 |
| Enc Time[%] | 97 % |  |  |  |  |
| Dec Time[%] | 106 %(Linux) ¦ 96 %(Win) |  |  |  |  |

Dec Time were measured both Linux and Windows. Linux Dec Time was confirmed by a cross-checker.

Enc Time was measured on Linux

The number of encoding passes is shown in Table 4.

Table 4: Number of encoding pass

|  | Num |
| --- | --- |
| Anchor | 16 |
| Proposal | 16 |

## Block size comparison

Additionally experiment, which compares several block sizes of 4x4, 4x2, 4x1, 2x2 and 1x1, were shown in Table 4.

The result shows the smaller block size is the better in terms of coding efficiency, while the encoding time and decoding time increases.

Table 5: Block size comparison

|  | Intra |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | 4x4※ | 4x2 | 4x1 | 2x2※ | 1x1 |
| Class A | -0.2 | -0.3 | -0.4 | -0.4 | -0.5 |
| Class B | -0.3 | -0.3 | -0.4 | -0.4 | -0.4 |
| Class C | -0.3 | -0.4 | -0.4 | -0.5 | -0.6 |
| Class D | 0.0 | -0.1 | -0.2 | -0.3 | -0.4 |
| Class E | -0.3 | -0.4 | -0.4 | -0.6 | -0.6 |
| All | -0.2 | -0.3 | -0.3 | -0.4 | -0.5 |
| Enc Time[ %] | 83 % | 86 % | 92 % | 94 % | 125 % |
| Dec Time[%] | 96 % | 96 % | 100 % | 103 % | 108 % |
|  | Random access |  |  |  |  |
|  | 4x4※ | 4x2 | 4x1 | 2x2※ | 1x1 |
| Class A | -0.7 | -0.8 | -0.9 | -0.8 | -0.9 |
| Class B | -0.8 | -1.0 | -1.0 | -1.1 | -1.1 |
| Class C | -0.4 | -0.6 | -0.6 | -0.7 | -0.8 |
| Class D | -0.2 | -0.3 | -0.4 | -0.5 | -0.6 |
| Class E |  |  |  |  |  |
| All | -0.5 | -0.7 | -0.7 | -0.8 | -0.9 |
| Enc Time[%] | 94 % | 95 % | 97 % | 97 % | 106 % |
| Dec Time[%] | 98 % | 98 % | 103 % | 104 % | 114 % |

Dec Time and Enc Time were measured on Linux

※: The results of the marked block sizes have been cross-checked.

# Conclusion

In this contribution, the region-based adaptive loop filter using two-dimensional feature [1][2] was evaluated as one of the CE8 subtest 2 proposals. The technique uses two-dimensional feature, activity and direction based on block level adaptation rather than pixel level adaptation, where block size is 4x4 and up to eighteen sets of filter coefficients are sent  In the case of 4x4 block, the coding gains of 0.2 %, 0.5 % and 0.6 % in HE IO, HE RA, HE LD were reported. The decoding time ratio between 96 % to 99 % and the encoding time ratio between 83 % and 94 % were reported in HE case. In the case of 2x2 block, the coding gains of 0.4 %, 0.8 % and 1.0 % in HE IO, HE RA, HE LD were reported. The decoding time ratio between 103 % and 106 % and the encoding time ratio between 94 % and 97 % were reported. On Windows platform, the decoding time ratio were reportedly 88% to 90% in 4x4 and 93% to 96% in 2x2. The WD2 base drafted text has been prepared. Thanks to Qualcomm’s cross-check[7] coding results and Enc/Dec time on Linux were confirmed.

In the implementation, it is asserted the proposed technique is compatible with the DF-combined ALF[4][5][6] where the number of input signals is two (pre-DF signal and post-DF signal) to achieve further coding efficiency improvement which is disabled at default with #define macro.

# Reference

- T.Yamakage, Y.W. Huang, I.S. Chong, T. Ikai, M. Narroschke, Wang L. Lai, Ali Tabatabai, Madhukar Budagavi, Faouzi Kossentini, “Description of CE8: Non-deblocking loop filtering”, JCTVC-D608, 4th JCT-VC Meeting, Daegu, January 2011.

- T. Yamakage, I. S. Chong, M. Narroschke, “CE8: Non-deblocking Loop Filtering - Summary Report ,” JCTVC-E028, 5th JCT-VC Meeting, Geneva, Switzerland, March 2011.

- HM 2.0 software: http://hevc.kw.bbc.co.uk/trac/browser/tags/HM-2.0

- T.Ikai and T.Yamamoto, “Parallel adaptive loop filter”, JCTVC-C173, 3rd JCT-VC Meeting, Guangzhou, October 2010.

- T.Ikai and T. Yamazaki, “CE8: DF-combined adaptive loop filter”, JCTVC-D115, 4th JCT-VC Meeting, Daegu, January 2011.

- T. Ikai and T. Yamazaki, “CE8.1: DF-combined adaptive loop filter”, JCTVC-E140, 5th JCT-VC Meeting, Geneva, Switzerland, March 2011.

- xxx, “xxx”, JCTVC-Exxx, 5th JCT-VC Meeting, Geneva, Switzerland, March 2011.

# Patent rights declaration(s)

SHARP Corporation may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).