| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 5th Meeting: Geneva, CH, 16-23 March, 2011 | Document: JCTVC-E142 |
| --- | --- |

| Title: | Dynamic range restriction of temporal motion vector |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Sung-Chang Lim Hui Yong Kim Jongho Kim Jinho Lee Jin Soo Choi ETRI, 138 Gajeongno, Yuseong-gu, Daejeon, 305-700, Korea | Tel: Email: | +82-42-860-5772 sclim@etri.re.kr hykim5@etri.re.kr pooney@etri.re.kr jinosoul@etri.re.kr jschoi@etri.re.kr |
| Source: | ETRI |  |  |

_____________________________

# Abstract

This contribution presents a dynamic range restriction of temporal motion vector. In HM 2.0, temporal motion vectors are used as a temporal predictor to improve coding efficiency both in advanced motion vector prediction (AMVP) and in prediction unit (PU) merge. In order to use the temporal motion vectors, they have to be stored in the memory before encoding and decoding of the current picture. In the last meeting, the temporal motion vector memory saving method (JCTVC-D072) was adopted in HM 2.0. This method reduces the spatial resolution of motion vector data to be stored in the memory. To further reduce the memory size and the memory access bandwidth, the dynamic range restriction of temporal motion vector is proposed in this contribution. The proposed method controls the bit width for storing each temporal motion vector component. The experimental results show that the proposed method additionally reduces the required memory size by about half on top of HM 2.0 with about 0.2% coding loss for random access and low delay test conditions on average.

# Introduction

In HM 2.0, temporal motion vectors, which are the motion vectors of reference picture, are used as a temporal predictor to improve coding efficiency both in AVMP and in PU merge. By using temporal motion vector predictor, it shows that the average 2.2% of BD-Rate saving is achieved for random access and low delay test conditions. However, the usage of the temporal motion vector leads to the need of additional memory requirement in inter prediction.

In the last meeting, the temporal motion vector memory saving method [1] was adopted in HM 2.0. This method reduces the spatial resolution of temporal motion vector data to be stored in the memory. Without using this method, the temporal motion vectors of every 4x4 PU have to be stored in the memory as described in Figure 1. In HM 2.0, the temporal motion vectors of the first 4x4 PU in the units of 16x16 size are stored in the memory with lower resolution as shown in Figure 2 [1]. Therefore, both the required memory size and the memory access bandwidth for the temporal motion vector data are reduced by 1/16.

Figure 1. Temporal motion vector per 4x4 PU in HM 1.0

Figure 2. Temporal motion vector per 16x16 PU in HM 2.0

As the picture sizes increases, the dynamic range of each motion vector component becomes larger. Thus, the memory size needed to store temporal motion vector is affected by spatial resolution of motion vector data as well as the bit width representing the value of each motion vector component. Table 1 shows the required memory size to store the temporal motion vector data. In Table 1, the dynamic range of a motion vector is computed in units of quarter luma sample. In worst case scenario, such as class A, the motion vector component x and y are represented in 15 bits and 14 bits, respectively and the memory size of the temporal motion vector is 59.39 Mbits/frame. For test sequences of 5 classes, 21.41 Mbits/frame is needed on average.

Table 1. Required memory size for storing temporal motion vector

|  | Width | Height | Dynamic range of X comp. (First CU) | Dynamic range of Y comp. (First CU) | Dynamic range of X comp. (Last CU) | Dynamic range of Y comp. (Last CU) | Bit width for X comp. | Bit width for Y comp. | Number of 4x4 blocks | MV per block | Number of ref. lists | Number of ref. pictures per ref. list | Required memory size (Mbits/frame) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Class A | 2560 | 1600 | -252 ~ 10236 | -252 ~ 6396 | -10460 ~ 28 | -6620 ~ 28 | 15 | 14 | 256000 | 2 | 2 | 2 | 59.39 |
| Class B | 1920 | 1080 | -252 ~ 7676 | -252 ~ 4316 | -7900 ~ 28 | -4540 ~ 28 | 14 | 14 | 129600 | 2 | 2 | 2 | 29.03 |
| Class C | 832 | 480 | -252 ~ 3324 | -252 ~ 1916 | -3548 ~ 28 | -2140 ~ 28 | 13 | 12 | 24960 | 2 | 2 | 2 | 4.99 |
| Class D | 416 | 240 | -252 ~ 1660 | -252 ~ 956 | -1884 ~ 28 | -1180 ~ 28 | 12 | 12 | 6240 | 2 | 2 | 2 | 1.20 |
| Class E | 1280 | 720 | -252 ~ 5116 | -252 ~ 2876 | -5340 ~ 28 | -3100 ~ 28 | 14 | 13 | 57600 | 2 | 2 | 2 | 12.44 |
| Average |  |  |  |  |  |  |  |  |  |  |  |  | 21.41 |

To further reduce both the memory size and the memory access bandwidth of the temporal motion vector data, the method to restrict the dynamic range of the temporal motion vector is proposed in this contribution.

# Proposal

## Algorithm description

In this contribution, the bit width for storing each motion vector component is controlled in order to restrict the minimum value and the maximum value of each motion vector component. And the required memory size and the memory access bandwidth can be reduced by restricting the dynamic range of motion vector value. The proposed method simply clips the value of each motion vector component according to the bit width information before saving it in the memory. The bit width information is signaled in sequence parameter set to reflect the motion characteristic of each sequence.

Each motion vector component is clipped to the bit width of N bits by the following equations:

| clippedMV_X = min( 1<<(N-1)-1, max( -1<<(N-1), MV_X ) ). | (1) |
| --- | --- |
| clippedMV_Y = min( 1<<(N-1)-1, max( -1<<(N-1), MV_Y ) ). | (2) |

MV_X and MV_Y represent the value of motion vector component x and y, respectively. After clipping of each motion vector component, the clipped values of motion vector component, clippedMV_X and clippedMV_Y, are stored in the memory with fixed bit width. Then, the clipped motion vectors are used as a temporal motion vector predictor.

For example, Figure 3 shows that the clipping operation for given motion vector component. Here, the value of motion vector component represented by the bit width of 13 bits is clipped in the range of -128 to +127. By doing so, every motion vector component can be stored with the fixed bit width of 8 bits.

Figure 3. Example of proposed dynamic range restriction.

## Syntax, semantics, and decoding process

### Proposed syntax

This contribution proposes that the bit width information to restrict the dynamic range of motion vector component can be signaled in sequence parameter set as described in Table 2.

Table 2. Proposed syntax design in sequence parameter set

| seq_parameter_set_rbsp( ) { | C | Descriptor |
| --- | --- | --- |
| ..... |  |  |
| bit_width_temporal_motion_vector_minus8 | 0 | se(v) |
| ..... |  |  |
| rbsp_trailing_bits( ) | 0 |  |
| } |  |  |

### Proposed semantics

And associated semantics is as follows:

bit_width_temporal_motion_vector_minus8 specifies the bit width of the temporal motion vector component. When bit_width_temporal_motion_vector_minus8 is not present, it shall be inferred to be equal to 0. The bit width of temporal motion vector component is specified as follows:

TMVBitWidth = bit_width_temporal_motion_vector_minus8 + 8

### Proposed decoding process

The proposed decoding process is as follows:

#### Clipping process for motion vector components

This process is invoked when slice_type is equal to B.

A clipping process shall be performed on a treeblock basis after the completion of the in-loop filter process. Each treeblock is processed on a coding unit basis with the same order as decoding process.

Inputs to this process are

- a luma location ( xP, yP ) specifying the top-left luma sample of the prediction unit relative to the top-left sample of the current picture,

- motion vector matrices MvL0 and MvL1.

Outputs of this process are the clipped motion vector matrices CMvL0 and CMvL1.

For X being replaced by either 0 or 1 in the matrices MvLX and CMvLX, the following applies.

mvLX = MvLX[ xP, yP ]

cmvLX[ 0 ] = Clip3( -1<<( TMVBitWidth -1), 1<<( TMVBitWidth -1)-1, mvLX[ 0 ] )

cmvLX[ 1 ] = Clip3( -1<<( TMVBitWidth -1), 1<<( TMVBitWidth -1)-1, mvLX[ 1 ] )

CMvLX [ xP, yP ] = cmvLX

# Experiments

The proposed method is implemented on HM 2.0 software. The experiments are conducted on the platform of Intel Core i7 CPU X980 @ 3.33GHz with 12GB RAM. Table 3 shows the summary result of the proposed method using the bit width of 8 bits compared to HM 2.0 anchor [2]. About 0.2% of coding loss is observed for four test conditions on average. In random access cases, the proposed method shows 0.3% of coding loss for high efficiency and 0.4% of coding loss for low complexity on average. In low delay cases, the proposed method shows no coding loss for high efficiency and low complexity on average. Table 4 and 5 show the summary result of the proposed method using the bit width of 6 bits and 10 bits, respectively. The detail results can be found in accompanied Excel files.

Table 3. Summary result of the proposed method (8 bits)

|  | Random access |  |  | Random access LoCo |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | 0.9 | 0.7 | 0.6 | 1.0 | 0.7 | 0.8 |
| Class B | 0.2 | 0.2 | 0.2 | 0.3 | 0.1 | 0.2 |
| Class C | 0.1 | 0.1 | 0.2 | 0.1 | 0.2 | 0.1 |
| Class D | 0.0 | 0.1 | 0.1 | 0.0 | -0.1 | 0.1 |
| Class E |  |  |  |  |  |  |
| All | 0.3 | 0.3 | 0.2 | 0.4 | 0.2 | 0.3 |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 101% |  |  | 100% |  |  |
|  | Low delay |  |  | Low delay LoCo |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A |  |  |  |  |  |  |
| Class B | 0.0 | 0.1 | -0.3 | 0.0 | 0.0 | 0.2 |
| Class C | 0.0 | 0.1 | 0.1 | -0.1 | -0.1 | -0.1 |
| Class D | -0.1 | 0.1 | -0.3 | 0.1 | 0.1 | 0.1 |
| Class E | 0.0 | -0.3 | -0.1 | -0.1 | -0.1 | 0.0 |
| All | 0.0 | 0.0 | -0.1 | 0.0 | 0.0 | 0.1 |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 100% |  |  | 98% |  |  |

Table 4. Summary result of the proposed method (6 bits)

|  | Random access |  |  | Random access LoCo |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | 2.1 | 1.6 | 1.5 | 2.4 | 2.1 | 2.3 |
| Class B | 1.2 | 0.7 | 0.8 | 1.6 | 1.0 | 1.0 |
| Class C | 1.1 | 1.0 | 1.2 | 1.4 | 1.3 | 1.4 |
| Class D | 0.4 | 0.3 | 0.5 | 0.5 | 0.4 | 0.5 |
| Class E |  |  |  |  |  |  |
| All | 1.2 | 0.9 | 1.0 | 1.5 | 1.2 | 1.3 |
| Enc Time[%] | #NUM! |  |  | #NUM! |  |  |
| Dec Time[%] | #NUM! |  |  | #NUM! |  |  |
|  | Low delay |  |  | Low delay LoCo |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A |  |  |  |  |  |  |
| Class B | 0.5 | 0.5 | 0.3 | 0.7 | 0.5 | 0.7 |
| Class C | 0.4 | 0.5 | 0.5 | 0.5 | 0.5 | 0.5 |
| Class D | 0.0 | 0.1 | 0.0 | 0.2 | 0.1 | 0.4 |
| Class E | -0.1 | -0.2 | 0.0 | 0.1 | 0.1 | 0.2 |
| All | 0.2 | 0.3 | 0.2 | 0.4 | 0.3 | 0.4 |
| Enc Time[%] | #NUM! |  |  | #NUM! |  |  |
| Dec Time[%] | #NUM! |  |  | #NUM! |  |  |

Table 5. Summary result of the proposed method (10 bits)

|  | Random access |  |  | Random access LoCo |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| Class B | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| Class C | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| Class D | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| Class E |  |  |  |  |  |  |
| All | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| Enc Time[%] | #NUM! |  |  | #NUM! |  |  |
| Dec Time[%] | #NUM! |  |  | #NUM! |  |  |
|  | Low delay |  |  | Low delay LoCo |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A |  |  |  |  |  |  |
| Class B | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.1 |
| Class C | 0.0 | 0.1 | 0.0 | 0.0 | -0.1 | -0.1 |
| Class D | 0.0 | 0.1 | -0.1 | 0.0 | 0.0 | 0.1 |
| Class E | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| All | 0.0 | 0.1 | 0.0 | 0.0 | 0.0 | 0.0 |
| Enc Time[%] | #NUM! |  |  | #NUM! |  |  |
| Dec Time[%] | #NUM! |  |  | #NUM! |  |  |

Since the proposed method simply clips the value of temporal motion vector component before it is stored in the memory, there is no additional complexity impact on software runtime in encoding and decoding as shown in Table 3. Regarding the encoding and decoding time of the proposed method, only 8 bits version is reliable as shown in Table 3.

Table 6 shows the required memory size for storing the temporal motion vector data and the percentage of memory reduction by using the proposed method. By using the proposed method using the bit width of 8 bits on top of HM 2.0, the memory size for storing the temporal motion vector is reduced by about half.

Table 6. Required memory size (Mbits/frame) and memory reduction (%)

|  | Reference sample storage (4:2:0, 8 bits) | Temporal motion vector storage |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | HM 1.0 | HM 2.0 | HM 2.0 + Proposal (6 bits) | HM 2.0 + Proposal (8 bits) | HM 2.0 + Proposal (10 bits) |
| Class A | 196.61 | 59.39 | 3.71 | 1.54 | 2.05 | 2.56 |
| Class B | 99.53 | 29.03 | 1.81 | 0.78 | 1.04 | 1.30 |
| Class C | 19.17 | 4.99 | 0.31 | 0.15 | 0.20 | 0.25 |
| Class D | 4.79 | 1.20 | 0.07 | 0.04 | 0.05 | 0.06 |
| Class E | 44.24 | 12.44 | 0.78 | 0.35 | 0.46 | 0.58 |
| Average | 72.87 | 21.41 | 1.34 | 0.57 | 0.76 | 0.95 |
| Memory size reduction |  | 100% | 6.25% | 2.66% | 3.55% | 4.43% |
|  |  | 1x | 16x | 37.61x | 28.21x | 22.57x |

# Conclusion

In this contribution, a dynamic range restriction of temporal motion vector is proposed and its coding performance and the memory saving are reported. The proposed method simply clips the value of each motion vector component according to the bit width information signaled in sequence parameter set. The proposed method further reduces the required memory size for storing temporal motion vector by about half on top of HM 2.0 with about 0.2% of coding loss. We suggest the proposed method to be included into the next version of HM.

# References

[1] Yeping Su and Andrew Segall, “CE9: Reduced resolution storage of motion vector data,” JCTVC-D072, Jan. 2011.

[2] Frank Bossen, "Common test conditions and software reference configurations," JCTVC-D600, Jan. 2011.

# Patent rights declaration(s)

ETRI may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).