| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 5th Meeting: Geneva, CH, 16-23 March, 2011 | Document: JCTVC-E143 |
| --- | --- |

| Title: | CE5: Counter based adaption for LCEC |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Liwei Guo Bin Li Xianglin Wang Marta Karczewicz Jizheng Xu | Email: | liweig@qualcomm.com xianglin@qualcomm.com jzxu@microsoft.com |
| Source: | Qualcomm University of Science and Technology of China Microsoft |  |  |

_____________________________

# Abstract

This document reports CE results on counter based adaptation for low complexity entropy coding (LCEC). In total there are 41 counters added and the bit-depth of each counter is no more than 5 bits. The average bit savings are 0.2%, 0.5% and 0.8% for AI_LC, RA_LC and LD_LC cases respectively. It is recommended this counter based scheme be integrated into HM software.

- Introduction

In the current HM-2.0, lookup table and codeword adaption is widely used in LCEC to improve the coding efficiency. When one codeword is decoded, it is swapped with the codeword in front of it to make the current code word have smaller index in the lookup table. But this direct swapping scheme may be too sensitive in some cases. In this document, a counter based adaptation scheme is introduced, which improves the RD performance without  noticeable running increment.

- Solution

In this joint proposal, 41 counters are added in the LCEC lookup table. These counters are used for CBF tables, split tables, block cbp tables and MI1 table (for inter prediction direction and reference frame index coding). For CBF, split and MI1 tables, counters are used for the first 4 codewords. For block cbp tables, counters are used for the first 2 codewords. Each of those counters is of 5 bits. The other codewords are still adapted using direct swapping. For each table, a sum counter is used to aggregate the sum of counters for that table, which is of 4 bits. Once the sum counter of a table reaches a user-defined threshold, all counters for that table will be divided by 2. There are in total 2 CBF, 4 split, 2 block cbp and 1 MI1 tables. Thus, the total number of counters is 41.

When one codeword associated with a counter comes, the counter of this codeword is increased by one. If the counter of the current codeword is equal or greater than the counter of the codeword immediately preceding it, these two codewords are swapped in the lookup table. When one codeword not associated with a counter comes, the direct swapping strategy is used, i.e., it is swapped with the codeword immediately preceding it. When a codeword associated with a counter is swapped with a codeword without a counter, the later codeword will inherit the counter of the former codeword.

- Experimental results

The proposed counter based algorithm was implemented on HM-2.0. Three low complexity tests are performed according to the common CE test condition. The anchor is the HM-2.0 default anchor. The summary result is shown in Table 1. The detailed result can be found in JCTVC-E143.xls

Table 1 Summary result of counter based LCEC (Y BD-rate)

|  | AI_LC | RA_LC | LD_LC |
| --- | --- | --- | --- |
| Class A | -0.2 | -0.5 | N/A |
| Class B | -0.1 | -0.5 | -0.9 |
| Class C | -0.1 | -0.6 | -1.0 |
| Class D | -0.1 | -0.6 | -0.7 |
| Class E | -0.3 | N/A | -0.8 |
| All | -0.2 | -0.5 | -0.8 |
| Enc Time | 100% | 98% | 99% |
| Dec Time | 100% | 100% | 100% |

It can be seen that the proposed counter based LCEC algorithm can bring bits saving without much impact on the complexity.

- Conclusions

- This contribution describes a counter based adaption scheme for LCEC coding. In total there are 41 counters introduced. The average bit savings are 0.2%, 0.5% and 0.8% for AI_LC, RA_LC and LD_LC cases respectively. The impact on the encoder/decoder complexity is very minor. In the experiments, we did not observe any running time increase. Thus we recommend this counter based adaptation be adopted by HM software.

# LCEC parsing process for syntax elements with counter-based codetable adaptation

This process is invoked when parsing syntax elements cbp_yuv_root, cbp_luma, cbp_cb, cbp_cr, split_coding_unit_mode, and inter_pred_dir_ref_id , and when entropy_coding_mode_flag is equal to 0.

Inputs to this process are bits from slice data, a request of a syntax element and values of prior parsed syntax elements.

Output of this process is the value of the syntax element.

When starting the parsing of the slice data of a slice, an initialization process of counters, sum counters and decoding tables is invoked as specified in Section 1.

The parsing of syntax elements proceeds as follows:

For each requested syntax element, a decoding table LCEC_decoding_table, a counter table LCEC_decoding_counter_table, a counter number LCEC_decoding_counter_num, and a sum counter LCEC_decoding_sum_counter are derived as specified in Section 2.

A codeword index is parsed as specified in Section 3.

A codetable adaptation process is specified in Section 4.

## Initialization of counters, sum counters and decoding tables

Outputs of this process are initialized LCEC counter-based adaptation decoding tables, LCEC counter tables, LCEC sum counters.

The initialization process proceeds as follows:

- Initialize Split_coding_unit_mode_decoding_table [4][7] as in Table 1.

- Initialize cbp_yuv_root_decoding_table[2][8] as in Table 2.

- Initialize cbp_decoding_table [2][15] as in Table 3.

- Initialize inter_dir_ref_id_decoding_table[9] as in Table 4.

- Set all elements of the following decoding counters to be 0:

Split_coding_unit_mode_decoding_counters[4][4],

cbp_yuv_root_decoding_counters[2][4],

cbp_decoding_counters [2][2],

inter_dir_ref_id_decoding_counters[4].

- Set all elements of the following decoding sum counters to be 0:

Split_coding_unit_mode_sum_decoding_counters[4],

cbp_yuv_root_decoding_sum_counters[2],

cbp_decoding_sum_counters [2],

inter_dir_ref_id_decoding_sum_counters.

| Syntax element value CodeNum | SplitVlcIndex |  |  |  |
| --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 |
| 0 | 0 | 0 | 0 | 1 |
| 1 | 1 | 1 | 1 | 2 |
| 2 | 2 | 2 | 2 | 3 |
| 3 | 3 | 3 | 3 | 4 |
| 4 | 4 | 4 | 4 | 5 |
| 5 | 5 | 5 | 5 | 6 |
| 6 | 6 | 6 | 6 | 0 |

Table 1 Initial values of Split_coding_unit_mode_decoding_table

| Syntax element value CodeNum | cbpVlcIndex |  |
| --- | --- | --- |
|  | 0 | 1 |
| 0 | 1 | 0 |
| 1 | 7 | 1 |
| 2 | 0 | 5 |
| 3 | 5 | 3 |
| 4 | 3 | 4 |
| 5 | 4 | 7 |
| 6 | 2 | 2 |
| 7 | 6 | 6 |

Table 2 Initial values of cbp_yuv_root_decoding_table

| Syntax element value CodeNum | blkcbpVlcIndex |  |
| --- | --- | --- |
|  | 0 | 1 |
| 0 | 14 | 0 |
| 1 | 13 | 1 |
| 2 | 12 | 3 |
| 3 | 10 | 7 |
| 4 | 6 | 2 |
| 5 | 11 | 4 |
| 6 | 9 | 5 |
| 7 | 8 | 8 |
| 8 | 5 | 9 |
| 9 | 4 | 11 |
| 10 | 2 | 6 |
| 11 | 7 | 10 |
| 12 | 3 | 12 |
| 13 | 1 | 13 |
| 14 | 0 | 14 |

Table 3 Initial values of cbp_decoding_table

| Syntax element value CodeNum | Case of number of reference frames |  |  |
| --- | --- | --- | --- |
|  | 0 | 1 | 2 |
| 0 | 0 | 0 | 0 |
| 1 | 4 | 2 | 1 |
| 2 | 1 | 4 | 4 |
| 3 | 2 | 1 | 6 |
| 4 | 3 | 3 | 5 |
| 5 | 6 | 6 | 7 |
| 6 | 5 | 5 | 2 |
| 7 | 7 | 7 | 3 |
| 8 | 8 | 8 | 8 |

Table 4 Initial values of inter_dir_ref_id_decoding_table for different cases

When there is no more than 1 reference frame in each list and two lists are identical, case 0 is used as the initial table; otherwise when there is no more than 1 reference frame in each list, case 1 is used as the initial table; otherwise case 2 is used as the initial table.

## Derivation process of counter table, counter number, sum counter and decoding table

Input to this process is a request of a syntax element and values of prior parsed syntax elements.

Output of this process is:

LCEC_decoding_table: decoding table mapping a CodeNum to a syntax element value

LCEC_decoding_counter_num:   the number of decoding counters

LCEC_decoding_counter_table:   a table of decoding counters

LCEC_decoding_sum_counter:   a counter to record the summation of decoding counters

The derivations for different syntax elements are different, and are specified in Section 2.1, 2.2, 2.3 and 2.4.

### Derivation process of syntax split_coding_unit_mode

The LCEC_decoding_table, LCEC_decoding_counter_num, LCEC_decoding_counter_table, LCEC_decoding_sum_counter are derived by

- LCEC_decoding_table is Split_coding_unit_mode_decoding_table[SplitVlcIndex];

- LCEC_decoding_counter_num = 4;

- LCEC_decoding_counter_table is     Split_coding_unit_mode_decoding_counters [SplitVlcIndex];

- LCEC_decoding_sum_counter is Split_coding_unit_mode_sum_decoding_counters [SplitVlcIndex]

The variable SplitVlcIndex is derived by

-SplitVlcIndex = CUDepth;

### Derivation process of syntax elements cbp_luma, cbp_cb, and cbp_cr

The LCEC_decoding_table, LCEC_decoding_counter_num, LCEC_decoding_counter_table, LCEC_decoding_sum_counter are derived by

- LCEC_decoding_table is cbp_decoding_table [blkcbpVlcIndex];

- LCEC_decoding_counter_num = 2;

- LCEC_decoding_counter_table is cbp_decoding_counters [blkcbpVlcIndex];

- LCEC_decoding_sum_counter is cbp_decoding_sum_counters [blkcbpVlcIndex];

The variable blkcbpVlcIndex for cbp_luma is derived by:

- if PredMode is MODE_INTRA, blkcbpVlcIndex = 0; otherwise, blkcbpVlcIndex = 1;

The variable blkcbpVlcIndex for cbp_cb and cbp_cr is derived by:

-blkcbpVlcIndex = 1;

### Derivation process of syntax element cbp_yuv_root

The LCEC_decoding_table, LCEC_decoding_counter_num, LCEC_decoding_counter_table, LCEC_decoding_sum_counter are derived by

- LCEC_decoding_table is cbp_yuv_root_decoding_table [cbpVlcIndex];

- LCEC_decoding_counter_num = 4;

- LCEC_decoding_counter_table is cbp_yuv_root_decoding_counters [cbpVlcIndex];

- LCEC_decoding_sum_counter is cbp_yuv_root_decoding_sum_counters [cbpVlcIndex];

The variable cbpVlcIndex is derived by:

- if PredMode is MODE_INTRA, cbpVlcIndex = 0; otherwise, cbpVlcIndex = 1;

### Derivation process of syntax element inter_pred_dir_ref_id

The LCEC_decoding_table, LCEC_decoding_counter_num, LCEC_decoding_counter_table, LCEC_decoding_sum_counter are derived by

- LCEC_decoding_table is set to be inter_dir_ref_id_decoding_table;

- LCEC_decoding_counter_num = 4;

- LCEC_decoding_counter_table is set to be inter_dir_ref_id_decoding_counters;

- LCEC_decoding_sum_counter is set to be inter_dir_ref_id_decoding_sum_counters;

## Parsing process of codeword index

Input to this process are bits from slice data, and syntax descriptor parameters (e.g., vlc(n, v), ).

Out of this process is a codeword index CodeNum, and a syntax element value CodeValue. The syntax element value CodeValue is assigned to the syntax element.

## Codetable adaptation process

Input to this process is CodeNum, LCEC_decoding_table, LCEC_decoding_counter_num, LCEC_decoding_counter_table, and LCEC_decoding_sum_counter.

Output of this process is LCEC_decoding_table, LCEC_decoding_counter_num, LCEC_decoding_counter_table, and LCEC_decoding_sum_counter.

The codetable adaptation process consists of normalization condition check, codetable adaptation and counter normalization processes. It is specified using the following pseudo codes:

Normalize = false;

if ( LCEC_decoding_counter_num > 0)

{

//Normalization condition check

if (CodeNum < LCEC_decoding_counter_num)

{

LCEC_decoding_counter_table [CodeNum]++;

if (LCEC_decoding_sum_counter != 15 )

{

LCEC_decoding_sum_counter ++;

}

else

{

Normalize = true;

}

//Codetable adaptation

if ( CodeNum > 0 && LCEC_decoding_counter_table [CodeNum] >= LCEC_decoding_counter_table [CodeNum -1])

{

swap();

swap_counter();

}

}

else	//(not in the range of the counter, directly swapping)

{

swap();

}

//Counter Normalization

if ( Normalize )

{

LCEC_decoding_sum_counter = 0;

for each element in LCEC_decoding_counter_table

element >>= 1;

}

}

else //(no counter, directly swapping)

swap();

swap () is the process swapping the entries in the decoding table LCEC_decoding_table. No output of this process. The process is specified using the following pseudo codes.

if (CodeNum > 0)

{

CodeNum_minus1= CodeNum – 1;

value_of_code_word = LCEC_decoding_table [CodeNum];

value_of_code_word_minus1 = LCEC_decoding_table [CodeNum_minus1];

LCEC_decoding_table [CodeNum] = value_of_code_word_minus1;

LCEC_decoding_table [CodeNum_minus1] = value_of_code_word;

}

else

{

Do nothing.

}

swap_counter () is the process swapping the entries in the decoding counter table LCEC_decoding_counter_table. No output of this process. The process is specified using the following pseudo codes.

if (CodeNum > 0)

{

CodeNum _minus1 = CodeNum – 1;

counter_of_code_word = LCEC_decoding_counter_table [CodeNum];

counter_of_code_word_minus1 = LCEC_decoding_counter_table [CodeNum _minus1];

LCEC_decoding_counter_table [CodeNum] = counter_of_code_word_minus1;

LCEC_decoding_counter_table [CodeNum _minus1] = counter_of_code_word;

}

else

{

Do nothing.

}

# Patent rights declaration(s)

Qualcomm Inc., University of Science and Technology of China, and Microsoft Corp. may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).