| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 5th Meeting: Geneva, CH, 16-23 March, 2011 | Document: JCTVC-E144 |
| --- | --- |

| Title: | CE12, Subset 1: Report of Deblocking for Large Size Blocks |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Zhongbo Shi Xiaoyan Sun Jizheng Xu | Email: | {xysun, jzxu}@microsoft.com |
| Source: | University of Science and Technology of China Microsoft |  |  |

_____________________________

# Abstract

This document presents a deblocking scheme for large size blocks to reduce blocking artifacts. For large smooth regions with small variation, an extra smoothing deblocking mode is introduced to suppress the visually severe blocking artifacts. It is observed that the proposed method can reduce blocking artifacts in smooth regions, which are usually more visible to human eyes.

# Introduction

Visual artifacts are more visible to human eyes in smooth regions. It is also the case for blocking artifacts. Although compared to AVC, the current HEVC scheme can yield much better decoded videos, the blocking artifacts are still annoying, especially at low bit-rates and in smooth region. Thus, we propose to use a stronger deblocking filter for blocks with large block size and in smooth region.

The scheme described in this document is based on what we have done in [1] and [2]. The difference is that we have integrated the proposed scheme into HM2.0 software and fine-tuned the parameters

# Algorithm description

Fig. 1. Extra smoothing deblocking (Mode B). A vertical boundary of a luminance block is taken for example.  are the pixels to be filtered. In this example, the filtered value of q4 is obtained by averaging .

There are two deblocking mode in the proposed scheme, the HEVC deblocking mode (Mode A), which performs exactly the same filtering as in the current HEVC deblocking, and the extra smoothing deblocking mode (Mode B), which performs a strong and long distance filtering across certain transform unit (TU) boundaries.

Deblocking Mode B is supposed to be applied in large smooth regions with small variation. In the proposed scheme, TU boundaries satisfy the following conditions will be processed with deblocking Mode B.

- The two adjacent TU’s related to the TU boundary are both in intra-coded CU’s;

- The smaller transform size between the two TU's is larger than a certain value (set to 8 in the proposed scheme);

- The larger transform size between the two TU's is larger than a certain value (set to 16 in the proposed scheme).

- The sum of absolute difference between the luminance values inside two TUs and average values of tow TU’s boundary pixels is smaller than a certain threshold (set to 512 in the proposed scheme).

For luminance, an additional condition should be satisfied:

- All the pixels at the TU boundaries should be processed as strong filtering in Mode A;

Other TU boundaries will be processed with deblocking Mode A.

For deblocking Mode B, the filtering is performed along lines perpendicular to the TU boundary. As shown in Fig. 1, suppose  are the pixels in the same row or column of two adjacent TU’s, during filtering their values are modified as

,

This is a heavy and long distance smoothing across the TU boundary and pixels in large smooth regions are preferred to be involved. For deblocking Mode B, no boundary strength and filter thresholds are needed.

Once the deblocking mode for each TU boundary is determined, the two different deblocking modes are simultaneously employed in a raster-scan order at the TU level for both luminance and chroma components. The reconstructed frame is obtained in a single deblocking pass.

# Compression performance discussion

Experiments are done with HM2.0 software platform. We compared our results with HM2.0 anchors.

## Objective compression performance

Following tables show the summary. Details can be found in JCTVC-E144.xls.

|  | Intra |  |  | Intra LoCo |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | 0.0 | -0.1 | -0.1 | 0.0 | 0.0 | 0.0 |
| Class B | 0.0 | 0.0 | -0.1 | 0.0 | 0.0 | 0.0 |
| Class C | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| Class D | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| Class E | 0.0 | -0.2 | -0.2 | 0.0 | 0.0 | 0.0 |
| All | 0.0 | -0.1 | -0.1 | 0.0 | 0.0 | 0.0 |
| Enc Time[%] | 100% |  |  | 101% |  |  |
| Dec Time[%] | 101% |  |  | 102% |  |  |
|  |  | Random access |  |  | Random access LoCo |  |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | 0.0 | 0.2 | -0.2 | 0.0 | 0.0 | 0.0 |
| Class B | 0.0 | 0.0 | -0.1 | 0.0 | 0.0 | 0.0 |
| Class C | 0.0 | -0.1 | -0.1 | 0.0 | 0.0 | 0.0 |
| Class D | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| Class E |  |  |  |  |  |  |
| All | 0.0 | 0.0 | -0.1 | 0.0 | 0.0 | 0.0 |
| Enc Time[%] | 100% |  |  | 98% |  |  |
| Dec Time[%] | 100% |  |  | 101% |  |  |
|  | Low delay |  |  | Low delay LoCo |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A |  |  |  |  |  |  |
| Class B | 0.0 | 0.0 | -0.1 | 0.0 | 0.0 | 0.1 |
| Class C | 0.0 | 0.2 | 0.2 | -0.1 | 0.0 | -0.2 |
| Class D | 0.0 | 0.0 | -0.2 | 0.0 | -0.1 | -0.1 |
| Class E | -0.1 | -0.2 | 0.4 | 0.0 | 0.2 | 0.2 |
| All | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 100% |  |  | 101% |  |  |

The impact on the running times and objective qualities are very marginal.

## Subjective compression performance

We observed visual improvements (less blocking artifacts) for Kimono (sky and face area) and Parkscene (trunk area), for all_intra, low_delay and random_access at low bit-rates (Qp37, and sometimes Qp32). Some examples taken from the required sequences included in the visual test of CE12 are shown below.

Kimono_1920x1080_24, All intra, QP = 37, frame #3, cropped. Left: anchor, right: proposed

Kimono_1920x1080_24, All intra, QP = 37, frame #16, cropped. Left: anchor, right: proposed

# Closing remarks

This document presents the proposed content-adaptive deblocking scheme in the current HEVC. Less visual artifacts are observed, while the complexity and the objective quality changes are margional.

# Annex

The proposed modifications are showed as following:

In 8.6.1

# The derivation process of the boundary filtering strength specified in subclause Error: Reference source not found is invoked with the luma location ( xC, yC ), horEdgeFlags and verEdgeFlags as inputs and an array of size (2)x(nS)x(nS), bS as output.

# The derivation process of the boundary filtering mode specidfied in subclause 8.6.1.4 is involded with the luma location (xC,yC), the coding unit size log2CUSize, horEdgeFlags, and verEdgeFlags as inputs and an array of size (2)x(nS)x(nS), FM(Filtering Mode) as output.

# The filtering process for coding unit specified in subclause 8.6.1.5 are invoked with the luma location ( xC, yC ) specifying the top-left luma sample of the current coding unit relative to the top left luma sample of the current picture, the coding unit size log2CUSize , the array bS and the array FM as inputs and the modified reconstructued picture as output.

In 8.6.1.4

8.6.1.4 Derivation process of filtering mode

Inputs of this process are:

–	a luma location ( xC, yC ) specifying the top-left luma sample of the current coding unit relative to the top-left luma sample of the current picture,

–	a variable log2CUSize specifying the size of the current coding unit,

–	a two-dimensional arrays of size (nS)x(nS), horEdgeFlags and verEdgeFlags.

Output of this process is an array of size (2)x(nS)x(nS), FM specifying the filtering mode.

Let ( xEk, yEj ) with k = 0..nE-1 and j = 0..nE-1 specify a set of edge sample locations where nE is set equal to ( ( 1 << log2CUSize ) >> 2 ), xE0 = 0, yE0 = 0, xEk+1 = xEk + 4 and yEj+1 = yEj + 4.

For ( xEk, yEj ) with k %4  is equal to  0 or j%4 is equal to 0, the following applies.

If horEdgeFlags[ xEk ][ yEj ] is equal to 1,

Set sample p0 = recPicture[ xC + xEk ][ yC + yEj – 1 ] and q0 = recPicture[ xC + xEk ][ yC + yEj ].

The variable filterDir is set equal to 1.

Otherwise, if verEdgeFlags[ xEk ][ yEj ] is equal to 1,

Set sample p0 = recPicture[ xC + xEk – 1 ][ yC + yEj ] and q0 = recPicture[ xC + xEk ][ yC + yEj ].

The variable filterDir is set equal to 0.

Depending on the value of filterDir, the variable FM[ filterDir ][ xEk ][ yEj ] is derived as follows.

If the following conditions are ture, the variable FM[ filterDir ][ xEk+i ][ yEj+h ], i,h = 0,..,3 is set equal to 1, specifying the filtering mode is ModeB.

The  transform unit TU0 which containts q0 and the trandform unit TU1 which contarints p0 are both in intra- code coding unit.

The smaller transform size between TU0 and TU1 is larger than 8.

The larger thransform size between TU0 and TU1 is larger than 16.

The variable boundaryMeanP and boundaryMeanQ are derived as follows.

If horEdgeFlags[ xEk ][ yEj ] is equal to 1,

Set sample pmn and qmn as follows,

pmn = recPicture[ xC + xEk + m ][ yC + yEj – 1 – n ]   m, n = 0,1,…,15.

qmn = recPicture[ xC + xEk  + m ][ yC + yEj + n ]  m, n = 0,1,…,15.

Otherwise, If verEdgeFlags[ xEk ][ yEj ] is equal to 1,

Set sample pmn and qmn as follows,

pmn = recPicture[ xC + xEk – 1 – m ][ yC + yEj + n ]   m, n = 0,1,…,15.

qmn = recPicture[ xC + xEk  + m ][ yC + yEj + n ]  m, n = 0,1,…,15.

boundaryMeanP = ( ∑pm0  + ∑ p0n ) >> 5 m, n = 0,1,…,15.                                                                    (8-1)

boundaryMeanQ = (∑qm0  + ∑q0n ) >>5 m, n = 0,1,…,15.                                                                      (8-2)

The variable boundaryVarP and boundaryVarQ are derived as follows.

If horEdgeFlags[ xEk ][ yEj ] is equal to 1,

Set sample pmn and qmn as follows,

pmn = recPicture[ xC + xEk + m ][ yC + yEj – 1 – n ]   m, n = 0,1,…,15.

qmn = recPicture[ xC + xEk  + m ][ yC + yEj + n ]  m, n = 0,1,…,15.

Otherwise, If verEdgeFlags[ xEk ][ yEj ] is equal to 1,

Set sample pmn and qmn as follows,

pmn = recPicture[ xC + xEk – 1 – m ][ yC + yEj + n ]   m, n = 0,1,…,15.

qmn = recPicture[ xC + xEk  + m ][ yC + yEj + n ]  m, n = 0,1,…,15.

boundaryVarP = ( ∑ abs ( pmn – boundaryMeanP )) m, n = 0,1,…,15.                                          (8-3)

boundaryVarQ = ( ∑ abs ( qmn – boundaryMeanQ )) m, n = 0,1,…,15.                                         (8-4)

The sum of boundaryVarP and boundaryVarQ is smaller than 512.

Otherwise, the variable FM[ filterDir ][ xEk+i ][ yEj+h ], i,h = 0,..,3 is set equal to 0, specifying the filtering mode is ModeA.

In 8.6.1.5

(1)

–	an array bS specifying the boundary filtering strength.

–	an array FM specifying the filtering mode.

Output of this process is:

(2)

    - Boundary filtering strength bSVer is derived as follows:

bSVer = Max( bS[ 0 ][ xDk ][ yDm + i ] ) for i = 0..7

    - Filtering process for luma block edge in subclause Error: Reference source not found is invoked with the luma location of the coding unit ( xC, yC ), the luma location of the block ( xDk, yDm ), a variable verticalEdgeFlag set equal to 1,  the boundary filtering strength bSVer and the filtering mode FMVer( equal to FM[0][ xDk ][ yDm ] ) as inputs and the modified luma picture buffer as output.

(3)

- For xDk set equal to xB+( k << 3 ), k=0..nD – 1, the following ordered steps apply:

    - Boundary filtering strength bSHor is derived as follows:

bSHor = Max( bS[ 1 ][ xDk + i][ yDm ] ) for i = 0..7

    - Filtering process for luma block edge in subclause Error: Reference source not found is invoked with the luma location of the coding unit ( xC, yC ), the luma location of the block ( xDk, yDm ), a variable verticalEdgeFlag set equal to 0,  the boundary filtering strength bSHor and the filtering mode FMHor( equal to FM[1][ xDk ][ yDm ] ) as inputs and the modified luma picture buffer as output.

(4)

- Boundary filtering strength bSVer is derived as follows:

bSVer = bS[ 0 ][ xDk*2 ][ yDm*2 ]

- Filtering process for chroma block edge in subclause Error: Reference source not found is invoked with the chroma location of the coding unit ( xC/2, yC/2 ), the chroma location of the block ( xDk, yDm ), a variable verticalEdgeFlag set equal to 1, a chroma component index cIdx set equal to 1, the boundary filtering strength bSVer and the filtering mode FMVer( equal to FM[ 0 ][ xDk*2 ][ yDm*2 ] ) as inputs and the modified chroma picture buffer as output.

- Filtering process for chroma block edge in subclause Error: Reference source not found is invoked with the chroma location of the coding unit ( xC/2, yC/2 ), the chroma location of the block ( xDk, yDm ), a variable verticalEdgeFlag set equal to 1, a chroma component index cIdx set equal to 2, the boundary filtering strength bSVer and the filgering mode FMVer( equal to FM[ 0 ][ xDk*2 ][ yDm*2 ] ) as inputs and the modified chroma picture buffer as output.

(5)

- For xDk set equal to ( xB / 2 )+( k << 2 ), k=0..nD*2 – 1, the following ordered steps apply:

- Boundary filtering strength bSHor is derived as follows:

bSHor = bS[ 1 ][ xDk*2 ][ yDm*2 ]

- Filtering process for chroma block edge in subclause Error: Reference source not found is invoked with the chroma location of the coding unit ( xC/2, yC/2 ), the chroma location of the block ( xDk, yDm ), a variable verticalEdgeFlag set equal to 0, a chroma component index cIdx set equal to 1, the boundary filtering strength bSHor and the filtring mode FMHor(equal to FM[ 1 ][ xDk*2 ][ yDm*2 ] ) as inputs and the modified chroma picture buffer as output.

- Filtering process for chroma block edge in subclause Error: Reference source not found is invoked with the chroma location of the coding unit ( xC/2, yC/2 ), the chroma location of the block ( xDk, yDm ), a variable verticalEdgeFlag set equal to 0, a chroma component index cIdx set equal to 2,  the boundary filtering strength bSHor and the filtering mode FMHor(equal to FM[ 1 ][ xDk*2 ][ yDm*2 ] ) as inputs and the modified chroma picture buffer as output.

In 8.6.1.5.1

(1)

–	a variable verticalEdgeFlag,

–	a variable bS specifying the boundary filtering strength,

–	an variable FM specifying the filtring mode,

Output of this process is:

(2)

Depending on verticalEdgeFlag and FM, the following applies:

–	If verticalEdgeFlag is equal to 1 and FM is equal to 0, the following ordered steps apply:

(3)

–	Otherwise (verticalEdgeFlag is equal to 0 and the FM is equal to 0), the following ordered steps apply:

- The sample values pi,k and qi,k with i = 0..3 and k = 0..7 are derived as follows:

qi,k = s’[ xC + xB +k, yC + yB + i ]

pi,k = s’[ xC + xB +k, yC + yB – i – 1 ]

(4)

–   Otherwise (verticalEdgeFlag is equal to 1 and the FM is equal to 1), the following ordered steps apply:

            - The sample values pi,k and qi,k with i = 0..N-1 and k = 0..7 are derived as follows:

qi,k = s’[ xC + xB +i, yC + yB + k ]

pi,k = s’[ xC + xB – i – 1, yC + yB + k ]

N is decided by qPL, when qPL < 30, N is equal to 8, otherwise N is equal to 16.

            - The Variable d is derived as follows:

d = | p2,2 – 2*p1,2 + p0,2 | + | q2,2 – 2*q1,2 + q0,2 | + | p2,5 – 2*p1,5 + p0,5 | + | q2,5 – 2*q1,5 + q0,5 |

3.    If d is less than ( β >> 2 ), | p3,k – p0,k | + | q0,k – q3,k | is less than ( β >> 3 ) and | p0,k – q0,k | is less than ( 5*tC + 1 ) >> 1, for all k = 0,…,7. The following ordered steps apply:

a. For each sample location ( xC + xB, yC + yB + k ), k = 0..7, the filtering prcess for a luma sample specified in subclause 8.6.1.5.4 is invoked with sample value pi,k, qi,k with i = 0..N-1, the boundary filtering strength bS as inputs, the output is the filtered sample values pi’ and qi’.

b.The filterd sample values pi’ and qi’ with i = 0..N/2 – 1, replace the corresponding samples inside the sample array s’ as follows:

s’[ xC + xB +i, yC + yB + k ] = qi’

s’[ xC + xB – i – 1, yC + yB + k ] = pi’

Otherwise, the following ordered steps apply:

a. the filtering process for a luma sample specified in subclause 8.6.1.5.3 is invoked with     sample values pi,k, qi,k with i = 0..3, the boundary filtering strength bS and the variables d, β and tC as inputs and the number of filtered samples nD and the filtered sample values pi’ and qi’ as outputs.

b. The filtered sample values pi’ and qi’ with i = 0..nD – 1 replace the corresponding samples inside the sample array s’ as follows:

s’[ xC + xB +i, yC + yB + k ] = qi’

s’[ xC + xB – i – 1, yC + yB + k ] = pi’

–	Otherwise (verticalEdgeFlag is equal to 0 and the FM is equal to 1), the following ordered steps apply:

- The sample values pi,k and qi,k with i = 0..N-1 and k = 0..7 are derived as follows:

qi,k = s’[ xC + xB +k, yC + yB + i ]

pi,k = s’[ xC + xB +k, yC + yB – i – 1 ]

N is decided by qPL, when qPL < 30, N is equal to 8, otherwise N is equal to 16.

2. The Variable d is derived as follows:

d = | p2,2 – 2*p1,2 + p0,2 | + | q2,2 – 2*q1,2 + q0,2 | + | p2,5 – 2*p1,5 + p0,5 | + | q2,5 – 2*q1,5 + q0,5 |

3. If d is less than ( β >> 2 ), | p3,k – p0,k | + | q0,k – q3,k | is less than ( β >> 3 ) and | p0,k – q0,k | is less than ( 5*tC + 1 ) >> 1, for all k = 0,…,7. The following ordered steps apply:

For each sample location ( xC + xB + k, yC + yB ), k = 0..7, the filtering prcess for a luma sample specified in subclause 8.6.1.5.4 is invoked with sample value pi,k, qi,k with i = 0..N-1, the boundary filtering strength bS as inputs, the output is the filtered sample values pi’ and qi’. The filterd sample values pi’ and qi’ with i = 0..N/2 – 1, replace the corresponding samples inside the sample array s’ as follows:

s’[ xC + xB +k, yC + yB + i ] = qi’

s’[ xC + xB +k, yC + yB – i – 1 ] = pi’

Otherwise, the following ordered steps apply:

a. the filtering process for a luma sample specified in subclause 8.6.1.5.3 is invoked with     sample values pi,k, qi,k with i = 0..3, the boundary filtering strength bS and the variables d, β and tC as inputs and the number of filtered samples nD and the filtered sample values pi’ and qi’ as outputs.

b. The filtered sample values pi’ and qi’ with i = 0..nD – 1 replace the corresponding samples inside the sample array s’ as follows:

s’[ xC + xB +k, yC + yB + i ] = qi’

s’[ xC + xB +k, yC + yB – i – 1 ] = pi’

In 8.6.1.5.2

(1)

–	a variable cIdx specifying the chroma component index.

–	a variable FM specifying the filtering mode.

Output of this process is:

(2)

Depending on verticalEdgeFlag, the following applies:

–	If verticalEdgeFlag is equal to 1 and the FM is equal to 0, for each sample location ( xC + xB, yC + yB + k ), k = 0..3, the following ordered steps apply:

- The sample values pi and qi with i = 0..1 are derived as follows:

(3)

Otherwise (verticalEdgeFlag is equal to 0 and the FM is equal to 0), for each sample location ( xC + xB +k, yC + yB ), k = 0..3, the following ordered steps apply:

- The sample values pi and qi with i = 0..1 are derived as follows:

(4)

    - The filtered sample values p0’ and q0’ replace the corresponding samples inside the sample array   s’ as follows:

s’[ xC + xB +k, yC + yB ] = q0’

s’[ xC + xB +k, yC + yB – 1 ] = p0’

–	Otherwise (verticalEdgeFlag is equal to 1 and the FM is equal to 1), for each sample location ( xC + xB , yC + yB +k ), k = 0..3, the following ordered steps apply:

- The sample values pi and qi with i = 0..7 are derived as follows:

qi = s’[ xC + xB +i, yC + yB + k ]

pi = s’[ xC + xB – i – 1, yC + yB + k ]

- The filtering process for a sample specified in subclause 8.6.1.5.6 is invoked whith sample values pi, qi, with i= 0,..7, the boundary filtering strength bS as inputs, the output is the filtered sample values pi’ and qi’, with i= 0,..3. The filterd sample values pi’ and qi’, with i= 0,..3 replace the correspoinding sample inside the sample array s’ as follows:

s’[ xC + xB + i, yC + yB + k ] = qi’

s’[ xC + xB – i – 1, yC + yB + k ] = pi’

–	Otherwise (verticalEdgeFlag is equal to 0 and the FM is equal to 1), for each sample location ( xC + xB + k, yC + yB ), k = 0..3, the following ordered steps apply:

- The sample values pi and qi with i = 0..7 are derived as follows:

qi = s’[ xC + xB +k, yC + yB + i ]

pi = s’[ xC + xB +k, yC + yB – i – 1 ]

- The filtering process for a sample specified in subclause 8.6.1.5.6 is invoked whith sample values pi, qi, with i= 0,..7, the boundary filtering strength bS as inputs, the output is the filtered sample values pi’ and qi’, with i= 0,..3. The filterd sample values pi’ and qi’, with i= 0,..3 replace the correspoinding sample inside the sample array s’ as follows:

s’[ xC + xB +k, yC + yB + i ] = qi’

s’[ xC + xB +k, yC + yB – i  – 1 ] = pi’

In 8.6.1.5.3

8.6.1.5.3 Filtering process for a luma sample in ModeA

In 8.6.1.5.4

8.6.5.4  Filtering process for a luma sample in ModeB

Inputs of this process are:

–	sample values, pi and qi with i = 0.. N/2 – 1,

–	a variable bS specifying the boundary filtering strength,

Output of this process is:

–	filtered sample values, pi’ and qi’ with i = 0.. N/2 – 1,

When the variable bS is not equal to 0, the following applies:

pi’ = ( ∑ pn + ∑ qm ) / N,  n = 0,..,i+N/2, m = 0,.., N/2 – i – 2                                                                                       (8-5)

qi’ = ( ∑ pn + ∑ qm ) / N,  n = 0,.., N/2 – i – 1, m = 0,..,i+N/2 – 1                                                                                (8-6)

In 8.6.1.5.5

8.6.5.5 Filtering process for a chroma sample in ModeA

In 8.6.1.5.6

8.6.1.5.6 Filtering process for a chroma sample in ModeB

[Ed: (WJ) no filtering when bS is equal or less than 2]

Inputs of this process are:

–	sample values, pi and qi with i = 0..7,

–	a variable bS specifying the boundary filtering strength.

Output of this process is:

–	The filtered sample values, pi’ and qi’ with i = 0..3.

When the variable bS is greater than 2, the filtered sample values p0’ and q0’ are derived by

pi’ = ( ∑ pn + ∑ qm ) / N,  n = 0,..,i+N/2, m = 0,.., N/2 – i – 2                                                                                (8-7)

qi’ = ( ∑ pn + ∑ qm ) / N,  n = 0,.., N/2 – i – 1, m = 0,..,i+N/2 – 1                                                                                (8-8)

N is equal to 8.

# References

[1] Z. Xiong, X. Sun, J. Xu, “TE10, Subset 1: Report of Content-Adaptive Deblocking,” JCTVC-C273, Oct. 2010, Guangzhou, China.

[2] Z. Shi, Z. Xiong, X. Sun, J. Xu, “CE8, Subset 1: Report of Content-Adaptive Deblocking,”JCTVC-D137, Jan.2011, Daegu, KR.

# Patent rights declaration(s)

University of Science and Technology of China, and Microsoft Corporation may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).