| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 5th Meeting: Geneva, CH, 16-23 March, 2011 | Document: JCTVC-E146 |
| --- | --- |

| Title: | On merge candidate construction |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Bin Li Jizheng Xu Feng Wu Houqiang Li | Email: | jzxu@microsoft.com |
| Source: | University of Science and Technology of China Microsoft |  |  |

_____________________________

# Abstract

In the current design of HEVC, HM-2.0, merge mode is widely used. From the 4th JCT-VC meeting, the concept of partial merge was introduced to HEVC, which means when one CU is larger than the smallest CU, and it has two partitions, the first part will be inferred as merge mode. But what will happen if the first part does not have any merge candidate is unclear. This contribution presents a method to avoid one PU is inferred as merge mode but without any available merge candidate. The proposed method does not have impact on complexity or RD performance.

- Introduction

In the current design of HEVC, HM-2.0, merge mode is widely used. From the 4th JCT-VC meeting, the concept of partial merge was introduced to HEVC, which means when one CU is larger than the smallest CU, and it has two partitions, the first part will be inferred as merge mode. But what will happen if the first part does not have any merge candidate is unclear. This contribution presents a method to avoid one PU is inferred as merge mode but without any available merge candidate. The proposed method does not have impact on complexity or RD performance.

- Solution

The current design of HEVC adopts up to 5 merge candidates, left, up, co-located, and two from the corner. When the block (or called PU) is inter (including skip) mode, it will be used as a merge candidate. So the number of merge candidate in the current design can be from 0 to 5.

In the proposed method, no syntax is changed. Only the semantics of getting merge candidates is slightly changed. After the existing process of getting merge candidate, if the set of merge candidate is empty, one merge candidate, with list 0 prediction, reference frame index 0, mv_x and mv_y are both 0, is added to the merge candidate. So the number of merge candidate in the proposed method can be from 1 to 5. So in the proposed method, the set of merge candidates will never be empty and the first part of the CU, which is larger than the smallest CU and has two partitions, can always be inferred as merge mode.

Also if we changed the context model of merge flag slightly, the decoding complexity can be also reduced. Two syntax elements are related to the merge mode, one is the merge flag, to indicate whether the current PU is merge mode or not, and the other is the merge index, to indicate which candidate the current PU merges to. Decoding of these two syntax elements depends on the number of merge candidate. When there is no merge candidate, the merge flag does not need to be decoded, and when there is only one merge candidate, the merge index does not need to be decoded.

As the number of merge candidate is always greater than 0, the merge flag will always be decoded. And the process of reconstructing the merge candidate can be done after decoding the merge flag when it is merge mode. The context model of merge flag will be slightly changed, to make it independent with the number of merge candidate. In this proposal, the context model of merge flag is changed to very similar with the context model of skip flag, using the left and top block as context.

The decoding process related to merge mode in HM-2.0 and the proposed algorithm can be seen in figure 1 and figure 2.

Figure 1 decoding process of HM-2.0

Figure 2 decoding process of the proposed algorithm

It can be seen that in the proposed algorithm, when it is not merge mode, the reconstruction of merge candidates is not necessary, which can decrease the decoding complexity.

- Experimental results

The proposed algorithm was implemented on HM-2.0. Four inter tests are tested according to the common test condition in [1]. The anchor is the HM-2.0 default anchor. The summary result is shown in Table 1 and 2. The low complexity result in table 2 is identical to that in table 1, as LCEC is used in low complexity configuration. The detailed result can be found in JVCVC-E146_1(2).xls.

Table 1 Summary result of adding zero merge candidate (Y BD-rate)

|  | RA_HE | RA_LC | LD_HE | LD_LC |
| --- | --- | --- | --- | --- |
| Class A | 0.0 | -0.1 |  |  |
| Class B | 0.0 | 0.0 | 0.0 | 0.0 |
| Class C | -0.1 | -0.1 | 0.0 | 0.0 |
| Class D | 0.0 | 0.0 | -0.1 | 0.0 |
| Class E |  |  | -0.2 | 0.0 |
| All | 0.0 | 0.0 | -0.1 | 0.0 |
| Enc Time | 102% | 99% | 101% | 100% |
| Dec Time | 100% | 100% | 100% | 100% |

Table 2 Summary result of adding zero merge candidate and context changing (Y BD-rate)

|  | RA_HE | RA_LC | LD_HE | LD_LC |
| --- | --- | --- | --- | --- |
| Class A | 0.0 | -0.1 |  |  |
| Class B | 0.0 | 0.0 | 0.0 | 0.0 |
| Class C | 0.0 | -0.1 | 0.0 | 0.0 |
| Class D | 0.0 | 0.0 | -0.1 | 0.0 |
| Class E |  |  | 0.1 | 0.0 |
| All | 0.0 | 0.0 | 0.0 | 0.0 |
| Enc Time | 101% | 99% | 102% | 100% |
| Dec Time | 100% | 100% | 100% | 100% |

When it is inter B slice, bi-prediction can be used instead of list 0 prediction. The summary result is shown in table 3. And the detailed result can be found in JVCVC-E146_3.xls. The same algorithm was also proposed in [2], and the same results are obtained.

Table 3 Summary result of adding zero merge candidate (bi) and context changing (Y BD-rate)

|  | RA_HE | RA_LC | LD_HE | LD_LC |
| --- | --- | --- | --- | --- |
| Class A | 0.1 | 0.0 |  |  |
| Class B | 0.0 | 0.0 | -0.1 | 0.0 |
| Class C | 0.0 | -0.1 | 0.0 | -0.1 |
| Class D | 0.0 | 0.0 | -0.1 | 0.0 |
| Class E |  |  | -0.1 | 0.0 |
| All | 0.0 | 0.0 | -0.1 | 0.0 |
| Enc Time |  |  |  |  |
| Dec Time |  |  |  |  |

It can be seen that add one zero merge candidate when the set of merge candidates is empty do not have much impact the RD performance and can decrease the decoding complexity.

Another test is that disabling partial merge. It is reported in Error: Reference source not found, the same results are obtained. The summary result can be found in table 4 and the detailed result can be found in JVCVC-E146_4.xls. It can be seen that disabling partial merge can lead to performance gain.

Table 4 Summary result of disabling partial merge (Y BD-rate)

|  | RA_HE | RA_LC | LD_HE | LD_LC |
| --- | --- | --- | --- | --- |
| Class A | -0.4 | -0.7 |  |  |
| Class B | -0.4 | -0.6 | -0.1 | -0.4 |
| Class C | -0.6 | -0.8 | -0.3 | -0.6 |
| Class D | -0.5 | -0.7 | -0.4 | -0.4 |
| Class E |  |  | 0.0 | -0.2 |
| All | -0.5 | -0.7 | -0.2 | -0.4 |
| Enc Time | 113% | 116% | 114% | 119% |
| Dec Time | 101% | 100% | 100% | 100% |

- Text Modification

- Add all zero merge candidate

For adding the all 0 merge candidate, no syntax changes are needed.

In 8.4.2.1.3,

3.	The merging candidate list, mergeCandList, is constructed of which elements are given as specified order:

1.	A, if availableFlagA is equal to 1

2.	B, if availableFlagB is equal to 1

3.	Col, if availableFlagCol is equal to 1

4.	C, if availableFlagC is equal to 1

5.	D, if availableFlagD is equal to 1

6. 	If A, B, Col, C, and D are all unavailable, one all_zero_merge_candidate is added to the set of merge candidate. The all_zero_merge_candidate is

When the current slice has two reference picture list (such as B slice), all_zero_merge_candidate is predFlagL0 = 1, predFlagL1 = 1, refIdxL0 = 0, refIdxL1 = 0, mvL0[0] = 0, mvL0[1] = 0, mvL1[0] = 0, mvL1[1] = 0.

When the current slice does not have two reference picture lists (such as P slice), all_zero_merge_candidate is predFlagL0 = 1, predFlagL1 = 0, refIdxL0 = 0, refIdxL1 = 0, mvL0[0] = 0, mvL0[1] = 0, mvL1[0] = 0, mvL1[1] = 0.

- Proposed merge candidates independent decoding of merge flag

As specified in 8.4.2.1.3, “This process is only invoked when PredMode is equal to MODE_ INTER and merge_flag [ xP ][ yP ] is equal to 1,”

In 7.3.7

| if( !InferredMergeFlag && NumMergeCand > 0 ) |  |
| --- | --- |
| merge_flag[ x0 ][ y0 ] | u(1) ¦ ae(v) |

The decoding of merge flag depends on the NumMergeCand.

So 8.4.2.1.3 conflicts with 7.3.7.

The text of the proposed method of decoding merge flag does not depends on the merge candidates list.

For the proposed decoding of merge flag and merge index, the text changes include:

| if( !InferredMergeFlag && NumMergeCand > 0 ) |  |
| --- | --- |
| merge_flag[ x0 ][ y0 ] | u(1) ¦ ae(v) |
| if( merge_flag[ x0 ][ y0 ] && NumMergeCand > 1 ) { |  |
| merge_idx[ x0 ][ y0 ] | ue(v) ¦ ae(v) |

For CABAC, the context model of merge flag is slightly changed to independent with NumMergeCand.

The context model of merge flag is similar to the context model with skip flag. The merge flags of the top block and the left block are used as proposed context model, as shown in the following table.

| Merge_flag of the upper block | Merge_flag of the left block | Ctx offset |
| --- | --- | --- |
| 0 | 0 | 0 |
| 0 | 1 | 1 |
| 1 | 0 | 1 |
| 1 | 1 | 2 |

For LCEC, no changes on the coding method of merge flag.

- References

- JCT-VC, “Common test conditions and software reference configurations”, JCTVC-D600, JCT-VC Meeting, Daegu, KR, Jan. 2011.

- JCT-VC, “A study on HM2.0 bitstream parsing and error resiliency issue”, JCTVC-E118, JCT-VC Meeting, Geneva, CH, March. 2011

# Patent rights declaration(s)

University of Science and Technology of China, and Microsoft Corp. may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).