| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 5th Meeting: Geneva, CH, 16-23 March, 2011 | Document: JCTVC-E147 |
| --- | --- |

| Title: | On motion information compression |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Bin Li Jizheng Xu Feng Wu Houqiang Li | Email: | jzxu@microsoft.com |
| Source: | University of Science and Technology of China Microsoft |  |  |

_____________________________

# Abstract

In the current design of HEVC, MV compress is used to save the memory required by the MV information in the reference frame. This contribution proposes to further reduce the memory requirement by compressing reference frame indexes and modes, while slight coding gain can be achieved. This document also presents a simple modification to improve the coding efficiency without complexity increasing. In the proposed method, on average 0.5%, 0.6%, 0.7% and 0.7% bit-savings can be achieved for RA_HE, RA_LC, LD_HE and LD_LC settings.

- Introduction

In the current design of HEVC, HM-2.0, MV compress is used to save the memory required by the MV information in the reference frame. One 16x16 block saves only one set of motion information, instead of saving 16 different sets of motion information. In the current design, the 16x16 block uses the motion vectors of the first block. Actually reference frame indexes and modes of the reference frame are also used in temporal motion vector prediction. Reference frame indexes are used to predict the current block’s reference frame index and influence the scaling factors for motion vector prediction; modes are used to determine if temporal motion vector predictors are available or not. While motion vectors have been compressed, reference frame indexes and modes are not. Instead, reference frame indexes and modes are still stored for each 4x4 block, which requires lots of memory for both the encoder and the decoder.

In this document, we propose to further compress reference frame indexes and modes in a similar way as motion vector compression. The memory requirement for reference frame indexes and modes can be reduced to 1/16 of the current one. Meanwhile, a slight coding gain is achieved.

We also present a simple modification to the current motion vector compression scheme to improve the coding performance. Instead of using the first 4x4 block as a representative within a 16x16 block, we use the last 4x4 block. By doing so, more than 0.5% bits can be saved for all the inter test conditions without increasing any complexity.

- Solution

As the MV is compressed, the information of mode and reference frame index should also be compressed. Because the following will use the mode information to decide using temporal MVP or not, and use the reference frame index to do the scaling process. If only MV is compressed, the MV and the mode, reference frame index will mismatch, which may lead to performance loss.

In the current design, the MV compression only uses the motion information of the first block. Using the motion information of the last block will provide extra performance improvement. Because when the PU size is greater than 16x16, the motion information of the last block is much nearer the center than the first block. For example in figure 1, a 32x32 block will use the motion information at position A. If we compress the MV using the first block, it is position B. But if we compress MV using the last block, it is just position A.

Figure 1 an example of 32x32 block

In the above step, only the mv in block 15 is used for compression. If it is not available (maybe it is intra mode or out of slice boundary), the mv of the whole region will be unavailable. We can use the scanning method to find the first available mv in the region. As the whole 8x8 blocks should have the same prediction mode, only block 15, 11, 7, and 3 needs to be checked to find the first available mv.

Figure 2 4x4 blocks in one 16x16 region

- Experimental results

The proposed algorithm was implemented on HM-2.0. Four inter tests are tested according to the common test condition in [1]. The anchor is the HM-2.0 default anchor. The summary result is shown in Table 1, 2, and 3. The detailed result can be found in JCTVC-E147_1(2, 3).xls.

Table 1 Summary result of compressing mode and reference frame index (Y BD-rate)

|  | RA_HE | RA_LC | LD_HE | LD_LC |
| --- | --- | --- | --- | --- |
| Class A | 0.1 | 0.1 |  |  |
| Class B | 0.0 | 0.0 | 0.0 | -0.1 |
| Class C | 0.0 | 0.0 | -0.1 | -0.1 |
| Class D | -0.1 | -0.1 | -0.2 | -0.1 |
| Class E |  |  | -0.1 | -0.1 |
| All | 0.0 | 0.0 | -0.1 | -0.1 |
| Enc Time | 100% | 98% | 100% | 100% |
| Dec Time | 101% | 101% | 101% | 101% |

Table 2 Summary result of compressing mode and reference frame index, and finding last (Y BD-rate)

|  | RA_HE | RA_LC | LD_HE | LD_LC |
| --- | --- | --- | --- | --- |
| Class A | -0.5 | -0.5 |  |  |
| Class B | -0.3 | -0.5 | -0.4 | -0.7 |
| Class C | -0.5 | -0.6 | -0.6 | -0.8 |
| Class D | -0.9 | -0.9 | -0.8 | -0.8 |
| Class E |  |  | -0.8 | -0.5 |
| All | -0.5 | -0.6 | -0.7 | -0.7 |
| Enc Time | 100% | 98% | 101% | 100% |
| Dec Time | 101% | 101% | 101% | 101% |

Table 3 Summary result of compressing mode and reference frame index, and scanning first (Y BD-rate)

|  | RA_HE | RA_LC | LD_HE | LD_LC |
| --- | --- | --- | --- | --- |
| Class A | -0.5 | -0.5 |  |  |
| Class B | -0.4 | -0.3 | -0.4 | -0.5 |
| Class C | -0.6 | -0.6 | -0.7 | -0.8 |
| Class D | -0.9 | -0.9 | -0.8 | -0.8 |
| Class E |  |  | -0.8 | -0.6 |
| All | -0.6 | -0.6 | -0.7 | -0.7 |
| Enc Time | 100% | 99% | 100% | 99% |
| Dec Time | 100% | 101% | 101% | 101% |

It can be seen that the RD performance can be improved with these simple changes and without complexity increasing. And the result in table 3 is only slightly better than the result in table 2. But the method used in table 3 needs to scan for the first available block to get the motion information. So we think, only using the mv of the last block for mv compression is good enough.

It should be noticed that disabling TMVP of merge mode will brings a little performance improvement; the summary result will be shown in table 4. And the proposed MV compress method does not conflict with disabling TMVP of merge mode. The summary result of the proposed method based on disabling TMVP of merge mode will be shown in table 5. The results in table 5 roughly equals to the results in table 2 adding the results in table 4.The detailed result can be found in JCTVC-E147_4(5).xls.

Table 4 Summary result of disabling TMVP of merge mode (Y BD-rate)

|  | RA_HE | RA_LC | LD_HE | LD_LC |
| --- | --- | --- | --- | --- |
| Class A | 0.0 | -0.1 |  |  |
| Class B | 0.0 | -0.2 | 0.0 | -0.5 |
| Class C | -0.1 | -0.2 | 0.0 | -0.3 |
| Class D | 0.0 | -0.2 | -0.3 | -0.4 |
| Class E |  |  | -0.3 | -0.4 |
| All | 0.0 | -0.2 | -0.1 | -0.4 |
| Enc Time | 96% | 97% | 96% | 100% |
| Dec Time | 101% | 100% | 99% | 100% |

Table 5 Summary result of proposed method based on disabling TMVP of merge mode (Y BD-rate)

|  | RA_HE | RA_LC | LD_HE | LD_LC |
| --- | --- | --- | --- | --- |
| Class A | -0.4 | -0.6 |  |  |
| Class B | -0.3 | -0.7 | -0.4 | -1.1 |
| Class C | -0.6 | -0.7 | -0.7 | -1.1 |
| Class D | -0.9 | -1.0 | -1.0 | -1.2 |
| Class E |  |  | -0.9 | -0.9 |
| All | -0.5 | -0.7 | -0.7 | -1.1 |
| Enc Time | 95% | 100% | 95% | 97% |
| Dec Time | 100% | 100% | 99% | 100% |

It also should be noticed that, if the center position of the temporal MVP in the reference frame is slightly changed, for position A to position B in figure 2, extra gain can also be obtained.

Figure 2 Proposed center position of temporal MVP

The summary result of reference frame index fix + mode fix + get the mv of last position + change the center position of temporal MVP is shown in table 6. The detailed result can be found in JCTVC-E147_6.xls.

Table 6 Summary result of all proposed method (Y BD-rate)

|  | RA_HE | RA_LC | LD_HE | LD_LC |
| --- | --- | --- | --- | --- |
| Class A | -0.6 | -0.6 |  |  |
| Class B | -0.5 | -0.7 | -0.5 | -0.8 |
| Class C | -0.6 | -0.7 | -0.7 | -0.8 |
| Class D | -1.0 | -1.0 | -0.9 | -0.8 |
| Class E |  |  | -0.6 | -0.8 |
| All | -0.7 | -0.7 | -0.7 | -0.8 |
| Enc Time | 99% | 98% | 98% | 99% |
| Dec Time |  |  |  |  |

- Text Modification

- Reference frame index compression

This modification seems like a bug fix. No modification in the current WD is needed.

The current WD in 8.4.2.1.7, deals with the reference frame index compression well.

- Mode compression

This modification seems like a bug fix. No modification in the current WD is needed.

The current WD in 8.4.2.1.7, deals with the mode compression well.

- Using the motion of the last block

In 8.4.2.1.7,

The variable colPu is set as the prediction unit covering the modified position given by ( ( xPCtr >> 4 ) << 4, ( yPCtr >> 4 ) << 4 ) inside the colPic. Suggest to be changed to

The variable colPu is set as the prediction unit covering the modified position given by ( ( ( xPCtr >> 4 ) << 4 ) + 12, ( ( yPCtr >> 4 ) << 4 ) + 12 ) inside the colPic.

And as the new position is the last block of the 16x16 region, it may out of the picture boundary. So, If colPu is coded in an intra prediction mode, both components of mvLXCol are set equal to 0 and availableFlagLXCol is set equal to 0. Suggest to be changed to

If colPu is coded in an intra prediction mode or is out of picture boundary, both components of mvLXCol are set equal to 0 and availableFlagLXCol is set equal to 0.

- Scan for the first available motion from the last block

The motion information compression based on the scanning for the first available motion from the last block should be done before the picture is stored in the DPB.

The block is scanned according to the order of 15, 11, 7, 3; the motion information of the first available block will be stored.

- Disabling TMVP in merge mode

No syntax changes.

In 8.4.2.1.3

The motion vectors mvL0 and mvL1, the reference indices refIdxL0 and refIdxL1, and the prediction utilization flags predFlagL0 and predFlagL1 are derived as specified by the following ordered steps:

1.	The derivation process for merging candidates from neighboring prediction unit partitions in subclause 8.4.2.1.4 is invoked with luma location ( xP, yP ), the width and the height of the prediction unit nPSW and nPSH and the partition index PartIdx as inputs and the output is assigned to the availability flags availableFlagN, the motion vectors mvL0N and mvL1N, the reference indices refIdxL0N and refIdxL1N and the prediction list utilization flags predFlagL0N and predFlagL1N with N being replaced by A, B, C or D.

2.	The derivation process for temporal luma motion vector prediction in subclause 8.4.2.1.7 is invoked with luma location ( xP, yP ), refIdxLXCol (with X being 0 or 1, respectively) equal to 0 as the inputs and with the output being the availability flag availableFlagLXCol and the temporal motion vector mvLXCol. The variables availableFlagCol and predFlagLXCol (with X being 0 or 1, respectively) are derived as specified below.

- Modification of center position in temporal MVP

In 8.4.2.1.7

1.	Central luma position of the current prediction unit is defined by

xPCtr = ( xP + ( nPSW >> 1 ) – 1

yPCtr = ( yP + ( nPSH >> 1 ) – 1

- References

- JCT-VC, “Common test conditions and software reference configurations”, JCTVC-D600, JCT-VC Meeting, Daegu, KR, Jan. 2011.

# Patent rights declaration(s)

University of Science and Technology of China, and Microsoft Corp. may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).