| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 5th Meeting: Geneva, CH, 16-23 March, 2011 | Document: JCTVC-E150 |
| --- | --- |

| Title: | CE11.B: Cross-check report of adaptive coefficients scanning for inter-frame coding (JCTVC-E296) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Bin Li Jizheng Xu | Email: | jzxu@microsoft.com |
| Source: | University of Science and Technology of China Microsoft |  |  |

_____________________________

# Abstract

This document reports cross-check results for Huawei’s proposal JCTVC-E296, which is designed to improve the inter frame coding efficiency by adaptive coefficient scanning.

# Introduction

We got the source code of JCTVC-E296 from Huawei and independently compiled and ran the code with the following running environment.

| Platform | CPU | Memory | Compiler |
| --- | --- | --- | --- |
| Windows Server 2008 R2 64 bits | 2xIntel Xeon L5420@2.5Ghz | 16GB | VS2010 |

For the testes we have performed, the results exactly match what are reported in JCTVC-E286 and show similar encoding/decoding characteristics.

# Results

Table 1 shows the summary. Details results are at JCTVC-E150_1.xls.

Table 1

|  |  | Random access |  |
| --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate |
| Class A | -0.1 | -0.1 | -0.2 |
| Class B | -0.3 | -0.1 | -0.1 |
| Class C | -0.4 | -0.1 | -0.2 |
| Class D | -0.4 | -0.1 | -0.2 |
| Class E |  |  |  |
| All | -0.3 | -0.1 | -0.1 |
| Enc Time[%] | 106% |  |  |
| Dec Time[%] | 102% |  |  |
|  | Low delay |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate |
| Class A |  |  |  |
| Class B | -0.7 | -0.1 | 0.0 |
| Class C | -0.7 | 0.2 | -0.1 |
| Class D | -0.9 | -0.2 | -0.8 |
| Class E | -0.8 | -0.1 | 0.7 |
| All | -0.8 | -0.1 | -0.1 |
| Enc Time[%] | 106% |  |  |
| Dec Time[%] | 102% |  |  |

Later we got an updated version of the code, which further applies adaptive coefficient scanning to Chroma signals. Table 2 shows the summary. Details results are at JCTVC-E150_2.xls.

Table 2

|  |  | Random access |  |
| --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate |
| Class A | -0.1 | -0.1 | -0.2 |
| Class B | -0.3 | -0.4 | -0.4 |
| Class C | -0.4 | -0.5 | -0.4 |
| Class D | -0.4 | -0.5 | -0.4 |
| Class E |  |  |  |
| All | -0.3 | -0.4 | -0.4 |
| Enc Time[%] | 106% |  |  |
| Dec Time[%] | 102% |  |  |
|  | Low delay |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate |
| Class A |  |  |  |
| Class B | -0.7 | -1.1 | -1.2 |
| Class C | -0.7 | -0.8 | -0.8 |
| Class D | -0.9 | -1.0 | -1.2 |
| Class E | -0.8 | -0.2 | -0.2 |
| All | -0.8 | -0.8 | -0.9 |
| Enc Time[%] | 105% |  |  |
| Dec Time[%] | 102% |  |  |