| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 5th Meeting: Geneva, 16-23 March, 2011 | Document: JCTVC-E154_r1 |
| --- | --- |

| Title: | CE1: Report of DMVD-based Bi-prediction |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Report/Proposal |  |  |
| Author(s) or Contact(s): | Chun-Chi Chen, Chung-Lin Lee, Wen-Hsiao Peng, Hsueh-Ming Hang 1001 Ta-Hsueh Rd., Hsinchu 30010, Taiwan | Tel: Email: | +886-955-028-902 pawn@mail.si2lab.org |
| Source: | National Chiao Tung University (NCTU)/ITRI International |  |  |

_____________________________

# Abstract

This contribution reports the experimental results on the DMVD-based Bi-prediction technique described in CE1. The technique forms a bi-prediction for a 2Nx2N PU based on two sets of motion parameters, one is specified in uni-prediction syntax and the other is signaled by the motion merging mechanism. Two predictors are created and combined linearly using OBMC. Depending on how the merged motion parameters are derived, a separate OBMC window function is applied. The compression performance and the complexity impacts of this technique are evaluated in two tests. Test #1 allows each 2Nx2N PU to choose from a set of six different window functions. Test #2 provides one additional set of window functions at the cost of extra signaling overhead. When compared with the HM-2.0 anchor in common test conditions, the technique is observed in Test #1 to have 0.4-3.2% BD-rate reductions, with an average of 1.6%. The encoding time is increased by 53% while the decoding time is increased by 4%. In Test #2, it achieves 0.6-3.7% BD-rate reductions, showing an average of 1.9%. The encoding time is doubled, whereas the decoding time is increased by 6%.

# Introduction

JCTVC-D175 [1] introduces a bi-prediction technique which requires a motion overhead as for uni-directional prediction. Figure 1 depicts its concept of operation. Like the conventional bi-prediction, it predicts a target PU based on two predictors. However, these predictors are weighted in a pixel-adaptive manner using OBMC. In particular, one of them is derived from the motion parameters specified in uni-prediction syntax while the other is obtained from the merged motion parameters. Depending on how the merged motion parameters are derived, a separate OBMC window function is applied. In addition, a modified block matching criterion is utilized to optimize the motion parameters to be signaled explicitly in syntax.

This contribution reports the compression performance and the complexity impacts of this technique based on the HM-2.0 software [2] and the test conditions specified in CE1 [3]. This technique is currently applied to 2Nx2N PUs only. A flag is additionally sent for each 2Nx2N PU to allow an adaptive switching between this mode and the usual inter-prediction mode. Two tests are conducted to evaluate this technique with different settings.

- Concept of the DMVD-based Bi-prediction.

# DMVD-based Bi-prediction with OBMC

## Selection of Window Functions

Test #1:

In Test #1, each 2Nx2N PU, when coded in the proposed mode, has six choices of window functions, denoted by wm,1 with m=1, …, 6. Their waveforms are illustrated in Figure 2, assuming that the 2Nx2N PU is of size 16x16. The selection of window functions is adaptive and depends on that of the merged motion parameters. The criterion is detailed as follows:

- If the motion parameters of the Left and Above candidates are identical, w6,1 is applied;

- Else

  - If the merged motion parameters are derived from the Left candidate, w1,1 is applied;

  - If the merged motion parameters are derived from the Above candidate, w2,1 is applied;

  - If the merged motion parameters are derived from the Co-located candidate, w3,1 is applied;

  - If the merged motion parameters are derived from the Above-Right candidate, w4,1 is applied;

  - If the merged motion parameters are derived from the Bottom-Left candidate, w5,1 is applied;

Test #2:

In Test #2, each 2Nx2N PU, when coded in the proposed mode, can select between two sets of window functions, denoted by wm,n with m=1, …, 6 and n=1,2. The window functions in the set wm,1, m=1, …, 6 are exactly the same as those used in Test #1, whereas those in the set wm,2, m=1, …,6 are defined in Figure 3 (assuming that the PU size is of 16x16). To indicate which set is in use, i.e., the value of n, one additional flag is sent for each 2Nx2N PU. For a chosen set of window functions, the parameter m is determined similarly as in Test #1.

| w1,1 | w2,1 | w3,1 |
| --- | --- | --- |
| w4,1 | w5,1 | w6,1 |

- The proposed window functions, wm,1, for 16x16 PUs.

| w1,2 | w2,2 | w3,2 |
| --- | --- | --- |
| w4,2 | w5,2 | w6,2 |

- The proposed window functions, wm,2, for 16x16 PUs.

- Candidates for motion merging.

## Derivation of Prediction Samples

With the proposed technique, each pixel in a 2Nx2N PU is a weighted sum of two prediction values q(i, j) and r(i, j), which are derived, respectively, from (1) the uni-directional motion vector of the target PU,, and (2) the motion vectors belonging to one of its five merge candidates, and, in the case of bi-prediction, .

Specifically, q(i, j) is computed as follows:

,

where

- (x0, y0) is the position (in a picture) of the top-left pixel of the target PU.

- (i, j), with i, j=0, …, 2N-1, is an integer position of a pixel in the target PU.

- p1 is a previously coded picture.

Using the same notation, r(i, j) is computed similarly:

- If r(i, j) is a uni-predicted sample,

;

- Otherwise (r(i, j) is a bi-predicted sample),

,

where p2 and p3 are previously coded pictures.

Given the q(i, j) and r(i, j) above, the creation of each pixel, ppred(x0+i, y0+i), in the target 2Nx2N PU is governed by the following equation:

.

The computation of ppred(x0+i, y0+i) can be done in fixed-point arithmetic (involving only binary shifts and integer additions), because the values of the window functions are currently restricted to be 0.875, 0.75, 0.5, and 0.125. For example,

- If wm,n(i, j)=0.875,

;

- If wm,n(i, j)=0.75,

;

- If wm,n(i, j)=0.5,

;

- Otherwise (wm,n(i, j)=0.125),

.

## Resizing Window Functions

In the proposed technique, the window functions wm,n need to be resized to fit the size of a target 2Nx2N PU, which currently ranges from 8x8 to 128x128 for the luma component (and ranges from 4x4 to 64x64 for the chroma components). This is accomplished by using a table to record, for every possible 2Nx2N PU size and for every window function, the integer pixel positions where a transition in the value of the window function takes place.

In the current design, a window function wm,n can take values from either the set (0.125, 0.5, 0.875) or the set (0.125, 0.5, 0.75). The distribution of its weighting coefficients forms a partitioning of a PU into three non-overlapping regions, each corresponds to the application of some specific coefficient. As an illustration, Figure 5 presents the various partitionings that can arise from the use of the window functions shown in Figure 2 and Figure 3. As can be seen, the coefficients 0.5 occupy a region that may distribute vertically, horizontally, or diagonally (at an angle of 45 or 135 degree). To resize a window function according to the target PU size, the start position a and the width b of this region are recorded (see Figure 5 for their definitions). The configuration of a for every possible 2Nx2N PU size is specified in Table 1, and the configuration of b is deduced as follows:

  - If the window function is either w3,1 or w3,2, b=2N;

  - Otherwise (neither w3,1 nor w3,2),

    - If the target PU, luma or chroma, is larger than 4x4, b=2;

    - Otherwise (the target PU, luma or chroma, is of 4x4),

      - For wm,1, b=1;

      - For wm,2, b=2.

Once these two parameters are known, the coefficients associated with the other two regions are derived using the following mapping:

- In region A1, wm,n(i, j)=0.75;

- In region A2, wm,n(i, j)=0.875;

- In region B, wm,n(i, j)=0.5;

- In region C, wm,n(i, j)=0.125,

where n=1 or 2.

| w1,n | w2,n | w3,n |
| --- | --- | --- |
| w4,n | w5,n | w6,n |

- Partitioning of a 2Nx2N PU due to the application of the proposed window functions.

- The start position a for various 2Nx2N PU sizes

| PU Size | w1,1 | w2,1 | w3,1 | w4,1 | w5,1 | w6,1 | PU Size | w1,2 | w2,2 | w3,2 | w4,2 | w5,2 | w6,2 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 128x128 | 31 | 31 | 0 | 31 | 31 | 95 | 128x128 | 63 | 63 | 0 | 0 | 0 | 126 |
| 64x64 | 15 | 15 | 0 | 15 | 15 | 47 | 64x64 | 31 | 31 | 0 | 0 | 0 | 62 |
| 32x32 | 7 | 7 | 0 | 7 | 7 | 23 | 32x32 | 15 | 15 | 0 | 0 | 0 | 30 |
| 16x16 | 3 | 3 | 0 | 3 | 3 | 11 | 16x16 | 7 | 7 | 0 | 0 | 0 | 14 |
| 8x8 | 1 | 1 | 0 | 1 | 1 | 5 | 8x8 | 3 | 3 | 0 | 0 | 0 | 6 |
| 4x4 | 1 | 1 | 0 | 1 | 1 | 2 | 4x4 | 1 | 1 | 0 | 0 | 0 | 2 |

# Block Matching Criterion

When the proposed prediction mode is in use, the search criterion of (mvxb, mvyb) is changed as follows in order to consider the contribution from the merged motion parameters, i.e., and, in the case of bi-prediction, :

,

where p denotes the current original picture. During the RDO mode decision, the encoder will compare the R-D costs of coding a 2Nx2N PU with various merge candidates, each may correspond to a distinct choice of (mvxb, mvyb)*.

# Experiments

Extensive experiments were carried out using the HM-2.0 software [2] and the common test conditions [4] to collect the BD-rate savings and the encoding and decoding times of the proposed techniques (referred hereafter to as TB-mode) in Tests #1 and #2. The specifications of the machines used to run the simulations are shown below:

OS: 64-bit Microsoft Windows 7 Enterprise + Microsoft Visual Studio 2008 (Compiler)

Platform: Intel(R) Core(TM) i7-860 (2.80GHz) processor and 16GB RAMs

For the decoding runtime measurements, a single machine was used to run the decoding for all test cases (and for all algorithms including the HM-2.0 anchor) in a single thread. However, the encoding runtime measurements were carried out with parallel encoding—i.e., multiple bit-streams may be generated simultaneously on one single machine in multiple threads. Thus, the encoding time reported in this contribution may not be reliable.

## Syntax Modifications

Figure 6 highlights the changes in syntax necessary for supporting the TB-mode. The semantics of the newly added syntax are as follows:

Test #1:

One flag, tb_flag, is coded right after the syntax elements for motion merging (merge_flag & merge_idx):

- If tb_flag =1, a 2Nx2N target PU is coded in TB-mode. In this case, the encoder will additionally send one complete set of (uni-directional) motion parameters for this PU;

- Otherwise (tb_flag =1), the usual merge mode is applied.

Test #2:

Similar to Test #1, one flag, tb_flag, is coded right after the syntax elements for motion merging (merge_flag & merge_idx) to indicate the use of TB_mode. When tb_flag is 1, one additional syntax element, tb_table_sel, is transmitted to choose between the two sets of window functions, wm,1 and wm,2:

- If tb_table_sel=0, wm,1 is selected;

- Otherwise (tb_table_sel=1), wm,2 is selected.

tb_table_sel is inferred as 0 if the merged candidate comes from the co-located PU.

| HM-2.0 | Test #1 | Test #2 |
| --- | --- | --- |

- Syntax modifications.

## Performance Evaluation

Table 2 and Table 3 detail the BD-rate savings and encoding and decoding time ratios relative to HM-2.0 anchors for Test #1 and #2, respectively.

As can be seen, Test #1 achieves an average BD-rate saving of 1.6% over all test cases (min: 0.4%; max: 3.2%). Due to the extra motion search needed for each merge candidate, the encoding time is increased by 53% while the decoding time is increased by 4%.

Comparing Table 2 and Table 3, Test #2 gives better compression performance than Test #1. Averagely, it performs 0.3% better in terms of BD-rate saving (avg.: 1.9%; min: 0.6%; max: 3.7%) with a decoding time only 6% slightly longer than the anchors’, whereas the encoding time is nearly doubled.

- BD-rate savings and processing time ratios for Test #1.

|  | Random Access |  |  | Random Access LoCo |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | -1.0 | -1.5 | -1.5 | -1.4 | -1.4 | -1.2 |
| Class B | -0.9 | -1.3 | -1.2 | -1.1 | -1.4 | -1.4 |
| Class C | -1.4 | -1.9 | -1.9 | -1.5 | -2.1 | -2.2 |
| Class D | -1.5 | -2.1 | -1.9 | -1.6 | -2.3 | -2.2 |
| Class E |  |  |  |  |  |  |
| All | -1.2 | -1.7 | -1.6 | -1.4 | -1.8 | -1.7 |
| Enc Time[%] | 162% |  |  | 154% |  |  |
| Dec Time[%] | 103% |  |  | 105% |  |  |
|  | Low Delay |  |  | Low Delay LoCo |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A |  |  |  |  |  |  |
| Class B | -1.2 | -2.1 | -2.3 | -2.0 | -2.5 | -2.0 |
| Class C | -1.7 | -2.2 | -2.4 | -2.3 | -2.8 | -2.9 |
| Class D | -1.9 | -1.8 | -2.5 | -2.3 | -2.8 | -2.4 |
| Class E | -1.9 | -3.1 | -2.5 | -2.6 | -3.2 | -2.8 |
| All | -1.6 | -2.2 | -2.4 | -2.3 | -2.8 | -2.5 |
| Enc Time[%] | 147% |  |  | 151% |  |  |
| Dec Time[%] | 103% |  |  | 106% |  |  |

- BD-rate savings and processing time ratios for Test #2.

|  | Random Access |  |  | Random Access LoCo |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | -1.2 | -1.8 | -1.9 | -1.5 | -1.6 | -1.3 |
| Class B | -1.1 | -1.5 | -1.4 | -1.3 | -1.6 | -1.6 |
| Class C | -1.7 | -2.2 | -2.3 | -1.8 | -2.5 | -2.6 |
| Class D | -1.8 | -2.1 | -2.2 | -1.9 | -2.6 | -2.5 |
| Class E |  |  |  |  |  |  |
| All | -1.4 | -1.9 | -1.9 | -1.6 | -2.0 | -2.0 |
| Enc Time[%] | 202% |  |  | 200% |  |  |
| Dec Time[%] | 105% |  |  | 107% |  |  |
|  | Low Delay |  |  | Low Delay LoCo |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A |  |  |  |  |  |  |
| Class B | -1.5 | -2.4 | -2.5 | -2.4 | -2.9 | -2.4 |
| Class C | -2.1 | -2.5 | -2.5 | -2.6 | -3.0 | -3.2 |
| Class D | -2.3 | -2.5 | -2.6 | -2.7 | -2.9 | -2.5 |
| Class E | -2.1 | -3.6 | -2.8 | -3.0 | -4.0 | -3.4 |
| All | -1.9 | -2.7 | -2.6 | -2.6 | -3.1 | -2.8 |
| Enc Time[%] | 183% |  |  | 194% |  |  |
| Dec Time[%] | 104% |  |  | 109% |  |  |

# Conclusion

In view of the promising results, we would recommend the group consider adopting the proposed technique into the next release of the HM software and the working draft.

# Appendix

## Test #1 HE-RA

|  | Sequences | BD-rate Y (%) | BD-rate U (%) | BD-rate V (%) | Enc-time | Dec-time (w/ yuv out) | Dec-time (w/o yuv out) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Class A | Traffic | -0.7 | -1.0 | -1.0 |  |  |  |
|  | PeopleOnStreet | -1.6 | -2.5 | -2.5 |  |  |  |
|  | Nebuta | -1.4 | -2.0 | -2.3 |  |  |  |
|  | SteamLocomotive | -0.4 | -0.5 | -0.5 |  |  |  |
| Class B | Kimono1 | -0.6 | -1.1 | -1.0 |  |  |  |
|  | ParkScene | -0.6 | -1.0 | -0.9 |  |  |  |
|  | Cactus | -1.2 | -1.8 | -1.7 |  |  |  |
|  | BasketballDrive | -1.0 | -1.5 | -1.6 |  |  |  |
|  | BQTerrace | -1.0 | -1.2 | -0.9 |  |  |  |
| Class C | BasketballDrill | -1.3 | -2.4 | -2.3 |  |  |  |
|  | BQMall | -1.2 | -1.4 | -1.4 |  |  |  |
|  | PartyScene | -1.5 | -1.8 | -1.8 |  |  |  |
|  | RaceHorses | -1.3 | -2.0 | -2.2 |  |  |  |
| Class D | BasketballPass | -1.1 | -2.0 | -2.0 |  |  |  |
|  | BQSquare | -1.9 | -1.8 | -1.5 |  |  |  |
|  | BlowingBubbles | -1.6 | -2.0 | -1.7 |  |  |  |
|  | RaceHorses | -1.3 | -2.6 | -2.5 |  |  |  |
| Average of Class A |  | -1.0 | -1.5 | -1.5 |  |  |  |
| Average of Class B |  | -0.9 | -1.3 | -1.2 |  |  |  |
| Average of Class C |  | -1.4 | -1.9 | -1.9 |  |  |  |
| Average of Class D |  | -1.5 | -2.1 | -1.9 |  |  |  |
| Overall Average |  | -1.2 | -1.7 | -1.6 | 162% | 102% | 103% |

## Test #1 HE-LD

|  | Sequences | BD-rate Y (%) | BD-rate U (%) | BD-rate V (%) | Enc-time | Dec-time (w/ yuv out) | Dec-time (w/o yuv out) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Class B | Kimono1 | -0.8 | -1.3 | -1.6 |  |  |  |
|  | ParkScene | -1.0 | -1.7 | -2.1 |  |  |  |
|  | Cactus | -2.1 | -3.2 | -3.3 |  |  |  |
|  | BasketballDrive | -1.8 | -2.8 | -2.5 |  |  |  |
|  | BQTerrace | -0.5 | -1.4 | -2.1 |  |  |  |
| Class C | BasketballDrill | -1.8 | -3.2 | -2.8 |  |  |  |
|  | BQMall | -1.7 | -1.9 | -2.2 |  |  |  |
|  | PartyScene | -1.8 | -1.9 | -2.4 |  |  |  |
|  | RaceHorses | -1.5 | -1.6 | -2.0 |  |  |  |
| Class D | BasketballPass | -1.9 | -2.9 | -2.2 |  |  |  |
|  | BQSquare | -1.7 | 0.7 | -3.3 |  |  |  |
|  | BlowingBubbles | -2.1 | -2.8 | -2.0 |  |  |  |
|  | RaceHorses | -2.0 | -2.1 | -2.5 |  |  |  |
| Class E | Vidyo1 | -2.5 | -3.4 | -2.9 |  |  |  |
|  | Vidyo3 | -1.9 | -3.6 | -2.4 |  |  |  |
|  | Vidyo4 | -1.3 | -2.2 | -2.2 |  |  |  |
| Average of Class B |  | -1.2 | -2.1 | -2.3 |  |  |  |
| Average of Class C |  | -1.7 | -2.2 | -2.4 |  |  |  |
| Average of Class D |  | -1.9 | -1.8 | -2.5 |  |  |  |
| Average of Class E |  | -1.9 | -3.1 | -2.5 |  |  |  |
| Overall Average |  | -1.6 | -2.2 | -2.4 | 147% | 103% | 103% |

## Test #1 LC-RA

|  | Sequences | BD-rate Y (%) | BD-rate U (%) | BD-rate V (%) | Enc-time | Dec-time (w/ yuv out) | Dec-time (w/o yuv out) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Class A | Traffic | -1.0 | -1.0 | -1.0 |  |  |  |
|  | PeopleOnStreet | -1.7 | -2.9 | -2.6 |  |  |  |
|  | Nebuta | -1.4 | -0.3 | -0.1 |  |  |  |
|  | SteamLocomotive | -1.3 | -1.3 | -1.1 |  |  |  |
| Class B | Kimono1 | -0.7 | -1.3 | -1.1 |  |  |  |
|  | ParkScene | -0.8 | -1.2 | -1.1 |  |  |  |
|  | Cactus | -1.3 | -2.1 | -2.1 |  |  |  |
|  | BasketballDrive | -1.0 | -1.7 | -1.6 |  |  |  |
|  | BQTerrace | -1.4 | -0.7 | -1.0 |  |  |  |
| Class C | BasketballDrill | -1.4 | -2.3 | -2.3 |  |  |  |
|  | BQMall | -1.5 | -2.0 | -2.0 |  |  |  |
|  | PartyScene | -1.6 | -1.7 | -1.9 |  |  |  |
|  | RaceHorses | -1.5 | -2.3 | -2.5 |  |  |  |
| Class D | BasketballPass | -1.1 | -2.4 | -2.2 |  |  |  |
|  | BQSquare | -1.7 | -1.7 | -1.8 |  |  |  |
|  | BlowingBubbles | -1.8 | -2.4 | -2.0 |  |  |  |
|  | RaceHorses | -1.7 | -2.8 | -2.7 |  |  |  |
| Average of Class A |  | -1.4 | -1.4 | -1.2 |  |  |  |
| Average of Class B |  | -1.1 | -1.4 | -1.4 |  |  |  |
| Average of Class C |  | -1.5 | -2.1 | -2.2 |  |  |  |
| Average of Class D |  | -1.6 | -2.3 | -2.2 |  |  |  |
| Overall Average |  | -1.4 | -1.8 | -1.7 | 154% | 105% | 105% |

## Test #1 LC-LD

|  | Sequences | BD-rate Y (%) | BD-rate U (%) | BD-rate V (%) | Enc-time | Dec-time (w/ yuv out) | Dec-time (w/o yuv out) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Class B | Kimono1 | -2.4 | -3.2 | -2.5 |  |  |  |
|  | ParkScene | -1.7 | -1.8 | -1.9 |  |  |  |
|  | Cactus | -2.7 | -3.3 | -3.2 |  |  |  |
|  | BasketballDrive | -2.5 | -3.4 | -3.1 |  |  |  |
|  | BQTerrace | -1.0 | -0.8 | 0.5 |  |  |  |
| Class C | BasketballDrill | -2.1 | -2.9 | -2.7 |  |  |  |
|  | BQMall | -2.5 | -2.8 | -3.0 |  |  |  |
|  | PartyScene | -2.1 | -2.5 | -2.4 |  |  |  |
|  | RaceHorses | -2.4 | -3.0 | -3.6 |  |  |  |
| Class D | BasketballPass | -2.6 | -2.8 | -2.9 |  |  |  |
|  | BQSquare | -1.8 | -2.4 | -0.4 |  |  |  |
|  | BlowingBubbles | -2.1 | -2.5 | -3.2 |  |  |  |
|  | RaceHorses | -2.5 | -3.3 | -3.0 |  |  |  |
| Class E | Vidyo1 | -2.4 | -3.4 | -2.6 |  |  |  |
|  | Vidyo3 | -3.2 | -3.6 | -2.6 |  |  |  |
|  | Vidyo4 | -2.2 | -2.5 | -3.3 |  |  |  |
| Average of Class B |  | -2.0 | -2.5 | -2.0 |  |  |  |
| Average of Class C |  | -2.3 | -2.8 | -2.9 |  |  |  |
| Average of Class D |  | -2.3 | -2.8 | -2.4 |  |  |  |
| Average of Class E |  | -2.6 | -3.2 | -2.8 |  |  |  |
| Overall Average |  | -2.3 | -2.8 | -2.5 | 151% | 105% | 106% |

## Test #2 HE-RA

|  | Sequences | BD-rate Y (%) | BD-rate U (%) | BD-rate V (%) | Enc-time | Dec-time (w/ yuv out) | Dec-time (w/o yuv out) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Class A | Traffic | -0.9 | -1.1 | -1.3 |  |  |  |
|  | PeopleOnStreet | -1.8 | -2.8 | -2.7 |  |  |  |
|  | Nebuta | -1.5 | -2.1 | -2.3 |  |  |  |
|  | SteamLocomotive | -0.6 | -1.3 | -1.3 |  |  |  |
| Class B | Kimono1 | -0.8 | -1.3 | -1.1 |  |  |  |
|  | ParkScene | -0.8 | -1.2 | -1.2 |  |  |  |
|  | Cactus | -1.5 | -2.1 | -2.0 |  |  |  |
|  | BasketballDrive | -1.2 | -1.7 | -1.8 |  |  |  |
|  | BQTerrace | -1.1 | -1.1 | -0.9 |  |  |  |
| Class C | BasketballDrill | -1.6 | -2.7 | -2.8 |  |  |  |
|  | BQMall | -1.5 | -1.6 | -1.6 |  |  |  |
|  | PartyScene | -1.9 | -2.1 | -2.1 |  |  |  |
|  | RaceHorses | -1.6 | -2.4 | -2.7 |  |  |  |
| Class D | BasketballPass | -1.3 | -1.9 | -2.2 |  |  |  |
|  | BQSquare | -2.3 | -1.5 | -1.8 |  |  |  |
|  | BlowingBubbles | -2.1 | -2.5 | -2.0 |  |  |  |
|  | RaceHorses | -1.7 | -2.6 | -2.8 |  |  |  |
| Average of Class A |  | -1.2 | -1.8 | -1.9 |  |  |  |
| Average of Class B |  | -1.1 | -1.5 | -1.4 |  |  |  |
| Average of Class C |  | -1.7 | -2.2 | -2.3 |  |  |  |
| Average of Class D |  | -1.8 | -2.1 | -2.2 |  |  |  |
| Overall Average |  | -1.4 | -1.9 | -1.9 | 202% | 102% | 105% |

## Test #2 HE-LD

|  | Sequences | BD-rate Y (%) | BD-rate U (%) | BD-rate V (%) | Enc-time | Dec-time (w/ yuv out) | Dec-time (w/o yuv out) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Class B | Kimono1 | -1.0 | -1.6 | -1.4 |  |  |  |
|  | ParkScene | -1.2 | -1.8 | -2.1 |  |  |  |
|  | Cactus | -2.4 | -3.8 | -3.8 |  |  |  |
|  | BasketballDrive | -2.0 | -2.7 | -2.8 |  |  |  |
|  | BQTerrace | -0.7 | -2.2 | -2.4 |  |  |  |
| Class C | BasketballDrill | -2.3 | -3.3 | -3.1 |  |  |  |
|  | BQMall | -2.1 | -2.2 | -2.5 |  |  |  |
|  | PartyScene | -2.2 | -2.6 | -2.4 |  |  |  |
|  | RaceHorses | -1.7 | -2.0 | -2.0 |  |  |  |
| Class D | BasketballPass | -2.2 | -3.1 | -2.6 |  |  |  |
|  | BQSquare | -2.3 | -1.7 | -2.4 |  |  |  |
|  | BlowingBubbles | -2.4 | -3.1 | -2.7 |  |  |  |
|  | RaceHorses | -2.2 | -2.1 | -2.7 |  |  |  |
| Class E | Vidyo1 | -2.4 | -2.7 | -3.6 |  |  |  |
|  | Vidyo3 | -2.2 | -4.3 | -1.8 |  |  |  |
|  | Vidyo4 | -1.7 | -3.7 | -2.9 |  |  |  |
| Average of Class B |  | -1.5 | -2.4 | -2.5 |  |  |  |
| Average of Class C |  | -2.1 | -2.5 | -2.5 |  |  |  |
| Average of Class D |  | -2.3 | -2.5 | -2.6 |  |  |  |
| Average of Class E |  | -2.1 | -3.6 | -2.8 |  |  |  |
| Overall Average |  | -1.9 | -2.7 | -2.6 | 183% | 101% | 104% |

## Test #2 LC-RA

|  | Sequences | BD-rate Y (%) | BD-rate U (%) | BD-rate V (%) | Enc-time | Dec-time (w/ yuv out) | Dec-time (w/o yuv out) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Class A | Traffic | -1.2 | -1.2 | -1.2 |  |  |  |
|  | PeopleOnStreet | -1.9 | -3.2 | -3.0 |  |  |  |
|  | Nebuta | -1.6 | -0.3 | -0.1 |  |  |  |
|  | SteamLocomotive | -1.2 | -1.5 | -1.1 |  |  |  |
| Class B | Kimono1 | -0.9 | -1.6 | -1.4 |  |  |  |
|  | ParkScene | -1.0 | -1.2 | -1.2 |  |  |  |
|  | Cactus | -1.6 | -2.4 | -2.4 |  |  |  |
|  | BasketballDrive | -1.3 | -2.2 | -2.0 |  |  |  |
|  | BQTerrace | -1.5 | -0.6 | -1.0 |  |  |  |
| Class C | BasketballDrill | -1.7 | -2.8 | -2.8 |  |  |  |
|  | BQMall | -1.7 | -2.1 | -2.2 |  |  |  |
|  | PartyScene | -1.9 | -2.0 | -2.2 |  |  |  |
|  | RaceHorses | -1.8 | -2.8 | -3.1 |  |  |  |
| Class D | BasketballPass | -1.4 | -2.8 | -2.3 |  |  |  |
|  | BQSquare | -2.0 | -2.2 | -2.3 |  |  |  |
|  | BlowingBubbles | -2.1 | -2.5 | -2.2 |  |  |  |
|  | RaceHorses | -2.0 | -2.9 | -3.0 |  |  |  |
| Average of Class A |  | -1.5 | -1.6 | -1.3 |  |  |  |
| Average of Class B |  | -1.3 | -1.6 | -1.6 |  |  |  |
| Average of Class C |  | -1.8 | -2.5 | -2.6 |  |  |  |
| Average of Class D |  | -1.9 | -2.6 | -2.5 |  |  |  |
| Overall Average |  | -1.6 | -2.0 | -2.0 | 200% | 104% | 107% |

## Test #2 LC-LD

|  | Sequences | BD-rate Y (%) | BD-rate U (%) | BD-rate V (%) | Enc-time | Dec-time (w/ yuv out) | Dec-time (w/o yuv out) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Class B | Kimono1 | -2.7 | -3.6 | -2.6 |  |  |  |
|  | ParkScene | -1.8 | -2.0 | -2.1 |  |  |  |
|  | Cactus | -3.1 | -4.0 | -3.6 |  |  |  |
|  | BasketballDrive | -3.0 | -3.9 | -3.6 |  |  |  |
|  | BQTerrace | -1.2 | -0.8 | -0.3 |  |  |  |
| Class C | BasketballDrill | -2.4 | -3.3 | -3.1 |  |  |  |
|  | BQMall | -2.8 | -3.0 | -3.4 |  |  |  |
|  | PartyScene | -2.6 | -2.4 | -2.5 |  |  |  |
|  | RaceHorses | -2.7 | -3.1 | -3.7 |  |  |  |
| Class D | BasketballPass | -3.0 | -3.6 | -3.2 |  |  |  |
|  | BQSquare | -2.5 | -2.0 | -0.5 |  |  |  |
|  | BlowingBubbles | -2.5 | -3.0 | -3.1 |  |  |  |
|  | RaceHorses | -2.8 | -3.1 | -3.2 |  |  |  |
| Class E | Vidyo1 | -2.7 | -4.4 | -3.3 |  |  |  |
|  | Vidyo3 | -3.7 | -4.2 | -3.6 |  |  |  |
|  | Vidyo4 | -2.5 | -3.5 | -3.4 |  |  |  |
| Average of Class B |  | -2.4 | -2.9 | -2.4 |  |  |  |
| Average of Class C |  | -2.6 | -3.0 | -3.2 |  |  |  |
| Average of Class D |  | -2.7 | -2.9 | -2.5 |  |  |  |
| Average of Class E |  | -3.0 | -4.0 | -3.4 |  |  |  |
| Overall Average |  | -2.6 | -3.1 | -2.8 | 194% | 105% | 109% |

# References

- C.-L. Lee, C.-C. Chen, Y.-W. C., M.-H. Wu, C.-H. Wu, W.-H. Peng and H.-M. Hang, “Bi-prediction combining template and block motion compensations,” JCTVC-D175, Daegu, KR, Jan. 2011.

- JCT-VC, “HM-2.0,” https://hevc.hhi.fraunhofer.de/svn/svn_HEVCSoftware/tags/HM-2.0/

- Y.-J. Chiu, H. Yu, Y.-W. Huang, S.-I. Sekiguchi and W.-H. Peng, “Description of Core Experiment 1: Decoder-Side Motion Vector Derivation,” JCTVC-D601, Daegu, Jan. 2011.

- F. Bossen, “Common test conditions and software reference configurations,” JCTVC-D600, Daegu, Jan. 2011.

# Patent rights declaration(s)

(NOTE – Activities in the JCT-VC and contributions to the JCT-VC are subject to the common patent policy for ITU-T/ITU-R/ISO/IEC. A statement of that policy can be found at
http://www.itu.int/ITU-T/dbase/patent/patent-policy.html, with further information available at http://www.itu.int/ITU-T/ipr/index.html and in the ISO/IEC Directives. The form to be used for the formal reporting of patent rights to ITU-T/ITU-R/ISO/IEC can be found at http://www.itu.int/ITU-T/ipr/index.html. Contributions to the JCT-VC proposing normative technical content shall contain a non-binding informal notice of whether the submitter may have patent rights that would be necessary for implementation of the resulting standard. The provided informal notice shall indicate the category of anticipated licensing terms according to the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form. This obligation to provide an informal notice is supplemental to, and does not replace, any existing obligations of parties with technology included in a final or draft standard to submit formal IPR declarations to ITU-T/ITU-R/ISO/IEC. An example of an informal IPR notification statement for a contribution is provided below.)

NCTU/ITRI may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

All the coefficient values are given in the attached archive, JCTVC-E154-Coeff.zip.