| Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11 5th Meeting: Geneva, CH, 16-23 March, 2011 | Document: JCTVC-E155 WG11 Number: m19672 |
| --- | --- |

| Title: | CE9: Verification of TI’s proposal (JCTVC-E116) |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Akira Fujibayashi TK Tan 3-6 Hikarinooka, Yokosuka-shi Kanagawa, 239 8536 Japan Frank Bossen 3240 Hillview Ave Palo Alto CA 94304 USA | Tel: Email: | +81 468 40 6833 fujibayashi@nttdocomo.co.jp +65 9799 3861 tktan.p@rd.nttdocomo.co.jp +1 650 496 4742 bossen@docomolabs-usa.com |
| Source: | NTT DOCOMO, Inc. and DOCOMO USA Labs |  |  |

_____________________________

# Abstract

This contribution reports on the simulation results for Core Experiment (CE) 9 as described in JCTVC-D609_r3. The proposed simplifications to the upper/left spatial predictive motion vector (PMV) candidate detection based on JCTVC-D055 were cross verified. Additionally, the combination of the proposed simplifications and the SKIP mode modification proposed by HHI defined as test 4.3 B in CE9 were also verified.

Reducing the number of partitions that are searched in the upper/left PMV candidate detection process results in no coding efficiency loss compared to the HM2.0 reference coding configuration. Similarly, adding this proposed simplification to the SKIP mode modification results in no additional coding efficiency loss compared to the reference HM2.0 coding configuration.

# Introduction

In section 4.3 of CE 9, 40 tests were proposed. Please refer to JCTVC-D609_r3 [1] for the details of the proposed tests. NTT DOCOMO participated in 4 tests. Two simplifications of upper/left PMV candidate detection based on JCTVC-D055 [2] defined as test 4.3E and 4.3F were cross verified.

As described in section 4.2 of JCTVC-D609_r3, motion vector derivation for SKIP mode and MERGE mode was verified as test 4.3B. According to this modification, combinations of the modification based on JCTVC-D055 and modified SKIP mode defined as test 4.3B was also verified as test 4.3G and 4.3H.

The following tasks were performed as an activity of the core experiment:

Cross verification of tests 4.3 E and 4.3 F

- Implemented and cross verified the Texas Instruments’ proposal method on top of the HM2.0 software (revision 573). NTT DOCOMO and Texas Instruments independently implemented the defined modifications. The simulations were also conducted independently.

- Compared the summary results in the excel sheets with that obtained by Texas Instruments.

Verification of tests 4.3 G and 4.3 H

- Implemented and verified the Texas Instruments’ proposal method on top of the modified HM2.0 software shared by HHI (HM2.0-CE9-mergeskip-HHI.zip). Modified code was shared through the mailing list of CE9 and provided to cross checker.

- Compared the summary results in the excel sheets with that obtained by cross checker (Canon).

- Cross-verification of modifications based on JCTVC-D055(Test 4.3 E/F)

The modifications defined as tests 4.3 E and 4.3F as described below were cross-verified.

## Test 4.3E: Reduction of search partition in upper/left PMV candidate detection (JCTVC-D055-b)

In test 4.3E, following modification was evaluated. As shown in Figure2, for the search of left predictor, instead of scanning all the left neighboring PUs from bottom to top, only up to three neighboring PUs, Am+1, Am,A0 are used. Similarly, For the search of top predictor, only up to four neighboring PUs,Bn+1,Bn,B0,B-1 ,are used. Other steps to select MVP candidate are unchanged from current default setting (HM2.0). In this test, this modification is applied to both Inter coding part and SKIP mode.

Figure2 Reduction of search partition in upper/left PMV candidate detection

## Test 4.3F: Further reduction of search partition in upper/left PMV candidate detection (JCTVC-D055-c)

In test 4.3F, as shown in Figure3, upper/left MVP candidate detection is further simplified. For the search of left predictor, instead of scanning all the left neighboring PUs from bottom to top, only up to two neighboring PUs, Am+1,Am, are used. Similarly, For the search of top predictor, only up to three neighboring PUs, Bn+1,Bn ,B-1　are used. Other steps to select MVP candidate are unchanged. In this test, this modification is applied to both Inter coding part and SKIP mode.

Figure 3 Further reduction of search partition in upper/left PMV candidate detection

- Verification for combination of modifications based on JCTVC-D055 and modified SKIP mode proposed by HHI (Test 4.3 G/H)

The modifications defined as tests 4.3 G and 4.3H as described below were verified as an activity of CE9.

## Test 4.3G: Combination of JCTVC-D055-b and modified SKIP mode defined as Test 4.3 B

In test 4.3 G, the same modification to test 4.3E (JCTVC-D055-b) was applied to Inter coding part of the modified HM2.0 which includes the SKIP mode modification proposed as test 4.3B.

## Test 4.3H: Combination of JCTVC-D055-c and modified SKIP mode defined as Test 4.3 B

In test 4.3 H, the same modification to test 4.3F (JCTVC-D055-c) was applied to Inter coding part of the modified HM2.0 which includes the SKIP mode modification proposed as test 4.3B.

- Results

The summary results for the 4 verifications under investigation using coding conditions defined in JCTVC-D600 [3] are shown in the Table1, Table2, Table3 and Table4 below. Table1 and Table 2 show the cross verification results of tests 4.3E and 4.3F, respectively.

Table 1: Summary results of BD-Rate comparing test 4.3E to HM2.0

| HM2.0_CE9_4.3E vs. HM2.0 default |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Random access |  |  | Random access LoCo |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | 0.0 | 0.1 | -0.1 | 0.0 | -0.1 | 0.0 |
| Class B | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.1 |
| Class C | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| Class D | 0.0 | 0.0 | -0.1 | 0.0 | 0.0 | 0.0 |
| Class E |  |  |  |  |  |  |
| All | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 100% |  |  | 99% |  |  |

| HM2.0_CE9_4.3E vs. HM2.0 default |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Low Delay |  |  | Low Delay LoCo |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A |  |  |  |  |  |  |
| Class B | 0.0 | 0.0 | -0.2 | 0.0 | 0.0 | 0.2 |
| Class C | 0.0 | 0.1 | 0.0 | 0.0 | 0.0 | 0.0 |
| Class D | -0.1 | -0.4 | 0.0 | 0.1 | 0.2 | 0.3 |
| Class E | -0.1 | 0.1 | 0.0 | -0.1 | -0.5 | 0.2 |
| All | 0.0 | -0.1 | 0.0 | 0.0 | 0.0 | 0.2 |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 100% |  |  | 100% |  |  |

Table 2: Summary results of BD-Rate comparing test 4.3F to HM2.0

| HM2.0 CE9_4.3F vs. HM2.0 default |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Random access |  |  | Random access LoCo |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | 0.1 | 0.0 | 0.0 | 0.0 | 0.0 | 0.1 |
| Class B | 0.0 | 0.0 | 0.1 | 0.0 | 0.1 | 0.1 |
| Class C | 0.0 | 0.0 | 0.0 | 0.0 | 0.1 | 0.1 |
| Class D | 0.0 | 0.0 | 0.1 | 0.0 | 0.1 | -0.1 |
| Class E |  |  |  |  |  |  |
| All | 0.0 | 0.0 | 0.0 | 0.0 | 0.1 | 0.1 |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 100% |  |  | 99% |  |  |

| HM2.0 CE9_4.3F vs. HM2.0 default |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Low Delay |  |  | Low Delay LoCo |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A |  |  |  |  |  |  |
| Class B | 0.0 | 0.1 | 0.0 | 0.0 | -0.2 | 0.1 |
| Class C | 0.0 | 0.1 | 0.1 | 0.0 | -0.1 | -0.3 |
| Class D | -0.1 | -0.2 | -0.2 | 0.1 | -0.2 | 0.2 |
| Class E | 0.1 | 0.0 | 1.0 | 0.0 | -0.4 | 0.0 |
| All | 0.0 | 0.0 | 0.2 | 0.0 | -0.2 | 0.0 |
| Enc Time[%] | 100% |  |  | 100% |  |  |
| Dec Time[%] | 99% |  |  | 99% |  |  |

The results in Error: Reference source not found1 and Table2 show almost no loss to current default setting though the number of partitions searched for the upper/left PMV candidates is reduced. The bitrate and PSNR of the coding results shown in Table1 and Table2 are identical as the data listed in the excel sheet provided by Texas Instruments.

Table3 and Table4 show the results of tests 4.3G and 4.3H, respectively. The excel sheet corresponding to these result is attached as part of the contribution.

Table 3: Summary results of BD-Rate comparing test 4.3G to HM2.0

| HM2.0 CE9_4.3G vs. HM2.0 default |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Random access |  |  | Random access LoCo |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | 0.2 | -0.1 | -0.2 | -0.1 | -0.3 | -0.3 |
| Class B | -0.1 | -0.3 | -0.4 | -0.7 | -0.6 | -0.5 |
| Class C | -0.1 | 0.0 | 0.0 | -0.3 | -0.2 | -0.3 |
| Class D | -0.1 | 0.0 | 0.0 | -0.3 | -0.3 | 0.0 |
| Class E |  |  |  |  |  |  |
| All | 0.0 | -0.1 | -0.1 | -0.4 | -0.4 | -0.3 |
| Enc Time[%] | 99% |  |  | 97% |  |  |
| Dec Time[%] | 99% |  |  | 99% |  |  |

| HM2.0 CE9_4.3G vs. HM2.0 default |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Low Delay |  |  | Low Delay LoCo |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A |  |  |  |  |  |  |
| Class B | -0.3 | 0.0 | -0.3 | 1.4 | 0.6 | 0.8 |
| Class C | -0.1 | 0.3 | 0.2 | 0.7 | 0.7 | 0.6 |
| Class D | -0.3 | 0.3 | 0.1 | 0.4 | 0.3 | -0.2 |
| Class E | -1.7 | -1.4 | -0.2 | 0.5 | -0.6 | 0.1 |
| All | -0.5 | -0.1 | 0.0 | 0.8 | 0.3 | 0.3 |
| Enc Time[%] | 99% |  |  | 98% |  |  |
| Dec Time[%] | 101% |  |  | 98% |  |  |

Table 4: Summary results of BD-Rate comparing test 4.3H to HM2.0

| HM2.0 CE9_4.3H vs. HM2.0 default |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Random access |  |  | Random access LoCo |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A | 0.1 | 0.0 | 0.3 | -0.2 | -0.3 | -0.1 |
| Class B | -0.1 | -0.2 | -0.3 | -0.6 | -0.6 | -0.6 |
| Class C | -0.1 | 0.0 | 0.0 | -0.3 | -0.2 | -0.2 |
| Class D | -0.1 | -0.1 | 0.2 | -0.3 | -0.3 | -0.1 |
| Class E |  |  |  |  |  |  |
| All | 0.0 | -0.1 | 0.0 | -0.4 | -0.4 | -0.3 |
| Enc Time[%] | 98% |  |  | 97% |  |  |
| Dec Time[%] | 99% |  |  | 99% |  |  |

| HM2.0 CE9_4.3H vs. HM2.0 default |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Low Delay |  |  | Low Delay LoCo |  |  |
|  | Y BD-rate | U BD-rate | V BD-rate | Y BD-rate | U BD-rate | V BD-rate |
| Class A |  |  |  |  |  |  |
| Class B | -0.3 | 0.1 | -0.1 | 1.4 | 0.8 | 0.8 |
| Class C | -0.2 | 0.5 | 0.3 | 0.7 | 0.6 | 0.6 |
| Class D | -0.4 | 0.5 | 0.1 | 0.3 | 0.1 | 0.2 |
| Class E | -1.5 | -1.1 | -0.4 | 0.6 | -0.6 | -0.1 |
| All | -0.5 | 0.1 | 0.0 | 0.8 | 0.3 | 0.4 |
| Enc Time[%] | 98% |  |  | 98% |  |  |
| Dec Time[%] | 100% |  |  | 99% |  |  |

The results in Error: Reference source not found3 and Table4 show small gain (0.4%~0.5% in average BD-rate) for Random Access LoCo and Low Delay cases but bring coding loss of 0.8% in average BD-rate for the Low Delay LoCo case when compared to the reference configuration. These results are confirmed to show no loss when compared to the result of test 4.3B.

This implies that the simplification of the upper/left PMV candidate detection brings no coding loss for both HM2.0 and HM2.0 with SKIP mode modification.

- Conclusions

This contribution confirms the results of tests 4.3E, 4.3F, 4.3G and 4.3H defined in the section 4.3 in CE9. The simplification for the upper/left PMV candidate detection process proposed by Texas Instruments was successfully cross verified. It is confirmed that their modification does not introduce coding loss compared to the reference coding performance.

Additionally, adding the simplification to the upper/left PMV candidate detection process to the SKIP mode modification proposed by HHI does not introduce additional coding loss compared to the reference coding performance.

# Patent rights declaration(s)

NTT DOCOMO, Inc. may have IPR relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

[1] .Jung,B.Bross, “Description of Core Experiment 9: MV Coding and Skip/Merge operations”, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11,JCTVC-D609r3,4th Meeting: Deagu, Korea, 20-28 January, 2011.

[2] M.Zhou , “Scalable motion vector competition and simplified MVP calculation”, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11,JCTVC-D055,4th Meeting: Deagu, Korea, 20-28 January, 2011.

[3] F. Bossen., “Common conditions and software reference configurations”, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JCTVC-D600, 4th Meeting: Daegu, Korea, 20-28 January, 2011.